| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0105 |
| --- | --- |

| Title: | AHG9: Modification of general constraint information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sean McCarthy, Peng Yin, Taoran Lu, Fangjun Pu, Walt Husak, Tao Chen 432 Lakeside Drive, Sunnyvale, CA, 940850 | Tel: Email: | {Sean.McCarthy,pyin}@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes modifications of syntax and semantics for signalling general constraint information (gci). The main aspect of the proposal is to signal categorized limited-scope general constraint information syntax structures within the existing general constraint information syntax structure, i.e., signal the following:

general_format_constraint_info( ) specifying only those gci flags related to formats

general_functionality_constraint_info( ) specifying only those gci flags related to functionality

general_NALU_constraint_info( ) specifying only those gci flags related to NAL unit type

general_partiton_constraint_info( ) specifying only those gci flags related to partitions

general_pred_mode_constraint_info( ) specifying only those gci flags related to prediction modes

general_intra_constraint_info( ) specifying only those gci flags related to intra prediction

general_inter_constraint_info( ) specifying only those gci flags related to inter prediction

general_tqr_constraint_info( ) specifying only those gci flags related to transform, quantization, and residual coding

general_loop_filter_constraint_info( ) specifying only those gci flags related to loop filters

general_layer_constraint_info( ) specifying only those gci flags related to layers

general_SEI_constraint_info( ) specifying only those gci flags related to SEI messages

This contribution also proposes additional general constraint information flags and minor modifications of the names or semantics of several existing general constraint information flags.

# Introduction

The VVC draft specification includes 62 general constraint flags that specify limitations on the behaviour of coding tools and values of syntax elements. The existing general constrain flags relates to NAL unit types, prediction modes, inter and intra prediction, transforms, quantization, loop filters, layers, SEI messages, and formats. All 62 general constraint flags are currently signalled in the general_constraint_info( ) syntax structure.

As currently specified, the list of general constraint flags may be confusing as there is no logical order to the list. The lack of order may discourage use of general constraint flags and thus limit their benefit.

The current proposal is motivated by a desire to make general_constraint_info( ) easier to parse and use, and less prone to user error. It is proposed to signal categorized limited-scope general constraint info syntax structures within the existing general constraint information syntax structure. For example, a general_intra_constraint_info( ) syntax structure -- specifying only those general constraint flags that are related to intra prediction -- is proposed to be signalled in general_constraint_info( ). Similar limited-scope gci syntax structures that group categorized gci flags are proposed for formats; functionality (scalability, 360-video, screen content coding); NAL unit types; partitions; prediction modes; inter prediction; transform, quantization, and residual coding; loop filters; layers; and SEI messages.

In an alternative proposal, it is proposed to also signal whether each limited-scope gci syntax structures is present or not.

It is proposed also to add general constraint information flags  for completeness: no_virtual_boundary_constraint_flag; no_weighted_prediction_constraint_flag; no_weighted_bipred_constraint_flag; and no_explicit_scaling_list_constraint_flag. Additionally, it is proposed to add the following general constraint information flags: no_scalability_constraint_flag (indicates that scalable and layered coding are disabled); no_360_video_constraint_flag (indicates that 360-video coding is disabled); no_SCC_constraint_flag (indicates that screen content coding is disabled); and flags that indicate that specific SEI messages specified by Rec. ITU-T H.VVC | ISO/IEC 23090-3 are not present in the bitstream.

It is proposed also that the following modifications be adopted to improve the clarity of the specification:

Change general_non_packed_constraint_flag to general_non_packed_SEI_constraint_flag

Change general_non_projected_constraint_flag to general_non_projected_SEI_constraint_flag

Change max_bitdepth_constraint_idc to max_bitdepth_minus8_constraint_idc

Correct the semantics for no_aps_constraint_flag to refer to sps_explicit_scaling_list_enabled_flag

# Proposal

Yellow highlighting is used below to indicate new syntax elements and modifications to existing semantics.

      - 7.3.3.2 General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| general_format_constraint_info( ) |  |
| general_functionality_constraint_info( ) |  |
| general_NALU_constraint_info( ) |  |
| general_partiton_constraint_info( ) |  |
| general_pred_mode_constraint_info( ) |  |
| general_intra_constraint_info( ) |  |
| general_inter_constraint_info( ) |  |
| general_tqr_constraint_info( ) |  |
| general_loop_filter_constraint_info( ) |  |
| general_layer_constraint_info( ) |  |
| general_SEI_constraint_info( ) |  |
| while( !byte_aligned( ) ) |  |
| gci_alignment_zero_bit | f(1) |
| gci_num_reserved_bytes | u(8) |
| for( i = 0; i < gci_num_reserved_bytes; i++ ) |  |
| gci_reserved_byte[ i ] | u(8) |
| } |  |

      - 7.3.3.2.1 General format constraint information syntax

| general_format_constraint_info( ) { | Descriptor |
| --- | --- |
| general_frame_only_constraint_flag | u(1) |
| max_bitdepth_minus8_constraint_idc | u(3) |
| max_chroma_format_constraint_idc | u(2) |
| } |  |

      - 7.3.3.2.2 General functionality information syntax

| general_functionality_constraint_info( ) { | Descriptor |
| --- | --- |
| general_one_picture_only_constraint_flag | u(1) |
| no_scalability_constraint_flag | u(1) |
| no_360_video_constraint_flag | u(1) |
| no_scc_constraint_flag | u(1) |
| } |  |

      - 7.3.3.2.3 General NAL unit type syntax constraint information syntax

| general_NALU_constraint_info( ) { | Descriptor |
| --- | --- |
| no_mixed_nalu_types_in_pic_constraint_flag | u(1) |
| no_trail_constraint_flag | u(1) |
| no_stsa_constraint_flag | u(1) |
| no_radl_constraint_flag | u(1) |
| no_rasl_constraint_flag | u(1) |
| no_idr_constraint_flag | u(1) |
| no_cra_constraint_flag | u(1) |
| no_gdr_constraint_flag | u(1) |
| no_aps_constraint_flag | u(1) |
| } |  |

      - 7.3.3.2.4 General partition constraint information syntax

| general_partition_constraint_info( ) { | Descriptor |
| --- | --- |
| one_tile_per_pic_constraint_flag | u(1) |
| pic_header_in_slice_header_constraint_flag | u(1) |
| one_slice_per_pic_constraint_flag | u(1) |
| one_subpic_per_pic_constraint_flag | u(1) |
| no_qtbtt_dual_tree_intra_constraint_flag | u(1) |
| no_partition_constraints_override_constraint_flag | u(1) |
| no_virtual_boundary_constraint_flag | u(1) |
| } |  |

      - 7.3.3.2.5 General prediction mode constraint information syntax

| general_pred_mode_constraint_info( ) { | Descriptor |
| --- | --- |
| intra_only_constraint_flag | u(1) |
| no_palette_constraint_flag | u(1) |
| no_ibc_constraint_flag | u(1) |
| } |  |

      - 7.3.3.2.6 General intra prediction constraint information syntax

| general_intra_constraint_info( ) { | Descriptor |
| --- | --- |
| no_mrl_constraint_flag | u(1) |
| no_isp_constraint_flag | u(1) |
| no_mip_constraint_flag | u(1) |
| no_cclm_constraint_flag | u(1) |
| } |  |

      - 7.3.3.2.7 General inter prediction constraint information syntax

| general_inter_constraint_info( ) { | Descriptor |
| --- | --- |
| no_ref_pic_resampling_constraint_flag | u(1) |
| no_res_change_in_clvs_constraint_flag | u(1) |
| no_ref_wraparound_constraint_flag | u(1) |
| no_temporal_mvp_constraint_flag | u(1) |
| no_sbtmvp_constraint_flag | u(1) |
| no_amvr_constraint_flag | u(1) |
| no_bdof_constraint_flag | u(1) |
| no_dmvr_constraint_flag | u(1) |
| no_affine_motion_constraint_flag | u(1) |
| no_mmvd_constraint_flag | u(1) |
| no_smvd_constraint_flag | u(1) |
| no_prof_constraint_flag | u(1) |
| no_bcw_constraint_flag | u(1) |
| no_ciip_constraint_flag | u(1) |
| no_gpm_constraint_flag | u(1) |
| no_weighted_pred_constraint_flag | u(1) |
| no_weighted_bipred_constraint_flag | u(1) |
| } |  |

      - 7.3.3.2.8 General transform, quantization, and residual constraint information syntax

| general_tqr_constraint_info( ) { | Descriptor |
| --- | --- |
| no_mts_constraint_flag | u(1) |
| no_sbt_constraint_flag | u(1) |
| no_lfnst_constraint_flag | u(1) |
| no_transform_skip_constraint_flag | u(1) |
| no_act_constraint_flag | u(1) |
| no_joint_cbcr_constraint_flag | u(1) |
| no_bdpcm_constraint_flag | u(1) |
| no_tsrc_constraint_flag | u(1) |
| no_cu_qp_delta_constraint_flag | u(1) |
| no_chroma_qp_offset_constraint_flag | u(1) |
| no_dep_quant_constraint_flag | u(1) |
| no_sign_data_hiding_constraint_flag | u(1) |
| no_explicit_scaling_list_constraint_flag | u(1) |
| } |  |

      - 7.3.3.2.9 General loop filter constraint information syntax

| general_loop_filter_constraint_info( ) { | Descriptor |
| --- | --- |
| no_sao_constraint_flag | u(1) |
| no_alf_constraint_flag | u(1) |
| no_ccalf_constraint_flag | u(1) |
| no_ladf_constraint_flag | u(1) |
| no_lmcs_constraint_flag | u(1) |
| } |  |

      - 7.3.3.2.10 General layer constraint information syntax

| general_layer_constraint_info( ) { | Descriptor |
| --- | --- |
| single_layer_constraint_flag | u(1) |
| all_layers_independent_constraint_flag | u(1) |
| } |  |

      - 7.3.3.2.11 General supplemental enhancement information message  constraint information syntax

| general_sei_constraint_info( ) { | Descriptor |
| --- | --- |
| no_scalable_nesting_SEI_constraint_flag | u(1) |
| no_subpic_level_SEI_constraint_flag | u(1) |
| no_filler_payload_SEI_constraint_flag | u(1) |
| no_user_data_reg_SEI_constraint_flag | u(1) |
| no_user_data_unreg_SEI_constraint_flag | u(1) |
| no_film_grain_SEI_constraint_flag | u(1) |
| general_non_packed_SEI_constraint_flag | u(1) |
| general_non_projected_SEI_constraint_flag | u(1) |
| no_parameter_set_incl_SEI_constraint_flag | u(1) |
| no_decoded_picture_hash_SEI_constraint_flag | u(1) |
| no_mdcv_SEI_constraint_flag | u(1) |
| no_cll_SEI_constraint_flag | u(1) |
| no_DRAP_constraint_SEI_flag | u(1) |
| no_alt_transfer_char_SEI_constraint_flag | u(1) |
| no_ambient_view_envir_SEI_constraint_flag | u(1) |
| no_ccv_SEI_constraint_flag | u(1) |
| no_omni_video_specific_SEI_constraint_flag | u(1) |
| no_field_frame_info_SEI_constraint_flag | u(1) |
| no_sar_SEI_constraint_flag | u(1) |
| } |  |

      - 7.4.4.2 General constraint information semantics

gci_alignment_zero_bits shall be equal to 0.

gci_num_reserved_bytes specifies the number of the reserved constraint bytes. The value of gci_num_reserved_bytes shall be 0. Other values of gci_num_reserved_bytes are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification.

gci_reserved_byte[ i ] may have any value. Its presence and value do not affect decoder conformance to profiles specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore the values of all the gci_reserved_byte[ i ] syntax elements.

      - 7.4.4.2.1 General format constraint information semantics

general_frame_only_constraint_flag equal to 1 specifies that OlsInScope conveys pictures that represent frames. general_frame_only_constraint_flag equal to 0 specifies that OlsInScope conveys pictures that may or may not represent frames. When general_frame_only_constraint_flag is equal to 1, the value of sps_field_seq_flag shall be equal to 0. When not present, general_frame_only_constraint_flag is inferred to be equal to 0.

NOTE 1 – Decoders may ignore the value of general_frame_only_constraint_flag, as there are no decoding process requirements associated with it.

max_bitdepth_minus8_constraint_idc specifies that sps_bit_depth_minus8 shall be in the range of 0 to max_bitdepth_minus8_constraint_idc, inclusive.

max_chroma_format_constraint_idc specifies that sps_chroma_format_idc shall be in the range of 0 to max_chroma_format_constraint_idc, inclusive.

      - 7.4.4.2.2 General functionality information semantics

general_one_picture_only_constraint_flag equal to 1 specifies that there is only one coded picture in the bitstream. general_one_picture_only_constraint_flag equal to 0 does not impose such a constraint. When not present, general_one_picture_only_constraint_flag is inferred to be equal to 0.

no_scalability_constraint_flag equal to 1 specifies that scalable and layered coding are disabled for the CVS. no_scalability_constraint_flag equal to 0 does not impose such a constraint.

The value of no_scalability_constraint_flag shall be equal to the value of the variable noScalabilityConstraint. The value of noScalabilityConstraint is derived as follows:

if( ( sps_video_parameter_set_id == 0 )  | |  ( vps_max_layers_minus1 == 0 )  | |  vps_all_independent_layers_flag )
	noScalabilityConstraint = 1
else 
	noScalabilityConstraint = 0

When not present, no_scalability_constraint_flag is inferred to be equal to 0.

no_360_video_constraint_flag equal to 1 specifies that sps_ref_wraparound_enabled_flag shall be equal to 0 and there shall not be any equirectangular projection SEI messages or generalized cubemap projection SEI messages present in the bitstream of the OlsInScope. no_360_video_constraint_flag equal to 0 does not impose such a constraint. When not present, no_360_video_constraint_flag is inferred to be equal to 0.

no_scc_constraint_flag equal to 1 specifies that sps_ibc_enabled_flag shall be equal to 0, sps_bdpcm_enabled_flag shall be equal to 0, and sps_palette_enabled_flag shall be equal to 0. no_scc_constraint_flag equal to 0 does not impose such a constraint. When not present, no_scc_constraint_flag is inferred to be equal to 0.

      - 7.4.4.2.3 General NAL unit type constraint information semantics

no_mixed_nalu_types_in_pic_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that pps_mixed_nalu_types_in_pic_flag shall be equal to 0. no_mixed_nalu_types_in_pic_constraint_flag equal to 0 does not impose such a constraint. When not present, no_mixed_nalu_types_in_pic_constraint_flag is inferred to be equal to 0.

no_trail_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to TRAIL_NUT present in OlsInScope. no_trail_constraint_flag equal to 0 does not impose such a constraint. When not present, no_trail_constraint_flag is inferred to be equal to 0.

no_stsa_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to STSA_NUT present in OlsInScope. no_stsa_constraint_flag equal to 0 does not impose such a constraint. When not present, no_stsa_constraint_flag is inferred to be equal to 0.

no_radl_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to RADL_NUT present in OlsInScope. no_radl_constraint_flag equal to 0 does not impose such a constraint. When not present, no_radl_constraint_flag is inferred to be equal to 0.

no_rasl_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to RASL_NUT present in OlsInScope. no_rasl_constraint_flag equal to 0 does not impose such a constraint. When not present, no_rasl_constraint_flag is inferred to be equal to 0.

no_idr_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to IDR_W_RADL or IDR_N_LP present in OlsInScope. no_idr_constraint_flag equal to 0 does not impose such a constraint. When not present, no_idr_constraint_flag is inferred to be equal to 0.

no_cra_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to CRA_NUT present in OlsInScope. no_cra_constraint_flag equal to 0 does not impose such a constraint. When not present, no_cra_constraint_flag is inferred to be equal to 0.

no_gdr_constraint_flag equal to 1 specifies that sps_gdr_enabled_flag shall be equal to 0 and that there shall be no NAL unit with nuh_unit_type equal to GDR_NUT in OlsInScope. no_gdr_constraint_flag equal to 0 does not impose such a constraint. When not present, no_gdr_constraint_flag is inferred to be equal to 0.

no_aps_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to PREFIX_APS_NUT or SUFFIX_APS_NUT present in OlsInScope, and the sps_lmcs_enabled_flag and sps_explicit_scaling_list_enabled_flag shall both be equal to 0. no_aps_constraint_flag equal to 0 does not impose such a constraint. When not present, no_aps_constraint_flag is inferred to be equal to 0.

      - 7.4.4.2.4 General partition constraint information semantics

one_tile_per_pic_constraint_flag equal to 1 specifies that each picture shall contain only one tile. one_tile_per_pic_constraint_flag equal to 0 does not impose such a constraint. When not present, one_tile_per_pic_constraint_flag is inferred to be equal to 0.

pic_header_in_slice_header_constraint_flag equal to 1 specifies that each picture shall contain only one slice and the value of sh_picture_header_in_slice_header_flag in each slice shall be equal to 1. pic_header_in_slice_header_constraint_flag equal to 0 does not impose such a constraint. When not present, pic_header_in_slice_header_constraint_flag is inferred to be equal to 0.

one_slice_per_pic_constraint_flag equal to 1 specifies that each picture shall contain only one slice. one_slice_per_pic_constraint_flag equal to 0 does not impose such a constraint. When pic_header_in_slice_header_constraint_flag is equal to 1, the value of one_slice_per_pic_constraint_flag shall be equal to 1. When not present, one_slice_per_pic_constraint_flag is inferred to be equal to 0.

one_subpic_per_pic_constraint_flag equal to 1 specifies that each picture shall contain only one subpicture and the value of sps_subpic_info_present_flag shall be equal to 0. one_subpic_per_pic_constraint_flag equal to 0 does not impose such a constraint. When one_slice_per_pic_constraint_flag is equal to 1, the value of one_subpic_per_pic_constraint_flag shall be equal to 1. When not present, one_subpic_per_pic_constraint_flag is inferred to be equal to 0.

no_qtbtt_dual_tree_intra_constraint_flag equal to 1 specifies that sps_qtbtt_dual_tree_intra_flag shall be equal to 0. no_qtbtt_dual_tree_intra_constraint_flag equal to 0 does not impose such a constraint. When max_chroma_format_constraint_idc is equal to 0, the value of no_qtbtt_dual_tree_intra_constraint_flag shall be equal to 1. When not present, no_qtbtt_daul_tree_intra_constraint_flag is inferred to be equal to 0.

no_partition_constraints_override_constraint_flag equal to 1 specifies that sps_partition_constraints_override_enabled_flag shall be equal to 0. no_partition_constraints_override_constraint_flag equal to 0 does not impose such a constraint. When not present, no_partition_constraints_override_constraint_flag is inferred to be equal to 0.

no_virtual_boundary_constraint_flag equal to 1 specifies that sps_virtual_boundaries_enabled_flag shall be equal to 0. no_virtual_boundary_constraint_flag equal to 0  does not impose such a constraint. When not present, no_virtual_boundary_constraint_flag is inferred to be equal to 0.

      - 7.4.4.2.5 General prediction mode constraint information semantics

intra_only_constraint_flag equal to 1 specifies that sh_slice_type shall be equal to I. intra_only_constraint_flag equal to 0 does not impose such a constraint. When general_one_picture_only_constraint_flag is equal to 1, the value of intra_only_constraint_flag shall be equal to 1. When not present, intra_only_constraint_flag is inferred to be equal to 0.

no_palette_constraint_flag equal to 1 specifies that sps_palette_enabled_flag shall be equal to 0. no_palette_constraint_flag equal to 0 does not impose such a constraint. When not present, no_palette_constraint_flag is inferred to be equal to 0.

no_ibc_constraint_flag equal to 1 specifies that sps_ibc_enabled_flag shall be equal to 0. no_ibc_constraint_flag equal to 0 does not impose such a constraint. When not present, no_ibc_constraint_flag is inferred to be equal to 0.

      - 7.4.4.2.6 General intra constraint information semantics

no_mrl_constraint_flag equal to 1 specifies that sps_mrl_enabled_flag shall be equal to 0.  no_mrl_constraint_flag equal to 0 does not impose such a constraint. When not present, no_mrl_constraint_flag is inferred to be equal to 0.

no_isp_constraint_flag equal to 1 specifies that sps_isp_enabled_flag shall be equal to 0.  no_isp_constraint_flag equal to 0 does not impose such a constraint. When not present, no_isp_constraint_flag is inferred to be equal to 0.

no_mip_constraint_flag equal to 1 specifies that sps_mip_enabled_flag shall be equal to 0.  no_mip_constraint_flag equal to 0 does not impose such a constraint. When not present, no_mip_constraint_flag is inferred to be equal to 0.

no_cclm_constraint_flag equal to 1 specifies that sps_cclm_enabled_flag shall be equal to 0. no_cclm_constraint_flag equal to 0 does not impose such a constraint. When max_chroma_format_constraint_idc is equal to 0, the value of no_cclm_constraint_flag shall be equal to 1. When not present, no_cclm_constraint_flag is inferred to be equal to 0.

      - 7.4.4.2.7 General inter constraint information semantics

no_ref_pic_resampling_constraint_flag equal to 1 specifies that sps_ref_pic_resampling_enabled_flag shall be equal to 0. no_ref_pic_resampling_constraint_flag equal to 0 does not impose such a constraint. When not present, no_ref_pic_resampling_constraint_flag is inferred to be equal to 0.

no_res_change_in_clvs_constraint_flag equal to 1 specifies that sps_res_change_in_clvs_allowed_flag shall be equal to 0. no_res_change_in_clvs_constraint_flag equal to 0 does not impose such a constraint. When no_ref_pic_resampling_constraint_flag is equal to 1, no_res_change_in_clvs_constraint_flag shall be equal to 1. When not present, no_res_change_in_clvs_constraint_flag is inferred to be equal to 0.

no_ref_wraparound_constraint_flag equal to 1 specifies that sps_ref_wraparound_enabled_flag shall be equal to 0. no_ref_wraparound_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_ref_wraparound_constraint_flag shall be equal to 1. When not present, no_ref_wraparound_constraint_flag is inferred to be equal to 0.

no_temporal_mvp_constraint_flag equal to 1 specifies that sps_temporal_mvp_enabled_flag shall be equal to 0. no_temporal_mvp_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_temporal_mvp_constraint_flag shall be equal to 1. When not present, no_temporal_mvp_constraint_flag is inferred to be equal to 0.

no_sbtmvp_constraint_flag equal to 1 specifies that sps_sbtmvp_enabled_flag shall be equal to 0. no_sbtmvp_constraint_flag equal to 0 does not impose such a constraint. When no_temporal_mvp_constraint_flag is equal to 1, the value of no_sbtmvp_constraint_flag shall be equal to 1. When not present, no_sbtmvp_constraint_flag is inferred to be equal to 0.

no_amvr_constraint_flag equal to 1 specifies that sps_amvr_enabled_flag shall be equal to 0. no_amvr_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_amvr_constraint_flag shall be equal to 1. When not present, no_amvr_constraint_flag is inferred to be equal to 0.

no_bdof_constraint_flag equal to 1 specifies that sps_bdof_enabled_flag shall be equal to 0. no_bdof_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_bdof_constraint_flag shall be equal to 1. When not present, no_bdof_constraint_flag is inferred to be equal to 0.

no_dmvr_constraint_flag equal to 1 specifies that sps_dmvr_enabled_flag shall be equal to 0. no_dmvr_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_dmvr_constraint_flag shall be equal to 1. When not present, no_dmvr_constraint_flag is inferred to be equal to 0.

no_affine_motion_constraint_flag equal to 1 specifies that sps_affine_enabled_flag shall be equal to 0. no_affine_motion_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_affine_motion_constraint_flag shall be equal to 1. When not present, no_affine_motion_constraint_flag is inferred to be equal to 0.

no_mmvd_constraint_flag equal to 1 specifies that sps_mmvd_enabled_flag shall be equal to 0. no_mmvd_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_mmvd_constraint_flag shall be equal to 1. When not present, no_mmvd_constraint_flag is inferred to be equal to 0.

no_smvd_constraint_flag equal to 1 specifies that sps_smvd_enabled_flag shall be equal to 0. no_smvd_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_smvd_constraint_flag shall be equal to 1. When not present, no_smvd_constraint_flag is inferred to be equal to 0.

no_prof_constraint_flag equal to 1 specifies that sps_affine_prof_enabled_flag shall be equal to 0. no_prof_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_prof_constraint_flag shall be equal to 1. When not present, no_prof_constraint_flag is inferred to be equal to 0.

no_bcw_constraint_flag equal to 1 specifies that sps_bcw_enabled_flag shall be equal to 0. no_bcw_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_bcw_constraint_flag shall be equal to 1. When not present, no_bcw_constraint_flag is inferred to be equal to 0.

no_ciip_constraint_flag equal to 1 specifies that sps_ciip_enabled_flag shall be equal to 0. no_cipp_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_cipp_constraint_flag shall be equal to 1. When not present, no_ciip_constraint_flag is inferred to be equal to 0.

no_gpm_constraint_flag equal to 1 specifies that sps_gpm_enabled_flag shall be equal to 0. no_gpm_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_gpm_constraint_flag shall be equal to 1. When not present, no_gpm_constraint_flag is inferred to be equal to 0.

no_weighted_pred_constraint_flag equal to 1 specifies that sps_weighted_pred_flag shall be equal to 0. no_weighted_pred_constraint_flag equal to 0 does not impose such a constraint. When not present, no_weighted_pred_constraint_flag is inferred to be equal to 0.

no_weighted_bipred_constraint_flag equal to 1 specifies that sps_weighted_bipred_flag shall be equal to 0. no_weighted_bipred_constraint_flag equal to 0 does not impose such a constraint. When not present, no_weighted_bipred_constraint_flag is inferred to be equal to 0.

      - 7.4.4.2.8 General transform, quantization, and residual constraint information semantics

no_mts_constraint_flag equal to 1 specifies that sps_mts_enabled_flag shall be equal to 0. no_mts_constraint_flag equal to 0 does not impose such a constraint. When not present, no_mts_constraint_flag is inferred to be equal to 0.

no_sbt_constraint_flag equal to 1 specifies that sps_sbt_enabled_flag shall be equal to 0. no_sbt_constraint_flag equal to 0 does not impose such a constraint. When not present, no_sbt_constraint_flag is inferred to be equal to 0.

no_lfnst_constraint_flag equal to 1 specifies that sps_lfnst_enabled_flag shall be equal to 0. no_lfnst_constraint_flag equal to 0 does not impose such a constraint. When not present, no_lfnst_constraint_flag is inferred to be equal to 0.

no_transform_skip_constraint_flag equal to 1 specifies that  sps_transform_skip_enabled_flag shall be equal to 0. no_transform_skip_constraint_flag equal to 0 does not impose such a constraint. When not present, no_transform_skip_constraint_flag is inferred to be equal to 0.

no_act_constraint_flag equal to 1 specifies that sps_act_enabled_flag shall be equal to 0. no_act_constraint_flag equal to 0 does not impose such a constraint. When not present, no_act_constraint_flag is inferred to be equal to 0.

no_joint_cbcr_constraint_flag equal to 1 specifies that sps_joint_cbcr_enabled_flag shall be equal to 0.  no_joint_cbcr_constraint_flag equal to 0 does not impose such a constraint. When max_chroma_format_constraint_idc is equal to 0, the value of no_joint_cbcr_constraint_flag shall be equal to 1.  When not present, no_joint_cbcr_constraint_flag is inferred to be equal to 0.

no_bdpcm_constraint_flag equal to 1 specifies that sps_bdpcm_enabled_flag shall be equal to 0. no_bdpcm_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, no_bdpcm_constraint_flag shall be equal to 1. When not present, no_bdpcm_constraint_flag is inferred to be equal to 0.

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 1. no_tsrc_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_tsrc_constraint_flag shall be equal to 1. When not present, no_tsrc_constraint_flag is inferred to be equal to 0.

no_cu_qp_delta_constraint_flag equal to 1 specifies that pps_cu_qp_delta_enabled_flag shall be equal to 0. no_cu_qp_delta_constraint_flag equal to 0 does not impose such a constraint. When not present, no_cu_qp_delta_constraint_flag is inferred to be equal to 0.

no_chroma_qp_offset_constraint_flag equal to 1 specifies that pps_cu_chroma_qp_offset_list_enabled_flag shall be equal to 0. no_chroma_qp_offset_constraint_flag equal to 0 does not impose such a constraint. When not present, chroma_qp_offset_constraint_flag is inferred to be equal to 0.

no_dep_quant_constraint_flag equal to 1 specifies that sps_dep_quant_enabled_flag shall be equal to 0. no_dep_quant_constraint_flag equal to 0 does not impose such a constraint. When not present, no_dep_quant_constraint_flag is inferred to be equal to 0.

no_sign_data_hiding_constraint_flag equal to 1 specifies that sps_sign_data_hiding_enabled_flag shall be equal to 0. no_sign_data_hiding_constraint_flag equal to 0 does not impose such a constraint. When not present, no_sign_data_hiding_constraint_flag is inferred to be equal to 0.

no_explicit_scaling_list_constraint_flag equal to 1 specifies that sps_explicit_scaling_list_enabled_flag shall be equal to 0. no_explicit_scaling_list_constraint_flag equal to 0 does not impose such a constraint. When no_aps_constraint_flag is equal to 1, the value of no_explicit_scaling_list_constraint_flag shall be equal to 1. When not present, no_explicit_scaling_list_constraint_flag is inferred to be equal to 0.

      - 7.4.4.2.9 General loop filter constraint information semantics

no_sao_constraint_flag equal to 1 specifies that sps_sao_enabled_flag shall be equal to 0. no_sao_constraint_flag equal to 0 does not impose such a constraint. When not present, no_sao_constraint_flag is inferred to be equal to 0.

no_alf_constraint_flag equal to 1 specifies that sps_alf_enabled_flag shall be equal to 0. no_alf_constraint_flag equal to 0 does not impose such a constraint. When not present, no_alf_constraint_flag is inferred to be equal to 0.

no_ccalf_constraint_flag equal to 1 specifies that sps_ccalf_enabled_flag shall be equal to 0. no_ccalf_constraint_flag equal to 0 does not impose such a constraint. When max_chroma_format_constraint_idc is equal to 0 or no_alf_constraint_flag is equal 1, the value of no_ccalf_constraint_flag shall be equal to 1. When not present, no_ccalf_constraint_flag is inferred to be equal to 0.

no_ladf_constraint_flag equal to 1 specifies that sps_ladf_enabled_flag  shall be equal to 0. no_ladf_constraint_flag equal to 0 does not impose such a constraint. When not present, no_ladf_constraint_flag is inferred to be equal to 0.

no_lmcs_constraint_flag equal to 1 specifies that sps_lmcs_enabled_flag shall be equal to 0. no_lmcs_constraint_flag equal to 0 does not impose such a constraint. When no_aps_constraint_flag is equal to 1, the value of no_lmcs_constraint_flag shall be equal to 1. When not present, no_lmcs_constraint_flag is inferred to be equal to 0.

      - 7.4.4.2.10 General layer constraint information semantics

single_layer_constraint_flag equal to 1 specifies that sps_video_parameter_set_id shall be equal to 0. single_layer_constraint_flag equal to 0 does not impose such a constraint. When general_one_picture_only_constraint_flag is equal to 1, the value of single_layer_constraint_flag shall be equal to 1. When not present, single_layer_constraint_flag is inferred to be equal to 0.

all_layers_independent_constraint_flag equal to 1 specifies that vps_all_independent_layers_flag, when present, shall be equal to 1 and, when not present, shall be inferred to be equal to 1. all_layers_independent_constraint_flag equal to 0 does not impose such a constraint. When not present, all_layers_independent_constraint_flag is inferred to be equal to 0.

      - 7.4.4.2.11 General supplemental enhancement information constraint information semantics

NOTE 1 – Decoders may ignore the value of the general supplemental enhancement information constraint information flags as there are no decoding process requirements associated with the presence or interpretation of the corresponding SEI messages

no_scalable_nesting_SEI_constraint_flag equal to 1 specifies that there shall not be any scalable nesting SEI messages present in the bitstream of the OlsInScope. no_scalable_nesting_SEI_constraint_flag equal to 0 does not impose such a constraint. When not present, no_scalable_nesting_SEI_constraint_flag is inferred to be equal to 0.

no_subpic_level_SEI_constraint_flag equal to 1 specifies that there shall not be any subpicture level SEI messages present in the bitstream of the OlsInScope. no_subpic_level_SEI_constraint_flag equal to 0 does not impose such a constraint. When not present, no_subpic_level_SEI_constraint_flag is inferred to be equal to 0.

no_filler_payload_SEI_constraint_flag equal to 1 specifies that there shall not be any filler payload SEI messages present in the bitstream of the OlsInScope. no_filler_payload_SEI_constraint_flag equal to 0 does not impose such a constraint. When not present, no_filler_payload_SEI_constraint_flag is inferred to be equal to 0.

no_user_data_reg_SEI_constraint_flag equal to 1 specifies that there shall not be any user data registered by Recommendation ITU T.35 SEI messages present in the bitstream of the OlsInScope. no_user_data_reg_SEI_constraint_flag equal to 0 does not impose such a constraint. When not present, no_user_data_reg_SEI_constraint_flag is inferred to be equal to 0.

no_user_data_unreg_SEI_constraint_flag equal to 1 specifies that there shall not be any user data unregistered SEI messages present in the bitstream of the OlsInScope. no_user_data_unreg_SEI_constraint_flag equal to 0 does not impose such a constraint. When not present, no_user_data_unreg_SEI_constraint_flag is inferred to be equal to 0.

no_film_grain_SEI_constraint_flag equal to 1 specifies that there shall not be any film grain characteristics SEI messages present in the bitstream of the OlsInScope. no_film_grain_SEI_constraint_flag equal to 0 does not impose such a constraint. When not present, no_film_grain_SEI_constraint_flag is inferred to be equal to 0.

general_non_packed_SEI_constraint_flag equal to 1 specifies that there shall not be any frame packing arrangement SEI messages present in the bitstream of the OlsInScope. general_non_packed_SEI_constraint_flag equal to 0 does not impose such a constraint. When not present, general_non_packed_SEI_constraint_flag is inferred to be equal to 0.

NOTE 2 – Decoders may ignore the value of general_non_packed_constraint_flag, as there are no decoding process requirements associated with the presence or interpretation of frame packing arrangement SEI messages.

general_non_projected_SEI_constraint_flag equal to 1 specifies that there shall not be any equirectangular projection SEI messages or generalized cubemap projection SEI messages present in the bitstream of the OlsInScope. general_non_projected_SEI_constraint_flag equal to 0 does not impose such a constraint. When not present, general_non_projected_SEI_constraint_flag is inferred to be equal to 0.

NOTE 3 – Decoders may ignore the value of general_non_projected_constraint_flag, as there are no decoding process requirements associated with the presence or interpretation of equirectangular projection SEI messages and generalized cubemap projection SEI messages.

no_parameter_set_incl_SEI_constraint_flag equal to 1 specifies that there shall not be any parameter sets inclusion indication SEI messages present in the bitstream of the OlsInScope. no_parameter_set_incl_SEI_constraint_flag equal to 0 does not impose such a constraint. When not present, no_parameter_set_incl_SEI_constraint_flag is inferred to be equal to 0.

no_decoded_picture_hash_SEI_constraint_flag equal to 1 specifies that there shall not be any decoded picture hash SEI messages present in the bitstream of the OlsInScope. no_decoded_picture_hash_SEI_constraint_flag equal to 0 does not impose such a constraint. When not present, no_decoded_picture_hash_SEI_constraint_flag is inferred to be equal to 0.

no_mcdv_SEI_constraint_flag equal to 1 specifies that there shall not be any mastering display colour volume SEI messages present in the bitstream of the OlsInScope. No_mcdv_SEI_constraint_flag equal to 0 does not impose such a constraint. When not present, no_mcdv_SEI_constraint_flag is inferred to be equal to 0.

no_cll_SEI_constraint_flag equal to 1 specifies that there shall not be any content light level SEI messages present in the bitstream of the OlsInScope. no_cll_SEI_constraint_flag equal to 0 does not impose such a constraint. When not present, no_cll_SEI_constraint_flag is inferred to be equal to 0.

no_DRAP_SEI_constraint_flag equal to 1 specifies that there shall not be any dependent random access point indication SEI messages present in the bitstream of the OlsInScope. no_DRAP_SEI_constraint_flag equal to 0 does not impose such a constraint. When not present, no_DRAP_SEI_constraint_flag is inferred to be equal to 0.

no_alt_transfer_char_SEI_constraint_flag equal to 1 specifies that there shall not be any alternative transfer characteristics SEI messages present in the bitstream of the OlsInScope. no_alt_transfer_char_SEI_constraint_flag equal to 0 does not impose such a constraint. When not present, no_alt_transfer_char_SEI_constraint_flag is inferred to be equal to 0.

no_ambient_view_envir_SEI_constraint_flag equal to 1 specifies that there shall not be any ambient viewwing environment SEI messages present in the bitstream of the OlsInScope. no_ambient_view_envir_SEI_constraint_flag equal to 0 does not impose such a constraint. When not present, no_ambient_view_envir_SEI_constraint_flag is inferred to be equal to 0.

no_ccv_SEI_constraint_flag equal to 1 specifies that there shall not be any content colour volume SEI messages present in the bitstream of the OlsInScope. no_ccv_SEI_constraint_flag equal to 0 does not impose such a constraint. When not present, no_ccv_SEI_constraint_flag is inferred to be equal to 0.

no_omni_video_specific_SEI_constraint_flag equal to 1 specifies that there shall not be any omnidirectional video specific SEI messages present in the bitstream of the OlsInScope. no_omni_video_specific_SEI_constraint_flag equal to 0 does not impose such a constraint. When not present, no_omni_video_specific_SEI_constraint_flag is inferred to be equal to 0.

no_field_frame_SEI_constraint_flag equal to 1 specifies that there shall not be any frame-field information SEI messages present in the bitstream of the OlsInScope. no_field_frame_SEI_constraint_flag equal to 0 does not impose such a constraint. When not present, no_field_frame_SEI_constraint_flag is inferred to be equal to 0.

no_sar_SEI_constraint_flag equal to 1 specifies that there shall not be any sample aspect ratio SEI messages present in the bitstream of the OlsInScope. no_sar_SEI_constraint_flag equal to 0 does not impose such a constraint. When not present, no_sar_SEI_constraint_flag is inferred to be equal to 0.

# Alternative proposal

      - 7.3.3.2 General constraint information syntax (Alternative proposal)

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| general_format_constraint_info_present_flag | u(1) |
| general_functionality_constraint_info_present_flag | u(1) |
| general_NALU_constraint_info_present_flag | u(1) |
| general_partiton_constraint_info_present_flag | u(1) |
| general_pred_mode_constraint_info_present_flag | u(1) |
| general_intra_constraint_info_present_flag | u(1) |
| general_inter_constraint_info_present_flag | u(1) |
| general_tqr_constraint_info_present_flag | u(1) |
| general_loop_filter_constraint_info_present_flag | u(1) |
| general_layer_constraint_info_present_flag | u(1) |
| general_SEI_constraint_info_present_flag | u(1) |
| general_reserved_zero_4bits | u(4) |
| if( general_format_constraint_info_present_flag ) |  |
| general_format_constraint_info( ) |  |
| if( general_functionality_constraint_info_present_flag ) |  |
| general_functionality_constraint_info( ) |  |
| if( general_NALU_constraint_info_present_flag ) |  |
| general_NALU_constraint_info( ) |  |
| if( general_partition_constraint_info_present_flag ) |  |
| general_partiton_constraint_info( ) |  |
| if( general_pred_mode_constraint_info_present_flag ) |  |
| general_pred_mode_constraint_info( ) |  |
| if( general_intra_constraint_info_present_flag ) |  |
| general_intra_constraint_info( ) |  |
| if( general_inter_constraint_info_present_flag ) |  |
| general_inter_constraint_info( ) |  |
| if( general_tqr_constraint_info_present_flag ) |  |
| general_tqr_constraint_info( ) |  |
| if( general_loop_filter_constraint_info_present_flag ) |  |
| general_loop_filter_constraint_info( ) |  |
| if( general_layer_constraint_info_present_flag ) |  |
| general_layer_constraint_info( ) |  |
| if( general_SEI_constraint_info_present_flag ) |  |
| general_SEI_constraint_info( ) |  |
| while( !byte_aligned( ) ) |  |
| gci_alignment_zero_bit | f(1) |
| gci_num_reserved_bytes | u(8) |
| for( i = 0; i < gci_num_reserved_bytes; i++ ) |  |
| gci_reserved_byte[ i ] | u(8) |
| } |  |

      - 7.3.3.2 General constraint information syntax (Alternative proposal)

general_format_constraint_info_present_flag equal to 1 specifies that the  general_format_constraint_info_present_flag( ) syntax structure is present. general_format_constraint_info_present_flag equal to 0 specifies that a general_format_constraint_info_present_flag( ) syntax structure is not present.

general_functionality_constraint_info_present_flag equal to 1 specifies that the  general_functionality_constraint_info_present_flag( ) syntax structure is present. general_functionality_constraint_info_present_flag equal to 0 specifies that a general_functionality_constraint_info_present_flag( ) syntax structure is not present. When general_functionality_constraint_info_present_flag is equal to 1, at least one general functionality constraint flag in the general_functionality_constraint_info_present_flag( ) syntax structure should be equal to 1.

general_NALU_constraint_info_present_flag equal to 1 specifies that the  general_NALU_constraint_info_present_flag( ) syntax structure is present. general_NALU_constraint_info_present_flag equal to 0 specifies that a general_NALU_constraint_info_present_flag( ) syntax structure is not present. When general_NALU_constraint_info_present_flag is equal to 1, at least one general NALU constraint flag in the general_NALU_constraint_info_present_flag( ) syntax structure should be equal to 1.

general_partition_constraint_info_present_flag equal to 1 specifies that the  general_partition_constraint_info_present_flag( ) syntax structure is present. general_partition_constraint_info_present_flag equal to 0 specifies that a general_partition_constraint_info_present_flag( ) syntax structure is not present. When general_partition_constraint_info_present_flag is equal to 1, at least one general partition constraint flag in the general_partition_constraint_info_present_flag( ) syntax structure should be equal to 1.

general_pred_mode_constraint_info_present_flag equal to 1 specifies that the  general_pred_mode_constraint_info_present_flag( ) syntax structure is present. general_pred_mode_constraint_info_present_flag equal to 0 specifies that a general_pred_mode_constraint_info_present_flag( ) syntax structure is not present. When general_pred_mode_constraint_info_present_flag is equal to 1, at least one general prediction mode constraint flag in the general_pred_mode_constraint_info_present_flag( ) syntax structure should be equal to 1.

general_intra_constraint_info_present_flag equal to 1 specifies that the  general_intra_constraint_info_present_flag( ) syntax structure is present. general_intra_constraint_info_present_flag equal to 0 specifies that a general_intra_constraint_info_present_flag( ) syntax structure is not present. When general_intra_constraint_info_present_flag is equal to 1, at least one general intra prediction constraint flag in the general_intra_constraint_info_present_flag( ) syntax structure should be equal to 1.

general_inter_constraint_info_present_flag equal to 1 specifies that the  general_inter_constraint_info_present_flag( ) syntax structure is present. general_inter_constraint_info_present_flag equal to 0 specifies that a general_inter_constraint_info_present_flag( ) syntax structure is not present. When general_inter_constraint_info_present_flag is equal to 1, at least one general inter prediction constraint flag in the general_inter_constraint_info_present_flag( ) syntax structure should be equal to 1.

general_tqr_constraint_info_present_flag equal to 1 specifies that the  general_tqr_constraint_info_present_flag( ) syntax structure is present. general_tqr_constraint_info_present_flag equal to 0 specifies that a general_tqr_constraint_info_present_flag( ) syntax structure is not present. When general_tqr_constraint_info_present_flag is equal to 1, at least one general transform, quantization, and residual constraint flag in the general_tqr_constraint_info_present_flag( ) syntax structure should be equal to 1.

general_loop_filter_constraint_info_present_flag equal to 1 specifies that the  general_loop_filter_constraint_info_present_flag( ) syntax structure is present. general_loop_filter_constraint_info_present_flag equal to 0 specifies that a general_loop_filter_constraint_info_present_flag( ) syntax structure is not present. When general_loop_filter_constraint_info_present_flag is equal to 1, at least one general loop filter constraint flag in the general_loop_filter_constraint_info_present_flag( ) syntax structure should be equal to 1.

general_layer_constraint_info_present_flag equal to 1 specifies that the  general_layer_constraint_info_present_flag( ) syntax structure is present. general_layer_constraint_info_present_flag equal to 0 specifies that a general_layer_constraint_info_present_flag( ) syntax structure is not present. When general_layer_constraint_info_present_flag is equal to 1, at least one general layer constraint flag in the general_layer_constraint_info_present_flag( ) syntax structure should be equal to 1.

general_SEI_constraint_info_present_flag equal to 1 specifies that the  general_SEI_constraint_info_present_flag( ) syntax structure is present. general_SEI_constraint_info_present_flag equal to 0 specifies that a general_SEI_constraint_info_present_flag( ) syntax structure is not present. When general_SEI_constraint_info_present_flag is equal to 1, at least one general SEI message constraint flag in the general_SEI_constraint_info_present_flag( ) syntax structure should be equal to 1.

general_reserved_zero_4bits, when present, shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for general_reserved_zero_4bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of general_reserved_zero_4bits.

gci_alignment_zero_bits shall be equal to 0.

gci_num_reserved_bytes specifies the number of the reserved constraint bytes. The value of gci_num_reserved_bytes shall be 0. Other values of gci_num_reserved_bytes are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification.

gci_reserved_byte[ i ] may have any value. Its presence and value do not affect decoder conformance to profiles specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore the values of all the gci_reserved_byte[ i ] syntax elements.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).