| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0115 |
| --- | --- |

| Title: | AHG9: On SPS cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Vadim Seregin, Muhammed Coban, Marta Karczewicz | Email: | yonghe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following cleanup changes on SPS.

- Signaling condition on sps_num_subpics_minues1

- Semantic constraint on sps_max_sublayers_minus1

# Proposals

## Condition on sps_num_subpics_minus1

In SPS, sps_subpic_ctu_top_left_x[ ] or sps_subpic_ctu_top_left_y[ ] is not explicitly signaled when the picture width or height is less than or equal to the CTB size. The similar design may apply to the syntax element sps_num_subpics_minus1, the value of sps_num_subpics_minus1 can be inferred to be equal to 0 when both picture width and height are less than or equal to CTB size.

The proposed signaling condition is shown in Table 1.

Table 1 Proposed SPS RBSP

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_subpic_info_present_flag | u(1) |
| if( sps_subpic_info_present_flag ) { |  |
| if( sps_pic_width_max_in_luma_samples > CtbSizeY ¦ ¦ sps_pic_height_max_in_luma_samples > CtbSizeY ) |  |
| sps_num_subpics_minus1 | ue(v) |
| if( sps_num_subpics_minus1 > 0 ) |  |
| sps_independent_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && sps_pic_width_max_in_luma_samples > CtbSizeY ) |  |
| sps_subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && sps_pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| sps_subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && sps_pic_width_max_in_luma_samples > CtbSizeY ) |  |
| sps_subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && sps_pic_height_max_in_luma_samples > CtbSizeY ) |  |
| sps_subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| sps_subpic_treated_as_pic_flag[ i ] | u(1) |
| sps_loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … | ue(v) |
| } |  |
| … |  |
| } |  |

Alternatively, semantic constraint may be imposed on the sps_num_subpics_minus1 as follows.

sps_num_subpics_minus1 plus 1 specifies the number of subpictures in each picture in the CLVS. The value of sps_num_subpics_minus1 shall be in the range of 0 to Ceil( sps_pic_width_max_in_luma_samples ÷ CtbSizeY ) * Ceil( sps_pic_height_max_in_luma_samples ÷ CtbSizeY ) − 1, inclusive. When not present, the value of sps_num_subpics_minus1 is inferred to be equal to 0. When the maximum picture width and height are less than or equal to CtbSizeY, the value of sps_num_subpics_minus1 shall be equal to 0.

## Semantic constraint on sps_max_sublyers_minus1

Maximum number of sub-layers can be signalled in VPS and SPS when sps_video_parameter_set_id is greater than 0. When vps_all_layers_same_num_sublayers_flag is equal to 1, the number of temporal sublayers is the same for all the layers specified by the VPS. It may be ambiduous if the value of sps_max_sublayers_minus1 does not match the value of vps_max_sublayers_minus1. The semantic constraint on sps_max_sublayers_minus1 is proposed as follows:

vps_max_sublayers_minus1 plus 1 specifies the maximum number of temporal sublayers that may be present in a layer specified by the VPS. The value of vps_max_sublayers_minus1 shall be in the range of 0 to 6, inclusive. When sps_video_parameter_set_id is greater than 0 and vps_all_layers_same_num_sublayers_flag is equal to 1, the value of sps_max_sublayers_minus1 shall be equal to the value of vps_max_sublayers_minus1.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).