| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0125-v2 |
| --- | --- |

| Title: | AHG9: On sub-picture constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Yong He, Yao-Jen Chang, Muhammed Coban, Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

There are multiple constraints applied for sub-pictures, some constraints are applied for sub-pictures treated as pictures, some other constraints are applied regardless of sps_subpic_treated_as_pic_flag value. In the latter case, the constraints are more restrictive.

This contribution proposes to add a condition sps_subpic_treated_as_pic_flag[ i ] equal to 1 for sub-picture constraints.

It also proposes to add a constraint that when sps_num_subpics_minus1 is greater than 0 and sps_subpic_treated_as_pic_flag[ i ] is equal to 1, the scaling window offsets of the current layer and the scaling window offsets of the layers that have the current layer as a reference shall be the same.

In the version 2 of the document, the proposed text is modified reflecting AhG recommendation made at the last meeting and offline discussion: scaling window change is removed, PPS related constraint is reworded, and sps_subpic_treated_as_pic_flag constraint is added.

# Introduction

In VVC specification, there are three places where constraints are expressed for sub-pictures.

sps_subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. sps_subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of sps_subpic_treated_as_pic_flag[ i ] is inferred to be equal to 1.

When sps_num_subpics_minus1 is greater than 0 and sps_subpic_treated_as_pic_flag[ i ] is equal to 1, for each CLVS of a current layer referring to the SPS, let targetAuSet be all the AUs starting from the AU containing the first picture of the CLVS in decoding order, to the AU containing the last picture of the CLVS in decoding order, inclusive, it is a requirement of bitstream conformance that all of the following conditions are true for the targetLayerSet that consists of the current layer and all the layers that have the current layer as a reference layer:

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of pps_pic_width_in_luma_samples and the same value of pps_pic_height_in_luma_samples.

–	All the SPSs referred to by the layers in targetLayerSet shall have the same value of sps_num_subpics_minus1 and shall have the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], sps_subpic_height_minus1[ j ], and sps_subpic_treated_as_pic_flag[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

### 8.3.2 Decoding process for reference picture lists construction

…

- When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 and sps_num_subpics_minus1 is greater than 0, either of the following two conditions (but not both) shall be true:

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] has the same subpicture layout as the current picture (i.e., the SPSs referred to by that picture and the current picture have the same value of sps_num_subpics_minus1 and the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], and sps_subpic_height_minus1[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive).

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] is an ILRP for which the value of sps_num_subpics_minus1 is equal to 0.

pps_subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the pps_subpic_id[ i ] syntax element is pps_subpic_id_len_minus1 + 1 bits.

The variable SubpicIdVal[ i ], for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, is derived as follows:

for( i = 0; i  <=  sps_num_subpics_minus1; i++ )
	if( sps_subpic_id_mapping_explicitly_signalled_flag )
		SubpicIdVal[ i ] = pps_subpic_id_mapping_present_flag ? pps_subpic_id[ i ] : sps_subpic_id[ i ]	(81)
	else
		SubpicIdVal[ i ] = i

It is a requirement of bitstream conformance that both of the following constraints apply:

–	For any two different values of i and j in the range of 0 to sps_num_subpics_minus1, inclusive, SubpicIdVal[ i ] shall not be equal to SubpicIdVal[ j ].

–	For each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, when the value of SubpicIdVal[ i ] of a current picture is not equal to the value of SubpicIdVal[ i ] of a reference picture, the active entries of the RPLs of the coded slices in the i-th subpicture of the current picture shall not include that reference picture.

# Proposal

## Aspect 1. Sub-picture constraints for sps_subpic_treated_as_pic_flag equal to 1

In the semantics of sps_subpic_treated_as_pic_flag, sub-picture constraints are expressed only for sps_subpic_treated_as_pic_flag[ i ] equal to 1 for a current layer and the layers that have the current layer as a reference layer.

In the “Decoding process for reference picture lists construction”, similar constraints are expressed for all sub-picture regardless of the value of sps_subpic_treated_as_pic_flag for a current layer and all reference layers. There is no explicit constraint imposed on the same value of pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples, which can be actually redundant since the sub-picture layout is the same which implicitly constrains that the picture sizes are the same as well.

In the pps_subpic_id, constraints are expressed for all sub-pictures regardless of the sps_subpic_treated_as_pic_flag value.

It is proposed to specify the sub-picture constraints for sub-pictures treated as pictures, i.e. sps_subpic_treated_as_pic_flag[ i ] equal to 1, and express the constraints using the current layer and all reference layer language.

Editorially, it would be more convenient to consolidate all sub-picture constraints in one place in the specification, for example in the “Decoding process for reference picture lists construction” section.

## Aspect 2. Scaling window constraint

Resolution change is disabled between the current layer and the layers that have the current layer as a reference layer when sps_num_subpics_minus1 is greater than 0 and sps_subpic_treated_as_pic_flag[ i ] is equal to 1. If sps_num_subpics_minus1 is greater than 0 and sps_subpic_treated_as_pic_flag[ i ] is equal to 1, there is a constraint that the values of picture width and height shall be the same across the layers (current layer and the layers that have the current layer as a reference layer). However, there is no constraint to have the same scaling windows, i.e., pps_scaling_win_left_offset, pps_scaling_win_right_offset, pps_scaling_win_top_offset, and pps_scaling_win_bottom_offset. Therefore, scaling ratio may be different from 1x, i.e. RPR is enabled, and this creates a problem in motion compensation decoding process because the current specification does not support aligning of the sub-picture positions of the current subpicture and reference subpicture for non 1x scaling ratio.

It is proposed that, when sps_ref_pic_resampling_enabled_flag is equal to 1, sps_num_subpics_minus1 is greater than 0, and sps_subpic_treated_as_pic_flag[ i ] is equal to 1, the scaling window offsets of the current layer and the scaling window offsets of the layers that have the current layer as a reference layer shall be the same.

# Specification change

The proposed changes are as follows.

sps_subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. sps_subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of sps_subpic_treated_as_pic_flag[ i ] is inferred to be equal to 1.

When sps_num_subpics_minus1 is greater than 0 and sps_subpic_treated_as_pic_flag[ i ] is equal to 1, for each CLVS of a current layer referring to the SPS, let targetAuSet be all the AUs starting from the AU containing the first picture of the CLVS in decoding order, to the AU containing the last picture of the CLVS in decoding order, inclusive, it is a requirement of bitstream conformance that all of the following conditions are true for the targetLayerSet that consists of the current layer and all the layers that have the current layer as a reference layer:

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of pps_pic_width_in_luma_samples and the same value of pps_pic_height_in_luma_samples.

–	All the SPSs referred to by the layers in targetLayerSet shall have the same value of sps_num_subpics_minus1 and shall have the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], sps_subpic_height_minus1[ j ], and sps_subpic_treated_as_pic_flag[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

### 8.3.2 Decoding process for reference picture lists construction

…

- When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0, and sps_num_subpics_minus1 is greater than 0 and the current subpicture with sub-picture index subPicIdx has sps_subpic_treated_as_pic_flag[ subPicIdx ] equal to 1, either of the following two conditions (but not both) shall be true:

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall have the same value of pps_pic_width_in_luma_samples and the same value of pps_pic_height_in_luma_samples as the current picture, shall have the same scaling windows offsets as the current picture (i.e., the SPSs referred to by that picture and the current picture have the same values of pps_scaling_win_left_offset, pps_scaling_win_right_offset, pps_scaling_win_top_offset, and pps_scaling_win_bottom_offset), shall have has the same subpicture layout as the current picture (i.e., the SPSs referred to by that picture and the current picture have the same value of sps_num_subpics_minus1 and the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], and sps_subpic_height_minus1[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive), and shall have the same value of sub-picture IDs as the current picture (i.e. SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive).

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] is an ILRP for which the value of sps_num_subpics_minus1 is equal to 0.

pps_subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the pps_subpic_id[ i ] syntax element is pps_subpic_id_len_minus1 + 1 bits.

The variable SubpicIdVal[ i ], for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, is derived as follows:

for( i = 0; i  <=  sps_num_subpics_minus1; i++ )
	if( sps_subpic_id_mapping_explicitly_signalled_flag )
		SubpicIdVal[ i ] = pps_subpic_id_mapping_present_flag ? pps_subpic_id[ i ] : sps_subpic_id[ i ]	(81)
	else
		SubpicIdVal[ i ] = i

It is a requirement of bitstream conformance that both of the following constraints apply:

–	For any two different values of i and j in the range of 0 to sps_num_subpics_minus1, inclusive, SubpicIdVal[ i ] shall not be equal to SubpicIdVal[ j ].

–	For each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, when the value of SubpicIdVal[ i ] of a current picture is not equal to the value of SubpicIdVal[ i ] of a reference picture, the active entries of the RPLs of the coded slices in the i-th subpicture of the current picture shall not include that reference picture.

# Specification change for the updated proposal

The updated proposed changes are as follows.

sps_subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. sps_subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of sps_subpic_treated_as_pic_flag[ i ] is inferred to be equal to 1.

When sps_num_subpics_minus1 is greater than 0 and sps_subpic_treated_as_pic_flag[ i ] is equal to 1, for each CLVS of a current layer referring to the SPS, let targetAuSet be all the AUs starting from the AU containing the first picture of the CLVS in decoding order, to the AU containing the last picture of the CLVS in decoding order, inclusive, it is a requirement of bitstream conformance that all of the following conditions are true for the targetLayerSet that consists of the current layer and all the layers that have the current layer as a reference layer:

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of pps_pic_width_in_luma_samples and the same value of pps_pic_height_in_luma_samples.

–	All the SPSs referred to by the layers in targetLayerSet shall have the same value of sps_num_subpics_minus1 and shall have the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], sps_subpic_height_minus1[ j ], and sps_subpic_treated_as_pic_flag[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

### 8.3.2 Decoding process for reference picture lists construction

…

- When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0, and sps_num_subpics_minus1 is greater than 0 and the current subpicture with sub-picture index subPicIdx has sps_subpic_treated_as_pic_flag[ subPicIdx ] equal to 1, either of the following two conditions (but not both) shall be true:

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] and the current picture have the same value for each of the following:

    - pps_pic_width_in_luma_samples

    - pps_pic_height_in_luma_samples

    - sps_num_subpics_minus1

    - sps_subpic_ctu_top_left_x[ i ], sps_subpic_ctu_top_left_y[ i ], sps_subpic_width_minus1[ i ], sps_subpic_height_minus1[ i ], sps_subpic_treated_as_pic_flag[ i ], respectively, for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive

    - SubpicIdVal[ subpicIdx ]

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] is an ILRP for which the value of sps_num_subpics_minus1 is equal to 0.

pps_subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the pps_subpic_id[ i ] syntax element is pps_subpic_id_len_minus1 + 1 bits.

The variable SubpicIdVal[ i ], for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, is derived as follows:

for( i = 0; i  <=  sps_num_subpics_minus1; i++ )
	if( sps_subpic_id_mapping_explicitly_signalled_flag )
		SubpicIdVal[ i ] = pps_subpic_id_mapping_present_flag ? pps_subpic_id[ i ] : sps_subpic_id[ i ]	(81)
	else
		SubpicIdVal[ i ] = i

It is a requirement of bitstream conformance that both of the following constraints apply:

–	For any two different values of i and j in the range of 0 to sps_num_subpics_minus1, inclusive, SubpicIdVal[ i ] shall not be equal to SubpicIdVal[ j ].

–	For each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, when the value of SubpicIdVal[ i ] of a current picture is not equal to the value of SubpicIdVal[ i ] of a reference picture, the active entries of the RPLs of the coded slices in the i-th subpicture of the current picture shall not include that reference picture.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).