| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0130-v3 |
| --- | --- |

| Title: | On chroma QP mapping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li, Xiang Li, Byeongdoo Choi, Stephan, Wenger, Shan Liu | Tel: Email: | {aurali, xlxiangli, bdchoi, swenger, shanl }@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

The contribution proposes following changes:

Aspect 1: bugfix for the range of sps_num_points_in_qp_table_minus1[ i ]

Aspect 2: introduce sps_chroma_qp_identical_to_luma_flag in SPS to have a shortcut when chroma quantization parameter are set equal to luma quantization parameter. It is asserted that this shortcut is the common case for 4:2:2 and 4:4:4, and substantive bits can be saved by skipping all the chroma QP mapping related syntax elements in SPS.

# Introduction

HEVC uses a fixed look-up table to convert the luma quantization parameter to chroma quantization parameter for 4:2:0 chroma format. For non 4:2:0, chroma quantization parameter are set equal to luma quantization parameter with clipping value set to 51.

In the VVC Draft 9, a more flexible luma-to-chroma QP mapping is used. Instead of having a fixed table, the luma-to-chroma QP mapping relationship is signalled in the SPS using a flexible piecewise linear model, with the only constraint on the linear model being that the slope of each piece cannot be negative (i.e., as luma QP increases, chroma QP must stay flat or increase, but cannot decrease). The QP mapping relationship can be signalled separately for Cb, Cr and joint Cb/Cr coding, or signalled once and used for all three types of residual coding.

Table 1 – chroma QP mapping related syntax elements in VVC Draft 9

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ChromaArrayType != 0 ) { |  |
| sps_joint_cbcr_enabled_flag | u(1) |
| sps_same_qp_table_for_chroma_flag | u(1) |
| numQpTables = sps_same_qp_table_for_chroma_flag ? 1 : ( sps_joint_cbcr_enabled_flag ? 3 : 2 ) |  |
| for( i = 0; i < numQpTables; i++ ) { |  |
| sps_qp_table_start_minus26[ i ] | se(v) |
| sps_num_points_in_qp_table_minus1[ i ] | ue(v) |
| for( j = 0; j <= sps_num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| sps_delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| sps_delta_qp_diff_val[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

sps_same_qp_table_for_chroma_flag equal to 1 specifies that only one chroma QP mapping table is signalled and this table applies to Cb and Cr residuals and additionally to joint Cb-Cr residuals when sps_joint_cbcr_enabled_flag is equal to 1. sps_same_qp_table_for_chroma_flag equal to 0 specifies that chroma QP mapping tables, two for Cb and Cr, and one additional for joint Cb-Cr when sps_joint_cbcr_enabled_flag is equal to 1, are signalled in the SPS. When not present, the value of sps_same_qp_table_for_chroma_flag is inferred to be equal to 1.

sps_qp_table_start_minus26[ i ] plus 26 specifies the starting luma and chroma QP used to describe the i-th chroma QP mapping table. The value of sps_qp_table_start_minus26[ i ] shall be in the range of −26 − QpBdOffset to 36 inclusive. When not present, the value of sps_qp_table_start_minus26[ i ] is inferred to be equal to 0.

sps_num_points_in_qp_table_minus1[ i ] plus 1 specifies the number of points used to describe the i-th chroma QP mapping table. The value of sps_num_points_in_qp_table_minus1[ i ] shall be in the range of 0 to 63 + QpBdOffset, inclusive. When not present, the value of sps_num_points_in_qp_table_minus1[ 0 ] is inferred to be equal to 0.

sps_delta_qp_in_val_minus1[ i ][ j ] specifies a delta value used to derive the input coordinate of the j-th pivot point of the i-th chroma QP mapping table. When not present, the value of sps_delta_qp_in_val_minus1[ 0 ][ j ] is inferred to be equal to 0.

sps_delta_qp_diff_val[ i ][ j ] specifies a delta value used to derive the output coordinate of the j-th pivot point of the i-th chroma QP mapping table.

The value of the luma and chroma quantization parameter range offset is QpBdOffset.

The i-th chroma QP mapping table ChromaQpTable[ i ] for i = 0..numQpTables − 1 is derived as follows:

qpInVal[ i ][ 0 ] = sps_qp_table_start_minus26[ i ] + 26
qpOutVal[ i ][ 0 ] = qpInVal[ i ][ 0 ]
for( j = 0; j  <=  sps_num_points_in_qp_table_minus1[ i ]; j++ ) {
	qpInVal[ i ][ j + 1 ] = qpInVal[ i ][ j ] + sps_delta_qp_in_val_minus1[ i ][ j ] + 1
	qpOutVal[ i ][ j + 1 ] = qpOutVal[ i ][ j ] +
		( sps_delta_qp_in_val_minus1[ i ][ j ] ^ sps_delta_qp_diff_val[ i ][ j ] )
}
ChromaQpTable[ i ][ qpInVal[ i ][ 0 ] ] = qpOutVal[ i ][ 0 ]
for( k = qpInVal[ i ][ 0 ] − 1; k  >=  −QpBdOffset; k − − )
	ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffset, 63, ChromaQpTable[ i ][ k + 1 ] − 1 )	(63)
for( j = 0; j  <=  sps_num_points_in_qp_table_minus1[ i ]; j++ ) {
	sh = ( sps_delta_qp_in_val_minus1[ i ][j ] + 1 )  >>  1
	for( k = qpInVal[ i ][ j ] + 1, m = 1; k  <=  qpInval[ i ][ j + 1 ]; k++, m++ )
		ChromaQpTable[ i ][ k ] = ChromaQpTable[ i ][ qpInVal[ i ][ j ] ] +
			( ( qpOutVal[ i ][j + 1] − qpOutVal[ i ][j ] ) * m + sh ) /   ( sps_delta_qp_in_val_minus1[ i ][ j ] + 1 )
}
for( k = qpInVal[ i ][ sps_num_points_in_qp_table_minus1[ i ] + 1 ] + 1; k  <=  63; k++ )
	ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffset, 63, ChromaQpTable[ i ][ k − 1 ] + 1 )

When sps_same_qp_table_for_chroma_flag is equal to 1, ChromaQpTable [ 1 ][ k ] and ChromaQpTable[ 2 ][ k ] are set equal to ChromaQpTable[ 0 ][ k ] for k in the range of −QpBdOffset to 63, inclusive.

It is a requirement of bitstream conformance that the values of qpInVal[ i ][ j ] and qpOutVal[ i ][ j ] shall be in the range of −QpBdOffset to 63, inclusive for i in the range of 0 to numQpTables − 1, inclusive, and j in the range of 0 to sps_num_points_in_qp_table_minus1[ i ] + 1, inclusive.

# Problem Statement

1) It is noticed when sps_num_points_in_qp_table_minus1[ i ] for i-th chroma QP mapping table has a value equal to 63+QpBdOffset, qpInVal[ i ][ j + 1 ] where j = sps_num_points_in_qp_table_minus1[ i ] has value exceeds the maximum QP value 63.

It is proposed to set the range of sps_num_points_in_qp_table_minus1[ i ] from 0 to 62 + QpBdOffset.

2) To simply have chroma quantization parameter set equal to luma quantization parameter for 4:2:2 and 4:4:4, there is no shortcut and all the chroma QP mapping related syntax need to be signalled. The worst-case range values for those syntaxs are specified below.

Table 2 – worst-case range value for chroma QP mapping related syntax elements in VVC Draft 9

| Syntax element | Range of value in worst case |
| --- | --- |
| sps_same_qp_table_for_chroma_flag | 0 to 1 |
| sps_qp_table_start_minus26[ i ] | -74 to 36 |
| sps_num_points_in_qp_table_minus1[ i ] | 0 to 111 |

It is proposed to have a shortcut for the case when chroma quantization parameter are set equal to luma quantization parameter.

# Proposed Spec Text

Changes compared to the latest VVC Draft 9 vA [1] are highlighted in yellow.

## Aspect 1: bugfix for the range of sps_num_points_in_qp_table_minus1[ i ]

sps_num_points_in_qp_table_minus1[ i ] plus 1 specifies the number of points used to describe the i-th chroma QP mapping table. The value of sps_num_points_in_qp_table_minus1[ i ] shall be in the range of 0 to 62 + QpBdOffset, inclusive. When not present, the value of sps_num_points_in_qp_table_minus1[ 0 ] is inferred to be equal to 0.

## Aspect 2: add a shortcut when chroma QP are set equal to luma QP

Table 3 – proposed shortcut when chroma QP are set equal to luma QP

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ChromaArrayType != 0 ) { |  |
| sps_joint_cbcr_enabled_flag | u(1) |
| sps_chroma_qp_identical_to_luma_flag | u(1) |
| if(sps_chroma_qp_identical_to_luma_flag != 1 ) { |  |
| sps_same_qp_table_for_chroma_flag | u(1) |
| numQpTables = sps_same_qp_table_for_chroma_flag ? 1 : ( sps_joint_cbcr_enabled_flag ? 3 : 2 ) |  |
| for( i = 0; i < numQpTables; i++ ) { |  |
| sps_qp_table_start_minus26[ i ] | se(v) |
| sps_num_points_in_qp_table_minus1[ i ] | ue(v) |
| for( j = 0; j <= sps_num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| sps_delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| sps_delta_qp_diff_val[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

sps_chroma_qp_identical_to_luma_flag equal to 1 specifies that chroma QP has the identical value with luma QP. sps_chroma_qp_identical_to_luma_flag equal to 0 specifies that chroma QP does not have the identical value with luma QP. When not present, the value of sps_chroma_qp_identical_to_luma_flag is inferred to be equal to 1.

sps_delta_qp_diff_val[ i ][ j ] specifies a delta value used to derive the output coordinate of the j-th pivot point of the i-th chroma QP mapping table. When sps_chroma_qp_identical_to_luma_flag equals to 1, the value of sps_delta_qp_diff_val[ i ][ j ] is inferred to be equal to 1.

# Analysis for aspect 2

Bit saving analysis is shown in below table.

Table 4 – Bit saving comparison for worst case QP configuration

| The number of bits used | VTM 9.0rc1 under worst case QP configuration* | Proposed under worst case QP configuration* |
| --- | --- | --- |
| sps_same_qp_table_for_chroma_flag | 1 bit | NA |
| sps_qp_table_start_minus26[ i ] | 13 bits | NA |
| sps_num_points_in_qp_table_minus1[ i ] | 13 bits | NA |
| sps_delta_qp_in_val_minus1[ i ][ j ] | 74 bits | NA |
| sps_delta_qp_diff_val[ i ][ j ] | 222 bits | NA |
| Overall | 323 bits | 1 bit |

The simulation results are also shown in Table 5~7. It is implemented on VTM 9.0rc1 and tested using non 420 test condition [2]. Only 1 frame is tested under AI configuration with worst cast QP configured.

Table 5 – Simulation results for computer generated 444 sequences

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | YUV 4:4:4 |  |  |  |  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM 1080p | -0.05% | -0.05% | -0.05% | 100% | 100% | -0.04% | -0.04% | -0.04% | 100% | 99% |
| TGM 720p | -0.13% | -0.13% | -0.13% | 99% | 99% | -0.09% | -0.09% | -0.09% | 100% | 98% |
| Animation | -0.07% | -0.07% | -0.07% | 100% | 100% | -0.04% | -0.04% | -0.04% | 99% | 102% |
| Mixed content | -0.04% | -0.04% | -0.04% | 100% | 102% | -0.02% | -0.02% | -0.02% | 101% | 102% |
| Overall | -0.08% | -0.07% | -0.07% | 100% | 100% | -0.05% | -0.05% | -0.05% | 100% | 100% |

Table 6 – Simulation results for sensor generated 444 sequences

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | -0.02% | -0.02% | -0.02% | 100% | 101% |
| Kimono | -0.10% | -0.05% | -0.10% | 99% | 107% |
| EBULupoCandlelight | -0.12% | -0.13% | -0.13% | 100% | 98% |
| EBURainFruits | -0.05% | -0.05% | -0.05% | 100% | 98% |
| VenueVu | -0.05% | -0.05% | -0.05% | 102% | 102% |
| BirdsInCage | -0.12% | -0.07% | -0.12% | 100% | 102% |
| CrowdRun | -0.02% | -0.01% | -0.02% | 100% | 100% |
| Overall | -0.07% | -0.05% | -0.07% | 100% | 101% |

Table 7 – Simulation results for sensor generated 422 sequences

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | -0.03% | -0.02% | -0.03% | 100% | 99% |
| Kimono | -0.12% | -0.08% | -0.11% | 99% | 100% |
| EBUHorse | -0.02% | -0.02% | -0.02% | 98% | 100% |
| EBUGraphics | -0.02% | -0.02% | -0.02% | 99% | 99% |
| EBUWaterRocksClose | -0.02% | -0.02% | -0.03% | 101% | 105% |
| EBUKidsSoccer | -0.02% | -0.02% | -0.02% | 101% | 101% |
| Seeking | -0.03% | -0.02% | -0.03% | 99% | 100% |
| Overall | -0.04% | -0.03% | -0.04% | 100% | 101% |

Table 8 – Bit saving comparison for CTC QP configuration

| The number of bits used | VTM 9.0rc1 under CTC QP configuration** | Proposed under CTC QP configuration** |
| --- | --- | --- |
| sps_same_qp_table_for_chroma_flag | 1 bit | NA |
| sps_qp_table_start_minus26[ i ] | 1 bit | NA |
| sps_num_points_in_qp_table_minus1[ i ] | 1 bit | NA |
| sps_delta_qp_in_val_minus1[ i ][ j ] | 1 bit | NA |
| sps_delta_qp_diff_val[ i ][ j ] | 3 bits | NA |
| Overall | 7 bits | 1 bit |

* The worst case QP configuration is sending all the QP entries in the chroma QP mapping table.

** CTC QP configuration is only having qpInVal and qpOutVal as value 26 and 26, respectively.

# Conclusion

Several aspects are proposed regarding chroma QP mapping. It is recommended to adopt this proposal into next version of VVC.

# Reference

- B. Bross, J. Chen, S. Liu, Y.K. Wang, “Versatile Video Coding (Draft 9)” Document of Joint Video Experts Team (JVET), JVET-R2001-vA, Teleconference, 2020.

- Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu, “JVET common test conditions and software reference configurations for non-4:2:0 colour formats”, JVET-R2013-v1, Teleconference, 2020

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).