| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0136 |
| --- | --- |

| Title: | AHG9: On signaling of PDPC enabling/disabling flag in SPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Email: | xiaoyuxiu@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to introduce one control flag in sequence parameter set (SPS) to adaptively enable/disable position dependent intra prediction combination (PDPC). When the flag is off, all the decoding process of the PDPC are bypassed. Additionally, another constraint flag no_pdpc_constraint_flag is added to general constraint syntax table.

Proposal

For the intra prediction in VVC draft 9 [1], position dependent intra prediction combination (PDPC) is forcibly applied when one current coding unit (CU) is predicted by Planar, DC, horizontal, vertical, bottom-left angular mode and its adjacent angular modes, and top-right angular mode and its adjacent angular modes.

Compared to camera-captured video content which usually shows smooth transitions across object boundaries, screen content contains abundant sharp edges. Since PDPC is essentially a lowpass filtering operation, for screen content coding the PDPC operation may undesirably blur the important high-frequency information of the reconstructed samples, therefore reduce the intra prediction accuracy and efficiency.

In this contribution, it is proposed to introduce one control flag in sequence parameter set (SPS) to control the enabling/disabling of the PDPC tool at sequence level. Furthermore, one additional constraint flag no_pdpc_constraint_flag is introduced into general constraint information table for the purpose of defining profile/sub-profiles.

Proposed spec changes

- 7.3.2.2 General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| ….. |  |
| no_pdpc_constraint_flag | u(1) |
| no_mrl_constraint_flag | u(1) |
| no_isp_constraint_flag | u(1) |
| no_mip_constraint_flag | u(1) |

- 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| sps_pdpc_enabled_flag | u(1) |
| sps_isp_enabled_flag | u(1) |
| sps_mrl_enabled_flag | u(1) |
| sps_mip_enabled_flag | u(1) |
| …… |  |

- 7.4.3.3 Sequence parameter set RBSP semantics

sps_pdpc_enabled_flag equal to 1 specifies that position dependent intra prediction combination is enabled. sps_pdpc_enabled_flag equal to 0 specifies that position dependent intra prediction combination is disabled.

- 7.4.1.1 General profile, tier, and level semantics

no_pdpc_constraint_flag equal to 1 specifies that sps_pdpc_enabled_flag shall be equal to 0.  no_pdpc_constraint_flag equal to 0 does not impose such a constraint.

- 8.4.2.5 General intra sample prediction

When all of the following conditions are true, the position-dependent prediction sample filtering process specified in clause 8.4.5.2.14 is invoked with the intra prediction mode predModeIntra, the transform block width nTbW, the transform block height nTbH, the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1, the reference sample width refW, the reference sample height refH, and the reference samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1 as inputs, and the output is the modified predicted sample array predSamples:

- sps_pdpc_enabled_flag is equal to 1

- nTbW is greater than or equal to 4 and nTbH is greater than or equal to 4

- refIdx is equal to 0 or cIdx is not equal to 0

- BdpcmFlag[ xTbCmp ][ yTbCmp ][ cIdx ] is equal to 0

- One of the following conditions is true:

- predModeIntra is equal to INTRA_PLANAR

- predModeIntra is equal to INTRA_DC

- predModeIntra is less than or equal to INTRA_ANGULAR18

- predModeIntra is greater than or equal to INTRA_ANGULAR50 and less than INTRA_LT_CCLM

Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 9)”, JVET-R2001, April 2020, Teleconference

- VTM-8.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-8.0.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).