| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0161 |
| --- | --- |

| Title: | Report of side activity on GCI syntax elements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce | Email: | jill.boyce@intel.com |
| Source: | Side activity |  |  |

_____________________________

# Abstract

Side activity was conducted via email to make a recommendation on inclusion of Global Constraint Indicator (GCI) syntax elements, for items 4-22 in the AHG report JVET-S0137.

The recommendations are summarized as follows:

- Add syntax elements for the following:

  - no_weighted_pred_constraint_flag for both sps_weighted_pred_flag and sps_weighted_bipred_flag (S0058)

  - no_virtual_boundaries_constraint_flag to constrain sps_virtual_boundaries_enabled_flag (S0050 #2, S0105, S0131)

  - no_explicit_scaling_list_constraint_flag to constrain sps_explicit_scaling_list_enabled_flag shall be equal to 0 (S0050 #2, S0058 S0066, S0067, S0105)

  - no_mtt_constraint_flag to constrain SPS MTT SEs sps_max_mtt_hierarchy_depth_intra_slice_luma, sps_max_mtt_hierarchy_depth_inter_slice and sps_max_mtt_hierarchy_depth_intra_slice_chroma (S0058)

  - no_luma_transform_size_64_constraint_flag to constrain sps_max_luma_transform_size_64_flag (S0058, S0066)

  - max_log2_ctu_size_constraint_idc (u(2)) to constrain sps_log2_ctu_size_minus5 (S0066)

  - no_rectangular_slice_constraint_flag to constrain pps_rect_slice_flag (S0113)

  - one_slice_per_subpicture_constraint_flag to constrain pps_single_slice_per_subpic_flag (S0113)

- Additional recommendations

  - Remove any constraints that existing constraint flags impose on other constraint flags

  - Rename max_bitdepth_constraint_idc to max_bitdepth_minus8_constraint_idc

  - Remove no_aps_constraint_flag

The attached document implements these changes in marked specification text, based upon JVET-R2001-vA. The attached text does not implement other GCI adoptions from the AHG, such as reordering of GCI syntax elements. No attempt was made to intelligently order the additional syntax elements in the attached text, because it is assumed that will be done as part of activity in reordering of the syntax elements.