| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0168-v1 |
| --- | --- |

| Title: | AHG9: On GDR subpictures for sub-bitstream extraction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, ChingYeh Chen, Chih-Wei Hsu, Olena Chubach, Shih-Ta Hsiang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, chingyeh.chen, cw.hsu, olena.chubach, shih-ta.hsiang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution discusses about the GDR subpictures for sub-bitstream extraction with three options depending on whether the group regards it as an issue or not.

Introduction

In the VVC Draft 9 [1], the GDR picture coding tool enabling is specified in the SPS and further signalling of its presence may be either in the SPS or in the PH. In the first case, when sps_res_change_in_clvs_allowed_flag is equal to 1, the value of sps_subpic_info_present_flag shall be equal to 0. In this case, there are no subpictures present in the bitstream and virtual boundaries may be specified in the PH. In the second case, when sps_subpic_info_present_flag is equal to 1, the value of sps_res_change_in_clvs_allowed_flag shall be equal to 0 and the value of ph_virtual_boundaries_present_flag shall be equal to 0. In this case, the picture spatial resolution is not changed within any CLVS referring to the SPS and virtual boundaries are specified in the SPS only.

Proposals

In this contribution the second case is discussed, in which subpictures are used with the virtual boundaries specified in the SPS. Generally there are following scenarios with a subpicture.

- A subpicture has virtual boundaries in it

- A subpicture does not have any virtual boundaries in it

- Subpicture boundaries coincide with the picture virtual boundaries

Therefore there are four cases as illustrated below.

Case 1 = A, Case 2 = B, Case 3 = A&C, and Case 4 = B&C.

These four cases may happen after the GDR subpicture extraction and each of them may serve as a CLVS. Unfortunately, there may be no information in the bitstream on whether the extracted GDR picture has a “clean area” in it, no matter what the virtual boundaries are specified and used. Thus there may be no “clean area” in an extracted GDR subpicture. This may not be a desired result or the design intent for the VVC.

The following options may be considered, depending on whether the group regards it as an issue or not.

Option 1

It is the design intent. Do nothing or do something as needed, e.g. modify the SPS virtual boundary information in terms of the extracted subpicture structure, including the size and position. Note that there may be no “clean area” in the extracted GDR subpicture or the following recovering subpictures. Therefore, at least the SPS needs to be modified after the extraction process.

Option 2

It may be an issue in terms of the design intent, meaning that the GDR picture should have an intra-coded area in it. In this case, it is suggested to do something as a guideline for sub-bitstream extraction. Note that the SPS and the PH need to be modified after the extraction process.

Add, as a general constraint, a requirement of bitstream conformance that the extracted first subpicture for the sub-bistream shall contain an intra-coded area and the following processing are required in sub-bitstream extraction.

- Locate the first subpicture with an intra-coded area in the GDR picture or the following recovering pictures. This located subpicture becomes the new GDR subpicture.

- Change the value of nal_unit_type of the new GDR subpicture to GDR_NUT if the new GDR subpicture is not in the current GDR picture.

- Rewrite the virtual boundary information in the SPS in terms of the new GDR subpicture.

- Adjust the value of ph_recovery_poc_cnt in the PH if the new GDR subpicture is not the current GDR picture.

Option 3 (on top of Option 2)

To facilitate the subpicture extraction process, from system point of view, it is desirable that encoding process is able to mark/label for each one of subpictures that it consists of a starting point of an intra-coded area in the GDR picture and the following recovering pictures until the recovery point picture. The suggested syntax and semantics are as follows. Note that the SPS and the PH need to be modified after the extraction process.

| Seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sublayers_minus1 | u(3) |
| sps_reserved_zero_4bits | u(4) |
| sps_ptl_dpb_hrd_params_present_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| profile_tier_level( 1, sps_max_sublayers_minus1 ) |  |
| sps_gdr_enabled_flag | u(1) |
| if( sps_gdr_enabled_flag ) |  |
| sps_gdr_subpic_intra_starting_present_flag | u(1) |
| … |  |

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| ph_gdr_or_irap_pic_flag | u(1) |
| ph_non_ref_pic_flag | u(1) |
| if( ph_gdr_or_irap_pic_flag ) |  |
| ph_gdr_pic_flag | u(1) |
| ph_inter_slice_allowed_flag | u(1) |
| if( ph_inter_slice_allowed_flag ) |  |
| ph_intra_slice_allowed_flag | u(1) |
| ph_pic_parameter_set_id | ue(v) |
| ph_pic_order_cnt_lsb | u(v) |
| if( ph_gdr_or_irap_pic_flag ) |  |
| ph_no_output_of_prior_pics_flag | u(1) |
| if( ph_gdr_pic_flag ) |  |
| ph_recovery_poc_cnt | ue(v) |
| if( ph_recovery_poc_cnt && sps_gdr_subpic_intra_starting_present_flag ) |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| ph_subpic_intra_starting_poc_cnt[ i ] | ue(v) |
| … |  |

sps_gdr_subpic_intra_starting_present_flag specifies that the syntax element ph_subpic_intra_starting_poc_cnt[ i ] is present in the PH. sps_gdr_subpic_intra_starting_present_flag equal to 0 specifies that the syntax element ph_subpic_intra_starting_poc_cnt[ i ] is not present in the PH. When not present, the value of sps_gdr_subpic_intra_starting_present_flag shall be inferred to be equal to 0.

ph_subpic_intra_starting_poc_cnt[ i ] specifies the intra-coded starting point of the i-th decoded subpicture in output order.

When the current subpicture is the i-th GDR subpicture, the variable intraStarginPointPocVal is derived as follows:

intraStarringPointPocVal = PicOrderCntVal + ph_subpic_intra_starting_poc_cnt[ i ]

If the current subpicture is a GDR subpicture, and there is a subpicture subpicA that follows the current GDR subpicture in decoding order in the CLVS that has PicOrderCntVal equal to intraStartingPointPocVal, the subpicture subpicA is referred to as the intra-coded starting point subpicture. The value of ph_subpic_intra_starting_poc_cnt[ i ] shall be in the range of 0 to ph_recovery_poc_cnt – 1, inclusive. The value of ph_recovery_poc_cnt for the extracted i-th new GDR subpicture is equal to ph_recovery_poc_cnt – intraStarringPointPocVal.

As noted, Option 2 and Option 3 need the SPS and PH rewrite. However this case just needs the PH rewrite for the first extracted subpicture in the sub-bitstream.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 9),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-R2001-vA, 18th Meeting: by teleconference, 15–24 April 2020.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).