| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0182-v1 |
| --- | --- |

| Title: | AHG9: On Reference Picture List Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Jonatan Samuelsson Andrew Segall Philip Cowan | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

We propose a modification to the reference picture list information signalling for cleanup.

The condition for signalling ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] is proposed to be modified such that it is signalled only when num_ref_entries[ listIdx ][ rplsIdx ] is greater than 0 in addition to other conditions.

# Introduction

VVC Draft text [1] includes reference picture management based on reference picture list signaling.

We propose a modification to the reference picture list information signaling for cleanup .

The condition for signalling ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] is proposed to be modified such that it is signalled only when num_ref_entries[ listIdx ][ rplsIdx ] is greater than 0 in addition to other conditions.

# Proposal

The condition for signalling ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] is proposed to be modified such that it is signalled only when num_ref_entries[ listIdx ][ rplsIdx ] is greater than 0 in addition to other conditions. It is asserted that when num_ref_entries[ listIdx ][ rplsIdx ] is equal to 0, there are no long-term reference picture entries in the particular reference picture list and the signalling of ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] is meaningless, confusing/ misleading and wastes a bit.

The proposed modification is shown highlighted compared to VVC Draft Text.

### 7.3.9 Reference picture list structure syntax

| ref_pic_list_struct( listIdx, rplsIdx ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| if( sps_long_term_ref_pics_flag && (num_ref_entries[ listIdx ][ rplsIdx ] > 0) && rplsIdx < sps_num_ref_pic_lists[ listIdx ] ) |  |
| ltrp_in_header_flag[ listIdx ][ rplsIdx ] | u(1) |
| for( i = 0, j = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if( sps_inter_layer_ref_pics_present_flag ) |  |
| inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( !inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| if( sps_long_term_ref_pics_flag ) |  |
| st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| if( AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] > 0 ) |  |
| strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| } else if( !ltrp_in_header_flag[ listIdx ][ rplsIdx ] ) |  |
| rpls_poc_lsb_lt[ listIdx ][ rplsIdx ][ j++ ] | u(v) |
| } else |  |
| ilrp_idx[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| } |  |
| } |  |

ltrp_in_header_flag[ listIdx ][ rplsIdx ] equal to 0 specifies that the POC LSBs of the LTRP entries indicated in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure are present in the same syntax structure. ltrp_in_header_flag[ listIdx ][ rplsIdx ] equal to 1 specifies that the POC LSBs of the LTRP entries indicated in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure are not present in the same syntax structure. When sps_long_term_ref_pics_flag is equal to 1 and rplsIdx is equal to sps_num_ref_pic_lists[ listIdx ], the value of ltrp_in_header_flag[ listIdx ][ rplsIdx ] is inferred to be equal to 1.

# Conclusion

This document proposes a modification to reference picture list signalling in VVC. It is proposed to adopt the proposals in the next version of VVC draft text.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-S0152, AHG2: Editorial Input of a text integration for the May 2020 HLS AHG meeting outcome, May 2020