| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0184-v1 |
| --- | --- |

| Title: | AHG9: On Virtual Boundary Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Jonatan Samuelsson Andrew Segall Philip Cowan | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Modification is proposed related to a virtual boundary constraint.

It is proposed to specify a conformance constraint regarding virtual boundary signalling when subpictures are present, directly on related SPS syntax elements.

# Introduction

VVC Draft text [1] includes signalling of virtual boundaries. Virtual boundaries can be signalled either in SPS or picture header (or not signalled).

Modification is proposed related to a virtual boundary constraint.

It is proposed to specify a conformance constraint regarding virtual boundary signalling when subpictures are present, directly on related SPS syntax elements.

# Proposal

Currently in VVC draft text 9, when sps_virtual_boundaries_enabled_flag  is equal to 1, virtual boundary information can be signalled in SPS or in picture header. A constraint in VVC draft text specifies that: It is a requirement of bitstream conformance that, when sps_subpic_info_present_flag is equal to 1, the value of ph_virtual_boundaries_present_flag shall be equal to 0.

It is asserted that this constraint regarding subpictures and signalling of virtual boundaries in picture header should not be specified on the ph_virtual_boundaries_present_flag. This is because the above constraint allows signalling sps_virtual_boundaries_enabled_flag equal to 1, and sps_virtual_boundaries_present_flag equal to 0 and signalling ph_virtual_boundaries_present_flag equal to 0 in each picture header. Instead the restriction should be imposed directly at the SPS level.

Thus it is proposed to specify a conformance constraint regarding virtual boundary information signalling when subpictures are present, directly on related SPS syntax elements.

The proposed text for this is shown highlighted below compared to JVET-S0152:

…

sps_subpic_info_present_flag equal to 1 specifies that subpicture information is present for the CLVS and there may be one or more than one subpicture in each picture of the CLVS. sps_subpic_info_present_flag equal to 0 specifies that subpicture information is not present for the CLVS and there is only one subpicture in each picture of the CLVS.

When sps_res_change_in_clvs_allowed_flag is equal to 1, the value of sps_subpic_info_present_flag shall be equal to 0.

…

sps_virtual_boundaries_enabled_flag equal to 1 specifies that disabling in-loop filtering across virtual boundaries is enabled and may be applied in the coded pictures in the CLVS. sps_virtual_boundaries_enabled_flag equal to 0 specifies that disabling in-loop filtering across virtual boundaries is disabled and not applied in the coded pictures in the CLVS. In-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations.

sps_virtual_boundaries_present_flag equal to 1 specifies that information of virtual boundaries is signalled in the SPS. sps_virtual_boundaries_present_flag equal to 0 specifies that information of virtual boundaries is not signalled in the SPS. When there is one or more than one virtual boundaries signalled in the SPS, the in-loop filtering operations are disabled across the virtual boundaries in pictures referring to the SPS. In-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of sps_virtual_boundaries_present_flag is inferred to be equal to 0.

It is a requirement of bitstream conformance that when the value of sps_res_change_in_clvs_allowed_flag is equal to 1, the value of sps_virtual_boundaries_present_flag shall be equal to 0.

It is a requirement of bitstream conformance that when sps_subpic_info_present_flag is equal to 1, and sps_virtual_boundaries_enabled_flag is equal to 1, the value of sps_virtual_boundaries_present_flag shall be equal to 1.

…

ph_virtual_boundaries_present_flag equal to 1 specifies that information of virtual boundaries is signalled in the PH. ph_virtual_boundaries_present_flag equal to 0 specifies that information of virtual boundaries is not signalled in the PH. When there is one or more than one virtual boundaries signalled in the PH, the in-loop filtering operations are disabled across the virtual boundaries in the picture. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of ph_virtual_boundaries_present_flag is inferred to be equal to 0.

It is a requirement of bitstream conformance that, when sps_subpic_info_present_flag is equal to 1, the value of ph_virtual_boundaries_present_flag shall be equal to 0.

# Conclusion

This document proposes modification to a constraint to related to virtual boundary. It is proposed to adopt the proposal in the next version of VVC Draft text.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-S0152, AHG2: Editorial Input of a text integration for the May 2020 HLS AHG meeting outcome, May 2020