| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0192 |
| --- | --- |

| Title: | AHG9: On reference picture list with generating unavailable reference picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes the following modifications to reference picture list construction and unavailable picture generation:

- Modify the decoding process for generating unavailable reference pictures to correctly set the values of nuh_layer_id and PicOrderCntVal of the unavailable interlayer reference pictures and marked those pictures as “used for long-term reference”.

- Modify the following sentence to clarify the intention,

“For each LTRP entry in RefPicList[ 0 ] or RefPicList[ 1 ], when the picture is an STRP with the same nuh_layer_id as the current picture, the picture is marked as "used for long-term reference".”

as

“For each LTRP entry in RefPicList[ 0 ] or RefPicList[ 1 ], when the picture is currently marked as "used for short-term reference" with the same nuh_layer_id as the current picture, the picture is marked as "used for long-term reference".”

- Add the following note to the decoding process for reference picture list construction, to inform that the bitstream conformance check of the reference picture list should be performed after generating unavailable pictures.

“NOTE 5 – When the bitstream conformance is checked with the following constraints, the conformance check should be done after invoking decoding process for generating unavailable reference pictures specified in 8.3.4.”

# Proposal-1

In the decoding process for reference picture lists construction, specified in 8.3.2, missing reference pictures in DPB are set equal to “no reference picture”. The unavailable pictures equal to “no reference picture” are generated through the decoding process for generating unavailable reference pictures, specified in 8.3.4, for conformance check purposes. An issue is that not only unavailable reference pictures in the same layer but also unavailable interlayer reference pictures in the referene layer are set equal to “no reference picture” through the 8.3.2, as below:

if( !inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
	if( st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
		RefPicPocList[ i ][ j ] = pocBase + DeltaPocValSt[ i ][ RplsIdx[ i ] ][ j ]
		if( there is a reference picture picA in the DPB with the same nuh_layer_id as the current picture
					and PicOrderCntVal equal to RefPicPocList[ i ][ j ] )
			RefPicList[ i ][ j ] = picA
		else
			RefPicList[ i ][ j ] = "no reference picture"	
		pocBase = RefPicPocList[ i ][ j ]
	} else {
		if( !delta_poc_msb_cycle_present_flag[ i ][ k ] ) {
			if( there is a reference picA in the DPB with the same nuh_layer_id as the current picture and								PicOrderCntVal & ( MaxPicOrderCntLsb − 1 ) equal to PocLsbLt[ i ][ k ] )
				RefPicList[ i ][ j ] = picA
			else
				RefPicList[ i ][ j ] = "no reference picture"
			RefPicLtPocList[ i ][ j ] = PocLsbLt[ i ][ k ]
		} else {
			if( there is a reference picA in the DPB with the same nuh_layer_id as the current picture and
							PicOrderCntVal equal to 	FullPocLt[ i ][ k ] )
				RefPicList[ i ][ j ] = picA
			else
				RefPicList[ i ][ j ] = "no reference picture"
			RefPicLtPocList[ i ][ j ] = FullPocLt[ i ][ k ]
		}
		k++
	}
} else {
	layerIdx = DirectRefLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ ilrp_idx[ i ][ RplsIdx ][ j ] ]
	refPicLayerId = vps_layer_id[ layerIdx ]
	if( there is a reference picture picA in the DPB with nuh_layer_id equal to refPicLayerId and
				the same PicOrderCntVal as the current picture )
		RefPicList[ i ][ j ] = picA
	else
		RefPicList[ i ][ j ] = "no reference picture"
}

However, in 8.3.4 decoding process for generating unavailable reference pictures, all unavailable pictures equal to “no reference picture” are handled as the reference pictures in the same layer, so that the nuh_layer_id and PicOrderCntVal values of the unavailable interlayer reference pictures are set to incorrrect values. Also, the unavailable interlayer reference pictures are not correctly marked as "used for long-term reference". These wrong values result in errors in decoding process of the following pictures and bitstream conformance check specified in 8.3.2. Therefore, it is asserted that the unavailable interlayer reference pictures equal to “no reference picture” should be correctly generated as follows:

Proposed Spec Text:

#### General decoding process for generating unavailable reference pictures

This process is invoked once per coded picture when the current picture is an IDR picture with sps_idr_rpl_present_flag equal to 1 or pps_rpl_info_in_ph_flag equal to 1, a CRA picture with NoOutputBeforeRecoveryFlag equal to 1, or a GDR picture with NoOutputBeforeRecoveryFlag equal to 1.

When this process is invoked, the following applies:

- For each 	RefPicList[ i ][ j ], with i in the range of 0 to 1, inclusive, and j in the range of 0 to num_ref_entries[ i ][ RplsIdx[ i ] ] − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in subclause 8.3.4.2 and the following applies:

- If inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 0, the value of nuh_layer_id for the generated picture is set equal to nuh_layer_id of the current picture.

- Otherwise (inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 1), value of nuh_layer_id for the generated picture is set equal to vps_layer_id[ DirectRefLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ ilrp_idx[ i ][ RplsIdx ][ j ] ]]

- If st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 1 and inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 0, the value of PicOrderCntVal for the generated picture is set equal to RefPicPocList[ i ][ j ] and the generated picture is marked as "used for short-term reference".

- Otherwise, when If st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 0 and inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 0, the value of PicOrderCntVal for the generated picture is set equal to RefPicLtPocList[ i ][ j ], the value of ph_pic_order_cnt_lsb for the generated picture is inferred to be equal to ( RefPicLtPocList[ i ][ j ] & ( MaxPicOrderCntLsb – 1 ) ), and the generated picture is marked as "used for long-term reference".

- Otherwise, if inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 1, the value of PicOrderCntVal for the generated picture is set equal to PicOrderCntVal of the current picture, the value of ph_pic_order_cnt_lsb for the generated picture is inferred to be equal to ph_pic_order_cnt_lsb of the current picture and the generated picture is marked as "used for long-term reference".

- The value of PictureOutputFlag for the generated reference picture is set equal to 0.

- RefPicList[ i ][ j ] is set to be the generated reference picture.

- The value of TemporalId for the generated picture is set equal to TemporalId of the current picture.

- The value of ph_non_ref_pic_flag for the generated picture is set equal to 0.

- The value of ph_pic_parameter_set_id for the generated picture is set equal to ph_pic_parameter_set_id of the current picture.

# Proposal-2

In decoding process for reference picture marking specified in 8.3.3, to clarify the marking process, the following sentence is modified as one of the proposed sentences:

- For each LTRP entry in RefPicList[ 0 ] or RefPicList[ 1 ], when the picture is an STRP with the same nuh_layer_id as the current picture, the picture is marked as "used for long-term reference".

Proposed Spec Text

Option-1

- For each LTRP entry in RefPicList[ 0 ] or RefPicList[ 1 ], when the picture is an STRP currently marked as "used for short-term reference" with the same nuh_layer_id as the current picture, the picture is marked as "used for long-term reference".

Option-2

- For each LTRP entry in RefPicList[ 0 ] or RefPicList[ 1 ], when the picture is is an STRP LTRP with the same nuh_layer_id as the current picture, the picture is marked as "used for long-term reference".

The meaning of the current sentence is somewhat confused, because it says that an STRP is marked as "used for long-term reference".

If the original intention of the sentence is to mark the LTRP picture of the current picture in RefPicList[ 0 ] or RefPicList[ 1 ] as "used for long-term reference", when the LTRP is currently marked as "used for short-term reference", the option-1 should be taken. Otherwise, the option-2 is recommended.

# Proposal-3

In HEVC, the reference picture lists are constructed after generating unavailable reference pictures in RPS. In VVC (current draft), the reference picture lists are constructed before generating unavailable reference pictures in RPS. If the bitstream conformance check is performed before generating unavailable reference pictures specified in 8.3.2, the bitstream cannot obey the constraints specified in 8.3.2. To clarify the order, it is proposed to add the following note:

Proposed Spec Text

### Decoding process for reference picture lists construction

…

It is a requirement of bitstream conformance that the following constraints apply:

NOTE 5 – When the bitstream conformance is checked with the following constraints, the conformance check should be done after invoking decoding process for generating unavailable reference pictures specified in 8.3.4.

- For each i equal to 0 or 1, num_ref_entries[ i ][ RplsIdx[ i ] ] shall not be less than NumRefIdxActive[ i ].

- The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall be present in the DPB and shall have TemporalId less than or equal to that of the current picture.

- The picture referred to by each entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall not be the current picture and shall have ph_non_ref_pic_flag equal to 0.

- An STRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of a picture and an LTRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of the same slice or a different slice of the same picture shall not refer to the same picture.

- There shall be no LTRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] for which the difference between the PicOrderCntVal of the current picture and the PicOrderCntVal of the picture referred to by the entry is greater than or equal to 224.

- Let setOfRefPics be the set of unique pictures referred to by all entries in RefPicList[ 0 ] that have the same nuh_layer_id as the current picture and all entries in RefPicList[ 1 ] that have the same nuh_layer_id as the current picture. The number of pictures in setOfRefPics shall be less than or equal to MaxDpbSize − 1, inclusive, where MaxDpbSize is as specified in clause A.4.2, and setOfRefPics shall be the same for all slices of a picture.

- When the current slice has nal_unit_type equal to STSA_NUT, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that has TemporalId equal to that of the current picture and nuh_layer_id equal to that of the current picture.

- When the current picture is a picture that follows, in decoding order, an STSA picture that has TemporalId equal to that of the current picture and nuh_layer_id equal to that of the current picture, there shall be no picture that precedes the STSA picture in decoding order, has TemporalId equal to that of the current picture, and has nuh_layer_id equal to that of the current picture included as an active entry in RefPicList[ 0 ] or RefPicList[ 1 ].

- When the current subpicture, with TemporalId equal to a particular value tId, nuh_layer_id equal to a particular value layerId, and subpicture index equal to a particular value subpicIdx, is a subpicture that follows, in decoding order, an STSA subpicture with TemporalId equal to tId, nuh_layer_id equal to layerId, and subpicture index equal to subpicIdx, there shall be no picture with TemporalId equal to tId and nuh_layer_id equal to layerId that precedes the picture containing the STSA subpicture in decoding order included as an active entry in RefPicList[ 0 ] or RefPicList[ 1 ].

- When the current picture, with nuh_layer_id equal to a particular value layerId, is an IRAP picture, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes, in output order or decoding order, any preceding IRAP picture with nuh_layer_id equal to layerId in decoding order (when present).

- When the current subpicture, with nuh_layer_id equal to a particular value layerId and subpicture index equal to a particular value subpicIdx, is an IRAP subpicture, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes, in output order or decoding order, any preceding picture, in decoding order (when present), containing an IRAP subpicture with nuh_layer_id equal to layerId and subpicture index equal to subpicIdx.

- When the current picture is not a RASL picture associated with a CRA picture with NoOutputBeforeRecoveryFlag equal to 1, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that was generated by the decoding process for generating unavailable reference pictures for the CRA picture associated with the current picture.

- When the current subpicture is not a RASL subpicture associated with a CRA subpicture in a CRA picture with NoOutputBeforeRecoveryFlag equal to 1, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that was generated by the decoding process for generating unavailable reference pictures for the CRA picture containing the CRA subpicture associated with the current subpicture.

- When the current picture, with nuh_layer_id equal to a particular value layerId, is not any of the following, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that was generated by the decoding process for generating unavailable reference pictures for the IRAP or GDR picture associated with the current picture:

  - An IDR picture with sps_idr_rpl_present_flag equal to 1 or pps_rpl_info_in_ph_flag equal to 1

  - A CRA picture with NoOutputBeforeRecoveryFlag equal to 1

  - A picture, associated with a CRA picture with NoOutputBeforeRecoveryFlag equal to 1, that precedes, in decoding order, the leading pictures associated with the same CRA picture

  - A leading picture associated with a CRA picture with NoOutputBeforeRecoveryFlag equal to 1

  - A GDR picture with NoOutputBeforeRecoveryFlag equal to 1

  - A recovering picture of a GDR picture with NoOutputBeforeRecoveryFlag equal to 1 and nuh_layer_id equal to layerId

- When the current subpicture, with nuh_layer_id equal to a particular value layerId and subpicture index equal to a particular value subpicIdx, is not any of the following, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that was generated by the decoding process for generating unavailable reference pictures for the IRAP or GDR picture containing the IRAP or GDR subpicture associated with the current subpicture:

  - An IDR subpicture in an IDR picture with sps_idr_rpl_present_flag equal to 1 or pps_rpl_info_in_ph_flag equal to 1

  - A CRA subpicture in a CRA picture with NoOutputBeforeRecoveryFlag equal to 1

  - A subpicture, associated with a CRA subpicture in a CRA picture with NoOutputBeforeRecoveryFlag equal to 1, that precedes, in decoding order, the leading pictures associated with the same CRA picture

  - A leading subpicture associated with a CRA subpicture in a CRA picture with NoOutputBeforeRecoveryFlag equal to 1

  - A GDR subpicture in a GDR picture with NoOutputBeforeRecoveryFlag equal to 1

  - A subpicure in a recovering picture of a GDR picture with NoOutputBeforeRecoveryFlag equal to 1 and nuh_layer_id equal to layerId

- When the current picture follows an IRAP picture having the same value of nuh_layer_id in both decoding order and output order, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes that IRAP picture in output order or decoding order.

- When the current subpicture follows an IRAP subpicture having the same value of nuh_layer_id and the same value of subpicture index in both decoding and output order, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the picture containing that IRAP subpicture in output order or decoding order.

- When the current picture follows an IRAP picture having the same value of nuh_layer_id and the leading pictures, if any, associated with that IRAP picture in both decoding order and output order, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes that IRAP picture in output order or decoding order.

- When the current subpicture follows an IRAP subpicture having the same value of nuh_layer_id and the same value of subpicture index and the leading subpictures, if any, associated with that IRAP subpicture in both decoding and output order, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the picture containing that IRAP subpicture in output order or decoding order.

- When the current picture is a RADL picture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is any of the following:

  - A RASL picture with pps_mixed_nalu_types_in_pic_flag is equal to 0

NOTE 5 – This means that an active entry of the RPLs of a RADL picture can refer to a RASL picture with pps_mixed_nalu_types_in_pic_flag equal to 1. However, when decoding starts from the associated CRA picture, such a RADL picture can still be correctly decoded, because the RADL subpicture(s) in that referenced RASL picture would be correctly decoded, as the RADL picture would only refer to the RADL subpictures in the referenced RASL picture, as imposed by the next constraint that disallows RADL subpictures referring to a RASL subpicture.

  - A picture that precedes the associated IRAP picture in decoding order

- When the current subpicture, with nuh_layer_id equal to a particular value layerId and subpicture index equal to a particular value subpicIdx, is a RADL subpicture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is any of the following:

  - A picture with nuh_layer_id equal to layerId containing a RASL subpicture with subpicture index equal to subpicIdx

  - A picture that precedes the picture containing the associated IRAP subpicture in decoding order

- The following constraints apply for the picture referred to by each ILRP entry, when present, in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of the current picture:

  - The picture shall be in the same AU as the current picture.

  - The picture shall be present in the DPB.

  - The picture shall have nuh_layer_id refPicLayerId less than the nuh_layer_id of the current picture.

  - Either of the following constraints applies:

  - The picture shall be an IRAP picture.

  - The picture shall have TemporalId less than or equal to Max( 0, vps_max_tid_il_ref_pics_plus1[ currLayerIdx ][ refLayerIdx ] − 1 ), where currLayerIdx and refLayerIdx are equal to GeneralLayerIdx[ nuh_layer_id ] and GeneralLayerIdx[ refpicLayerId ], respectively.

- Each ILRP entry, when present, in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice shall be an active entry.

- When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 and sps_num_subpics_minus1 is greater than 0, either of the following two conditions (but not both) shall be true:

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] has the same subpicture layout as the current picture (i.e., the SPSs referred to by that picture and the current picture have the same value of sps_num_subpics_minus1 and the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], and sps_subpic_height_minus1[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive).

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] is an ILRP for which the value of sps_num_subpics_minus1 is equal to 0.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).