| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0210-v1 |
| --- | --- |

| Title: | On definition of the VVC Still Picture profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Rickard Sjöberg Mitra Damghanian Jack Enhorn Jacob Ström Ruoyang Yu Du Liu | Email: | martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com jack.enhorn@ericsson.com jacob.strom@ericsson.com ruoyang.yu@ericsson.com du.liu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The contribution asserts that a single IRAP picture or GDR picture with ph_recovery_poc_cnt equal to 0, extracted from a bitstream conforming to the Main 10 profile in VVC is not required to be decodable by a decoder only supporting the Main 10 Still Picture profile, unless the general_profile_idc is rewritten for the extracted bitstream. It is also asserted that a similar situation applies for the Main 4:4:4 10 Still Picture profile in VVC.

This contribution proposes that a bitstream containing a single IRAP picture or GDR picture with ph_recovery_poc_cnt equal to 0, and conforming to the Main 10 profile, should also conform to the Main 10 Still Picture profile. Likewise, it is also proposed that a bitstream, containing a single IRAP picture or GDR picture with ph_recovery_poc_cnt equal to 0, and conforming to the Main 4:4:4 10 profile, should also conform to the Main 4:4:4 10 Still Picture profile.

Two options are proposed on how to additionally specify in VVC that a bitstream containing a single IRAP picture or GDR picture with ph_recovery_poc_cnt equal to 0, and conforming to the Main 10 profile, also conforms to the Main 10 Still Picture profile (corresponding options are also proposed for the Main 4:4:4 10 Still Picture profile):

- The bitstream is indicated to conform to the Main 10 profile and the bitstream contains only one picture and nal_unit_type of all VCL NAL units are in the range of IDR_W_RADL to GDR_NUT, inclusive, and ph_recovery_poc_cnt is equal to 0.

- The bitstream is indicated to conform to the Main 10 profile and the bitstream contains only one picture and ph_gdr_or_irap_pic_flag is equal to 1 and ph_recovery_poc_cnt is equal to 0.

In both options, the value of ph_recovery_poc_cnt is proposed to be inferred to 0 when not present.

# Introduction and Problem Statement

The current version of the VVC specification (JVET-S0152-v5) defines four profiles; the Main 10 profile, the Main 10 Still Picture profile, the Main 4:4:4 10 profile and the Main 4:4:4 10 Still Picture profile. The specification text for the four VVC profiles are shown in section 3 of this document. An indication of which profile a bitstream conforms to is signaled using the general_profile_idc syntax element in the profile_tier_level() structure in DCI, VPS, or SPS.

A bitstream conforming to the Main 10 Still Picture profile also conforms to the Main 10 profile and a bitstream conforming to the Main 4:4:4 10 Still Picture profile also conforms to the Main 4:4:4 10 profile.

However, if a single IRAP picture or GDR picture with ph_recovery_poc_cnt equal to 0, and the corresponding parameter sets are extracted from a bitstream conforming to the Main 10 profile into a new bitstream, the new bitstream may not be supported by a decoder only supporting the Main 10 Still picture profile, unless the general_profile_idc syntax element is rewritten in the extracted bitstream.

Likewise, a decoder only conforming to the Main 4:4:4 10 Still Picture profile may not support decoding a single IRAP picture or GDR picture with ph_recovery_poc_cnt equal to 0, extracted from a Main 4:4:4 10 profile, unless general_profile_idc is rewritten in the extracted bitstream.

We believe that having to rewrite the value of the general_profile_idc syntax element is undesirable in order for a still picture decoder to support decoding a single IRAP picture or GDR picture with ph_recovery_poc_cnt equal to 0, from a video bitstream.

# Proposal

We propose to include in the definition of the Main 10 Still Picture profile that a bitstream containing a single IRAP picture or GDR picture with ph_recovery_poc_cnt equal to 0, and conforming to the Main 10 profile, also conforms to the Main 10 Still Picture profile. Likewise, we also propose to include in the definition of the Main 4:4:4 10 Still Picture profile that a bitstream containing a single IRAP picture or GDR picture with ph_recovery_poc_cnt equal to 0, and conforming to the Main 4:4:4 10 profile, also conforms to the Main 4:4:4 10 Still Picture profile.

Two options are proposed of how to additionally specify in VVC that a bitstream containing a single IRAP picture or GDR picture with ph_recovery_poc_cnt equal to 0, and conforming to the Main 10 profile, also conforms to the Main 10 Still Picture profile (corresponding options are also proposed for the Main 4:4:4 10 Still Picture profile):

- The bitstream is indicated to conform to the Main 10 profile and the bitstream contains only one picture and nal_unit_type of all VCL NAL units are in the range of IDR_W_RADL to GDR_NUT, inclusive, and ph_recovery_poc_cnt is equal to 0.

- The bitstream is indicated to conform to the Main 10 profile and the bitstream contains only one picture and ph_gdr_or_irap_pic_flag is equal to 1 and ph_ recovery_poc_cnt is equal to 0.

In both options, the value of ph_recovery_poc_cnt is proposed to be inferred to 0 when not present.

Proposed text on top of JVET-S0152-v5 for the two options is shown below with added text in yellow mark-up:

## Option 1

ph_recovery_poc_cnt specifies the recovery point of decoded pictures in output order.

When the current picture is a GDR picture, the variable recoveryPointPocVal is derived as follows:

recoveryPointPocVal = PicOrderCntVal + ph_recovery_poc_cnt	(83)

If the current picture is a GDR picture, and there is a picture picA that follows the current GDR picture in decoding order in the CLVS that has PicOrderCntVal equal to recoveryPointPocVal, the picture picA is referred to as the recovery point picture. Otherwise, the first picture in output order that has PicOrderCntVal greater than recoveryPointPocVal in the CLVS is referred to as the recovery point picture. The recovery point picture shall not precede the current GDR picture in decoding order. The pictures that are associated with the current GDR picture and have PicOrderCntVal less than recoveryPointPocVal are referred to as the recovering pictures of the GDR picture. The value of ph_recovery_poc_cnt shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive. When not present, the value of ph_recovery_poc_cnt is inferred to be equal to 0.

Main 10 and Main 10 Still Picture profiles

Bitstreams conforming to the Main 10 or Main 10 Still Picture profile shall obey the following constraints:

- …

Decoders conforming to the Main 10 Still Picture profile at a specific level of a specific tier shall be capable of decoding all bitstreams for which all of the following conditions apply:

- The bitstream is indicated to

  - conform to the Main 10 Still Picture profile; or

  - conform to the Main 10 profile and the bitstream contains only one picture and nal_unit_type of all VCL NAL units are in the range of IDR_W_RADL to GDR_NUT, inclusive, and ph_recovery_poc_cnt is equal to 0.

- The bitstream is indicated to conform to a tier that is lower than or equal to the specified tier.

- The bitstream is indicated to conform to a level that is not level 15.5 and is lower than or equal to the specified level.

Main 4:4:4 10 and Main 4:4:4 10 Still Picture profiles

Bitstreams conforming to the Main 4:4:4 10 or Main 4:4:4 10 Still Picture profile shall obey the following constraints:

- …

Decoders conforming to the Main 4:4:4 10 Still Picture profile at a specific level of a specific tier shall be capable of decoding all bitstreams for which all of the following conditions apply:

- The bitstream is indicated to

  - conform to the Main 4:4:4 10 Still Picture or the Main 10 Still Picture profile; or

  - conform to the Main 10 profile and the bitstream contains only one picture and nal_unit_type of all VCL NAL units are in the range of IDR_W_RADL to GDR_NUT, inclusive, and ph_recovery_poc_cnt is equal to 0; or

  - conform to the Main 4:4:4 10 profile and the bitstream contains only one picture and nal_unit_type of all VCL NAL units are in the range of IDR_W_RADL to GDR_NUT, inclusive, and ph_recovery_poc_cnt is equal to 0.

- The bitstream is indicated to conform to a tier that is lower than or equal to the specified tier.

- The bitstream is indicated to conform to a level that is not level 15.5 and is lower than or equal to the specified level.

## Option 2

ph_recovery_poc_cnt specifies the recovery point of decoded pictures in output order.

When the current picture is a GDR picture, the variable recoveryPointPocVal is derived as follows:

recoveryPointPocVal = PicOrderCntVal + ph_recovery_poc_cnt	(83)

If the current picture is a GDR picture, and there is a picture picA that follows the current GDR picture in decoding order in the CLVS that has PicOrderCntVal equal to recoveryPointPocVal, the picture picA is referred to as the recovery point picture. Otherwise, the first picture in output order that has PicOrderCntVal greater than recoveryPointPocVal in the CLVS is referred to as the recovery point picture. The recovery point picture shall not precede the current GDR picture in decoding order. The pictures that are associated with the current GDR picture and have PicOrderCntVal less than recoveryPointPocVal are referred to as the recovering pictures of the GDR picture. The value of ph_recovery_poc_cnt shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive. When not present, the value of ph_recovery_poc_cnt is inferred to be equal to 0.

Main 10 and Main 10 Still Picture profiles

Bitstreams conforming to the Main 10 or Main 10 Still Picture profile shall obey the following constraints:

- …

Decoders conforming to the Main 10 Still Picture profile at a specific level of a specific tier shall be capable of decoding all bitstreams for which all of the following conditions apply:

- The bitstream is indicated to

  - conform to the Main 10 Still Picture profile; or

  - conform to the Main 10 profile and the bitstream contains only one picture and ph_gdr_or_irap_pic_flag is equal to 1 and ph_recovery_poc_cnt is equal to 0.

- The bitstream is indicated to conform to a tier that is lower than or equal to the specified tier.

- The bitstream is indicated to conform to a level that is not level 15.5 and is lower than or equal to the specified level.

Main 4:4:4 10 and Main 4:4:4 10 Still Picture profiles

Bitstreams conforming to the Main 4:4:4 10 or Main 4:4:4 10 Still Picture profile shall obey the following constraints:

- …

Decoders conforming to the Main 4:4:4 10 Still Picture profile at a specific level of a specific tier shall be capable of decoding all bitstreams for which all of the following conditions apply:

- The bitstream is indicated to

  - conform to the Main 4:4:4 10 Still Picture or the Main 10 Still Picture profile; or

  - conform to the Main 10 profile and the bitstream contains only one picture and ph_gdr_or_irap_pic_flag is equal to 1 and ph_recovery_poc_cnt is equal to 0; or

  - conform to the Main 4:4:4 10 profile and the bitstream contains only one picture and ph_gdr_or_irap_pic_flag is equal to 1 and ph_recovery_poc_cnt is equal to 0.

- The bitstream is indicated to conform to a tier that is lower than or equal to the specified tier.

- The bitstream is indicated to conform to a level that is not level 15.5 and is lower than or equal to the specified level.

# Appendix: Current Profile Definitions in VVC

The profiles in VVC are defined as follows in JVET-S0152-v5.

## Main 10 and Main 10 Still Picture profiles

Bitstreams conforming to the Main 10 or Main 10 Still Picture profile shall obey the following constraints:

- In a bitstream conforming to the Main 10 Still Picture profile, the bitstream shall contain only one picture.

- Referenced SPSs shall have sps_chroma_format_idc equal to 0 or 1.

- Referenced SPSs shall have sps_bitdepth_minus8 in the range of 0 to 2, inclusive.

- In a bitstream conforming to the Main 10 Still Picture profile, the referenced SPS shall have max_dec_pic_buffering_minus1[ sps_max_sublayers_minus1 ] equal to 0. [Ed. (YK): Maybe it's better not to have this constraint for the Main 10 Still Picture profile, such that when extracting an intra picture from a Main 10 bitstream to form a Main 10 Still Picure bitstream, the extractor/"encoder" does not have to change the value of max_dec_pic_buffering_minus1[ ] in the SPS.]

- Referenced SPSs shall have sps_palette_enabled_flag equal to 0.

- In a bitstream conforming to the Main 10 profile that do not conform to the Main 10 Still Picture profile, general_level_idc and sublayer_level_idc[ i ] for all values of i in the referenced VPS (when available) and in the referenced SPSs shall not be equal to 255 (which indicates level 15.5).

- The tier and level constraints specified for the Main 10 or Main 10 Still Picture profile in clause A.4, as applicable, shall be fulfilled.

Conformance of a bitstream to the Main 10 profile is indicated by general_profile_idc being equal to 1.

Conformance of a bitstream to the Main 10 Still Picture profile is indicated by general_profile_idc being equal to 3.

NOTE – When the conformance of a bitstream to the Main 10 Still Picture profile is indicated as specified above, and the indicated level is not level 15.5, the conditions for indication of the conformance of the bitstream to the Main 10 profile are also fulfilled.

Decoders conforming to the Main 10 profile at a specific level of a specific tier  shall be capable of decoding all bitstreams for which all of the following conditions apply:

- The bitstream is indicated to conform to the Main 10 or Main 10 Still Picture profile.

- The bitstream is indicated to conform to a tier that is lower than or equal to the specified tier.

- The bitstream is indicated to conform to a level that is not level 15.5 and is lower than or equal to the specified level.

Decoders conforming to the Main 10 Still Picture profile at a specific level of a specific tier shall be capable of decoding all bitstreams for which all of the following conditions apply:

- The bitstream is indicated to conform to the Main 10 Still Picture profile.

- The bitstream is indicated to conform to a tier that is lower than or equal to the specified tier.

- The bitstream is indicated to conform to a level that is not level 15.5 and is lower than or equal to the specified level.

## Main 4:4:4 10 and Main 4:4:4 10 Still Picture profiles

Bitstreams conforming to the Main 4:4:4 10 or Main 4:4:4 10 Still Picture profile shall obey the following constraints:

- In a bitstream conforming to the Main 4:4:4 10 Still Picture profile, the bitstream shall contain only one picture.

- Referenced SPSs shall have sps_chroma_format_idc in the range of 0 to 3, inclusive.

- Referenced SPSs shall have sps_bitdepth_minus8 in the range of 0 to 2, inclusive.

- In a bitstream conforming to the Main 4:4:4 10 Still Picture profile, the referenced SPS shall have max_dec_pic_buffering_minus1[ sps_max_sublayers_minus1 ] equal to 0. [Ed. (YK): Maybe it's better not to have this constraint for the Main 4:4:4 10 Still Picture profile, such that when extracting an intra picture from a Main 4:4:4 10 bitstream to form a Main 4:4:4 10 Still Picure bitstream, the extractor/"encoder" does not have to change the value of max_dec_pic_buffering_minus1[ ] in the SPS.]

- In a bitstream conforming to the Main 4:4:4 10 profile that do not conform to the Main 4:4:4 10 Still Picture profile, general_level_idc and sublayer_level_idc[ i ] for all values of i in the referenced VPS (when available) and in the referenced SPSs shall not be equal to 255 (which indicates level 15.5).

- The tier and level constraints specified for the Main 4:4:4 10 or Main 4:4:4 10 Still Picture profile in clause A.4, as applicable, shall be fulfilled.

Conformance of a bitstream to the Main 4:4:4 10 profile is indicated by general_profile_idc being equal to 2.

Conformance of a bitstream to the Main 4:4:4 10 Still Picture profile is indicated by general_profile_idc being equal to 4.

NOTE – When the conformance of a bitstream to the Main 10 4:4:4 Still Picture profile is indicated as specified above, and the indicated level is not level 15.5, the conditions for indication of the conformance of the bitstream to the Main 10 4:4:4 profile are also fulfilled.

Decoders conforming to the Main 4:4:4 10 profile at a specific level of a specific tier shall be capable of decoding all bitstreams for which all of the following conditions apply:

- The bitstream is indicated to conform to the Main 4:4:4 10, Main 10, Main 4:4:4 10 Still Picture, or Main 10 Still Picture profile.

- The bitstream is indicated to conform to a tier that is lower than or equal to the specified tier.

- The bitstream is indicated to conform to a level that is not level 15.5 and is lower than or equal to the specified level.

Decoders conforming to the Main 4:4:4 10 Still Picture profile at a specific level of a specific tier shall be capable of decoding all bitstreams for which all of the following conditions apply:

- The bitstream is indicated to conform to the Main 4:4:4 10 Still Picture or Main 10 Still Picture profile.

- The bitstream is indicated to conform to a tier that is lower than or equal to the specified tier.

- The bitstream is indicated to conform to a level that is not level 15.5 and is lower than or equal to the specified level.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).