| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0217 |
| --- | --- |

| Title: | On deblocking filter for ACT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | S. Iwamura S. Nemoto A. Ichigaya | Tel: Email: | iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution proposes cleanup of deblocking filter condition for blocks coded with adaptive colour transform (ACT). When input signal format is 4:4:4 RGB and ACT is activated, boundary strength (bS) derivation is done in YCgCo domain when ACT is applied to the current coding block. However, deblocking filter is applied in RGB domain according to bS value derived in YCgCo domain. This discrepancy may cause undesirable deblocking filter operation. To avoid this, two solutions are proposed in this contribution.

- solution 1: Additionally check whether ACT is enabled or not for the current coding block in bS derivation

- solution 2: When ACT is enabled for the current block, modify tu_coded_flag equal to 1.

The simulation results of shows X.X% BD-rate change without encoding and decoding time increase.

Problem Statement

Table 1 shows derivation process of boundary strength for deblocking filter process in the current VVC draft specifications. As shown in the table, bS is set equal to 1 when one or more non-zero transform coefficients are contained in the transform block. This condition check is done for each component individually.

Table 1. Conditions of boundary strength derivation

| Priority | Conditions | bS |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| 6 | At least one of the adjacent blocks is coded with intra or CIIP mode | 2 | 2 | 2 |
| 5 | At least one of the adjacent blocks has non-zero transform coefficients | 1 | 1 | 1 |
| 4 | One of the adjacent blocks is coded in IBC prediction mode and the other is coded in inter prediction mode | 1 | 1 | 1 |
| 3 | Absolute difference between the motion vectors that belong to the adjacent blocks is greater than or equal to one half luma sample | 1 | 0 | 0 |
| 2 | Reference pictures the two adjacent blocks refers to are different | 1 | 0 | 0 |
| 1 | Otherwise | 0 | 0 | 0 |

However, when adaptive colour transform (ACT) is applied to the current coding block, the colour space of the transform coefficients is YCgCo, while that of non-ACT blocks is RGB (GBR). This means that deblocking filter is applied in the original colour space, nevertheless, the condition check of boundary strength derivation is done in the transform domain. That is, when input signal format is 4:4:4 RGB, boundary strength derivation is done in YCgCo domain when ACT is applied, while deblocking filter is applied in RGB domain. This discrepancy may cause insufficient deblocking filter operation.

Proposed modification

solution 1 (Add one condition)

Solution 1 is to add one condition check to bS derivation. When the residual signals in one component in YCgCo domain may be spread in all R, G, and B components. Therefore when either one of the adjacent blocks is ACT coded, bS is set equal to 1.

8.8.3.5 Derivation process of boundary filtering strength

…

- Otherwise, if the sample p0 or q0 is in a coding block coded with cu_act_enabled_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 1.

…

solution 2 (Modify tu_coded flag if ACT is applied)

To avoid additional check in the deblocking filter process, tu_coded_flag for all three component are modified to be equal to 1 when ACT is applied to the current block.

8.4.1 General decoding process for coding units coded in intra prediction mode

…

- The residual modification process for residual blocks using colour space conversion as specified in clause 8.7.4.6 is invoked with the variable nTbW set equal to cbWidth, the variable nTbH set equal to cbHeight, the array rY set equal to resSamplesL, the array rCb set equal to resSamplesCb, and the array rCr set equal to resSamplesCr as inputs, and the output are modified versions of the arrays resSamplesL, resSamplesCb and resSamplesCr.

- tu_y_coded_flag[ xCb ][ yCb ], tu_cb_coded_flag[ xCb ][ yCb ], and tu_cr_coded_flag[ xCb ][ yCb ] are set equal to 1.

…

8.5.1 General decoding process for coding units coded in inter prediction mode

…

- When cu_act_enabled_flag[ xCb ][ yCb ] is equal to 1, following steps applies:

  - the residual modification process for residual blocks using colour space conversion as specified in clause 8.7.4.6 is invoked with the variable nTbW set equal to cbWidth, the variable nTbH set equal to cbHeight, the array rY set equal to resSamplesL, the array rCb set equal to resSamplesCb, and the array rCr set equal to resSamplesCr as inputs, and the output are modified versions of the arrays resSamplesL, resSamplesCb and resSamplesCr.

  - tu_y_coded_flag[ xCb ][ yCb ], tu_cb_coded_flag[ xCb ][ yCb ], and tu_cr_coded_flag[ xCb ][ yCb ] are set equal to 1.

…

8.8.3.5 Derivation process of boundary filtering strength

- Otherwise, if the block edge is also a transform block edge, following applies:

  - If cIdx is equal to 0 and the sample p0 or q0 is in a transform block which contains one or more non-zero transform coefficient levels coded with tu_y_coded_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 1.

  - Otherwise, if cIdx is equal to 1 and the sample p0 or q0 is in a transform block coded with tu_cb_coded_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 1.

  - Otherwise, if cIdx is equal to 2 and the sample p0 or q0 is in a transform block coded with tu_cr_coded_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 1.

Experimental results

Table 2. BD-rate result of Solution 1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | 0.01% | 0.29% | 0.17% | 101% | 100% |
| Kimono | 0.05% | -0.21% | -0.66% | 100% | 99% |
| EBULupoCandlelight | 0.04% | 1.03% | 0.64% | 101% | 100% |
| EBURainFruits | 0.12% | 1.47% | 1.40% | 100% | 104% |
| VenueVu | -0.08% | -0.39% | -0.05% | 100% | 102% |
| DucksAndLegs | 0.02% | 0.02% | 0.03% | 101% | 100% |
| OldTownCross | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| ParkScene | 0.06% | 0.05% | 0.25% | 100% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).