| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0224-v1 |
| --- | --- |

| Title: | AHG16: Performance of a reasonably fast VVC software decoder |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | +1 647 745 1691 fbossen@sharpsec.com |
| Source: | Sharp Electronics of Canada, Ltd. |  |  |

_____________________________

# Abstract

Performance of a reasonably fast VVC software decoder is provided: it is asserted to run about 30 times faster than VTM on an 8-core processor when processing 4K bitstreams (CTC RA). No claim of optimality is made as the optimized decoder was written from scratch in a short amount of time.

# Introduction

While the VTM reference decoder includes a fair share of SIMD-optimized code, its runtime performance falls short of “real time” by a wide margin. One reason is the lack of multithreading. There have been reports of multithreaded implementations based on VTM, but these still fall short of “real time”.

An optimized VVC decoder was developed from scratch to form a better view of the potential performance of VVC software decoders.

# Results

The table below lists 4K bitstreams (generated with VTM version 9.0, RA configuration of common test conditions) for which the bit rate is 40Mbps or less (maximum rate permitted by Level 5.1). Experiments were run on a computer featuring a single Xeon W processor with 8 cores (Skylake). “Real time” performance (60 frames per second or more) is achieved in all cases with the optimized decoder, while VTM processes between 2.8 and 4.6 frames per second.

| Sequence | QP | Frames | Bitrate [Mbps] | VTM [s] | VTM [fps] | Optimized [fps] |
| --- | --- | --- | --- | --- | --- | --- |
| Tango | 22 | 294 | 27.9 | 93.792 | 3.13 | 95.4 |
| FoodMarket | 22 | 300 | 17.0 | 89.978 | 3.33 | 103.8 |
| Campfire | 27 | 300 | 30.7 | 93.977 | 3.19 | 110.8 |
| CatRobot | 22 | 300 | 33.7 | 103.032 | 2.91 | 86.9 |
| DaylightRoad | 27 | 300 | 10.3 | 81.181 | 3.70 | 109.3 |
| ParkRunning | 32 | 300 | 22.2 | 107.255 | 2.80 | 92.1 |
| DayStreet | 22 | 300 | 20.4 | 88.508 | 3.39 | 103.4 |
| FlyingBirds | 22 | 300 | 15.9 | 69.801 | 4.30 | 121.6 |
| PeopleInShoppingCenter | 22 | 300 | 11.6 | 65.853 | 4.56 | 127.2 |
| SunsetBeach | 27 | 300 | 26.6 | 80.717 | 3.72 | 109.2 |

The figure below shows a rough breakdown of the time spend in each broad category of functions for one of the data points in the table above. Note that the level of optimization is not homogeneous across the code.