| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0266-v1 |
| --- | --- |

| Title: | AHG9: Inferring defaults values for VUI syntax elements and adding VUI length |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Gary Sullivan Virginie Drugeon | Tel: Email: | yekui.wang@bytedance.com garysull@microsoft.com virginie.drugeon@eu.panasonic.com |
| Source: | Bytedance, Microsoft, Panasonic |  |  |

_____________________________

# Abstract

This contribution aims at answering the following two requests from the meeting report:

Request 1:

Make sure that each VUI syntax element that describes a property of the bitstream has an inferred/default value when sps_vui_parameters_present_flag is equal to 0. (S0160 aspect 8)

Revisit after offline study to determine whether action is needed. Defaults should indicate unspecified and/or unconstrained indications.

Request 2:

It was asked whether we have addressed the VUI length issue (see section 2.12 of the report of the previous meeting).

Decision (cleanup): Add a length field in the VVC specification prior to the VUI. YKW was asked to prepare text for revisit review.

The present document proposes the following modifications:

- Modification to the SPS syntax to signal the length of the VUI in bytes and add alignment bits to make sure that the VUI syntax structure starts at a byte aligned position;

- New vui_payload syntax to allow extensibility of the VUI syntax;

- Inference of VUI syntax elements as unspecified or determined by the application when not present;

- Editorial modifications to the SEI specification to clarify that both VUI and SEI can be extended by specifying an appropriate container;

- Addition of vui_non_packed_constraint_flag and vui_non_projected_constraint_flag to the VUI syntax structure.

The attached documents contain all the changes proposed in both VVC and SEI specifications.

# Modifications to SPS

It is suggested to modify the SPS as follows to signal the length of the VUI in bytes and add alignment bits to make sure that the VUI syntax structure starts at a byte aligned position:

| sps_vui_parameters_present_flag | u(1) |
| --- | --- |
| if( sps_vui_parameters_present_flag ) { |  |
| sps_vui_payload_size_minus1 | ue(v) |
| while( !byte_aligned( ) ) |  |
| sps_vui_alignment_zero_bit | f(1) |
| vui_payload( sps_vui_payload_size_minus1 + 1 )vui_parameters( ) |  |
| } |  |

With the following semantics:

sps_vui_payload_size_minus1 plus 1 specifies the number of RBSP bytes in the vui_payload( ) syntax structure. The value of sps_vui_payload_size_minus1 shall be in the range of 0 to 1023, inclusive.

NOTE 8 – The SPS NAL unit byte sequence containing the vui_payload( ) syntax structure might include one or more emulation prevention bytes (represented by emulation_prevention_three_byte syntax elements). Since the payload size of the vui_payload( ) syntax structure is specified in RBSP bytes, the quantity of emulation prevention bytes is not included in the size payloadSize of the the vui_payload( ) syntax structure.

sps_vui_alignment_zero_bit shall be equal to 0.

# New vui_payload syntax structure containing the vui_parameters

A new vui_payload syntax structure is introduced as container for the vui_parameters syntax structure to allow extensibility:

| vui_payload( payloadSize ) { | Descriptor |
| --- | --- |
| vui_parameters( payloadSize ) /* Specified in ITU-T H.SEI ¦ ISO/IEC 23002-7 */ |  |
| if( more_data_in_payload( ) ) { |  |  |
| if( payload_extension_present( ) ) |  |  |
| vui_reserved_payload_extension_data | u(v) |  |
| vui_payload_bit_equal_to_one /* equal to 1 */ | f(1) |  |
| while( !byte_aligned( ) ) |  |  |
| vui_payload_bit_equal_to_zero /* equal to 0 */ | f(1) |  |
| } |  |  |
| } |  |

With the following semantics:

vui_reserved_payload_extension_data shall not be present in bitstreams conforming to this version of this Specification. However, decoders conforming to this version of this Specification shall ignore the presence and value of vui_reserved_payload_extension_data. When present, the length, in bits, of vui_reserved_payload_extension_data is equal to 8 * payloadSize − nEarlierBits − nPayloadZeroBits − 1, where nEarlierBits is the number of bits in the vui_payload( ) syntax structure that precede the vui_reserved_payload_extension_data syntax element, and nPayloadZeroBits is the number of vui_payload_bit_equal_to_zero syntax elements at the end of the vui_payload( ) syntax structure.

vui_payload_bit_equal_to_one shall be equal to 1.

vui_payload_bit_equal_to_zero shall be equal to 0.

# Inference of VUI syntax elements

It is suggested to modify clause D.8.”Use of ITU-T H.SEI | ISO/IEC 23002-7 VUI parameters” as follows:

The VUI parameters specified in ITU-T H.SEI | ISO/IEC 23002-7 may be used together with bitstreams specified by this Specification.

The vui_parameters( ) syntax structure as specified in ITU-T H.SEI | ISO/IEC 23002-7 is may be included in the vui_payload( ) syntax specified in clause 7.3.2.22, which may be included in the SPS syntax as specified in clause 7.3.2.3.

When the SPS does not contain a vui_payload( ) syntax structure (i.e sps_vui_parameters_present_flag is equal to 0), the video usability information is inferred as follows unless determined by the application by external means:

‒	vui_progressive_source_flag and vui_interlaced_source_flag are both inferred to be equal to 0 (source scan type interpreted as unknown or unspecified).

‒	vui_non_packed_constraint_flag and vui_non_projected_constraint_flag are both inferred to be equal to 0 (no constraint imposed).

‒	vui_aspect_ratio_constant_flag is inferred to be equal to 0 (no constraint imposed).

‒	vui_aspect_ratio_idc is inferred to be equal to 0 (unspecified).

‒	vui_overscan_info_present_flag is inferred to be equal to 0 (preferred display method for the video signal is unspecified).

‒	vui_colour_primaries is inferred to be equal to 2 (unspecified).

‒	vui_transfer_characteristics is inferred to be equal to 2 (unspecified).

‒	vui_matrix_coeffs is inferred to be equal to 2 (unspecified).

‒	vui_full_range_flag is inferred to be equal to 0 (a value that has no effect when vui_matrix_coeffs is equal to 2).

‒	vui_chroma_sample_loc_type_top_field and vui_chroma_sample_loc_type_bottom_field are both inferred to be equal to 6 (unspecified).

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Microsoft Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).