| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S2017 |
| --- | --- |

| Title: | Technologies under consideration for VSEI |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Jill Boyce Ye-Kui Wang | Email: | jill.boyce@intel.com yekui.wang@bytedance.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document provides draft text of Technologies under Consideration for VSEI. It contains SEI messages proposed in input contributions JVET-S0107 “AHG9/AHG12: Recommended multi-layer composite picture SEI messages”, JVET-S0213 “AHG8/AHG9: Refinement of proposed positioning information SEI message of output independent layers with example bitstreams”, and JVET-S0269 "AHG17/AHG9: SEI manifest and SEI prefix indication SEI messages".

# Recommended multi-layer composite picture SEI messages from JVET-S0107

## Layer composite position info SEI message syntax

| layer_composite_position_info( ) { | Descriptor |
| --- | --- |
| lcpi_param_num_bits_minus1 | u(12) |
| lcpi_top_left_ pos_in_units_ver[ NuhLayerId ] | u(v) |
| lcpi_top_left_pos_in_units_hor[ NuhLayerId ] | u(v) |
| lcpi_height_in_units[ NuhLayerId ] | u(v) |
| lcpi_width_in_units[ NuhLayerId ] | u(v) |
| } |  |

Use of this SEI message requires the definition of the following variables:

–	NuhLayerId, the layer ID of the current layer

The layer composite position info SEI message describes the recommended position and size of the decoded picture of the current layer within a recommended composite picture comprised of decoded pictures from multiple layers.

lcpi_param_num_bits_minus1 plus 1 specifies the number of bits used to represent the lcpi_top_left_pos_ver[ NuhLayerId ], cpi_top_left_pos_ver[ NuhLayerId ], lcpi_width[ NuhLayerId ], and lcpi_height[ NuhLayerId ] syntax elements.

lcpi_top_left_pos_ver[ NuhLayerId ] and lcpi_top_left_pos_ver[ NuhLayerId ] indicate the recommended composite display vertical and horizontal positions, respectively, for the decoded picture of the current layer. The number of bits to represent the syntax elements is lcpi_param_num_bits_minus1 + 1.

lcpi_width_in_units[ NuhLayerId ] and lcpi_height_in_units[ NuhLayerId ] indicate the recommended composite display width and height, respectively, for the decoded picture of the current layer. The number of bits to represent the syntax elements is lcpi_param_num_bits_minus1 + 1.

## Recommended composite layers info SEI message syntax

| recommended_composite_layers_info( ) { | Descriptor |
| --- | --- |
| rcli_cancel_flag | u(1) |
| if( !rcli_cancel_flag ) { |  |
| rcli_persistence_flag | u(1) |
| rcli_layer_scaling_enabled_flag | u(1) |
| rcli_layer_overlap_enabled_flag | u(1) |
| rcli_unit_size_present_flag | u(1) |
| rcli_composite_size_present_flag | u(1) |
| rcli_offset_present_flag | u(1) |
| num_olss_minus1 | u(8) |
| for( i = 0; i <= num_olss_minus1; i++ ) { |  |
| if( rcli_unit_size_present_flag ) { |  |
| rcli_unit_size_ver[ i ] | u(16) |
| rcli_unit_size_hor[ i ] | u(16) |
| } |  |
| if( rcli_composite_size_present_flag ) { |  |
| rcli_composite_size_ver[ i ] | u(16) |
| rcli_composite_size_hor[ i ] | u(16) |
| } |  |
| if( rcli_offset_present_flag ) { |  |
| rcli_offset_ver[ i ] | s(16) |
| rcli_offset_hor[ i ] | s(16) |
| } |  |
| } |  |
| } |  |
| } |  |

## Recommended composite layers info SEI message semantics

The recommended composite layers info SEI message, with the layer composite position information SEI message, describes a layout of decoded pictures from the layers of an OLS within a recommended composite picture, according to the composition process of sub-clause X.

Use of this SEI message requires the definition of the following variables:

–	A list of OLSes, each OLS containing a set of target layers. [Ed. (JB: Need to add definition for OLS in VSEI.]

–	For each AU, a picture width and picture height in units of luma samples, denoted herein by LayerCodedPicWidth[ j ] and LayerCodedPicHeight [ j ], respectively, for each picture present in the AU in the layer with layer ID equal to j.

rcli_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous recommended composite layers info SEI message in output order.

rcli_persistence_flag specifies the persistence of recommended composite layers info SEI message.

rcli_persistence_flag equal to 0 specifies that the recommended composite layers info SEI message applies to the current AU only.

rcli_persistence_flag equal to 1 specifies that the recommended composite layers info SEI message applies to the current access unit and persists all subsequent access units in output order until one or more of the following conditions are true:

–	A new CVS begins.

–	The bitstream ends.

–	A picture in an AU associated with a recommended composite layers info SEI message is output that follows the current picture in output order

rcli_layer_scaling_enabled_flag equal to 0 indicates that LayerPicSizeInCompositeHeight[ i ] and LayerPicSizeInCompositeWidth[ i ] derived in subclause X shall be equal to LayerCodedPicHeight[ j ] and LayerCodedPicWidth[ j ] height, respectively,  of the coded picture with NuhLayerId equal to i. rcli_layer_scaling_enabled_flag equal to 1 indicates that LayerPicSizeInCompositeHeight[ i ] and LayerPicSizeInCompositeWidth[ i ] may differ from the width and height, respectively,  of the coded picture with NuhLayerId equal to i.

rcli_layer_overlap_enabled_flag equal to 0 indicates that all values of Count[ y ][‍ x  ] shall be les than or equal to 1, as derived in subclause X . rcli_layer_overlap_enabled_flag equal to 1 does not impose a restriction.

rcli_unit_size_present_flag equal to 1 specifies that the rcli_unit_size_ver[ i ]  and rcli_ unit_size_hor[ i ]  syntax elements are present. rcli_unit_size_present_flag equal to 0 specifies that the rcli_ unit_size_ver[ i ] and rcli_unit_size_hor[ i ] syntax elements are not  present.

rcli_composite_size_present_flag equal to 1 specifies that the rcli_composite_size_ver[ i ]  and rcli_composite_size_hor[ i ]  syntax elements are present. rcli_composite_size_present_flag equal to 0 specifies that the rcli_composite_size_ver[ i ] and rcli_composite_size_hor[ i ] syntax elements are not  present.

rcli_offset_present_flag equal to 1 specifies that the rcli_offset_ver[ i ]  and rcli_offset_hor[ i ]  syntax elements are present. rcli_offset_present_flag equal to 0 specifies that the rcli_offset_ver[ i ] and rcli_offset_hor[ i ] syntax elements are not  present.

num_olss_minus1 indicates the number of OLSs for which syntax elements are present in the SEI message.

rcli_unit_size_ver[ i ] and rcli_unit_size_hor[ i ] indicate vertical and horizontal unit size parameters respectively,  used in the composition process in subclause X for the i-th OLS. When not present, the values of rcli_offest_ver[ i ]  and rcli_offset_hor[ i ]  may be determined by external means.

rcli_composite_size_ver[ i ] and rcli_composite_size_hor[ i ] indicate the vertical and horizontal size, respectively, of the recommended composite picture in luma samples used in the composition process in subclause X for the i-th OLS. When not present, the values of rcli_offest_ver[ i ]  and rcli_offset_hor[ i ]  may be determined by external means.

rcli_offest_ver[ i ] and rcli_offset_hor[ i ] indicate vertical and horizontal offsets, respectively, of the positions of the decoded layer pictures used in the composition process in subclause X for the i-th OLS. When not present, the values of rcli_offest_ver[ i ]  and rcli_offset_hor[ i ]  are inferred to be equal to 0.

#### Recommended composition process

This subclause describes a composition process to derive sample values for a recommended composite picture, CompositePicture[ ], for the i-th OLS.

CompositePicture[ 0 ] is a 2-D sample array of size rcli_composite_size_hor x rcli_composite_size_ver, of the luma samples of CompositePicture. CompositePicture[ cIdx ]  for cIdx in 1..2 are 2-D sample arrays of size rcli_composite_size_hor/ SubWidthC x rcli_composite_size_ver/ SubHeightC.

For each layer j included in the i-th OLS, if a picture is present in the AU with NuhLayerId equal to j,   the recommended size of the representation of the decoded picture in the composite picture, ScaledLayerPic[ j ], is LayerPicSizeInCompositeWidth[j ] x LayerPicSizeInCompositeHeight[ j ], in luma samples, as derived below.

LayerPicSizeInCompositeHeight[ j ] = lcpi_height_in_units[j ] * rcli_unit_size_ver[ i ]

LayerPicSizeInCompositeWidth[ j ] = lcpi_width_in_units[ j ] * rcli_unit_size_hor[ i ]

When rcli_layer_scaling_enabled_flag equal to 0, ScaledLayerPic[ j ] is set to the decoded picture.

Otherwise, ScaledLayerPic[ j ] is derived by scaling the decoded picture.

ScaledLayerPic[ j ] is a picture of size LayerPicSizeInCompositeWidth[ j ] x LayerPicSizeInCompositeHeight[ j ], in the luma samples.

The sample values of CompositePicture are derived as follows:

for (y = 0; y < rcli_composite_size_ver i ];  y++ )

for (x = 0; x < rcli_composite_size_hor[ i ]; x++ )

Count[ y ][ x ] = 0

for (j = 0; j< 64; j++)

if( j is in the i-th OLS && a picture is present in the AU with NuhLayerId = j )

comp_y = rcli_offset_ver[ i ] + lcpi_top_left_pos_in_units_ver[ j ] * rcli_unit_size_ver

comp_x = rcli_offset_hor[ i ] + lcpi_top_left_pos_in_units_hor[ j ] * rcli_unit_size_hor

for (y = 0; y < LayerPicSizeInCompositeHeight[ j ]  ; y++ )

for ( x = 0; x < LayerPicSizeInCompositeWidth[ j ]; x++ )

CompositePicture[ 0 ] [ comp_y + y ][ comp_x + x ] = ScaledLayerPic[ 0 ][ j ][ y ][ x ]

Count[ [ comp_y + y ][ comp_x + x ] ]++

for ( y = 0; y < LayerPicSizeInCompositeHeight[ j ] / SubWidth * ; y++ )

for ( x = 0; x < LayerPicSizeInCompositeWidth[ j ] / SubHeight; x++ )

CompositePicture[ 1 ][ comp_y / SubHeight + y ][ comp_x / SubWidth + x ] =

ScaledLayerPic[ 1 ][ j ][ y ][ x ]

CompositePicture[ 2 ][ comp_y/SubHeight + y ][ comp_x/SubWidth + x ] =

ScaledLayerPic[ 2 ][ j ][ y ][ x ]

The value of samples of CompositePicture[cIdx] not assigned above are undefined.

## Addition to Table 4

Table 4 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Layer composite position info | The CLVS containing the SEI message |
| Recommended composite layers info | Specified by the syntax of the SEI message |

# Independent layer info SEI message from JVET-S0213

## Independent layer info SEI message syntax

| independent_layer_info( payloadSize ) { | Descriptor |
| --- | --- |
| boundary_identifier_north_present_flag | u(1) |
| if( boundary_identifier_north_present_flag ) |  |
| boundary_identifier_north | u(16) |
| boundary_identifier_east_present_flag | u(1) |
| if( boundary_identifier_east_present_flag ) |  |
| boundary_identifier_east | u(16) |
| boundary_identifier_south_present_flag | u(1) |
| if( boundary_identifier_south_present_flag ) |  |
| boundary_identifier_south | u(16) |
| boundary_identifier_west_present_flag | u(1) |
| if( boundary_identifier_west_present_flag ) |  |
| boundary_identifier_west | u(16) |
| } |  |

## Independent layer info SEI message semantics

boundary_identifier_north_present_flag, boundary_identifier_east_present_flag, boundary_identifier_south_present_flag and boundary_identifier_west_present_flag equal to 1 specify that the SEI message contains a boundary identifier, respectively, for the north, east, south and west boundary.

boundary_identifier_north, boundary_identifier_east, boundary_identifier_south and boundary_identifier_west specifiy the boundary identifier, respecitvely, at the north, east, south and west boundary of the decoded picture of the associated layer; the associated layer being the layer whose nuh_layer_id is equal to the nuh_layer_id of the SEI message. If not present, the boundary identifiers respecitvely at the north, east, south and west boundary of the decoded picture of the associated layer are not defined.

Figure X - Representation of where the bououndaries are on the layers

For two layers, the i-th and j-th layers, when the pair of the boundary_identifier_north value of the i-th layer and the boundary_identifier_south value of the j-th layer are equal then the decoded picture of the i-th layer and the decoded picture of the j-th layer are adjacent in the composed output picture and they share a common boundary at the boundary north/south. For i-th and j-th layers, when the pair of the boundary_identifier_east value of the i-th layer and the boundary_identifier_west value of the j-th layer are equal then the decoded picture of the i-th layer and the decoded picture of the j-th layer are adjacent in the composed output picture and they share a common boundary at the boundary east/west.

Two decoded pictures adjacent by the north/south boundary are aligned on their west boundary in the final output picture. Two decoded pictures adjacent by the east/west boundary are aligned on their north boundary in the final output picture.

All the Independent layer info SEI message present in layers of an OLS shall collectively describe a 4-connected graph and each layer of the OLS shall be connected to the graph.

### D.x.2 Process for generating the aggregated output picture

The process for generating the final output picture is informative. The following section provides the expected operations performed for generating the final output picture based on the decoded pictures of each layer from a selected OLS:

- For each Access Unit: [Ed. (JB): Needs to be rewritten to reside in VSEI, with variables as input instead of syntax structures such as VPS, PPS.]

  - If VPS present, parse VPS and store the list of layers in the bitstream.

  - For each present PPS, determine the size in luma samples of the corresponding layer.

  - For each present Independent layer info SEI, parse the payload and store the boundary identifiers for the corresponding layer.

  - If any of VPS, PPS or Independent layer info SEI is present in the current Access Unit, execute calculate_layer_layout_in_final_output_picture(), that returns the horizontal, XPos, and vertical, YPos, position of the top-left corner of each cropped decoded picture per layer in the final output picture.

  - Initialize a picture buffer of size FinalWidth of width and FinalHeight of height for the final output picture.

  - For each Picture Unit:

    - Decode the coded picture.

  - If pictures are ready for output:

    - For each layer in selected OLS.

      - Apply conformance window cropping on the decoded picture of the current layer.

      - Retrieve XPos and YPos, the positions of the current layer in the final output picture in luma sample.

      - Copy cropped decoded picture in final output picture buffer at position (XPos, YPos) corresponding to the top-left corner of the cropped decoded picture.

### D.x.2.1 Process for calculating the layer layout in the final output picture

Input to this process are:

- the list of layer identifiers included in the VPS, LayerIdList

- the list of cropped picture size of each layer, LayerPicSizeList

- the boundary identifiers for each layer.

Outputs of this process are:

- the horizontal and vertical position in luma sample of the cropped decoded picture in each layer in the final output picture, [{layer_id, XPos, YPos}][NbLayer]

- the size in luma samples of the final output picture, {FinalWidth, FinalHeight}

NOTE: There may be several ways to implement the function that returns the correct output given the input above. The following is an exemplary algorithm.

The steps of the process are:

- Select a layer as starting point, whose layer identifier is layer0. The top-left corner of this layer will be the origin for calculating the coordinates top-left corner of each layer.

- Set curLayerId to layer0.

- Until all layers are traversed, do:

  - Set XPos and Ypos for the current layer using LayerPicSizeList[curLayerId].

NOTE: For layer0, XPos are Ypos set to ‘0’. Otherwise, the rule defined in the semantic defines hwo to derive XPos and YPos. That is, for two vertical neighbors (North or South), the top-left corner of both layer have the same horizontal coordinate. Similarly, for two horizonatal neighbors (East or West), the top-left corner of both layer have the same vertical coordinate. The second component of the top-left corner position, vertical or horizontal, is determined by using the width or height of the cropped decoded picture of the layers.

  - Find a neaighbor of the current layer, and set curLayerId to this layer id.

- At this point XPos and YPos are relative to the top-left corner layer0. Apply a change of origin such that all the XPos and YPos values are positive or null.

- Set FinalWidth to the maximum value calculated by summing each row of grid of horizontal neighbor layers (layers having the same YPos)

- Set FinalHeight to the maximum value calculated by summing each height of vertical neighbor layers (layers having the same XPos)

## Addition to Table 4

Table 4 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Independent layer info | The CLVS containing the SEI message |

# SEI manifest and SEI prefix indication SEI messages from JVET-S0269

## SEI manifest SEI message

### SEI manifest SEI message syntax

| sei_manifest( payloadSize ) { | Descriptor |
| --- | --- |
| manifest_num_sei_msg_types | u(16) |
| for( i = 0; i < manifest_num_sei_msg_types; i++ ) { |  |
| manifest_sei_payload_type[ i ] | u(16) |
| manifest_sei_description[ i ] | u(8) |
| } |  |
| } |  |

### SEI manifest SEI message semantics

The SEI manifest SEI message conveys information on SEI messages that are indicated as expected (i.e., likely) to be present or not present. Such information may include the following:

–	The indication that certain types of SEI messages are expected (i.e., likely) to be present (although not guaranteed to be present) in the CVS.

–	For each type of SEI message that is indicated as expected (i.e., likely) to be present in the CVS, the degree of expressed necessity of interpretation of the SEI messages of this type, as follows:

- The degree of necessity of interpretation of an SEI message type may be indicated as "necessary", "unnecessary", or "undetermined".

- An SEI message is indicated by the encoder (i.e., the content producer) as being "necessary" when the information conveyed by the SEI message is considered as necessary for interpretation by the decoder or receiving system in order to properly process the content and enable an adequate user experience; it does not mean that the bitstream is required to contain the SEI message in order to be a conforming bitstream. It is at the discretion of the encoder to determine which SEI messages are to be considered as necessary in a particular CVS. However, it is suggested that some SEI messages, such as the frame packing arrangement, segmented rectangular frame packing arrangement, and omnidirectional projection indication SEI messages, should typically be considered as necessary.

–	The indication that certain types of SEI messages are expected (i.e., likely) not to be present (although not guaranteed not to be present) in the CVS.

NOTE – An example of such a usage of an SEI manifest SEI message is to express the expectation that there are no frame packing arrangement SEI messages or omnidirectional projection indication SEI messages in the CVS, and therefore that the rendering of the decoded video pictures for display purposes would not need any of the additional post-processing that is commonly associated with the interpretation of these SEI messages.

The content of an SEI manifest SEI message may, for example, be used by transport-layer or systems-layer processing elements to determine whether the CVS is suitable for delivery to a receiving and decoding system, based on whether the receiving system can properly process the CVS to enable an adequate user experience or whether the CVS satisfies the application needs.

When an SEI manifest SEI message is present in any access unit of a CVS, an SEI manifest SEI message shall be present in the first access unit of the CVS. The SEI manifest SEI message persists in decoding order from the current access unit until the end of the CVS. When there are multiple SEI manifest SEI messages present in a CVS, they shall have the same content.

An SEI NAL unit containing an SEI manifest SEI message shall not contain any other SEI messages other than SEI prefix indication SEI messages. When present in an SEI NAL unit, the SEI manifest SEI message shall be the first SEI message in the SEI NAL unit.

manifest_num_sei_msg_types specifies the number of types of SEI messages for which information is provided in the SEI manifest SEI message.

manifest_sei_payload_type[ i ] indicates the payloadType value of the i-th type of SEI message for which information is provided in the SEI manifest SEI message. The values of manifest_sei_payload_type[ m ] and manifest_sei_payload_type[ n ] shall not be identical when m is not equal to n.

manifest_sei_description[ i ] provides information on SEI messages with payloadType equal to manifest_sei_payload_type[ i ] as specified in Table X.

Table X – Interpretation of manifest_sei_description[ i ]

| - Value | - Description |
| --- | --- |
| 0 | Indicates that there is no SEI message with payloadType equal to manifest_sei_payload_type[ i ] expected to be present in the CVS. |
| 1 | Indicates that there are SEI messages with payloadType equal to manifest_sei_payload_type[ i ] expected to be present in the CVS, and these SEI messages are considered as necessary. |
| 2 | Indicates that there are SEI messages with payloadType equal to manifest_sei_payload_type[ i ] expected to be present in the CVS, and these SEI messages are considered as unnecessary. |
| 3 | Indicates that there are SEI messages with payloadType equal to manifest_sei_payload_type[ i ] expected to be present in the CVS, and the necessity of these SEI messages is undetermined. |
| 4..255 | Reserved |

The value of manifest_sei_description[ i ] shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this version of this Specification. Other values for manifest_sei_description[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall allow the value of manifest_sei_description[ i ] greater than or equal to 4 to appear in the syntax and shall ignore all information for payloadType equal to manifest_sei_payload_type[ i ] signalled in the SEI manifest SEI message and shall ignore all SEI prefix indication SEI messages with prefix_sei_payload_type equal to manifest_sei_payload_type[ i ] when manifest_sei_description[ i ] is greater than or equal to 4.

## SEI prefix indication SEI message

### SEI prefix indication SEI message syntax

| sei_prefix_indication( payloadSize ) { | Descriptor |
| --- | --- |
| prefix_sei_payload_type | u(16) |
| num_sei_prefix_indications_minus1 | u(8) |
| for( i = 0; i <= num_sei_prefix_indications_minus1; i++ ) { |  |
| num_bits_in_prefix_indication_minus1[ i ] | u(16) |
| for( j = 0; j <= num_bits_in_prefix_indication_minus1[ i ]; j++ ) |  |
| sei_prefix_data_bit[ i ][ j ] | u(1) |
| while( !byte_aligned( ) ) |  |
| byte_alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| } |  |
| } |  |

### SEI prefix indication SEI message semantics

The SEI prefix indication SEI message carries one or more SEI prefix indications for SEI messages of a particular value of payloadType. Each SEI prefix indication is a bit string that follows the SEI payload syntax of that value of payloadType and contains a number of complete syntax elements starting from the first syntax element in the SEI payload.

Each SEI prefix indication for an SEI message of a particular value of payloadType indicates that one or more SEI messages of this value of payloadType are expected (i.e., likely) to be present in the CVS and to start with the provided bit string. A starting bit string would typically contain only a true subset of an SEI payload of the type of SEI message indicated by the payloadType, may contain a complete SEI payload, and shall not contain more than a complete SEI payload. It is not prohibited for SEI messages of the indicated value of payloadType to be present that do not start with any of the indicated bit strings.

These SEI prefix indications should provide sufficient information for indicating what type of processing is needed or what type of content is included. The former (type of processing) indicates decoder-side processing capability, e.g., whether some type of frame unpacking is needed. The latter (type of content) indicates, for example, whether the bitstream contains subtitle captions in a particular language.

The content of an SEI prefix indication SEI message may, for example, be used by transport-layer or systems-layer processing elements to determine whether the CVS is suitable for delivery to a receiving and decoding system, based on whether the receiving system can properly process the CVS to enable an adequate user experience or whether the CVS satisfies the application needs (as determined in some manner by external means outside the scope of this Specification).

In one example, when the payloadType indicates the frame packing arrangement SEI message, an SEI prefix indication should include up to at least the syntax element frame_packing_arrangement_type; and when the payloadType indicates the omnidirectional projection indication SEI message, an SEI prefix indication should include up to at least the syntax element projection_type.

In another example, for user data registered SEI messages that are used to carry captioning information, an SEI prefix indication should include up to at least the language code; and for user data unregistered SEI messages extended for private use, an SEI prefix indication should include up to at least the UUID.

When an SEI prefix indication SEI message is present in any access unit of a CVS, an SEI prefix indication SEI message shall be present in the first access unit of the CVS. The SEI prefix indication SEI message persists in decoding order from the current access unit until the end of the CVS. When there are multiple SEI prefix indication SEI messages present in a CVS for a particular value of payloadType, they shall have the same content.

An SEI NAL unit containing an SEI prefix indication SEI message for a particular value of payloadType shall not contain any other SEI messages other than an SEI manifest SEI message and SEI prefix indication SEI messages for other values of payloadType.

prefix_sei_payload_type indicates the payloadType value of the SEI messages for which one or more SEI prefix indications are provided in the SEI prefix indication SEI message. When an SEI manifest SEI message is also present for the CVS, the value of prefix_sei_payload_type shall be equal to one of the manifest_sei_payload_type[ m ] values for which manifest_sei_description[ m ] is equal to 1 to 3, inclusive, as indicated by an SEI manifest SEI message that applies to the CVS.

num_sei_prefix_indications_minus1 plus 1 specifies the number of SEI prefix indications.

num_bits_in_prefix_indication_minus1[ i ] plus 1 specifies the number of bits in the i-th SEI prefix indication.

sei_prefix_data_bit[ i ][ j ] specifies the j-th bit of the i-th SEI prefix indication.

The bits sei_prefix_data_bit[ i ][ j ] for j ranging from 0 to num_bits_in_prefix_indication_minus1[ i ], inclusive, follow the syntax of the SEI payload with payloadType equal to prefix_sei_payload_type, and contain a number of complete syntax elements starting from the first syntax element in the SEI payload syntax, and may or may not contain all the syntax elements in the SEI payload syntax. The last bit of these bits (i.e., the bit sei_prefix_data_bit[ i ][ num_bits_in_prefix_indication_minus1[ i ] ]) shall be the last bit of a syntax element in the SEI payload syntax, unless it is a bit within an itu_t_t35_payload_byte or user_data_payload_byte.

NOTE – The exception for itu_t_t35_payload_byte and user_data_payload_byte is provided because these syntax elements may contain externally-specified syntax elements, and the determination of the boundaries of such externally-specified syntax elements is a matter outside the scope of this Specification.

byte_alignment_bit_equal_to_one shall be equal to 1.

## Addition to Table 4

Table 4 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| SEI manifest | The CLVS containing the SEI message |
| SEI prefix indication | The CLVS containing the SEI message |