| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0055-v1 |
| --- | --- |

| Title: | Some errata items for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Zhipin Deng2 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports some asserted bugs in VVC 1st edition and proposes fixes for the asserted bugs. The asserted bugs and proposed fixes are summarized as follows:

- The current constraint on POC values in clause C.4 unnecessarily requires the prevTid0Pic to be within the POC distance of MaxPicOrderCntLsb / 2 even when the current picture has the ph_poc_msb_cycle_val signalled or has an inter-layer reference picture, under either condition the tracking of POC MSB is not needed. Consequently, the constraint disallows the intended use of the syntax element vps_max_tid_il_ref_pics_plus1[ i ][ j ] equal to 0 without having a great value of sps_log2_max_pic_order_cnt_lsb_minus4.

The proposed fix for this asserted bug is to require the prevTid0Pic to be within the POC distance of MaxPicOrderCntLsb / 2 only when the current picture does not have the ph_poc_msb_cycle_val signalled and does not have an inter-layer reference picture.

- In the general sub-bitstream extraction process, the step 9, which removes from outBitstream all SEI NAL units that contain a scalable nesting SEI message that has sn_ols_flag equal to 0 and there is no value in the list NestingLayerId equal to a value in the list LayerIdInOls[ targetOlsIdx ] is redundant.

The proposed fix for this asserted bug is to remove the step 9 of the general sub-bitstream extraction process.

- In the subpicture sub-bitstream extraction process, when at least one VCL NAL unit has been removed by the step 2, the step 5 removes from outBitstream all SEI NAL units that contain scalable nesting SEI messages with sn_subpic_flag equal to 0, including nested SLI SEI messages. However, nested SLI SEI messages, when present, may be needed by the step 6 for parameter sets rewritting.

The proposed fix for this asserted bug is to move step 5 after step 6 in the subpicture sub-bitstream extraction process.

- It is allowed that both nested and non-nested HRD-related SEI messages are present and apply to the same OLS. Similarly, it is also allowed that both nested and non-nested non-HRD-related SEI messages are present and apply to the same layer. However, there lacks a constraint to require that the content of such SEI messages of a particular payload type to be the same.

The proposed fix for this asserted bug is to require such nested and non-nested SEI messages to have the same SEI payload content.

# The constraint on POC values in clause C.4

## Description of the problem

Clause C.4 of the VVC spec includes the following constraint related to POC:

Let currPicLayerId be equal to the nuh_layer_id of the current picture.

For each current picture, let the variables maxPicOrderCnt and minPicOrderCnt be set equal to the maximum and the minimum, respectively, of the PicOrderCntVal values of the following pictures with nuh_layer_id equal to currPicLayerId:

–	The current picture.

–	The previous picture in decoding order that has TemporalId and ph_non_ref_pic_flag both equal to 0 and is not a RASL or RADL picture.

–	The STRPs referred to by all entries in RefPicList[ 0 ] and all entries in RefPicList[ 1 ] of the current picture.

–	All pictures n that have PictureOutputFlag equal to 1, AuCpbRemovalTime[ n ] less than AuCpbRemovalTime[ currPic ] and DpbOutputTime[ n ] greater than or equal to AuCpbRemovalTime[ currPic ], where currPic is the current picture.

...

For each current picture that is not a CLVSS picture, the value of maxPicOrderCnt − minPicOrderCnt shall be less than MaxPicOrderCntLsb / 2.

...

The above constraint disallows the following bitstreams that would be confirming bitstreams if the second bullet item above is relaxed:

- An original bitstream is a single-layer bitstream, and a sub-bitstream outBitstream is extracted out of the bitstream by removing all the non-intra-coded pictures. There is at least one picture picA in outBitstream such that the difference of the POC values between the picture picA and the picture prevTid0Pic, i.e., the previous picture in decoding order in the same layer that has TemporalId and ph_non_ref_pic_flag both equal to 0 and is not a RASL or RADL picture, is greater than MaxPicOrderCntLsb / 2. According to the above constrraint, this bitstram outBitstream is then not a conforming bistream, as it violates the constraint.

However, if picA has the POC MSB value signalled in the PH, then its POC can be correctly derived. Therefore, it the second bullet item is changed to be as follows, the bitstram outBitstream may become a conforming bitstream:

–	When ph_poc_msb_cycle_val is not present for the current picture, the previous picture in decoding order that has TemporalId and ph_non_ref_pic_flag both equal to 0 and is not a RASL or RADL picture.

- An original bitstream is a multi-layer bitstream, a layer with layer index j is a direct reference layer of another layer with layer index layerB in the bitstream, and vps_max_tid_il_ref_pics_plus1[ i ][ j ] is equal to 0. Then in the extracted bitstream outBitstream, per the general sub-bitstream extraction process, for an OLS for which layer j is an output layer while layer i is not, there are only GDR picture with ph_recovery_poc_cnt equal to 0 or IRAP pictures in layer i. In this case, similarly as above, if there is at least one picture picA in layer i in outBitstream such that the difference of the POC values between the picture picA and the picture prevTid0Pic, i.e., the previous picture in decoding order in the same layer that has TemporalId and ph_non_ref_pic_flag both equal to 0 and is not a RASL or RADL picture, is greater than MaxPicOrderCntLsb / 2. According to the above constrraint, this bitstram outBitstream is then not a conforming bistream, as it violates the constraint. Since such an extracted bitstream like outBitstream is required to be a conforming bitstream if the origina bistream was a conforming bitstream, consequently, the original bitsream is not a confirming bitstream, either.

However, if picA has the POC MSB value signalled in the PH, then its POC can be correctly derived. Again, if the second bullet item above is changed to be as follows, both the bitstream outBitstream and the original bitstream in this example may become a conforming bitstream:

–	When ph_poc_msb_cycle_val is not present for the current picture, the previous picture in decoding order that has TemporalId and ph_non_ref_pic_flag both equal to 0 and is not a RASL or RADL picture.

- Similar original bitstream and outBitstream as in examples a and b above, but containing multiple layers, and for the picture picA, there is another picture picB in the same AU and that picture belongs to a reference layer of the layer containing picA. In other words, the POC value of picA would be derived to be equal to the POC value of picB. In this case, even the picture POC difference between picA and the prevTid0Pic is greater than MaxPicOrderCntLsb / 2, the POC value of picA can still be correctly derived, as long as the POC value of picB can be correctly derived.

Therefore, if the second bullet item above is changed to be as follows, the bitstream outBitstream in such an example may become a conforming bitstream:

–	When in the current AU there is no picture belonging to a reference layer of the current layer, the previous picture in decoding order that has TemporalId and ph_non_ref_pic_flag both equal to 0 and is not a RASL or RADL picture.

## Proposed fix

The proposed fix for this asserted bug is to require the prevTid0Pic to be within the POC distance of MaxPicOrderCntLsb / 2 only when the current picture does not have the ph_poc_msb_cycle_val signalled and does not have an inter-layer reference picture.

The proposed text changes are as follows.

#### 8.3.1	Decoding process for picture order count

Output of this process is PicOrderCntVal, the picture order count of the current picture.

Each coded picture is associated with a picture order count variable, denoted as PicOrderCntVal.

Let the variable currLayerIdx be set equal to GeneralLayerIdx[ nuh_layer_id ].

PicOrderCntVal is derived as follows:

–	If vps_independent_layer_flag[ currLayerIdx ] is equal to 0 and there is a picture picA in the current AU with nuh_layer_id equal to layerIdA such that GeneralLayerIdx[ layerIdA ] is in the list ReferenceLayerIdx[ currLayerIdx ], PicOrderCntVal is derived to be equal to the PicOrderCntVal of picA, the variable PocFromIlrpFlag is set equal qual to 1, and the value of ph_pic_order_cnt_lsb shall be the same in all VCL NAL units of the current AU.

–	Otherwise, PocFromIlrpFlag is set equal qual to 0, and PicOrderCntVal of the current picture is derived as specified in the remainder of this subclause.

When ph_poc_msb_cycle_val is not present and the current picture is not a CLVSS picture, the variables prevPicOrderCntLsb and prevPicOrderCntMsb are derived as follows:

- Let prevTid0Pic be the previous picture in decoding order that has nuh_layer_id equal to the nuh_layer_id of the current picture, has TemporalId and ph_non_ref_pic_flag both equal to 0, and is not a RASL or RADL picture.

NOTE 1 – In a sub-bitstream consisting of only intra pictures, extracted from a single-layer bitstream and used in intra-picture-only trick play, the prevTid0Pic is the previous intra picture in decoding order that has TemporalId equal to 0. To ensure correct POC derivation, encoders could choose either to include ph_poc_msb_cycle_val for each intra picture, or set the value of sps_log2_max_pic_order_cnt_lsb_minus4 to be large enough such that the POC difference between the current picture and the prevTid0Pic is less than MaxPicOrderCntLsb / 2.

NOTE 2 – When vps_max_tid_il_ref_pics_plus1[ i ][ j ] is equal to 0 for any value of i among the layer indices and j equal to the layer index of the current layer, the prevTid0Pic in the extracted sub-bitstream for some of the OLSs would be the previous IRAP or GDR picture with ph_recovery_poc_cnt equal to 0 in the current layer in decoding order. To ensure such a sub-bitstream to be a conforming bitstream, encoders could choose either to include ph_poc_msb_cycle_val for each IRAP or GDR picture with ph_recovery_poc_cnt equal to 0, or set the value of sps_log2_max_pic_order_cnt_lsb_minus4 to be large enough such that the POC difference between the current picture and the prevTid0Pic is less than MaxPicOrderCntLsb / 2.

- The variable prevPicOrderCntLsb is set equal to ph_pic_order_cnt_lsb of prevTid0Pic.

- The variable prevPicOrderCntMsb is set equal to PicOrderCntMsb of prevTid0Pic.

...

#### C.4	Bitstream conformance

A bitstream of coded data conforming to this Specification shall fulfil all requirements specified in this subclause.

The bitstream shall be constructed according to the syntax, semantics and constraints specified in this Specification outside of this annex.

The first coded picture in a bitstream shall be an IRAP picture (i.e., an IDR picture or a CRA picture) or a GDR picture.

The bitstream is tested by the HRD for conformance as specified in subclause C.1.

Let currPicLayerId be equal to the nuh_layer_id of the current picture.

For each current picture, let the variables maxPicOrderCnt and minPicOrderCnt be set equal to the maximum and the minimum, respectively, of the PicOrderCntVal values of the following pictures with nuh_layer_id equal to currPicLayerId:

–	The current picture.

–	When ph_poc_msb_cycle_val is not present for the current picture and PocFromIlrpFlag of the current picture is equal to 0, the The previous picture in decoding order that has TemporalId and ph_non_ref_pic_flag both equal to 0 and is not a RASL or RADL picture.

–	The STRPs referred to by all entries in RefPicList[ 0 ] and all entries in RefPicList[ 1 ] of the current picture.

–	All pictures n that have PictureOutputFlag equal to 1, AuCpbRemovalTime[ n ] less than AuCpbRemovalTime[ currPic ] and DpbOutputTime[ n ] greater than or equal to AuCpbRemovalTime[ currPic ], where currPic is the current picture.

All of the following conditions shall be fulfilled for each of the bitstream conformance tests:

...

- For each current picture that is not a CLVSS picture, the value of maxPicOrderCnt − minPicOrderCnt shall be less than MaxPicOrderCntLsb / 2.

...

# On removal of SEI NAL units in the general sub-bitstream extraction process

## Description of the problem

In the general sub-bitstream extraction process, the step 9 removes from outBitstream all SEI NAL units that contain a scalable nesting SEI message that has sn_ols_flag equal to 0 and there is no value in the list NestingLayerId equal to a value in the list LayerIdInOls[ targetOlsIdx ].

However, this step is actually not needed, because: 1) For an SEI NAL unit containing a scalable nesting SEI message with sn_ols_flag equal to 0, NestingLayerId would always include the layer ID of the SEI NAL unit. 2) If an SEI NAL unit has layer ID not in the list LayerIdInOls[ targetOlsIdx ], it would have been removed already by step 3.

## Proposed fix

The proposed fix for this asserted bug is to remove step 9 of the general sub-bitstream extraction process. The proposed text changes are as follows.

#### C.6 General sub-bitstream extraction process

...

The output sub-bitstream OutBitstream is derived by applying the following ordered steps:

- The bitstream outBitstream is set to be identical to the bitstream inBitstream.

- Remove from outBitstream all NAL units with TemporalId greater than tIdTarget.

- Remove from outBitstream all NAL units that have nuh_layer_id not included in the list LayerIdInOls[ targetOlsIdx ], and are not DCI, OPI, VPS, AUD, or EOB NAL units, and are not SEI NAL units containing non-scalable-nested SEI messages with payload PayloadType equal to 0, 1, 130, or 203.

- Remove from outBitstream all APS and VCL NAL units for which all of the following conditions are true, and the associated non-VCL NAL units of these VCL NAL units with nal_unit_type equal to PH_NUT or FD_NUT, or with nal_unit_type equal to SUFFIX_SEI_NUT or PREFIX_SEI_NUT and containing SEI messages with PayloadType not equal to any of 0 (BP), 1 (PT), 130 (DUI), and 203 (SLI):

–	nal_unit_type is equal to APS_NUT, TRAIL_NUT, STSA_NUT, RADL_NUT, or RASL_NUT, or nal_unit_type is equal to GDR_NUT and the associated ph_recovery_poc_cnt is greater than 0.

–	TemporalId is greater than or equal to NumSubLayersInLayerInOLS[ targetOlsIdx ][ GeneralLayerIdx[ ‌nuh_layer_id ] ].

- When all VCL NAL units of an AU are removed by steps 2, 3, or 4 above and an AUD or OPI NAL unit is present in the AU, remove the AUD or OPI NAL unit from outBitstream.

- For each OPI NAL unit in outBitstream, set opi_htid_info_present_flag equal to 1, set opi_ols_info_present_flag equal to 1, set opi_htid_plus1 equal to tIdTarget + 1, and set opi_ols_idx equal to targetOlsIdx.

- When an AUD is present in an AU in outBitstream and the AU becomes an IRAP or GDR AU, set aud_irap_or_gdr_flag of the AUD equal to 1.

- Remove from outBitstream all SEI NAL units that contain a scalable nesting SEI message that has sn_ols_flag equal to 1 and there is no value of i in the range of 0 to sn_num_olss_minus1, inclusive, such that NestingOlsIdx[ i ] is equal to targetOlsIdx.

- Remove from outBitstream all SEI NAL units that contain a scalable nesting SEI message that has sn_ols_flag equal to 0 and there is no value in the list NestingLayerId equal to a value in the list LayerIdInOls[ targetOlsIdx ].

- When LayerIdInOls[ targetOlsIdx ] does not include all values of nuh_layer_id in all VCL NAL units in the bitstream inBitstream, the following applies in the order listed:

...

# On SLI SEI messages in the subpicture sub-bitstream extraction process

## Description of the problem

In the subpicture sub-bitstream extraction process, when at least one VCL NAL unit has been removed by the step 2, the step 5 removes from outBitstream all SEI NAL units that contain scalable nesting SEI messages with sn_subpic_flag equal to 0, including nested SLI SEI messages. However, nested SLI SEI messages, when present, may be needed by the step 6 for parameter sets rewritting.

## Proposed fix

The proposed fix for this asserted bug is to move step 5 after step 6 in the subpicture sub-bitstream extraction process. The proposed text changes are as follows.

#### C.7 Subpicture sub-bitstream extraction process

...

The output sub-bitstream outBitstream is derived by the following order steps:

- The sub-bitstream extraction process, specified in Annex C.6, is invoked with inBitstream, targetOlsIdx, and tIdTarget as inputs and the output of the process is assigned to outBitstream.

- For each value of i in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1, inclusive, remove from outBitstream all VCL NAL units with nuh_layer_id equal to LayerIdInOls[ targetOlsIdx ][ i ] and sh_subpic_id not equal to SubpicIdVal[ subpicIdxTarget[ i ] ], their associated filler data NAL units, and their associated SEI NAL units that contain filler payload SEI messages.

- When an SLI SEI message that applies to the target OLS is present and sli_cbr_constraint_flag of the SLI SEI message is equal to 0, remove all NAL units with nal_unit_type equal to FD_NUT and SEI NAL units containing filler payload SEI messages.

- Remove from outBitstream all SEI NAL units that contain scalable nesting SEI messages with sn_subpic_flag equal to 1 and none of the sn_subpic_id[ j ] values for j from 0 to sn_num_subpics_minus1, inclusive, is equal to any of the SubpicIdVal[ subpicIdxTarget[ i ] ] values for the layers in the multiSubpicLayers.

5.	When at least one VCL NAL unit has been removed by step 2, remove from outBitstream all SEI NAL units that contain scalable nesting SEI messages with sn_subpic_flag equal to 0.

6 5.	If some external means not specified in this document is available to provide replacement parameter sets for the sub-bitstream outBitstream, replace all parameter sets with the replacement parameter sets. Otherwise, the following ordered steps apply:

...

6.	When at least one VCL NAL unit has been removed by step 2, remove from outBitstream all SEI NAL units that contain scalable nesting SEI messages with sn_subpic_flag equal to 0.

7.	When at least one VCL NAL unit has been removed by step 2, the following applies in the order listed:

...

# On scalable-nested and non-scalable-nested SEI messages applying to the same OLS or layer

## Description of the problem

It is allowed that both nested and non-nested HRD-related SEI messages are present and apply to the same OLS. Similarly, it is also allowed that both nested and non-nested non-HRD-related SEI messages are present and apply to the same layer. However, there lacks a constraint to require that the content of such SEI messages of a particular payload type to be the same.

## Proposed fix

The proposed fix for this asserted bug is to add the following constraint:

When there are multiple SEI messages with a particular value of payloadType not equal to 133 that are associated with a particular AU or DU and apply to a particular OLS or layer, regardless of whether some or all of these SEI messages are scalable-nested, the SEI messages shall have the same SEI payload content.

Note that 113 is the payloadType value of the scalable nesting SEI message.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).