| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0093 |
| --- | --- |

| Title: | AHG12: On signalling for high-bit depth |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kei Kawamura Kyohei Unno 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-6887-2592 yo-kidani@kddi.com ki-kawamura@kddi.com ky-unno@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes a signalling for high-bit depth video coding in VVC. In the previous meeting, JVET-S0243 presentation slide reported that coding gains of some tools are reduced or lost as QP decreases in high-bit depth video sequences. JCCR is one of the reported tools, and the verification effort can be reduced if JCCR is disabled in low QP range for high-bit depth coding. In this contribution, the additional applicable condition of JCCR with the pre-defined QP threshold is proposed against VVC ver.1. With this condition, JCCR can be disabled when block based QP is equal to or smaller than the pre-defined QP threshold. Simulation results under JVET-T0047 reportedly show the coding loss is negligible without one PQ test sequence (Market33Clip4000r2).

Introduction

In the previous meeting, JVET-S0243 [1] presentation slide reported that coding gains of some tools are reduced or lost as QP decreases in high-bit depth video sequences as the following table. JCCR is one of the reported tools, and the verification effort can be reduced if JCCR is disabled in low QP range for high-bit depth coding.

Table 1 Declining gains from tools as QP decrease (CrowdRun + Ballown) reported by JVET-S0243.

Proposed Method

This contribution proposes the additional applicable condition of JCCR with the pre-defined QP threshold against VVC ver.1 [2] as shown in Table 2. The yellow highlight line indicates the difference from VVC ver.1. The details of the condition are as follows. UFirst, qp_check_enabled_flag specifies whether JCCR is applied or not with the comparison of pre-defined threshold (QpTh) and the transform block based quantization parameter (qP). qp_check_enabled_flag is parsed only if sps_joint_cbcr_enabled_flag equals to 1. If qp_check_enabld_flag is not present, qp_check_enabld_flag is inferred to 0. Second, tu_joint_cbcr_redisual_flag is parsed when qp_check_enabled_flag euqals to 0 or qP is higher than QpTh. This means that JCCR can be disabled when qP is equal or smaller than the pre-defined threshold.

Table 2 Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| ... |  |
| if( sps_joint_cbcr_enabled_flag && ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && ( tu_cb_coded_flag[ xC ][ yC ] ¦ ¦ tu_cr_coded_flag[ xC ][ yC ] ) ) ¦ ¦ ( tu_cb_coded_flag[ xC ][ yC ] && tu_cr_coded_flag[ xC ][ yC ] ) ) && chromaAvailable) |  |
| if( !qp_check_enabled_flag ¦ ¦ qP > QpThres ) |  |
| tu_joint_cbcr_residual_flag[ xC ][ yC ] | ae(v) |
| ... |  |
| } |  |
| } |  |

Simulation Results

Simulation Condition

The proposed method is implemented on the top of VTM 10.0 [3]. The anchor is VTM 10.0, which test conditions are partially followed to JVET-T0047 [4]. It means that only the simulation settings where internal bit-depth equals to 12 described in JVET-T0047 are conducted. The value of QPTh is selected by the preliminary and exhaustive analysis using SVT class.

Simulation Results (InternalBitDepth = 12)

Table 3 shows the simulation results of AI condition where internal bit-depth equals to 12. SVT class simulations are ongoing, so to be update. It is observed that coding loss is negligible without one PQ test sequence (Market3Clip4000r2). It is considered that one of the causes is that value of QPTh is selected only by SVT classes.

Table 3 Simulation results of AI condition where InternalBitDepth =12.

|  | AI-12 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| OldTownCross | TBD | TBD | TBD | TBD | TBD |
| CrowdRun | TBD | TBD | TBD | TBD | TBD |
| DucksTakeOff | TBD | TBD | TBD | TBD | TBD |
| InToTree | TBD | TBD | TBD | TBD | TBD |
| ParkJoy | TBD | TBD | TBD | TBD | TBD |
| BalloonFestival | 0.04% | 0.01% | -0.01% | 92% | 98% |
| FireEater2Clip4000r1 | 0.08% | -0.04% | 0.07% | 88% | 102% |
| Market3Clip4000r2 | 1.62% | 0.63% | 0.63% | 88% | 100% |
| EBU_04_Hurdles | 0.05% | -0.07% | 0.06% | 86% | 100% |
| EBU_06_Starting | 0.02% | -0.01% | 0.00% | 90% | 99% |
| BalloonFestival | 0.04% | 0.01% | 0.00% | 94% | 100% |
| FireEater2Clip4000r1 | 0.04% | -0.01% | 0.03% | 92% | 102% |
| Market3Clip4000r2 | 1.09% | 0.40% | 0.41% | 92% | 102% |
| EBU_04_Hurdles | 0.08% | -0.02% | 0.01% | 93% | 99% |
| EBU_06_Starting | 0.03% | -0.01% | 0.03% | 93% | 100% |
| Overall | TBD | TBD | TBD | 91% | 100% |

Conclusion

In this contribution, the additional applicable condition of JCCR with the pre-defined QP threshold was proposed against VVC ver.1. It can reduce the verification effort for high bit-depth video coding under low QP range. Simulation results under JVET-T0047 reportedly show the coding loss is negligible without one PQ test sequence (Market33Clip4000r2).

Further study on the value of the pre-defined QP thresholds is recommended if profiles above 10-bit are to be considered in any future version 2.

This work was supported by Ministry of Internal Affairs and Communications (MIC) of Japan (Grant no. JPJ000595).

References

- A. Browne, S. Keating, K. Sharman, “On operation beyond 10-bit,” JVET-S0243, by teleconference, 22 June -1 July 2020.

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 10),” JVET-S2001, by teleconference, 22 June - 1 July 2020.

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 10 (VTM 10),” JVET-S2002, by teleconference, 22 June - 1 July 2020.

- T. Ikai, A. Browne, X. Xiu, “AHG12: Preliminary common test conditions for high bitdepth video coding,” by teleconference, 7-16 October 2020.

Patent rights declaration

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).