| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0042 |
| --- | --- |

| Title: | Status Report on HDR Verification Test Preparation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mathias Wien Vittorio Baroncini Andrew Segall | Tel: Email: | wien@lfb.rwth-aachen.de baroncini@gmx.com asegall@sharplabs.com |
| Source: | Verification Test Coordinators |  |  |

_____________________________

# Abstract

This document summarizes the status of test sequence selection and rate point determination for the VVC verification test of the HDR-HLG and HDR-PQ categories.  For HDR-HLG, a set of test sequences has been identified and visually inspected by JVET experts.  Rate points have been identified for three of the sequences.  During evaluation, a color artifact was identified in two additional test sequences and is currently being studied and corrected.  For HDR-PQ, a set of test candidates has been visually inspected by JVET experts.  Unfortunately, only one or two of the available sequences were determined to be suitable for further consideration.  Additional sources for sequences have been identified, and these sequences are currently under study.

# Introduction

In the context of AHG4 HDR-HLG and HDR-PQ activities, the verification test coordinators have assessed and discussed the available test sequences in multiple online meeting together with JVET experts. Furthermore, action has been taken to acquire more test material suitable for usage in the VVC verification tests.

# HDR-HLG

A set of three test sequences as listed in the table below have been identified out of the available set of proposed test sequences.

| Test sequence | Resolution | fps | Frames | Transfer Function |
| --- | --- | --- | --- | --- |
| H3_AMS01_3840x2160_10bit_420_HLG | 3840x2160 | 60 | 600 | EOTF |
| H3_AMS06_3840x2160_10bit_420_HLG | 3840x2160 | 60 | 600 | EOTF |
| H3_AMS11_3840x2160_10bit_420_HLG | 3840x2160 | 60 | 600 | EOTF |

The sequences are available on the JVET ftp site in the /ahg/verificationtests/candidates_hdr/HLG directory.

The test sequences have been encoded using the Random-Access configuration of the VTM-11.0 software and the HM-16.22 software, and an overview of the general rate-distortion characteristics of the studied sequences is provided in the figure below:

A selection of proposed QP points was generated and reviewed in a session of experts. The proposed QP settings are listed in the table below:

| Sequence | HM QPs | VTM QPs | QP Parameters suitable for Dry Run |
| --- | --- | --- | --- |
| H3_AMS01_3840x2160_10bit_420_HLG | 39, 36, 32, 28, 24 | 42, 38, 34, 30, 26 | Yes |
| H3_AMS06_3840x2160_10bit_420_HLG | 40, 36, 32, 28, 24 | 42, 38, 34, 30, 26 | Yes |
| H3_AMS11_3840x2160_10bit_420_HLG | 39, 35, 31, 27, 23 | 42, 38, 34, 30, 26 | Evaluating larger spacing for higher QP values |

## Additional Candidates

In addition to the three sequences described above, two more sequences were identified as suitable for the visual evaluation.  These sequences are:

| Test sequence | Resolution | fps | Frames | Transfer Function |
| --- | --- | --- | --- | --- |
| RiverByBoat_3840x2160_60fps_10bit_HLG_420_type2 | 3840x2160 | 60 | 500 | HLG |
| WaterFallWide_3840x2160_60fps_10bit_HLG_420_type2 | 3840x2160 | 60 | 500 | HLG |

The sequences are also available on the JVET ftp site in the /ahg/verificationtests/candidates_hdr/HLG directory.

Initial evaluation of the content suggested it was suitable for the verification test.  However, it was reported by the original providers that the content used a type-0 chroma location sampling type and, thus, did not conform to Recommendation BT.2100 for high dynamic range television parameters.  Type-2 chroma location sampling to conform to the Recommendation was subsequently provided.

Unfortunately, during evaluation of candidate QP values for the dry run, it was discovered that the type-2 versions of the content had a chroma artifact.  The origin of the chroma artifact is currently under investigation.  It is anticipated that the sequences will be regenerated and usable for the verification test.

## HM Chroma Quality

Evaluation of the HM-16.22 encodings of the HLG content revealed chroma artifacts at the lowest bit-rates.  While this appears to be representative of the HM-16.22 performance, it is noted that the default HM-16.22 configuration does not include any special handling for the BT.2100 colour gamut for HLG content.  This is unlike the default HDR-PQ configuration in HM-16.22 that does include special handing for wider colour gamut.  It is possible that an adjustment of the bit-allocation for the HLG content using HM-16.22 could remove the artifact with negligible impact on objective luma performance.

The authors recommend that this should be discussed in a larger group.

# HDR-PQ

A set of HDR-PQ candidate test sequences was visually evaluated in multiple online meeting together with JVET experts.  The evaluated sequences were:

| Test sequence | Resolution | fps | Frames |
| --- | --- | --- | --- |
| Chimera HDR3 | 3840x2160 | 60 | 600 |
| Chimera HDR5 | 3840x2160 | 60 | 600 |
| Chimera HDR7 | 3840x2160 | 60 | 600 |
| Meridian HDR2 | 3840x2160 | 60 | 600 |
| Meridian HDR5 | 3840x2160 | 60 | 600 |

The conclusion of the evaluation that only 1-2 sequences were suitable for further consideration.  These sequences were Meridian2 and (possibly) Chimera 7.

Given the conclusion, the HDR-PQ activity focused on identifying and securing additional content for the verification test.  Three sources for new content were identified, and the evaluation status is as follows:

## Netflix Open Source Content

The Meridian and Chimera sequences were graciously provided by Netflix as part of their Open Content initiative.  As part of this initiative, a sequence titled Sparks has also been identified as potentially suitable for the verification test activity.  The content has been reviewed and four potential clips have been identified.  The sequences are:

| Test sequence | Resolution | fps | Frames |
| --- | --- | --- | --- |
| Sparks Welding | 3840x2160 | 60 | 600 |
| Sparks Elevator | 3840x2160 | 60 | 567 |
| Sparks TruckPack | 3840x2160 | 60 | 554 |
| Sparks DirtLot | 3840x2160 | 60 | 600 |

These sequences are provided with the PQ transfer function and graded with a peak luminance of 1,000 nits.  It is noted that the sequences are all longer than 9 seconds, though some are not 10 seconds in length (as all other sequences in the verification activity).

A visual example of each sequence is show below.  Note that these sequences have not been tone-mapped for display in this document.  This is why they have low contrast.

Sparks Welding

Sparks Elevator

Sparks TruckPack

Sparks DirtLot

## BT.2245-9

ITU-R BT.2245-9 lists additional content that may be available for JVET use.  Initial contact with owners of the HDR content was made and initial screening of the content identified five sequences of interest.  A visual example of these sequences is below.

Sequence 7 - Drama

Sequence 11 – Swim Race Backstroke

Sequence 14 - Volleyball

Sequence 17 – Horse Race Homestretch

Sequence 16 - Paddock

## Blender

The Blender Foundation was contacted regarding content from their Tears of Steel project.  An HDR version is not currently available.  However, it was reported that the original version of the film is available in EXR format.  There was an expression of interest with working with the JVET community to create an HDR version, and it is anticipated that this discussion will continue.

# Conclusions

It is suggested to complete the evaluation of the HDR-HLG candidate sequences and quantization parameters to enable a dry run experiment in order to assert the suitability of the corresponding quality range. It is further suggested to evaluate the additional HDR-PQ candidates to determine if they can be used for the subjective assessment.