| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0044 |
| --- | --- |

| Title: | AHG4: Agenda and report of the AHG meeting on the SDR HD and 360 video verification test preparation on 2020-12-09 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mathias Wien Vittorio Baroncini Yan Ye | Tel: Email: | wien@lfb.rwth-aachen.de baroncini@gmx.com yan.ye@alibaba-inc.com |
| Source: | AHG |  |  |

_____________________________

# Agenda and Notes

The meeting was held online on Wed, 2020-12-09, 17:00h UTC with the connection information provided via the JVET mailing list.

- Review of contributions JVET-U0041 and JVET-U0043

- Discussion on next steps

| JVET-U0041 | m55695 | 2020-12-08 14:48:09 | 2020-12-09 17:44:38 | 2020-12-09 17:44:38 | Status Report on SDR HD and 360 Video Verification Test Preparation | M. Wien, V. Baroncini, Y. Ye | JVET-U0041. |
| --- | --- | --- | --- | --- | --- | --- | --- |

| JVET-U0043 | m55697 | 2020-12-09 07:17:07 | 2020-12-09 07:28:30 | 2020-12-09 17:40:44 | Game video sequences proposal for the SDR HD low delay VVC verification test | F. Le Léannec, G. Martin-Cocher (InterDigital) | JVET-U0043. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion JVET-U0043:

Contribution was welcomed. Preference for the StarCraft sequence expressed compared to the MineCraft sequence. Both sequences appear to be usable for assessment. MineCraft may be a bit more challenging for naïve subjects. It was commented that third-person perspective content might be generally more suitable for subjective evaluation than content from first-person perspective.

It was commented that for the gaming content, a comparison using SCC might be interesting. It was commented that SCC coding might preferably be assessed using 4:4:4 content.

For the time being, the verification tests focus on the Main 10 profile of VVC. It was commented that the consideration of specific SCC tools might be transferred to testing of other profiles, specifically the Main 10 4.4:4 profile. It was further commented that in that case, SCC tools available in the HEVC specification might be considered as an additional configuration to be tested.

Discussion of JVET-U0041:

It was remarked that VTM-11.0 is available in the meantime. As a main difference to VTM-10.0, VTM-11.0 includes the option of applying MCTF pre-filtering. The question was raised which configuration should be used in the verification tests. The VTM-11.0 random access configuration of the CTC includes GOP32 and MCTF. It was noted that GOP32 already has been used in the SDR UHD verification tests with VTM-10.0. MCTF is a pre-processing tool which would equally be applied for HM and VTM encoding.

Support was expressed for activating MCTF in all configuration where applicable. This applies specifically for random access configurations. It was reported that small RD gains can be observed for the low delay configuration. Further study evaluating the subjective impact of using MCTF in the low delay configuration is encouraged.

It was suggested to use MCTF for random access (for both, HM and VTM). Unless evidence is brought that it is visually beneficial, it was suggested to proceed without MCTF for the low delay configuration.

Concerns were raised that usage of MCTF on 360 content has not been studied yet and that the benefit of application of this pre-filtering for this category should not be considered without additional evidence of its visual benefit. It was suggested to continue without MCTF for the 360° video category at this point in time.

It was emphasized that for the low delay configuration, a GOP size of 4 was found to be visually better than GOP8. This assessment was done on the set of sequences previously considered for the SDR HD LD setting. It was pointed out that the bitstreams used for visual assessment before this AHG meeting have been encoded using GOP8. Action to be taken: perform encodings with the GOP4 configuration for VTM-11.0 and HM-16.22 and conduct a visual assessment for confirmation.

It was commented that for the 360° video, VTM-10.0 bitstreams using GOP32 are readily available. It was suggested to proceed with these streams unless evidence would be brought that suggests moving to VTM-11.0.

Next steps:

- Work out a proposed set of four candidate gaming sequences for the next meeting in order to enable the composition of the final SDR HD LD set.

- Re-encode the conversational SDR HD LD sequences with GOP4 and double-check the visual impact

- Determine rate points for the proposed SDR HD RA sequences.

- Possibly conduct dry run assessments on all proposed test cases.

AHG meeting was closed at approximately 18:50 UTC.

# List of AHG Meeting Participants

(in alphabetical order)

- Andrew Segall

- Benjamin Bross (HHI -DE) (Benjamin Bross)

- Edouard François (InterDigital - France) (Edouard François)

- Fabrice Le Léannec (InterDigital - FR)

- Han Boon Teo (Panasonic - SG) (Han Boon Teo (Panasonic) - SG)

- Jens-Rainer Ohm (RWTH - DE) (Jens-Rainer Ohm)

- Jie Chen (Alibaba -CN) (Jie Chen)

- Kenneth Andersson (Ericsson - SE)

- Koohyar Minoo (IR-Koohyar Minoo)

- Lulin Chen (MediaTek - US) (Lulin Chen (MediaTek))

- Mathias Wien (RWTH - DE)

- Olena Chubach (MediaTek Inc. - US)

- Peng Yin (Dolby - US) (Peng Yin (Dolby))

- Ru-Ling Liao (Alibaba - CN)

- Tomohiro Ikai (Sharp - JP)

- US: Gary Sullivan

- Vittorio Baroncini

- Yan Ye （Alibaba - US)