| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0047 |
| --- | --- |

| Title: | Updating StreamMergeApp to VTM 11.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Emmanuel Thomas Karim El Assal | Email: | emmanuel.thomas@tno.nl karim.elassal@tno.nl |
| Source: | TNO |  |  |

_____________________________

# Abstract

The StreamMergeAp allows to merge multiple single layer bitstreams into a single output multi-layer bitstream.

This contribution provides software updates to make the StreamMergeApp functional again with VTM 11.0.

The contribution also describes the decisions that were made in the software implementation to ensure the validity of the generated output bitstream.

# Introduction

The StreamMergeApp tool merges several single-layer input bitstreams into a single output multi-layer bitstream. Each of the input bitstream constitutes an independent layer in the output multi-layer bitstream. Figure 1 depicts this merging process.

Figure 1- StreamMergeApp operation

# Software update

## Merging process steps

The current version of the StreamMergeApp which was identified to be dysfunctional at least from with VTM 10.0 follows these steps:

- A new VPS is created for the output bitstream

- One NAL unit is read in each input bitstream in a round-robin fashion and written to the output bitstream, hence interleaving the NAL units.

- The layer id in the original NAL unit is overwritten to a new arbitrary value.

- When a VPS NAL is encountered in the input bitstream, the new VPS is written to the output bitstream with the layer id corresponding to the current layer. The new VPS is thus duplicated in all the layers of the output bitstream.

The following problems were detected:

- The rewriting of the layer id in the NAL unit header takes place at the wrong position in the NAL unit

- NAL units of different layers are unconditionally interleaved leading to breaking Picture Units, e.g. two consecutive PH NAL units.

- Parameters sets ids may collide if the same values are present in the input bitstreams

The software changes provided along with this contribution updates the merging process as follows:

- (unchanged) A new VPS is created for the output bitstream.

- (updated) One Access Unit is read in each input bitstream and interleaved in the output bitstream.

- (updated) The layer id in the original NAL units is overwritten to a new arbitrary value at the right position in the NAL unit header.

- (unchanged) When a VPS NAL is encountered in the input bitstream, the new VPS is written to the output bitstream with the layer id corresponding to the current layer. The new VPS is thus duplicated in all the layers of the output bitstream.

- (new) Parameter set ids are rewritten so that they are all unique in the bitstream and that each layer has self-contained parameter sets.

## Remark on parameter set ids rewriting in picture header structure

The rewriting of parameter set ids to avoid id collision requires rewriting the picture header structure so that the correct value of the parameter set id is signaled.

From the point of view of an application implementer (not an encoder/decoder), this task was generally deemed cumbersome since the picture header structure is a large data structure for which the StreamMergeApp has no use. Furthermore, the picture header structure may be located both in the slice header or in the picture header NAL unit. In the case that the picture header structure is located in the slice header, rewriting just the parameter set id impacts the byte alignment at the end of the slice header (somewhere in the slice data) which effectively requires the entire parsing of it if one desires to operate such targeted rewriting on the parameter set id. Alternatively, when the picture header structure is present as a NAL unit, the rewriting of byte alignment structure is easier in this case since it takes place at the end of the NAL unit which is known by the application parsing the bitstream.

# Availability of the software update

The merge request for the StreamMergeApp is available at: [soon to be provided]

# Conclusion

We recommend adopting this merge request to make StreamMergeApp functional again with the latest VTM version.

# Patent rights declaration(s)

TNO does not have any current or pending patent rights relating to the technology described in this contribution.