| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0056-v2 |
| --- | --- |

| Title: | [AHG10] GOP-based temporal filter improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P. Wennersten C. Hollmann J. Ström | Email: | per.wennersten@ericsson.com christopher.hollmann@ericsson.com jacob.strom@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes a set of changes to the GOP-based temporal filter in VTM that aim at improving the BD-rate performance.

The method was reportedly tested under VTM-11.0 RA test configurations. The filter is not used for AI or LD.

The average Y/U/V BD-rates for the common test conditions (CTC) are reported to be −1.3%/−1.0%/−1.1% for RA. All BDR numbers were computed using unfiltered source sequences as references.

It is proposed to adopt the proposed changes into the VTM software.

# Introduction

The VTM [1] encoder minimizes the rate*lambda + distortion metric for each block.

If a current frame is followed by an identical frame, the coded blocks in the current frame would be re-used identically in the following frame. In this case, rate*lambda+2*distortion should be minimized instead. VTM approximates this for RA by setting different QP values for the temporal layers.

If the following frame is only slightly different, the coded block is still likely to be skipped. In this case, rate*lambda + distortion1 + distortion2 should be minimized. Rather than coding the block as close as possible to the original block of the current frame, and hoping that reusing the block in the following frame will result in a small error against the original block there, it makes sense to code the block as close as possible to the average of the original versions of the two frames. This can be seen since the expression (origA-recA)2 + (origB-recA)2 is equal to ½ (origA-origB)2 + 2 (½(origA+origB) – recA)2, which is trivially minimized by setting recA=½(origA+origB). This is one motivation for temporally filtering the original frames.

The filter strength should drop off rapidly if the original images are different, as it is no longer likely that the decoded sample values for the first picture will be re-used for the second picture. In the current filter, this is modelled by a per-pixel weight depending on the difference between temporally adjacent pixels. Each adjacent pixel is weighed by

wri,a=bw∙so∙sr(i,a)∙e-∆I(i)2sw∙σl(QP)2

Where the base weight bw is 0.4 and the sigma weight sw is 2.

In this proposal, we also adjust the weight depending on the total difference in the entire block containing the pixel. We also adjust the weight based on a simple frequency measurement, with weaker filtering when the temporal difference is of low frequency, since such differences tend to be cheaper to code, and thus are more likely to be coded. Correspondingly, we use stronger filtering when the temporal difference has higher frequency.

In addition, we increase the number of reference frames on either side of the frame to be filtered from 2 to 4, and adjust the weights based on the relative prediction errors for different reference frames.

In addition, we made a few minor changes to motion estimation.

# Proposal

## Block error weighing

Here, we compute an error E for each 8x8 block and reference frame. Let SSD be the sum of squared differences between the original block and the corresponding block in the reference frame after motion compensation, and let V be the variance of the original block. Then E is computed as:

E=0.2∙SSD+5V+5+SSD3200

If E is less than 50, the base weight for pixels from this reference frame in this block is multiplied by 1.2. If E is more than 100, the base weight is multiplied by 0.6. If E is more than 50, the sigma weight is multiplied by 0.8.

## Block frequency weighing

Here, we compute a frequency metric F for each 8x8 block and reference frame. To do this, we first compute the pixel-wise difference between the original block and the corresponding block in the reference frame after motion compensation, to create a difference block D. We calculate the sum of squared differences between horizontally neighboring values in D, called HSD, and the same for the vertically neighboring values, VSD. Finally, we calculate the normal sum of squared values of D, SSD. We then compute F as

F= 300∙SSD+5010∙HSD∙VSD+50

If F is larger or equal 25, the base weight for pixels from this reference frame in this block is multiplied by 0.6, and sigma weight is multiplied by 0.8.

## Reference frame weighing

The existing filter in VTM has a weight so that depends on the temporal distance between the original frame and the reference frame. When reference frames are only available in one direction, so is 1.2 and 1.0 for distance 1 and 2 respectively. When reference frames are available in both directions, the weights are 0.85 and 0.6.

We have modified the filter to use up to use four frames in either direction, when available. The new base weights are 1.13, 0.97, 0.81 and 0.57 for distances 1, 2, 3 and 4 when only one direction is available, and 0.85, 0.57, 0.41 and 0.33 otherwise.

In addition, the weight is modified based on the block error E described in section 2.1. We compute the minimum E for each temporally adjacent block to the current block. Then the weights in each block are multiplied by

minE+1E+1

## Motion estimation changes

We made three changes to motion estimation performed by the filter. First, the pass to determine the final 8x8 motion vector for each block no longer does any full-pel motion search. Second, the zero-motion vector is always evaluated after retrieving the best nearby vector from the higher temporal layer. Finally, after fractional motion estimation, we consider switching to the final motion vectors from the above and left blocks.

# Results

The proposal was implemented on top of VTM-11.0. The method is applied for RA coding only. The BD-rate result relative to VTM-11.0 is reported below.

|  | Over VTM-11.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.79% | -1.77% | -2.56% | 89% | 104% |
| Class A2 | -1.03% | -0.70% | -0.76% | 94% | 120% |
| Class B | -1.32% | -1.08% | -1.07% | 87% | 100% |
| Class C | -1.04% | -0.53% | -0.34% | 98% | 108% |
| Class E |  |  |  |  |  |
| Overall | -1.28% | -0.99% | -1.11% | 92% | 107% |
| Class D | -1.31% | 0.01% | 0.25% | 100% | 111% |
| Class F | 0.00% | 0.00% | 0.00% | 96% | 119% |

In addition, these are the results when compared to VTM-11.0 without temporal filtering:

|  | Over VTM-11.0 with MCTF = 0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.50% | -8.81% | -8.66% | 86% | 89% |
| Class A2 | -6.17% | -10.76% | -10.56% | 89% | 101% |
| Class B | -7.60% | -9.30% | -9.47% | 87% | 92% |
| Class C | -2.68% | -4.33% | -3.89% | 98% | 94% |
| Class E |  |  |  |  |  |
| Overall | -5.38% | -8.17% | -8.04% | 90% | 94% |
| Class D | -0.64% | -3.44% | -3.09% | 99% | 96% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 100% |

# Conclusion

In this document changes to the temporal filter were introduced which provide improved compression efficiency, with no discernable impact on complexity. It is proposed to adopt the proposed method into the VTM software.

# References

[1] VTM-11.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).