| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0061 |
| --- | --- |

| Title: | EE1.1-related: BD-Rate improvements to JVET-T0057 neural network based in-loop filter using depthwise separable convolution and regular convolution |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung, Xiang Li, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 {cauyeung, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution presents BD-Rate improvements to the neural-network-based in loop filter in EE1.1 with the same amount of memory and the same number of GMAC by the addition of clipping to the neural-network. In an average of the results from RA/LB/LP/AI configurations, the luma BD-Rate with DSC changed from -0.99% in EE1.1 to -1.19%, and the luma BD-Rate with regular convolution changed from -1.30% in EE1.1 to -1.56%. The average decoding time of RA/LB/LP/AI width DSC changed from 3837% in EE1.1 to 3784. The average decoding time of RA/LB/LP/AI width regular convolution changed from 4831% in EE1.1 to 5017%. The results from this contribution that DSC is better than RC in terms of BDR reduction per %DecT for RA, and RC is better for LB and LP, and both perform similarly for AI.

# Introduction

The JVET-U0060 [3] provides description of the NN-based in-loop filter in EE1.1 [1]. The EE1.1 compares the complexity and coding performance of the in-loop filter when the depthwise separable convolution (DSC) is replaced by regular convolution (RC) in JVET-T0057 [2].

This contribution provides additional results from improved training and minor changes to the neural network in JVET-U0060 [3].

JVET-U0060 [3] reported the results of adding an additional filter called Dense Residual Convolutional Neural Network Based In-loop Filter (DRNLF) inserted between deblocking filter and SAO as shown in Figure 1. As shown in Figure 2, the DRNLF includes a QP Map Quantizer unit, a Pre-Processing unit, a Main-processing unit and a Post-Processing unit.

Figure 1: DRNLF in JVET-U0060 is an in-loop filter in between deblocking filter and SAO. The inputs to the DRMLF is the output of the deblocking filter and the QP map of the picture.

Figure 2: DRNLF in JVET-U0060 consists of a QP Map Quantizer, a Pre-Processing unit, a Main Processing unit and a Post-Processing unit.

# Modifications to JVET-U0060

In this contribution, a clipping step is added to the Pre-Processing unit depicted in Figure 3 from JVET-U0060 [3] to obtain the Pre-Procssing unit in Figure 4 . The clipping is intended to impove the results in the training and the inference process of the neural network.

Figure 3: DRNLF Pre-Processing in JVET-U0060.

Figure 4:Clipping is added to the Pre-Processing to impove the results of the neural network.

# Network information in training stage

The network information for the NN-based video coding tool testing with DSC and RC in training stage are the same except that the learning-rates are different. The learning rate of DSC and RC are 1e-3 and 2e-4, respectively.

The hardware used in this contribution was changed from the hardware in EE1.1. In EE1.1, two Tesla V100 GPU was used in training for 24 hours. In this contribution, one GTX 1080Ti was used in training for 48 hours.

## Network information for the NN-based in-loop filter with DSC in training stage

Table 1: Network information for the NN-based video coding tool testing with DSC in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Intel Core i7-8700 3.20 GHz (6 cores), 32GB RAM and one 11 GB Nvidia GTX 1080Ti |
|  | Framework: | PyTorch v1.4.0 |
|  | Number of GPUs per Task | 1 Nvidia GTX 1080Ti was used for training |
|  | Epoch: | 300 |
|  | Batch size: | 16 |
|  | Training time: | 48h |
|  | Training data information: | DIV2K still images |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP = 22, 27, 32, 37 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 64x64 |
|  | Learning rate: | 1.00E-03 |
|  | Optimizer: | ADAM |
|  | Loss function: | L1 |
|  | Preprocessing: | randomly cropped and rotated |
|  | Other information: |  |

## Network information for the NN-based in-loop filter with RC in training stage

Table 2: Network information for the NN-based video coding tool testing with DSC in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Intel Core i7-8700 3.20 GHz (6 cores), 32GB RAM and one 11GB Nvidia GTX 1080Ti |
|  | Framework: | PyTorch v1.4.0 |
|  | Number of GPUs per Task | 1 Nvidia GTX 1080Ti was used for training |
|  | Epoch: | 300 |
|  | Batch size: | 16 |
|  | Training time: | 48h |
|  | Training data information: | DIV2K still images |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP = 22, 27, 32, 37 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 64x64 |
|  | Learning rate: | 2.00E-04 |
|  | Optimizer: | ADAM |
|  | Loss function: | L1 |
|  | Preprocessing: | randomly cropped and rotated |
|  | Other information: |  |

# Network information in inference stage

The network information in inference stage for DSC and RC is the same as those reported in JVET-U0060 [3] for EE1.1.

## Network information for the NN-based in-loop filter with DSC in inference stage

Table 3: Network information for the NN-based video coding tool testing in inference stage with DSC

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Pytorch 1.2.0 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 22371 |
|  | Parameter Precision (Bits) in Float | 32 |
|  | Memory Parameter (MB) | 0.085338593 |
|  | MAC (Giga) per 3840x2160 pixels | 186.64 |
| Optional |  |  |
|  | Total Conv. Layers | 13 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size |  |
|  | Changes to network configuration or weights required to generate rate points | (e.g. ) |
|  | Peak Memory Usage |  |
|  | Other information: |  |

## Network information for the NN-based in-loop filter with regular convolution (RC) in inference stage

Table 4: Network information for the NN-based video coding tool testing in inference stage with regular convolution (RC)

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Pytorch 1.2.0 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 77443 |
|  | Parameter Precision (Bits) in Float | 32 |
|  | Memory Parameter (MB) | 0.2954216 |
|  | MAC (Giga) per 3840x2160 pixels | 643.43 |
| Optional |  |  |
|  | Total Conv. Layers | 13 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size |  |
|  | Changes to network configuration or weights required to generate rate points | (e.g. ) |
|  | Peak Memory Usage |  |
|  | Other information: |  |

# Results

The software was integrated with VTM10.0 using DSC and RC and it was tested using VTM10.0 as anchor.

The DSC and RC comparison of the overall BD-Rate is shown in Table 5 and Figure 5. As shown in Figure 5, using %BDR per %DecTime as a metric, RC is better than DSC for LB and LP, DSC is better than RC for RA. DSC and RC are about the same for AI.

Table 5: Overall BD-Rate comparison of DSC and RC.

|  |  | DSC |  |  | RC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Random access Main10 | -1.25% | -5.75% | -4.34% | -1.58% | -5.34% | -4.72% |
| Low delay B Main10 | -0.91% | -8.48% | -6.11% | -1.32% | -7.74% | -6.50% |
| Low delay P Main10 | -0.97% | -7.81% | -5.99% | -1.41% | -6.82% | -6.59% |
| All Intra Main10 | -1.61% | -4.48% | -4.12% | -1.94% | -4.34% | -4.70% |
| Average | -1.19% | -6.63% | -5.14% | -1.56% | -6.06% | -5.63% |

Table 6: Overall encoding time, decoding time and bit difference comparison of DSC and RC.

|  |  | DSC |  |  | RC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | EncT | DecT | bit DIFF | EncT | DecT | bit DIFF |
| Random access Main10 | 106% | 3487% | 0.18% | 107% | 4750% | 0.22% |
| Low delay B Main10 | 106% | 3965% | 0.24% | 108% | 5447% | 0.30% |
| Low delay P Main10 | 109% | 4302% | 0.28% | 111% | 5881% | 0.37% |
| All Intra Main10 | 108% | 3382% | 0.12% | 109% | 3989% | 0.13% |
| Average | 107% | 3784% | 0.21% | 109% | 5017% | 0.25% |

Figure 5: A comparison of DSC and RC for RA, LB, LP and AI configurations.

The DSC and RC comparison of the overall encoding time, decoding time, bit difference is shown in Table 6. The overall luma DecT with DSC for RA/LB/LP/AI is 3487%/3965%/4302%/3382%. The overall luma DecT with RC for RA/LB/LP/AI is 4750%/5447%/5881%/5989%.

A comparison of the results from this contribution and other NN-based in-loop filters from JVET-U0060 JVET-T0069, JVET-T0079, JVET-T0088, JVET-T0094 and JVET-T00128 are shown in Table 7. The comparison, without JVET-T0094 and JVET-T00128, is also shown in  Figure 6 for RA and Figure 7 for AI configuration.

The results from JVET-T0094 and JVET-T00128 are treated as outlier and are not shown in Figure 6 and Figure 7. The results from JVET-T0094 is treated as outlier because of the relatively large decoding time. The results form JVET-T00128 is treated as outlier because decoding time is not available.

The results in Figure 6 for RA and Figure 7 for AI demonstrated that DSC is better than RC in terms of BDR reduction per %DecT. The results from this contribution has larger BDR reduction per %DecT than the results reported in JVET-T0069, JVET-T0079 but smaller than JVET-T0088, which did not apply NN-based filtering to all pictures in RA configuration.

Table 7: Comparison with other NN-based loop filters.

|  | RA Dec Time | RA BDR Y | AI Dec Time | AI BDR Y | comments |
| --- | --- | --- | --- | --- | --- |
| U0060-DSC | 3471% | -0.99% | 3393% | -1.28% | 1 model |
| U0060-RC | 4379% | -1.28% | 4135% | -1.57% | 1 model |
| U0061-DSC | 3487% | -1.25% | 3382% | -1.61% | 1 model |
| U0061-RC | 4750% | -1.58% | 3989% | -1.94% | 1 model |
| T0069 | 27499% | -1.84% | 25695% | -2.11% | 1 model, QP=22,27,32,37 Loss = 0.2*SSIM + 0.8*L1 |
| T0079 | 14058% | -4.11% | 7877% | -3.63% | 6 DIV2K models + 6 BVI-DVC refined models |
| T0088 | 1040% | -3.86% | 15738% | -8.24% | Over VTM9.0 |
| T0094 | 754013% | -3.92% | 775056% | -4.99% | Over VTM9.0 |
| T0128 | ? | -5.57% | ? | ? | Over VTM9.3, Class B only (QP=27,32,37,42) |

Figure 6: A comparison of other NN-based in-loop filters in RA configuration.

Figure 7: A comparison of other NN-based in-loop filters in AI configuration.

## Results with DSC

Table 8: Results with DSC

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -1.84% | -4.29% | -4.34% | 107% | 4548% | 0.19% |
| Class A2 | -1.40% | -5.78% | -3.88% | 106% | 3351% | 0.21% |
| Class B | -0.84% | -6.22% | -4.22% | 105% | 2833% | 0.17% |
| Class C | -1.22% | -6.23% | -4.82% | 105% | 3818% | 0.18% |
| Class E |  |  |  |  |  |  |
| Overall | -1.25% | -5.75% | -4.34% | 106% | 3487% | 0.18% |
| Class D | -1.42% | -4.67% | -4.67% | 105% | 2385% | 0.22% |
| Class F | -0.45% | -2.00% | -2.07% | 110% | 1670% | 0.09% |
| Class H |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.61% | -7.69% | -4.98% | 105% | 4527% | 0.20% |
| Class C | -0.80% | -9.46% | -7.52% | 103% | 5009% | 0.25% |
| Class E | -1.56% | -3.96% | -4.41% | 113% | 2329% | 0.30% |
| Overall | -0.91% | -8.48% | -6.11% | 106% | 3965% | 0.24% |
| Class D | -1.03% | -6.61% | -6.64% | 104% | 3250% | 0.39% |
| Class F | -0.46% | -3.44% | -2.99% | 110% | 2359% | 0.27% |
| Class H |  |  |  |  |  |  |
|  | Low delay P Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.70% | -8.33% | -5.60% | 108% | 4921% | 0.24% |
| Class C | -0.80% | -10.19% | -7.64% | 104% | 5284% | 0.28% |
| Class E | -1.67% | -3.77% | -4.43% | 120% | 2614% | 0.35% |
| Overall | -0.97% | -7.81% | -5.99% | 109% | 4302% | 0.28% |
| Class D | -1.23% | -7.30% | -6.52% | 106% | 3991% | 0.37% |
| Class F | -0.47% | -3.71% | -4.30% | 113% | 2468% | 0.24% |
| Class H |  |  |  |  |  |  |
|  | All Intra Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -1.41% | -3.07% | -3.63% | 115% | 3701% | 0.09% |
| Class A2 | -1.34% | -3.81% | -2.97% | 108% | 2780% | 0.04% |
| Class B | -1.04% | -4.85% | -4.27% | 106% | 2819% | 0.05% |
| Class C | -2.04% | -6.53% | -5.57% | 104% | 3405% | 0.25% |
| Class E | -2.45% | -3.18% | -3.57% | 107% | 5046% | 0.13% |
| Overall | -1.61% | -4.48% | -4.12% | 108% | 3382% | 0.12% |
| Class D | -2.49% | -4.82% | -5.81% | 104% | 3098% | 0.28% |
| Class F | -0.94% | -2.58% | -2.49% | 104% | 2429% | 0.11% |
| Class H |  |  |  |  |  |  |

## Results with regular convolution

Table 9: Results with regular convolution.

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -1.93% | -4.55% | -5.14% | 109% | 6939% | 0.23% |
| Class A2 | -1.75% | -5.79% | -4.29% | 108% | 4746% | 0.22% |
| Class B | -1.20% | -5.55% | -4.81% | 107% | 3690% | 0.19% |
| Class C | -1.65% | -5.34% | -4.60% | 105% | 4903% | 0.24% |
| Class E |  |  |  |  |  |  |
| Overall | -1.58% | -5.34% | -4.72% | 107% | 4750% | 0.22% |
| Class D | -1.92% | -4.71% | -4.62% | 105% | 3166% | 0.27% |
| Class F | -0.57% | -1.70% | -2.13% | 112% | 1870% | 0.11% |
| Class H |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.89% | -7.40% | -6.10% | 107% | 6530% | 0.23% |
| Class C | -1.15% | -8.15% | -7.00% | 103% | 6401% | 0.30% |
| Class E | -2.26% | -3.39% | -5.31% | 115% | 3246% | 0.42% |
| Overall | -1.32% | -7.74% | -6.50% | 108% | 5447% | 0.30% |
| Class D | -1.39% | -6.48% | -5.84% | 104% | 4343% | 0.48% |
| Class F | -0.56% | -3.81% | -2.84% | 111% | 2692% | 0.28% |
| Class H |  |  |  |  |  |  |
|  | Low delay P Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -1.03% | -7.64% | -6.67% | 110% | 7034% | 0.30% |
| Class C | -1.20% | -8.06% | -7.11% | 106% | 6770% | 0.36% |
| Class E | -2.33% | -3.81% | -5.75% | 122% | 3616% | 0.51% |
| Overall | -1.41% | -6.82% | -6.59% | 111% | 5881% | 0.37% |
| Class D | -1.67% | -7.83% | -6.23% | 106% | 5098% | 0.51% |
| Class F | -0.59% | -3.56% | -3.89% | 115% | 2792% | 0.29% |
| Class H |  |  |  |  |  |  |
|  | All Intra Main10 |  |  |  |  |  |
|  | BD-rate Over VTM-10.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| Class A1 | -1.49% | -3.43% | -4.56% | 117% | 3994% | 0.10% |
| Class A2 | -1.59% | -3.98% | -3.28% | 109% | 3296% | 0.04% |
| Class B | -1.34% | -4.65% | -4.92% | 107% | 3536% | 0.06% |
| Class C | -2.51% | -5.74% | -5.55% | 105% | 4032% | 0.27% |
| Class E | -3.00% | -3.20% | -4.75% | 109% | 5816% | 0.14% |
| Overall | -1.94% | -4.34% | -4.70% | 109% | 3989% | 0.13% |
| Class D | -3.12% | -5.35% | -6.05% | 104% | 3506% | 0.30% |
| Class F | -1.12% | -2.36% | -2.71% | 104% | 2512% | 0.12% |
| Class H |  |  |  |  |  |  |

# Conclusions

This contribution improved the results from JVET-U0060. The results from this contribution demonstrated that DSC is better than RC in terms of BDR reduction per %DecT for RA. The results from this contribution and JVET-U0060 has larger BDR reduction per %DecT than the results reported in JVET-T0069, JVET-T0079 but smaller than JVET-T0088, which did not apply NN-based filtering to all pictures in RA.

# References

- E. Alshina, S. Liu, W. Chen, Y. Li, R.-L. Liao, Z. Ma and H. Wang, “Description of exploration experiments on NN-based video coding,” JVET-T2023_r2, 20th Meeting, by teleconference, 7 – 16 Oct. 2020.

- Cheung Auyeung, Wei Wang, Wei Jiang, Xiang Li, Shan Liu,”AHG11: A case study to reduce computation of neural network based in-loop filter by pruning”, JVET-T0057_r1, 20th Meeting, by teleconference, 7 – 16 Oct. 2020.

- Cheung Auyeung, Wei Wang, Wei Jiang, Xiang Li, Shan Liu,”EE1.1: A comparison of depthwise separable convolution and regular convolution with the JVET-T0057 neural network based in-loop filter”, JVET-U0060, 21st Meeting, by teleconference, 6–15 Jan. 2021.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).