| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0064 |
| --- | --- |

| Title: | CE-1.1 and CE-1.2: On the Rice parameter derivation for high bit-depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy Luong Pham Van Muhammed Coban Marta Karczewicz | Email: | dmytror@qti.qualcomm.com lphamvan@qti.qualcomm.com mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc |  |  |

_____________________________

# Abstract

This contribution describes a modification to the Rice parameters selection method for regular residual coding (RRC) being tested in CE-1.1 and CE-1.2 for purposes of High Bit Depth and High Bit Rate Coding. Simulation results, and assessments of complexity and memory requirement change are provided as per CE requirement in JVET-T2022.

# Introduction

In the current VVC [1], rice parameter for RRC is derived using look-up table with considering of coefficient value of the neighboring transform coefficients in a template. The template of the neighbouring coefficients is presented in Figure 1.

First, locSumAbs which is the sum of absolute of five available neighbouring coefficients in the template is calculated.

The locSumAbs is then normalized (subtraction and clip) as following:

locSumAbs = Clip3( 0, 31, locSumAbs - baseLevel * 5 )

The rice parameter is derived using this locSumAbs with look-up table (Table 1). With the current design, the rice parameter range is constrained from 0 to 3.

Figure 1 Neighbouring coefficients which are using in calculation of the localSumAbs of the current  coefficient

Table 1. Look-up table for Rice Parameter based on locSumAbs in current spec

| locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |

When the bitdepth of the input video is increased, or the extended precision is enabled, or the quantization parameter is set very low, the range of the coefficients and therefore locSumAbs value may significantly increase. In such cases, the current VVC range of allowed rice parameter may not be sufficient. This limitation causes a coding performance drop of VVC in high bit-depth coding.

# Test CE-1.1

Initial design tested in CE-1.1 was proposed in JVET-T0105. To extend the range of the Rice parameters, it is proposed to scale values locSumAbs by a scalling factor sh_shift_rice if locSumAbs exceed certain threshold, such that scaled value would fit in the allowed range of the Table 128 of the VVC specficiation without clipping. Following this, an output of the Table 128 is being adjusted by incrementing with shift value if scaling process was taken a place for a given locSumAbs value. Utilized value sh_shift_rice is signalled as syntax element in the slice header and being derived at the encoder side as a function of input data bit depth.

The specification text is accordingly modified as follows:

7.3.7	Slice header syntax

| … |  |
| --- | --- |
| if( sps_dep_quant_enabled_flag ) |  |
| sh_dep_quant_used_flag | u(1) |
| if( sps_high_bit_depth ) |  |
| sh_shift_rice | ue(v) |
| if( sps_sign_data_hiding_enabled_flag && !sh_dep_quant_used_flag ) |  |
| sh_sign_data_hiding_used_flag | u(1) |

sh_shift_rice specifies the shift value utilized for derivation of the Rice parameter, the value of sh_shift_rice shall be in the range of 0 to 4, inclusive. When sh_shift_rice is not present, it is inferred to be equal to 0.

Values RiceT are initialized as following:

RiceT = {{32, 52}, {32 << sh_shift_rice, 52 << sh_shift_rice}}

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ] Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the Rice parameter cRiceParam. Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo-code process: locSumAbs = 0 if( xC < ( 1 << log2TbWidth ) − 1 ) { locSumAbs += AbsLevel[ xC + 1 ][ yC ] if( xC < ( 1 << log2TbWidth ) − 2 ) locSumAbs += AbsLevel[ xC + 2 ][ yC ] if( yC < ( 1 << log2TbHeight ) − 1 ) locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] (1517) } if( yC < ( 1 << log2TbHeight ) − 1 ) { locSumAbs += AbsLevel[ xC ][ yC + 1 ] if( yC < ( 1 << log2TbHeight ) − 2 ) locSumAbs += AbsLevel[ xC ][ yC + 2 ] } Variable shift is derived as following for idx set equal to 0, if baseLevel is equal to 0, and set equal to 1, otherwise: shift = (localSumAbs < RiceT[0][idx]) ? 0 : (localSumAbs < RiceT[1][idx]) ? sh_shift_rice : (sh_shift_rice << 1) locSumAbs = locSumAbs >> shift Variable localSumAbs is updated as follows: locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 ) Given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 128. cRiceParam is then is refined as: cRiceParam = cRiceParam + shift

Worst case complexity analysis:

It is assessed that worst case complexity increase is accounted for 2 conditions, 1 shift operations and 1 add for each Golomb-Rice coded transform coefficient. Which results in 32 conditions, 16 shifts and 16 adds for 4x4 TU, 32/16/16 for condition/shift/adds respectively.

Memory increase analysis:

It is estimated that ROM increase for proposed method is 32 bits.

# CE-1.1 Simulation results for HBD CTC

Simulation results have been conducted according to JVET-T2018. Presented run time estimates may not be accurate and can be updated.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.55% | -0.60% | -0.60% | -0.54% | -0.61% | -0.61% | 81% | 89% |
| PQ422 | -0.66% | -0.73% | -0.74% | -0.66% | -0.73% | -0.75% | 83% | 90% |
| Overall | -0.60% | -0.67% | -0.67% | -0.60% | -0.67% | -0.68% | 82% | 90% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.04% | -0.06% | -0.09% | -0.04% | -0.06% | -0.08% | 101% | 98% |
| PQ422 | -0.06% | -0.07% | -0.05% | -0.06% | -0.06% | -0.05% | 98% | 93% |
| Overall | -0.05% | -0.06% | -0.07% | -0.05% | -0.06% | -0.06% | 99% | 95% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.12% | -0.13% | -0.15% | -0.12% | -0.13% | -0.15% | 91% | 99% |
| PQ422 | -0.13% | -0.15% | -0.16% | -0.13% | -0.15% | -0.16% | 87% | 92% |
| Overall | -0.12% | -0.14% | -0.16% | -0.12% | -0.14% | -0.15% | 89% | 95% |
| Overall PQ | -0.35% | -0.38% | -0.36% | -0.34% | -0.39% | -0.39% | 88% | 92% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.20% | -0.22% | -0.22% | 83% | 88% |
| HLG422 | -0.24% | -0.25% | -0.26% | 83% | 86% |
| Overall | -0.22% | -0.24% | -0.24% | 83% | 87% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.03% | -0.04% | -0.04% | 95% | 101% |
| HLG422 | -0.02% | -0.05% | -0.04% | 93% | 101% |
| Overall | -0.03% | -0.04% | -0.04% | 94% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| HLG422 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall HLG | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -25.10% | -23.98% | -23.99% | 98% | 76% |
| SVT12 | -2.62% | -2.48% | -2.48% | 83% | 89% |
| Overall | -13.86% | -13.23% | -13.23% | 90% | 83% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -16.56% | -15.20% | -15.47% | 100% | 90% |
| SVT12 | -0.29% | -0.38% | -0.38% | 103% | 98% |
| Overall | -8.43% | -7.79% | -7.92% | 101% | 94% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -15.36% | -15.50% | -15.54% | 90% | 83% |
| SVT12 | -0.44% | -0.45% | -0.45% | 92% | 91% |
| Overall | -7.90% | -7.98% | -8.00% | 91% | 87% |
| Overall RGB | -11.33% | -10.85% | -10.89% | 93% | 86% |

# CE-1.1 Simulation results for HBD lossless

Simulation results have been conducted according to JVET-T2022. Presented run time estimates may not be accurate and can be updated.

| PQ | All Intra |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-1.1 |  | VTM11.0rc1 | CE-1.1 |  |
| PQ444 | 2.4 | 2.5 | -2.68% | 3.0 | 0.0 | 0.00% |
| PQ422 | 2.2 | 2.3 | -3.79% | 2.8 | 0.0 | 0.00% |
| Overall | 2.3 | 2.4 | -3.23% | 2.9 | 0.0 | 0.00% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
| HLG | All Intra |  |  | Low delay B |  |  |
|  | Ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-1.1 |  | VTM11.0rc1 | CE-1.1 |  |
| HLG444 | 1.9 | 1.9 | -1.91% | 2.1 | 0.0 | 0.00% |
| HLG422 | 1.8 | 1.8 | -2.77% | 2.0 | 0.0 | 0.00% |
| Overall | 1.8 | 1.9 | -2.34% | 2.0 | 0.0 | 0.00% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
| SVT | All Intra |  |  | Low delay B |  |  |
|  | Ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-1.1 |  | VTM11.0rc1 | CE-1.1 |  |
| SVT16 | 0.7 | 1.2 | -41.40% | 0.7 | 1.2 | -43.90% |
| SVT12 | 1.0 | 1.2 | -21.28% | 1.0 | 1.3 | -20.86% |
| Overall | 0.8 | 1.2 | -31.34% | 0.8 | 1.2 | -32.38% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

# Test CE-1.2.a

Initial design tested in CE-1.2 was presented in the JVET-T0105. Similarly to the Test CE-1.1, to extend the range of the Rice parameters, it is proposed to scale values locSumAbs with a shift, if locSumAbs exceed certain threshold, such that scaled value would fit in the allowed range of the Table 128 of the VVC specficiation without clipping. Following this, an output of the Table 128 is being adjusted by incrementing with shift value if scaling process was taken a place for a given locSumAbs value.

The difference with CE-1.1 is limmited the way, shift parameter for locSumAbs scaling and cRiceParam increment is derived. In CE-1.2, it is tested as follows, with syntax element sh_scale_rice taking values 1 or 2.

scalledCoef = (localSumAbs * sh_scale_rice) >> 3

shift = (scalledCoef == 0) ? 0 : Floor( Log2( scalledCoef) )

Utilized value sh_scale_rice is signalled as syntax element in the slice header and being derived at the encoder side as a function of QP values.

The specification text is accordingly modified as follows:

7.3.7	Slice header syntax

| … |  |
| --- | --- |
| if( sps_dep_quant_enabled_flag ) |  |
| sh_dep_quant_used_flag | u(1) |
| if( sps_high_bit_depth ) |  |
| sh_scale_rice | ue(v) |
| if( sps_sign_data_hiding_enabled_flag && !sh_dep_quant_used_flag ) |  |
| sh_sign_data_hiding_used_flag | u(1) |

sh_scale_rice specifies the scale value utilized for derivation of the Rice parameter, the value of sh_scale_rice shall be in the range of 1 to 2, inclusive. When sh_scale_rice is not present, it is inferred to be equal to 0.

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ] Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the Rice parameter cRiceParam. Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo-code process: locSumAbs = 0 if( xC < ( 1 << log2TbWidth ) − 1 ) { locSumAbs += AbsLevel[ xC + 1 ][ yC ] if( xC < ( 1 << log2TbWidth ) − 2 ) locSumAbs += AbsLevel[ xC + 2 ][ yC ] if( yC < ( 1 << log2TbHeight ) − 1 ) locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] (1517) } if( yC < ( 1 << log2TbHeight ) − 1 ) { locSumAbs += AbsLevel[ xC ][ yC + 1 ] if( yC < ( 1 << log2TbHeight ) − 2 ) locSumAbs += AbsLevel[ xC ][ yC + 2 ] } Variable shift is derived as following: scalledCoef = (localSumAbs * sh_scale_rice) >> 3 shift = (scallPreCoef == 0) ? 0 : Floor( Log2( scalledCoef) ) locSumAbs = locSumAbs >> shift Variable localSumAbs is updated as follows: locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 ) Given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 128. cRiceParam is then is refined as: cRiceParam = cRiceParam + shift

Worst case complexity analysis:

It is assessed that worst case complexity increase is accounted for 1 condition, 2 shift operations, 1 add and 1 floorLog2() (equivalent of MSB find operation) for each Golomb-Rice coded transform coefficient. For worst case scenario (4x4 TU with each coefficient codded with Rice), complexity can be assessed in 16/32/16/16 for condition/shift/adds/MSB find respectively.

Memory increase analysis:

It is estimated that method requires no ROM increase.

# CE-1.2.a Simulation results for HBD CTC

Simulation results have been conducted according to JVET-T2018. Presented run time estimates may not be accurate and can be updated.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.91% | -0.80% | -0.80% | -0.92% | -0.80% | -0.79% | 106% | 104% |
| PQ422 | -0.99% | -0.93% | -0.96% | -1.01% | -0.93% | -0.96% | 104% | 104% |
| Overall | -0.95% | -0.87% | -0.88% | -0.96% | -0.87% | -0.88% | 105% | 104% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.04% | -0.05% | -0.08% | -0.04% | -0.05% | -0.06% | 101% | 102% |
| PQ422 | -0.03% | -0.05% | -0.05% | -0.04% | -0.05% | -0.05% | 100% | 98% |
| Overall | -0.03% | -0.05% | -0.06% | -0.04% | -0.05% | -0.05% | 101% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.17% | -0.16% | -0.16% | -0.18% | -0.16% | -0.16% | 104% | 106% |
| PQ422 | -0.17% | -0.16% | -0.17% | -0.18% | -0.17% | -0.17% | 105% | 106% |
| Overall | -0.17% | -0.16% | -0.16% | -0.18% | -0.16% | -0.16% | 104% | 106% |
| Overall PQ | -0.53% | -0.49% | -0.44% | -0.54% | -0.49% | -0.49% | 104% | 103% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.44% | -0.30% | -0.31% | 104% | 102% |
| HLG422 | -0.49% | -0.34% | -0.34% | 104% | 102% |
| Overall | -0.46% | -0.32% | -0.32% | 104% | 102% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.05% | 0.06% | 0.05% | 102% | 102% |
| HLG422 | 0.04% | 0.06% | 0.06% | 105% | 109% |
| Overall | 0.05% | 0.06% | 0.06% | 103% | 105% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.02% | 0.03% | 0.02% | 110% | 116% |
| HLG422 | 0.01% | 0.03% | 0.05% | 109% | 109% |
| Overall | 0.02% | 0.03% | 0.03% | 110% | 113% |
| Overall HLG | -0.22% | -0.14% | -0.14% | 105% | 105% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -27.55% | -25.76% | -25.76% | 102% | 94% |
| SVT12 | -4.20% | -3.78% | -3.78% | 94% | 101% |
| Overall | -15.88% | -14.77% | -14.77% | 98% | 97% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -18.05% | -17.29% | -17.30% | 105% | 108% |
| SVT12 | -1.01% | -1.00% | -1.01% | 99% | 106% |
| Overall | -9.53% | -9.14% | -9.15% | 102% | 107% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -17.13% | -17.31% | -17.25% | 100% | 98% |
| SVT12 | -1.16% | -1.04% | -1.05% | 90% | 97% |
| Overall | -9.15% | -9.18% | -9.15% | 95% | 98% |
| Overall RGB | -12.97% | -12.28% | -12.27% | 98% | 100% |

# CE-1.2.b Simulation results for HBD CTC lossless

Simulation results have been conducted according to JVET-T2022. Presented run time estimates may not be accurate and can be updated.

| PQ | All Intra |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-1.2.a |  | VTM11.0rc1 | CE-1.2.a |  |
| PQ444 | 2.4 | 2.5 | -3.34% | 3.0 | 3.0 | -0.82% |
| PQ422 | 2.2 | 2.3 | -4.41% | 2.8 | 2.8 | -1.29% |
| Overall | 2.3 | 2.4 | -3.88% | 2.9 | 2.9 | -1.06% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
| HLG | All Intra |  |  | Low delay B |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-1.2.a |  | VTM11.0rc1 | CE-1.2.a |  |
| HLG444 | 1.9 | 2.0 | -2.90% | 2.1 | 2.1 | -0.92% |
| HLG422 | 1.8 | 1.8 | -2.52% | 2.0 | 2.0 | -0.68% |
| Overall | 1.8 | 1.9 | -2.71% | 2.0 | 2.1 | -0.80% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
| SVT | All Intra |  |  | Low delay B |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-1.2.a |  | VTM11.0rc1 | CE-1.2.a |  |
| SVT16 | 0.7 | 1.2 | -41.15% | 0.7 | 1.2 | -43.56% |
| SVT12 | 1.0 | 1.3 | -21.92% | 1.0 | 1.3 | -21.25% |
| Overall | 0.8 | 1.2 | -31.54% | 0.8 | 1.2 | -32.41% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

# Test CE-1.2.b

Motivated by the history based method of rice derivation in HEVC and CE-1.4/1.5 tests, the test CE-1.2.b was designed as extension of CE1.2 by introducing Rice parameter replacement with a history value for coded transform coefficients, with no reliable template estimation can be achieved. In test CE-1.2.b, history rice parameter was utilized for the 1st Golomb-Rice coded coefficient ( abs_remainder[ ] or dec_abs_level[ ] ).

History maintenance:

A single counter per color component is utilized, StatCoeff[3]. History counters are being reset with a default value at the beginning of each CTU.

History update:

History counter update is conducted once per TU from the first, non-zero, Golomb-Rice coded transform coefficient (abs_remainder[ ] or dec_abs_level[ ]) as follows.

When first, non-zero, Golomb-Rice coded transform coefficient in TU is coded as abs_remainder, the history counter for color component cIdx is updated as following:

StatCoeff[cIdx] = ( StatCoeff[cIdx] + Floor(Log2(abs_remainder[ ])) + 2 ) >> 1

When first, non-zero, Golomb-Rice coded transform coefficient in TU is coded as abs_remainder, the history counter for color component cIdx is updated as following:

StatCoeff[cIdx] = ( StatCoeff[cIdx] + Floor(Log2(dec_abs_level[ ])) ) >> 1

History usage:

When first, non-zero, Golomb-Rice coded transform coefficient in TU is coded as abs_remainder[], the history counter for color component cIdx is used to provide Rice parameter value, replacing template based derivations of clause 9.3.3.2.

cRiceParam = StatCoeff [ cIdx ]

cRiceParam = (cRiceParam - 2 < 0) ? 0 : cRiceParam - 2

When first, non-zero, Golomb-Rice coded transform coefficient in TU is coded as abs_remainder[], the history counter for color component cIdx is used to provide rice parameter value, replacing template based derivations of clause 9.3.3.3.

cRiceParam = StatCoeff [ cIdx ]

Worst case complexity analysis:

Complexity of this method is being affected by a history update, conducted once per TU and by removal of one of the template call. Number of operations of the method for 4x4 TU, as a/b/c/d for condition/shift/adds/MSB find respectively:

Total number of operations increase vs. VVC anchor:      10/31/12/16

Memory increase analysis:

It is estimated that proposed method requires additional 12 bits for history counters in RAM.

# CE-1.2.b Simulation results for HBD CTC

Simulation results have been conducted according to JVET-T2018, proposed history method of was enabled for coding of 16 bit data, where 12 bit data was coded with CE-1.2.a. Presented run time estimates may not be accurate and can be updated.

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0rc1 CTC |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -26.47% | -25.04% | -25.05% | 97% | 80% |
| SVT12 | -3.11% | -2.91% | -2.91% | 99% | 103% |
| Overall | -14.79% | -13.97% | -13.98% | 98% | 91% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-11.0rc1 CTC |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -17.37% | -16.57% | -16.60% | 129% | 107% |
| SVT12 | -0.51% | -0.56% | -0.56% | 100% | 106% |
| Overall | -8.94% | -8.57% | -8.58% | 114% | 106% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-11.0rc1 CTC |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -16.43% | -16.58% | -16.54% | 101% | 94% |
| SVT12 | -0.69% | -0.67% | -0.67% | 80% | 86% |
| Overall | -8.56% | -8.62% | -8.60% | 91% | 90% |
| Overall RGB | -12.11% | -11.58% | -11.58% | 99% | 94% |

# References

[1]	Benjamin Bross, Jianle Chen, Shan Liu and Ye-Kui Wang, “Versatile Video Coding (Draft 10)” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET- S2001, 19th Meeting: Teleconference, 22 June – 01 July 2020.

[2] JVET-T0105, AHG12: On the Rice parameter derivation for high bit-depth coding, L. Pham Van, D. Rusanovskyy, M. Karczewicz (Qualcomm).

[3]	JVET-T2018, "Common test conditions for high bit depth and high bit rate video coding", A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu.

[4]	JVET-T2022, "CE on Entropy Coding for High Bit Depth and High Bit Rate Coding", A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy.

[5]	JVET-U0022, "CE: Summary Report on Entropy Coding for High Bit Depth and High Bit Rate Coding", A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).