| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0066 |
| --- | --- |

| Title: | CE-3.5 and CE-3.6: Combination of CE-1.2, CE-1.4/1.5 and CE-1.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy Luong Pham Van Muhammed Coban Marta Karczewicz | Email: | dmytror@qti.qualcomm.com lphamvan@qti.qualcomm.com mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc |  |  |

_____________________________

# Abstract

This contribution describes a modification to the Rice parameters selection method for regular residual coding (RRC) tested in CE-3.5 and CE-3.6 for purposes of High Bit Depth and High Bit Rate Coding. The test of CE-3.5 is presenting combination of the proposed RRC method of CE-1.2.b with classification elements of CE-1.4/1.5. The test CE-3.6 is a combination of the Rice derivation method presented in CE-1.3, history rice method of CE-3.5. Simulation results, and assessments of complexity and memory requirement change are provided as per CE requirement in JVET-T2022.

# Introduction

Detailed description of CE-1.2.b. is presented in JVET-U0064 and descriptions of CE-1.4/1.5 are presented in JVET-U0057.

# Test CE-3.5

In test CE-3.5, a development of test CE-1.2.b with incorporation of classification elements CE1.4 is tested.

In test CE-1.2.b, history Rice information was used to improve accuracy of Rice derivation for 1st Golomb-Rice coded coefficient, for which no accurate template information may be present under high bit depth coding. History is updated from a first coded transform coefficient in TU through the mean of exponential moving average. History counter is updated with default value at every CTU.

In the test CE-1.4/1.5, history Rice information was used to adjust scaling results of template derivation. For RRC coding, coefficients are classified in 3 classes, depending on their spatial position in coded TU, as shown in Figure below. Separate history counter for each class, is being updated either from every coded coefficient as in CE-1.4, or from coefficient located at the main diagonal of TU. History update is conducted either at every coefficient or after current TU is being decoded and used for following TU. History is maintained across full slice.

16x16 transform block

Figure 1. Visualization of the coefficient classification (into 3 classes for RRC) utilized in CE-1.4/CE-1.5 and samples providing input to history update located at the main diagonal of TU.

In CE-3.5, it is assessed that inaccurate Rice parameter template based derivation may potentially affect not only first coded transform coefficient (which is addressed in CE-1.2.b), but also coefficients at the TU boundaries. To address this issue, transform coefficients in TU are classified in 3 classes, and a history rice value is adaptively utilized for each of them. Visualization of utilized in CE-3.5 coefficient classification into 3 classes and source for history update is shown in Figure 2.

C3

C1

C2

C2

Figure 2. Visualization of the classification concept utilized in CE-3.5 and an example of sample location providing an input to history update shown in green.

History value utilization is integrated into the template derivation, clause 9.3.3.2, to reuse existing classification. Implementation of this method is shown VVC specification text extract for clause 9.3.3.2, with text highlighted with green is related to changes from CE-1.2.b and yellow highlighted text associated with classification utilized in CE-3.5. Variable g_historyValue is derived prior to coding of current TU from history counter associated with current color component id.

History counters update and maintenance process is identical to what is used in CE-1.2.b.

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ] Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the Rice parameter cRiceParam. Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo-code process: locSumAbs = 0 if( xC < ( 1 << log2TbWidth ) − 1 ) { locSumAbs += AbsLevel[ xC + 1 ][ yC ] if( xC < ( 1 << log2TbWidth ) − 2 ) locSumAbs += AbsLevel[ xC + 2 ][ yC ] else locSumAbs += histValue if( yC < ( 1 << log2TbHeight ) − 1 ) locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] (1517) else locSumAbs += histValue } else locSumAbs += 2 * histValue if( yC < ( 1 << log2TbHeight ) − 1 ) { locSumAbs += AbsLevel[ xC ][ yC + 1 ] if( yC < ( 1 << log2TbHeight ) − 2 ) locSumAbs += AbsLevel[ xC ][ yC + 2 ] else locSumAbs += histValue } else locSumAbs += histValue Variable shift is derived as following: scalledCoef = (localSumAbs * sh_scale_rice) >> 3 shift = (scallPreCoef == 0) ? 0 : Floor( Log2( scalledCoef) ) locSumAbs = locSumAbs >> shift Variable localSumAbs is updated as follows: locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 ) Given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 128. cRiceParam is then is refined as: cRiceParam = cRiceParam + shift

Worst case complexity analysis:

Number of operations for each component of the method for 4x4 TU, as c/s/a/m for condition/shift/adds/MSB find respectively:

CE-1.2.a:                             16/32/16/16

History update:								 1/1/2/1

History usage:								 0/1/20/0

Total number of operations increase for TU 4x4 vs. anchor:      17/34/38/17

Memory increase analysis:

It is estimated that proposed method requires additional 12 bits for history counters in RAM.

# CE-3.5 Simulation results for HBD CTC

Simulation results have been conducted according to JVET-T2018. Presented run time estimates may not be accurate and can be updated.

| HDR PQ |  |  | AI |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU |  | psnrV |  | EncT | DecT |
| PQ444 | -0.91% | -0.80% | -0.80% | -0.92% | -0.80% |  | -0.79% |  | 106% | 104% |
| PQ422 | -0.99% | -0.93% | -0.96% | -1.01% | -0.93% |  | -0.96% |  | 104% | 104% |
| Overall | -0.95% | -0.87% | -0.88% | -0.96% | -0.87% |  | -0.88% |  | 105% | 104% |
|  |  |  | LDB |  |  |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU |  | psnrV |  | EncT | DecT |
| PQ444 | -0.04% | -0.05% | -0.08% | -0.04% | -0.05% |  | -0.06% |  | 101% | 102% |
| PQ422 | -0.03% | -0.05% | -0.05% | -0.04% | -0.05% |  | -0.05% |  | 100% | 98% |
| Overall | -0.03% | -0.05% | -0.06% | -0.04% | -0.05% |  | -0.05% |  | 101% | 100% |
|  |  |  | RA |  |  |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU |  | psnrV |  | EncT | DecT |
| PQ444 | -0.17% | -0.16% | -0.16% | -0.18% | -0.16% |  | -0.16% |  | 104% | 106% |
| PQ422 | -0.17% | -0.16% | -0.17% | -0.18% | -0.17% |  | -0.17% |  | 105% | 106% |
| Overall | -0.17% | -0.16% | -0.16% | -0.18% | -0.16% |  | -0.16% |  | 104% | 106% |
| Overall PQ | -0.53% | -0.49% | -0.44% | -0.54% | -0.49% |  | -0.49% |  | 104% | 103% |
| HDR HLG |  |  | AI |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |
|  | psnrY | psnrU | psnrV | EncT |  | DecT |  |
| HLG444 | -0.44% | -0.30% | -0.31% | 104% |  | 102% |  |
| HLG422 | -0.49% | -0.34% | -0.34% | 104% |  | 102% |  |
| Overall | -0.46% | -0.32% | -0.32% | 104% |  | 102% |  |
|  |  |  | LDB |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |
|  | psnrY | psnrU | psnrV | EncT |  | DecT |  |
| HLG444 | 0.05% | 0.06% | 0.05% | 102% |  | 102% |  |
| HLG422 | 0.04% | 0.06% | 0.06% | 105% |  | 109% |  |
| Overall | 0.05% | 0.06% | 0.06% | 103% |  | 105% |  |
|  |  |  | RA |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |
|  | psnrY | psnrU | psnrV | EncT |  | DecT |  |
| HLG444 | 0.02% | 0.03% | 0.02% | 110% |  | 116% |  |
| HLG422 | 0.01% | 0.03% | 0.05% | 109% |  | 109% |  |
| Overall | 0.02% | 0.03% | 0.03% | 110% |  | 113% |  |
| Overall HLG | -0.22% | -0.14% | -0.14% | 105% |  | 105% |  |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -27.55% | -25.76% | -25.76% | 102% | 94% |
| SVT12 | -4.20% | -3.78% | -3.78% | 94% | 101% |
| Overall | -15.88% | -14.77% | -14.77% | 98% | 97% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -18.05% | -17.29% | -17.30% | 105% | 108% |
| SVT12 | -1.01% | -1.00% | -1.01% | 99% | 106% |
| Overall | -9.53% | -9.14% | -9.15% | 102% | 107% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -17.13% | -17.31% | -17.25% | 100% | 98% |
| SVT12 | -1.16% | -1.04% | -1.05% | 90% | 97% |
| Overall | -9.15% | -9.18% | -9.15% | 95% | 98% |
| Overall RGB | -12.97% | -12.28% | -12.27% | 98% | 100% |

# CE-3.5 Simulation results for HBD CTC lossless

Simulation results have been conducted according to JVET-T2022. Presented run time estimates may not be accurate and can be updated.

| PQ | All Intra |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-3.5 |  | VTM11.0rc1 | CE-3.5 |  |
| PQ444 | 2.4 | 2.5 | -3.25% | 3.0 | 3.0 | -0.63% |
| PQ422 | 2.2 | 2.3 | -4.61% | 2.8 | 2.8 | -0.97% |
| Overall | 2.3 | 2.4 | -3.93% | 2.9 | 2.9 | -0.80% |
| Enc Time[%] | 120% |  |  | 125% |  |  |
| Dec Time[%] | 117% |  |  | 0% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-3.5 |  | VTM11.0rc1 | CE-3.5 |  |
| HLG444 | 1.9 | 1.9 | -2.21% | 2.1 | 2.1 | 0.16% |
| HLG422 | 1.8 | 1.8 | -3.62% | 2.0 | 2.0 | -0.43% |
| Overall | 1.8 | 1.9 | -2.91% | 2.0 | 2.0 | -0.13% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
| SVT | All Intra |  |  | Low delay B |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-3.5 |  | VTM11.0rc1 | CE-3.5 |  |
| SVT16 | 0.7 | 1.2 | -41.90% | 0.7 | 1.2 | -43.96% |
| SVT12 | 1.0 | 1.3 | -23.26% | 1.0 | 1.3 | -21.87% |
| Overall | 0.8 | 1.2 | -32.58% | 0.8 | 1.3 | -32.91% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

# Test CE-3.6

In test CE-3.7, a combination of Rice derivation design of CE-1.3 is combined with history-based Rice adjustment tested in CE-3.5. Detailed description of the CE-1.3 is available in JVET-U0050, detailed description of the CE-3.5 is provided in this document.

# CE-3.6 Simulation results for HBD CTC

Simulation results have been conducted according to JVET-T2018. Presented run time estimates may not be accurate and can be updated.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -1.01% | -0.85% | -0.82% | -0.95% | -0.79% | -0.77% | 105% | 104% |
| PQ422 | -1.10% | -0.99% | -1.00% | -1.04% | -0.92% | -0.91% | 101% | 97% |
| Overall | -1.05% | -0.92% | -0.91% | -0.99% | -0.86% | -0.84% | 103% | 101% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.08% | 0.08% | 0.06% | 0.13% | 0.13% | 0.12% | 100% | 98% |
| PQ422 | 0.13% | 0.16% | 0.17% | 0.20% | 0.22% | 0.24% | 99% | 96% |
| Overall | 0.10% | 0.12% | 0.12% | 0.16% | 0.18% | 0.18% | 99% | 97% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.04% | 0.00% | -0.01% | 0.02% | 0.05% | 0.05% | 106% | 106% |
| PQ422 | 0.02% | 0.07% | 0.09% | 0.09% | 0.13% | 0.15% | 107% | 108% |
| Overall | -0.01% | 0.03% | 0.04% | 0.05% | 0.09% | 0.10% | 107% | 107% |
| Overall PQ | -0.50% | -0.42% | -0.37% | -0.44% | -0.36% | -0.35% | 103% | 101% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.54% | -0.35% | -0.35% | 102% | 101% |
| HLG422 | -0.57% | -0.39% | -0.39% | 104% | 101% |
| Overall | -0.56% | -0.37% | -0.37% | 103% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.03% | 0.05% | 0.05% | 108% | 108% |
| HLG422 | -0.01% | 0.05% | 0.05% | 108% | 111% |
| Overall | 0.01% | 0.05% | 0.05% | 108% | 110% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| HLG422 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall HLG | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -27.36% | -25.84% | -25.85% | 122% | 89% |
| SVT12 | -4.16% | -3.75% | -3.75% | 108% | 102% |
| Overall | -15.76% | -14.79% | -14.80% | 115% | 96% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -18.15% | -17.43% | -17.46% | 134% | 112% |
| SVT12 | -0.77% | -0.77% | -0.78% | 119% | 117% |
| Overall | -9.46% | -9.10% | -9.12% | 127% | 115% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -17.21% | -17.39% | -17.35% | 115% | 94% |
| SVT12 | -0.90% | -0.76% | -0.77% | 104% | 100% |
| Overall | -9.05% | -9.08% | -9.06% | 110% | 97% |
| Overall RGB | -12.87% | -12.26% | -12.26% | 116% | 100% |

# CE-3.6 Simulation results for HBD CTC lossless

Simulation results have been conducted according to JVET-T2022. Presented run time estimates may not be accurate and can be updated.

| PQ | All Intra |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-3.6 |  | VTM11.0rc1 | CE-3.6 |  |
| PQ444 | 2.4 | 2.5 | -4.56% | 3.0 | 3.1 | -4.17% |
| PQ422 | 2.2 | 2.3 | -5.76% | 2.8 | 2.9 | -4.56% |
| Overall | 2.3 | 2.4 | -5.16% | 2.9 | 3.0 | -4.37% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
| HLG | All Intra |  |  | Low delay B |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-3.6 |  | VTM11.0rc1 | CE-3.6 |  |
| HLG444 | 1.9 | 2.0 | -4.05% | 2.1 | 0.0 | 0.00% |
| HLG422 | 1.8 | 1.9 | -5.27% | 2.0 | 0.0 | 0.00% |
| Overall | 1.8 | 1.9 | -4.66% | 2.0 | 0.0 | 0.00% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
| SVT | All Intra |  |  | Low delay B |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-3.6 |  | VTM11.0rc1 | CE-3.6 |  |
| SVT16 | 0.7 | 1.2 | -42.08% | 0.7 | 1.2 | -44.41% |
| SVT12 | 1.0 | 1.3 | -23.58% | 1.0 | 1.3 | -22.79% |
| Overall | 0.8 | 1.2 | -32.83% | 0.8 | 1.3 | -33.60% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

# References

[1]	Benjamin Bross, Jianle Chen, Shan Liu and Ye-Kui Wang, “Versatile Video Coding (Draft 10)” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET- S2001, 19th Meeting: Teleconference, 22 June – 01 July 2020.

[2] JVET-T0105, AHG12: On the Rice parameter derivation for high bit-depth coding, L. Pham Van, D. Rusanovskyy, M. Karczewicz (Qualcomm).

[3]	JVET-T2018, "Common test conditions for high bit depth and high bit rate video coding", A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu.

[4]	JVET-T2022, "CE on Entropy Coding for High Bit Depth and High Bit Rate Coding", A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy.

[5]	JVET-U0022, "CE: Summary Report on Entropy Coding for High Bit Depth and High Bit Rate Coding", A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).