| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0067-v1 |
| --- | --- |

| Title: | AHG8: On ALF clipping of high bit-depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer Yan Ye 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1-408-858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

It was observed that the coding gain of ALF is significantly reduced at high bit-depth coding. At the very high bit-rate, the reconstructed frame consists of lot of details and high frequency components. Applying ALF may remove the details/high frequencies which ultimately impact the coding performance. In order to avoid the over filtering, this contribution proposes to adjust the ALF clipping values in high bit-depth coding.

Following results are reported:

- PQ sequences: -0.15% (Y), -0.72% (U), -0.72% (V)

- HLG sequences: -0.02% (Y), -0.02% (U), -0.11% (V)

- SVT sequences: 0.00%(G), 0.00% (B), 0.00% (R)

# Problem Statement

The coding gain of ALF is reduced if the coding bit-depth/bit-rate is increased. It was observed that for the similar content the coding gain of very high bit-rate is significantly reduced as compared to low bit-rate. This is because of the nature of the ALF filtering process. At the very high bit-rate, the reconstructed frame consists of lot of details and high frequency components. Applying ALF (i.e. low pass filter) may remove the details/high frequencies which ultimately impact the coding performance.

In VVC, the clipping parameters are signaled in the bit-stream along with the filter coefficients. The clipping operation introduces non-linearity to make ALF more efficient by reducing the impact of neighbor sample values that are too different with the current sample value. The current VVC specification supports 4 clipping values ( shown in Table 1) based on the bit-depth.

Table 1: Clipping values supported by VVC

| BitDepth | clipIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 8 | 28 | 25 | 23 | 21 |
| 9 | 29 | 26 | 24 | 22 |
| 10 | 210 | 27 | 25 | 23 |
| 11 | 211 | 28 | 26 | 24 |
| 12 | 212 | 29 | 27 | 25 |
| 13 | 213 | 210 | 28 | 26 |
| 14 | 214 | 211 | 29 | 27 |
| 15 | 215 | 212 | 210 | 28 |
| 16 | 216 | 213 | 211 | 29 |

# Proposed methods

In order to reduce the over-filtering of high bit-depth and high bit-rate operating range, in the proposed method, the effect of over filtering process is reduced by adjusting the clipping values.

The clipping values are adjusted as follows:

c'k, l=ck.l≫clipShift

where, ck.l is the clipping values of current VVC specification and c'k, l is the clipping values of the proposed method.

In the proposed method, if non-linear ALF process is disabled in the APS syntax or the default filter set is used, the clipping values are not adjusted. Mathematically,

if ck, l<2BitDepth,

c'k, l=ck.l≫clipShift

The clipShift is derived as follows:

clipShift=BitDepth≤10?0 :BitDepth-9

# Simulation results

The proposed method is implemented on top of VTM-11.0. The performance of the proposed method is  evaluated based on the common test conditions suggested in [1].

Table 2, Table 3 and Table 4 show the results of the proposed method of PQ, HLG, and SVT sequences, respectively. As shown in Table 2, the proposed method can improve the compression performance of PQ sequences -0.15% based on PSNRY and -0.20% based on wPSNRY. For HLG sequences, the proposed method can achieve very minor coding gain (-0.02%, Y). For SVT sequence, there is no performance different because ALF does not have any noticeable coding gain for SVT sequences ( as shown in Table 7).

Table 2: Results of the proposed method of PQ sequences

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.04% | -0.16% | -0.24% | -0.03% | -0.13% | -0.20% | 100% | 102% |
| PQ422 | -0.04% | -0.17% | -0.28% | -0.02% | -0.13% | -0.22% | 100% | 102% |
| Overall | -0.04% | -0.17% | -0.26% | -0.02% | -0.13% | -0.21% | 100% | 102% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM-11.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.30% | -1.26% | -1.17% | -0.23% | -1.09% | -1.06% | 101% | 103% |
| PQ422 | -0.28% | -1.09% | -0.96% | -0.22% | -0.94% | -0.87% | 101% | 102% |
| Overall | -0.29% | -1.17% | -1.07% | -0.22% | -1.02% | -0.97% | 101% | 103% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM-11.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.27% | -1.22% | -1.16% | -0.20% | -1.06% | -1.05% | 101% | 103% |
| PQ422 | -0.26% | -1.14% | -1.02% | -0.19% | -0.97% | -0.92% | 101% | 102% |
| Overall | -0.26% | -1.18% | -1.09% | -0.19% | -1.02% | -0.99% | 101% | 102% |
| Overall PQ | -0.20% | -0.84% | -0.81% | -0.15% | -0.72% | -0.72% | 101% | 102% |

Table 3: Results of the proposed method of HLG sequences

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.01% | 0.01% | -0.01% | 100% | 103% |
| HLG422 | -0.01% | 0.02% | 0.01% | 100% | 102% |
| Overall | -0.01% | 0.02% | 0.00% | 100% | 102% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.04% | -0.02% | -0.12% | 101% | 102% |
| HLG422 | -0.03% | -0.03% | -0.13% | 101% | 102% |
| Overall | -0.04% | -0.03% | -0.13% | 101% | 102% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.02% | -0.04% | -0.20% | 98% | 99% |
| HLG422 | -0.01% | -0.04% | -0.22% | 100% | 100% |
| Overall | -0.02% | -0.04% | -0.21% | 99% | 100% |
| Overall HLG | -0.02% | -0.02% | -0.11% | 100% | 101% |

Table 4: Results of the proposed method of SVT sequences

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 105% |
| SVT12 | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 104% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.01% | 0.00% | 0.00% | 101% | 105% |
| SVT12 | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Overall | -0.01% | 0.00% | 0.00% | 101% | 104% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 101% | 105% |
| SVT12 | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 104% |
| Overall RGB | 0.00% | 0.00% | 0.00% | 101% | 104% |

# Additional results

Table 5, Table 6, and Table 7 shows the simulation results of disabling ALF of PQ, HLG, and SVT sequences. The common test conditions described in high bit-depth CTC are used in this evaluation [1]. In overall, ALF can achieve 0.90% (Y), 1.82% (U) and 2.36% (V) coding gain of PQ sequences based on PSNR value. The gain of ALF for HLG sequences are 1.57% (Y), 3.92% (U), and 4.75% (V). The gain of ALF for SVT sequences are 0.01% (Y), 0.00% (U), and 0.00%(V) which are not noticeable.

Table 5: Performance of ALF of  PQ sequences

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.14% | 0.41% | 0.81% | 0.10% | 0.39% | 0.80% | 99% | 90% |
| PQ422 | 0.13% | 0.40% | 0.71% | 0.10% | 0.37% | 0.70% | 99% | 92% |
| Overall | 0.14% | 0.41% | 0.76% | 0.10% | 0.38% | 0.75% | 99% | 91% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM-11.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.61% | 3.43% | 3.50% | 1.52% | 3.10% | 3.35% | 99% | 83% |
| PQ422 | 1.47% | 2.41% | 3.54% | 1.38% | 2.19% | 3.36% | 100% | 85% |
| Overall | 1.54% | 2.92% | 3.52% | 1.45% | 2.65% | 3.36% | 99% | 84% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM-11.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 1.31% | 3.00% | 3.00% | 1.24% | 2.71% | 2.87% | 100% | 83% |
| PQ422 | 1.14% | 2.37% | 3.28% | 1.07% | 2.14% | 3.05% | 99% | 85% |
| Overall | 1.22% | 2.69% | 3.14% | 1.15% | 2.42% | 2.96% | 100% | 84% |
| Overall PQ | 0.97% | 2.01% | 2.48% | 0.90% | 1.82% | 2.36% | 99% | 86% |

Table 6: Performance of ALF of  HLG sequences

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.24% | 1.57% | 2.15% | 100% | 91% |
| HLG422 | 0.24% | 0.85% | 1.12% | 99% | 92% |
| Overall | 0.24% | 1.21% | 1.63% | 99% | 91% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 2.25% | 6.13% | 7.37% | 100% | 80% |
| HLG422 | 2.17% | 4.23% | 4.92% | 100% | 83% |
| Overall | 2.21% | 5.18% | 6.14% | 100% | 82% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 2.32% | 6.33% | 7.67% | 99% | 79% |
| HLG422 | 2.22% | 4.42% | 5.27% | 99% | 83% |
| Overall | 2.27% | 5.38% | 6.47% | 99% | 81% |
| Overall HLG | 1.57% | 3.92% | 4.75% | 99% | 85% |

Table 7: Performance of ALF of  SVT sequences

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-11.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| SVT12 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| SVT12 | 0.02% | 0.00% | 0.00% | 100% | 98% |
| Overall | 0.01% | 0.00% | 0.00% | 100% | 99% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM-11.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| SVT12 | 0.02% | 0.00% | 0.00% | 100% | 98% |
| Overall | 0.01% | 0.00% | 0.00% | 100% | 99% |
| Overall RGB | 0.01% | 0.00% | 0.00% | 100% | 99% |

# Conclusions

This contribution proposes to adjust the clipping values of ALF for high bit-depth coding. It is reported that the proposed method can achieve -0.15% (Y) coding gain based on PSNR metric for PQ sequences.

# References

A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-T2018, Oct 2020.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Specification changes

Following is the specification changes as compared to current VVC. The highlighted portion shows the changes as compared to current version.

#### 8.8.5.2 Coding tree block filtering process for luma samples

Inputs of this process are:

- a reconstructed luma picture sample array recPicture prior to the adaptive loop filtering process,

- a filtered reconstructed luma picture sample array alfPictureL,

- a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top-left sample of the current picture.

Output of this process is the modified filtered reconstructed luma picture sample array alfPictureL.

The derivation process for filter index clause 8.8.5.3 is invoked with the location ( xCtb, yCtb ) and the reconstructed luma picture sample array recPicture as inputs, and filtIdx[ x ][ y ] and transposeIdx[ x ][ y ] with x, y = 0..CtbSizeY − 1 as outputs.

The variable clipShift is derived as follows:

  - if BitDepth <= 10, clipShift is set to 0

  - Otherwise, clipShift is set to (BitDepth -9)

For the derivation of the filtered reconstructed luma samples alfPictureL[ xCtb + x ][ yCtb + y ], each reconstructed luma sample inside the current luma coding tree block recPicture[ xCtb + x ][ yCtb + y ] is filtered as follows with x, y = 0..CtbSizeY − 1:

  - The array of luma filter coefficients f[ j ] and the array of luma clipping values c[ j ] corresponding to the filter specified by filtIdx[ x ][ y ] is derived as follows with j = 0..11:

  - If AlfCtbFiltSetIdxY[ xCtb  >>  CtbLog2SizeY ][ yCtb  >>  CtbLog2SizeY ] is less than 16, the following applies:

i = AlfCtbFiltSetIdxY[ xCtb  >>  CtbLog2SizeY ][ yCtb  >>  CtbLog2SizeY ]	(1424)

f[ j ] = AlfFixFiltCoeff[ AlfClassToFiltMap[ i ][ filtIdx[ x ][ y ] ] ][ j ]	(1425)

c[ j ] = 2BitDepth		(1426)

  - Otherwise (AlfCtbFiltSetIdxY[ xCtb  >>  CtbLog2SizeY ][ yCtb  >>  CtbLog2SizeY ] is greater than or equal to 16, the following applies:

i = sh_alf_aps_id_luma[ AlfCtbFiltSetIdxY[ xCtb  >>  CtbLog2SizeY ][ yCtb  >>  CtbLog2SizeY ] − 16 ]
		(1427)

f[ j ] = AlfCoeffL[ i ][ filtIdx[ x ][ y ] ][ j ]		(1428)

c[ j ] = AlfClipL[ i ][ filtIdx[ x ][ y ] ][ j ]		(1429)

if (c[j] < 2BitDepth)

c[j] = c[j] >> clipShift

  - The luma filter coefficients and clipping values index idx are derived depending on transposeIdx[ x ][ y ] as follows:

  - If transposeIdx[ x ][ y ] is equal to 1, the following applies:

idx[ ] = { 9, 4, 10, 8, 1, 5, 11, 7, 3, 0, 2, 6 }		(1430)

  - Otherwise, if transposeIdx[ x ][ y ] is equal to 2, the following applies:

idx[ ] = { 0, 3, 2, 1, 8, 7, 6, 5, 4, 9, 10, 11 }		(1431)

  - Otherwise, if transposeIdx[ x ][ y ] is equal to 3, the following applies:

idx[ ] = { 9, 8, 10, 4, 3, 7, 11, 5, 1, 0, 2, 6 }		(1432)

  - Otherwise, the following applies:

idx[ ] = { 0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11 }		(1433)

      - 8.8.5.4. Coding tree block filtering process for chroma samples

Inputs of this process are:

- a reconstructed chroma picture sample array recPicture prior to the adaptive loop filtering process,

- a filtered reconstructed chroma picture sample array alfPicture,

- a chroma location ( xCtbC, yCtbC ) specifying the top-left sample of the current chroma coding tree block relative to the top-left sample of the current picture,

- an alternative chroma filter index altIdx.

Output of this process is the modified filtered reconstructed chroma picture sample array alfPicture.

The width and height of the current chroma coding tree block ctbWidthC and ctbHeightC is derived as follows:

ctbWidthC = CtbSizeY / SubWidthC		(1471)

ctbHeightC = CtbSizeY / SubHeightC		(1472)

The variable clipShift is derived as follows:

  - if BitDepth <= 10, clipShift is set to 0

  - Otherwise, clipShift is set to (BitDepth -9)

For the derivation of the filtered reconstructed chroma samples alfPicture[ xCtbC + x ][ yCtbC + y ], each reconstructed chroma sample inside the current chroma coding tree block recPicture[ xCtbC + x ][ yCtbC + y ] is filtered as follows with x = 0..ctbWidthC − 1, y = 0..ctbHeightC − 1:

  - The variables clipLeftPos, clipRightPos, clipTopPos, clipBottomPos, clipTopLeftFlag and clipBotRightFlag are derived by invoking the ALF boundary position derivation process as specified in clause 8.8.5.5 with ( xCtbC * SubWidthC, yCtbC * SubHeightC ), ( x * SubWidthC, y *SubHeightC ) and the variable vbOffset set equal to 2 * SubHeightC as inputs.

  - The locations ( hx + i, y + j, vy + j ) for each of the corresponding chroma samples ( x, y ) inside the given array recPicture of chroma samples with i, j = −2..2 are derived as follows:

hx + i, y + j = Clip3( 0, pps_pic_width_in_luma_samples / SubWidthC − 1, xCtbC + x + i )	(1473)

vy + j = Clip3( 0, pps_pic_height_in_luma_samples / SubHeightC − 1, yCtbC + y + j )	(1474)

  - The location ( hx + i, y + j, vy + j ) is modified by invoking the ALF sample padding process as specified in clause 8.8.5.6 with ( xCtbC * SubWidthC, yCtbC * SubHeightC ), ( hx + i, y + j, vy + j ), the variable isChroma set equal to 1, clipLeftPos, clipRightPos, clipTopPos, clipBottomPos, clipTopLeftFlag and clipBotRightFlag as inputs.

  - The variable applyAlfLineBufBoundary is derived as follows:

  - If the bottom boundary of the current coding tree block is the bottom boundary of the picture and pps_pic_height_in_luma_samples − ( yCtbC * SubHeightC ) < CtbSizeY − 4, applyAlfLineBufBoundary is set equal to 0.

  - Otherwise, applyAlfLineBufBoundary is set equal to 1.

  - The vertical sample position offsets y1, y2 and the variable alfShiftC are specified in Table 46 according to the vertical chroma sample position y and applyAlfLineBufBoundary.

  - The variable curr is derived as follows:

curr = recPicture[ hx, y ][ vy ]		(1475)

  - The array of chroma filter coefficients f[ j ] and the array of chroma clipping values c[ j ] is derived as follows with j = 0..5:

f[ j ] = AlfCoeffC[ sh_alf_aps_id_chroma ][ altIdx ][ j ]	(1476)

c[ j ] = AlfClipC[ sh_alf_aps_id_chroma ][ altIdx ][ j ]	(1477)

if (c[j] < 2BitDeptth)

c[j] = c[j] >> clipShift

  - The variable sum is derived as follows:

sum = f[ 0 ] * ( Clip3( −c[ 0 ], c[ 0 ], recPicture[ hx, y + y2 ][ vy + y2 ] − curr ) +
		Clip3( −c[ 0 ], c[ 0 ], recPicture[ hx, y − y2 ][ vy − y2 ] − curr ) ) +
	f[ 1 ] * ( Clip3( −c[ 1 ], c[ 1 ], recPicture[ hx + 1, y + y1 ][ vy + y1 ] − curr ) +
		Clip3( −c[ 1 ], c[ 1 ], recPicture[ hx − 1, y − y1 ][ vy − y1 ] − curr ) ) +
	f[ 2 ] * ( Clip3( −c[ 2 ], c[ 2 ], recPicture[ hx, y + y1 ][ vy + y1 ] − curr ) +
		Clip3( −c[ 2 ], c[ 2 ], recPicture[ hx, y − y1 ][ vy − y1 ] − curr ) ) +	(1478)
	f[ 3 ] * ( Clip3( −c[ 3 ], c[ 3 ], recPicture[ hx − 1, y + y1 ][ vy + y1 ] − curr ) +
		Clip3( −c[ 3 ], c[ 3 ], recPicture[ hx + 1, y − y1 ][ vy − y1 ] − curr ) ) +
	f[ 4 ] * ( Clip3( −c[ 4 ], c[ 4 ], recPicture[ hx + 2, y ][ vy ] − curr ) +
		Clip3( −c[ 4 ], c[ 4 ], recPicture[ hx − 2, y ][ vy ] − curr ) ) +
	f[ 5 ] * ( Clip3( −c[ 5 ], c[ 5 ], recPicture[ hx + 1, y ][ vy ] − curr ) +
		Clip3( −c[ 5 ], c[ 5 ], recPicture[ hx − 1, y ][ vy ] − curr ) )

sum = curr + ( ( sum + ( 1  <<  ( alfShiftC − 1 ) ) )  >>  alfShiftC )	(1479)

  - The modified filtered reconstructed chroma picture sample alfPicture[ xCtbC + x ][ yCtbC + y ] is derived as follows:

alfPicture[ xCtbC + x ][ yCtbC + y ] = Clip3( 0, ( 1  <<  BitDepth ) − 1, sum )	(1480)