| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0070r1 |
| --- | --- |

| Title: | CE Related: On signaling and encoder optimization for Rice parameter derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy Muhammed Coban Marta Karczewicz | Email: | dmytror@qti.qualcomm.com mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc |  |  |

_____________________________

# Abstract

This contribution presents additional results for CE-3.5 on High Bit Depth and High Bit Rate Coding. Proposed in this contribution a content adaptive, encoder side parameters derivation algorithm and simplified slice level signaling (removal of Rice derivation parameter) reportedly provide for currently completed classes and configurations additional 0.04% bd-rate gain for all PQ classes, and around 0.1% gain for HLG AI and LDB classes of High Bit Depth CTC. In the lossless coding configuration (required by CE), additional 1.32% and 1.82% of bd-rate gains for AI PQ and AI HLG classes, respectively, are reported. Comparing to the VVC anchor, following gains are reported for HBD/HBR CTC:

- 1.0 %, 0.6 % and 15.9 % in bdrateY in PQ, HLG and SVT for AI

- 0.1 %, 0.0 % and 9.5 % in bdrateY in PQ, HLG and SVT for LDB

- 0.2 %, xx% and xx % in bdrateY in PQ, HLG and SVT for RA

In lossless coding mode, following gains are reported:

- 5.2 %, 4.7% and 32.6% in PQ, HLG and SVT for AI

- 2.3%, xx% and 32.9 % in PQ, HLG and SVT for LDB.

It is proposed to include provided results to the CE-report as additional test for CE-3.5.

# Introduction

Core Experiment on High Bit Depth and High Bit Rate coding was established at the 20th JVET meeting. CE3.5 is one of the combined tests based on the CE-1.2 and CE-1.4/1.5 formulated at T4 (Dec. 16th). Due to relatively long run time for CTC simulation and short time allowed for combined tests testing, an improved encoder side algorithm became available and its description and simulation results are provided as additional test in this contribution.

# Proposed algorithm

In test CE-3.5, derivation of variable shift for template output locSumAbs normalization is dependent on scale_rice parameter, which can take value 0, 1 or 2 and is adapting to the bitdepth of input data and level of the quantization error.

scalledCoef = (localSumAbs * scale_rice) >> 3

shift = (scallPreCoef == 0) ? 0 : Floor( Log2( scalledCoef) )

locSumAbs = locSumAbs >> shift

To avoid dependency of the Rice derivation method on syntax ellements defining bidepth in SPS and QP paramters, and allow encoder flexibility at the slice level, a dedicated syntax ellement sh_scale_rice was introduced in CE-3.5. Encoder was deriving scale_rice as a function of bitdepth and QP.

Additionally, CE-3.5 utilizes a history counter StatCoeff[cIdx] that accumulate Rice value estimates for high frequency transform coefficients and provide localSumAbs adjustments for coefficients located at the boundary of coded TU, as shown in Figure 1. History counter StatCoeff[cIdx] is being reset at start of each CTU with a default value sh_rice_init which is determined at the encoder side and signalled at the slice header.

C3

C1

C2

C2

Figure 1. Visualization of the classification concept utilized in CE-3.5, with Rice derivation templates for samples in C2 and C3 areas are being adjusted with value from history counter.

It is assessed that StatCoeff[cIdx] statistics provide sufficient information to derive scale_rice parameter, thus omitting dependency on bitdepth and QP.

In this contribution, is proposed:

- Collect StatCoeff[cIdx] statistics for each CTU at the encoder side.

- Derive default value for StatCoeff[cIdx] as average counter value of previously coded frame and signal it to the decoder at the slice header.

- Remove signaling of sh_scale_rice.

- Derive ScaleRice parameter from default value sh_rice_init for StatCoeff[cIdx] at the decoder side.

| if( sps_high_bit_depth ) { |  |
| --- | --- |
| sh_scale_rice | ue(v) |
| sh_rice_init | ue(v) |
| } |  |

sh_scale_rice specifies the scale value utilized for derivation of the Rice parameter, the value of sh_scale_rice shall be in the range of 1 to 2, inclusive. When sh_scale_rice is not present, it is inferred to be equal to 0.

sh_rice_init specifies default value StatCoeff[cIdx] initialization. The value of sh_rice_init shall be in the range of 0 to 15, inclusive. When sh_rice_init is not present, it is inferred to be equal to 0.

Variable ScaleRice is derived as follows:

ScaleRice = (sh_rice_init >= 4) ? 2 : 1

# Simulation results for HBD CTC

Simulation results for proposed test have been conducted according to JVET-T2018. Presented run time estimates may not be accurate and can be updated.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.94% | -0.83% | -0.83% | -0.95% | -0.84% | -0.83% | 106% | 110% |
| PQ422 | -1.04% | -0.97% | -1.00% | -1.05% | -0.97% | -1.01% | 104% | 104% |
| Overall | -0.99% | -0.90% | -0.92% | -1.00% | -0.90% | -0.92% | 105% | 107% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.08% | -0.09% | -0.12% | -0.08% | -0.09% | -0.10% | 102% | 101% |
| PQ422 | -0.08% | -0.10% | -0.11% | -0.09% | -0.10% | -0.10% | 101% | 101% |
| Overall | -0.08% | -0.10% | -0.11% | -0.08% | -0.10% | -0.10% | 102% | 101% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.19% | -0.18% | -0.18% | -0.20% | -0.18% | -0.19% | 102% | 102% |
| PQ422 | -0.20% | -0.19% | -0.20% | -0.21% | -0.20% | -0.20% | 104% | 105% |
| Overall | -0.20% | -0.18% | -0.19% | -0.20% | -0.19% | -0.19% | 103% | 104% |
| Overall PQ | -0.56% | -0.52% | -0.48% | -0.57% | -0.52% | -0.53% | 104% | 105% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.52% | -0.38% | -0.39% | 105% | 110% |
| HLG422 | -0.58% | -0.43% | -0.43% | 105% | 106% |
| Overall | -0.55% | -0.41% | -0.41% | 105% | 108% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.01% | 0.00% | -0.01% | 107% | 104% |
| HLG422 | -0.03% | 0.00% | 0.00% | 107% | 108% |
| Overall | -0.02% | 0.00% | 0.00% | 107% | 106% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 |  |  |  |  |  |
| HLG422 |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Overall HLG |  |  |  |  |  |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -27.53% | -25.72% | -25.72% | 118% | 96% |
| SVT12 | -4.18% | -3.77% | -3.78% | 106% | 106% |
| Overall | -15.86% | -14.75% | -14.75% | 112% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 | -18.02% | -17.24% | -17.26% | 126% | 117% |
| SVT12 | -1.01% | -1.02% | -1.03% | 116% | 118% |
| Overall | -9.52% | -9.13% | -9.14% | 121% | 118% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrR | psnrG | psnrB | EncT | DecT |
| SVT16 |  |  |  |  |  |
| SVT12 |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Overall RGB |  |  |  |  |  |

# Simulation results for HBD CTC lossless

Simulation results for proposed test have been conducted according to JVET-T2022.

Presented run time estimates may not be accurate and can be updated.

| PQ | All Intra |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-3.5-Related |  | VTM11.0rc1 | CE-3.5-Related |  |
| PQ444 | 2.4 | 2.5 | -4.62% | 3.0 | 3.1 | -2.16% |
| PQ422 | 2.2 | 2.3 | -5.80% | 2.8 | 2.9 | -2.35% |
| Overall | 2.3 | 2.4 | -5.21% | 2.9 | 3.0 | -2.25% |
| Enc Time[%] | 118% |  |  | 124% |  |  |
| Dec Time[%] | 107% |  |  | 118% |  |  |
| HLG | All Intra |  |  | Low delay B |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-3.5-Related |  | VTM11.0rc1 | CE-3.5-Related |  |
| HLG444 | 1.9 | 2.0 | -4.09% | 2.1 |  |  |
| HLG422 | 1.8 | 1.9 | -5.28% | 2.0 |  |  |
| Overall | 1.8 | 1.9 | -4.69% | 2.0 |  |  |
| Enc Time[%] | 121% |  |  |  |  |  |
| Dec Time[%] | 118% |  |  |  |  |  |
| SVT | All Intra |  |  | Low delay B |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM11.0rc1 | CE-3.5-Related |  | VTM11.0rc1 | CE-3.5-Related |  |
| SVT16 | 0.7 | 1.2 | -41.91% | 0.7 | 1.2 | -43.96% |
| SVT12 | 1.0 | 1.3 | -23.28% | 1.0 | 1.3 | -21.87% |
| Overall | 0.8 | 1.2 | -32.60% | 0.8 | 1.3 | -32.92% |
| Enc Time[%] | 105% |  |  | 102% |  |  |
| Dec Time[%] | 203% |  |  | 88% |  |  |

# References

[1]	Benjamin Bross, Jianle Chen, Shan Liu and Ye-Kui Wang, “Versatile Video Coding (Draft 10)” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET- S2001, 19th Meeting: Teleconference, 22 June – 01 July 2020.

[2] JVET-T0105, AHG12: On the Rice parameter derivation for high bit-depth coding, L. Pham Van, D. Rusanovskyy, M. Karczewicz (Qualcomm).

[3]	JVET-T2018, "Common test conditions for high bit depth and high bit rate video coding", A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu.

[4]	JVET-T2022, "CE on Entropy Coding for High Bit Depth and High Bit Rate Coding", A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy.

[5]	JVET-U0022, "CE: Summary Report on Entropy Coding for High Bit Depth and High Bit Rate Coding", A. Browne, T. Hashimoto, H.-J. Jhu, D. Rusanovskyy.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).