| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0072 |
| --- | --- |

| Title: | AHG8: Tool Off Tests for High Bit-depth |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Steve Keating Kenji Kondo | Tel: Email: | steve.keating@sony.com kenji.kondo@sony.com |
| Source: | Sony |  |  |

_____________________________

# Abstract

This contribution reports the test results of various tools at high bit depths / rates. The reported gains and run times may help in considering disabling tools for future high bit depth / high bit rate simulations.

# Introduction

At the previous meeting, AHG13 reported tool off test results for the Version 1 4:2:0 CTC JVET-M1010 as well as for non-4:2:0 colour format contents [1]. This contribution reports tool off test results of various tools at high bit depths / rates according to the JVET-T2018 CTC [2].

# Tests

A selection of VTM tools was tested at either AI, LDB or RA; tests were conducted on VTM-10.2 software with VTM configuration by switching off or on specific tool(s) either in configuration files or in macros. Only 4:4:4 results are reported for PQ/HLG for LDB and RA, only 12-bit results are reported for SVT for LDB and RA, 4:4:4 and 4:2:2 PQ/HLG and 12/16-bit SVT results are reported for AI.

The tested tools are listed in the tables below.

## Tools included in VTM (Tool off test vs VTM Anchor)

Table 1 List of adoptions included in VTM (Tool off test (unless specified) vs VTM anchor)

| Tool Name | Acronym | Document reference(s) | AI | LDB | RA |
| --- | --- | --- | --- | --- | --- |
| Cross-component linear model | LMChroma | JVET-O1124, JVET-Q0500, JVET-Q0194, | X |  |  |
| Adaptive colour transform | ACT | JVET-P0517, JVET-Q0820, JVET-Q0353, JVET-Q0512, | X |  |  |
| Multiple transform set | MTS | JVET-O0294, JVET-O0474, JVET-O0541, JVET-Q0055, JVET-Q0516, | X |  |  |
| Joint coding of chrominance residuals | JCbCr | JVET-N0054 | X |  |  |
| Matrix based intra prediction | MIP | JVET-P0054, JVET-P0199, JVET-P0803, JVET-Q0446 | X |  |  |
| Intra sub-partitioning | ISP | JVET-O0106, JVET-O0341, JVET-O0502 | X |  |  |
| Low frequency non-separable transform * | LFNST | JVET-P1026, JVET-P0350, JVET-Q0784, | X |  |  |
| Adaptive loop filter Cross component adaptive loop filter | ALF_CCALF | JVET-P0162, JVET-P0164, JVET-P0505, JVET-P0554, JVET-P0665, JVET-P1038, JVET-Q0249, JVET-Q0150, JVET-Q0495, JVET-Q0795; JVET-Q0795 | X | X | X |
| Luma mapping with chroma scaling ** | LMCS | JVET-P0254, JVET-P0371 | X |  |  |
| Multi-reference line prediction | MRL | JVET-P0418 | X |  |  |
| Dependent quantization | DepQuant | JVET-P0170, JVET-Q0433, | X | X | X |
| Chroma separate tree | DualITree | JVET-P0063, JVET-P0406 | X | X | X |
| Adaptive motion vector resolution | IMV | JVET-O0057 |  | X | X |
| Combined intra/inter prediction | CIIP | JVET-O0108, JVET-O0681 |  | X | X |
| Bi-predictive with CU weights | BCW | JVET-P0280 |  | X | X |
| Merge with MVD | MMVD | JVET-P1023 |  | X | X |
| Geometric partition mode | Geo | JVET-Q0806 |  | X | X |
| Sub-block transform | SBT | JVET-P1026 |  | X | X |
| Affine motion model Prediction refinement using optical flow | Affine-PROF | JVET-O0070; JVET-P0409, JVET-P0057, JVET-P0154, JVET-P0491, JVET-P0653 |  |  | X |
| Symmetric motion vector difference | SMVD | JVET-O0284, JVET-O0414, JVET-O0567, JVET-O0572 |  |  | X |
| Bi-directional optical flow | BIO | JVET-P0091, JVET-P0519, JVET-P1023, JVET-Q0128, |  |  | X |
| Decoder motion vector refinement | DMVR | JVET-O0297, JVET-O0590, JVET-O0634 |  |  | X |

* Tool on test as LFNST is disabled in CTC

** Tool on test for SVT content as LMCS is disabled for SVT in CTC.

# Results

The results of the tests are summarized in Table 3-5 below. The attached spreadsheet provides some additional data. Scatter plots (Figs. 1-9) are also provided for the tested tools in each configuration, comparing PSNR-Y,U,V or PSNR-G,B,R based BD-rate on the Y-axis vs.  Enc runtime ratio.

Table 3. Simulation results in all intra configuration (AI). (VTM anchor)

|  |  | AI HDR PQ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | psnrY | psnrU | psnrV | EncT | DecT |
| LMChroma | PQ444 | 1.74% | 2.90% | 2.39% | 90% | 100% |
|  | PQ422 | 1.25% | 1.81% | 1.73% | 96% | 99% |
|  | Overall | 1.49% | 2.36% | 2.06% | 93% | 100% |
| MTS | PQ444 | 1.28% | 0.43% | 0.43% | 77% | 99% |
|  | PQ422 | 1.31% | 0.52% | 0.50% | 64% | 99% |
|  | Overall | 1.30% | 0.47% | 0.46% | 70% | 99% |
| JCbCr | PQ444 | 0.82% | 0.14% | 0.06% | 89% | 100% |
|  | PQ422 | 0.67% | -0.09% | -0.17% | 94% | 101% |
|  | Overall | 0.75% | 0.03% | -0.05% | 92% | 101% |
| MIP | PQ444 | 0.17% | 0.08% | 0.10% | 90% | 101% |
|  | PQ422 | 0.21% | 0.12% | 0.11% | 85% | 101% |
|  | Overall | 0.19% | 0.10% | 0.10% | 87% | 101% |
| ISP | PQ444 | 0.02% | 0.04% | 0.05% | 96% | 98% |
|  | PQ422 | 0.03% | 0.04% | 0.04% | 95% | 99% |
|  | Overall | 0.03% | 0.04% | 0.04% | 95% | 99% |
| LFNST * | PQ444 | -0.33% | -0.02% | 0.03% | 93% | 100% |
|  | PQ422 | -0.15% | -0.14% | 0.12% | 96% | 100% |
|  | Overall | -0.24% | -0.08% | 0.07% | 94% | 100% |
| ALF_CCALF | PQ444 | 0.10% | 0.39% | 0.80% | 97% | 91% |
|  | PQ422 | 0.10% | 0.37% | 0.70% | 96% | 91% |
|  | Overall | 0.10% | 0.38% | 0.75% | 97% | 91% |
| LMCS ** | PQ444 | -5.08% | -4.32% | -4.74% | 92% | 94% |
|  | PQ422 | -4.69% | -3.86% | -4.19% | 95% | 94% |
|  | Overall | -4.88% | -4.09% | -4.46% | 93% | 94% |
| MRL | PQ444 | 0.20% | 0.08% | 0.09% | 99% | 100% |
|  | PQ422 | 0.21% | 0.11% | 0.11% | 100% | 101% |
|  | Overall | 0.21% | 0.09% | 0.10% | 100% | 101% |
| DepQuant | PQ444 | -0.24% | 2.85% | 2.60% | 76% | 100% |
|  | PQ422 | 0.31% | 3.38% | 3.42% | 80% | 101% |
|  | Overall | 0.04% | 3.11% | 3.01% | 78% | 101% |
| DualITree | PQ444 | 1.02% | 1.08% | 1.07% | 80% | 89% |
|  | PQ422 | 0.63% | 0.48% | 0.54% | 102% | 94% |
|  | Overall | 0.82% | 0.78% | 0.80% | 91% | 91% |

*Tool on test

**A bug was reported with LMCS in VTM10.2, the test has been repeated with VTM11.0 and zero loss/gain was found and Enc/Dec run times were 100%

|  |  | AI HDR HLG |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | psnrY | psnrU | psnrV | EncT | DecT |
| LMChroma | HLG444 | 1.52% | 1.65% | 1.92% | 90% | 101% |
|  | HLG422 | 0.48% | 0.56% | 0.62% | 95% | 99% |
|  | Overall | 1.00% | 1.10% | 1.27% | 93% | 100% |
| MTS | HLG444 | 1.77% | 0.50% | 0.50% | 77% | 99% |
|  | HLG422 | 1.72% | 0.58% | 0.57% | 66% | 98% |
|  | Overall | 1.75% | 0.54% | 0.54% | 72% | 99% |
| JCbCr | HLG444 | 0.02% | 0.00% | -0.01% | 87% | 100% |
|  | HLG422 | 0.02% | -0.01% | -0.02% | 92% | 99% |
|  | Overall | 0.02% | -0.01% | -0.01% | 89% | 100% |
| MIP | HLG444 | 0.19% | 0.09% | 0.09% | 90% | 101% |
|  | HLG422 | 0.19% | 0.11% | 0.11% | 85% | 100% |
|  | Overall | 0.19% | 0.10% | 0.10% | 88% | 100% |
| ISP | HLG444 | 0.02% | 0.01% | 0.01% | 96% | 99% |
|  | HLG422 | 0.02% | 0.01% | 0.01% | 93% | 98% |
|  | Overall | 0.02% | 0.01% | 0.01% | 95% | 99% |
| LFNST * | HLG444 | -0.54% | 0.06% | 0.08% | 89% | 100% |
|  | HLG422 | -0.43% | 0.09% | 0.12% | 91% | 100% |
|  | Overall | -0.48% | 0.08% | 0.10% | 90% | 100% |
| ALF_CCALF | HLG444 | 0.24% | 1.57% | 2.15% | 98% | 91% |
|  | HLG422 | 0.24% | 0.85% | 1.12% | 98% | 92% |
|  | Overall | 0.24% | 1.21% | 1.63% | 98% | 91% |
| LMCS | HLG444 | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | HLG422 | 0.00% | 0.00% | 0.00% | 100% | 99% |
|  | Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| MRL | HLG444 | 0.10% | 0.03% | 0.03% | 99% | 100% |
|  | HLG422 | 0.10% | 0.04% | 0.04% | 99% | 99% |
|  | Overall | 0.10% | 0.04% | 0.03% | 99% | 100% |
| DepQuant | HLG444 | 0.25% | 4.71% | 4.82% | 76% | 102% |
|  | HLG422 | 0.98% | 4.93% | 5.21% | 79% | 105% |
|  | Overall | 0.61% | 4.82% | 5.01% | 77% | 104% |
| DualITree | HLG444 | 2.73% | 0.63% | 0.82% | 71% | 91% |
|  | HLG422 | 1.17% | 0.75% | 0.78% | 93% | 100% |
|  | Overall | 1.95% | 0.69% | 0.80% | 82% | 95% |

|  |  | AI SVT RGB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | psnrG | psnrB | psnrR | EncT | DecT |
| LMChroma | SVT16 | -0.37% | 2.06% | 2.11% | 95% | 101% |
|  | SVT12 | -0.16% | 1.73% | 1.74% | 96% | 100% |
|  | Overall | -0.27% | 1.89% | 1.92% | 95% | 101% |
| ACT ** | SVT16 | 1.26% | -0.42% | -0.46% | 72% | 119% |
|  | SVT12 | 0.77% | -0.08% | -0.08% | 65% | 115% |
|  | Overall | 1.02% | -0.25% | -0.27% | 68% | 117% |
| MTS | SVT16 | 1.04% | 0.57% | 0.56% | 66% | 98% |
|  | SVT12 | 0.98% | 0.28% | 0.28% | 66% | 99% |
|  | Overall | 1.01% | 0.43% | 0.42% | 66% | 98% |
| JCbCr | SVT16 | 0.00% | 0.00% | 0.00% | 88% | 100% |
|  | SVT12 | -0.01% | 0.00% | 0.00% | 88% | 100% |
|  | Overall | -0.01% | 0.00% | 0.00% | 88% | 100% |
| MIP | SVT16 | 0.58% | 0.30% | 0.30% | 79% | 99% |
|  | SVT12 | 0.46% | 0.17% | 0.16% | 80% | 100% |
|  | Overall | 0.52% | 0.23% | 0.23% | 79% | 100% |
| ISP | SVT16 | 0.00% | 0.04% | 0.05% | 97% | 100% |
|  | SVT12 | 0.00% | 0.01% | 0.01% | 97% | 99% |
|  | Overall | 0.00% | 0.03% | 0.03% | 97% | 100% |
| LFNST * | SVT16 | -0.51% | -0.19% | -0.19% | 68% | 100% |
|  | SVT12 | -0.57% | 0.12% | 0.14% | 69% | 99% |
|  | Overall | -0.54% | -0.03% | -0.02% | 68% | 99% |
| ALF_CCALF | SVT16 | 0.00% | 0.00% | 0.00% | 98% | 99% |
|  | SVT12 | 0.30% | 0.00% | 0.00% | 99% | 99% |
|  | Overall | 0.15% | 0.00% | 0.00% | 98% | 99% |
| LMCS * | SVT16 | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | SVT12 | -1.66% | -0.02% | -0.02% | 105% | 100% |
|  | Overall | -0.83% | -0.01% | -0.01% | 103% | 100% |
| MRL | SVT16 | 0.04% | 0.03% | 0.03% | 116% | 100% |
|  | SVT12 | -0.10% | -0.02% | -0.02% | 108% | 100% |
|  | Overall | -0.03% | 0.01% | 0.01% | 112% | 100% |
| DepQuant | SVT16 | 2.04% | 2.45% | 2.52% | 94% | 98% |
|  | SVT12 | 1.97% | 2.75% | 2.81% | 85% | 99% |
|  | Overall | 2.00% | 2.60% | 2.67% | 89% | 98% |
| DualITree | SVT16 | 0.00% | 0.00% | 0.00% | 116% | 100% |
|  | SVT12 | -0.12% | -0.03% | -0.03% | 108% | 99% |
|  | Overall | -0.06% | -0.02% | -0.02% | 112% | 99% |

* Tool on test

** Note that enabling ACT requires DualITree to be disabled.

Table 4. Simulation results in low delay B configuration (LDB). (VTM anchor)

|  | LDB HDR PQ444 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| IMV | 0.11% | 0.08% | 0.09% | 88% | 99% |
| CIIP | 0.33% | 0.54% | 0.59% | 96% | 100% |
| BCW | 0.06% | 0.03% | 0.01% | 94% | 101% |
| MMVD | 0.21% | 0.01% | 0.00% | 93% | 99% |
| Geo | 0.12% | 0.15% | 0.10% | 94% | 100% |
| SBT | -0.02% | 0.24% | 0.12% | 96% | 100% |
| Affine-PROF | 0.00% | -0.03% | 0.00% | 90% | 98% |
| ALF_CCALF | 1.52% | 3.11% | 3.36% | 97% | 84% |
| DepQuant | 2.38% | 3.24% | 2.94% | 89% | 101% |
| DualITree | 0.05% | 0.06% | 0.05% | 99% | 100% |

|  | LDB HDR HLG444 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| IMV | 0.06% | 0.28% | 0.28% | 88% | 101% |
| CIIP | 0.24% | 0.19% | 0.16% | 93% | 100% |
| BCW | 0.03% | 0.10% | 0.10% | 90% | 99% |
| MMVD | -0.05% | -0.01% | -0.01% | 95% | 100% |
| Geo | 0.02% | 0.01% | 0.01% | 94% | 100% |
| SBT | 0.00% | 0.00% | 0.00% | 97% | 100% |
| Affine-PROF | 0.00% | 0.05% | 0.07% | 89% | 97% |
| ALF_CCALF | 2.25% | 6.14% | 7.37% | 99% | 82% |
| DepQuant | 3.61% | 4.96% | 5.22% | 84% | 100% |
| DualITree | 0.21% | 0.05% | 0.06% | 97% | 99% |

|  | LDB SVT RGB12 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| IMV | -0.01% | 0.00% | 0.00% | 92% | 98% |
| CIIP | 0.07% | 0.03% | 0.03% | 103% | 98% |
| BCW | 0.00% | 0.01% | 0.01% | 93% | 101% |
| MMVD | 0.07% | 0.09% | 0.09% | 103% | 99% |
| Geo | 0.04% | 0.05% | 0.05% | 96% | 100% |
| SBT | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Affine-PROF | -0.01% | 0.03% | 0.03% | 100% | 98% |
| ALF_CCALF | 0.38% | 0.00% | 0.00% | 98% | 98% |
| DepQuant | 2.46% | 3.13% | 3.22% | 87% | 96% |
| DualITree | 0.00% | 0.00% | 0.00% | 106% | 99% |

Table 5. Simulation results in random access configuration (RA). (VTM anchor)

|  | RA HDR PQ444 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| IMV | 0.10% | 0.03% | 0.06% | 95% | 109% |
| CIIP | 0.26% | 0.44% | 0.49% | 96% | 99% |
| BCW | 0.03% | 0.04% | 0.06% | 96% | 100% |
| MMVD | 0.01% | -0.19% | -0.20% | 92% | 99% |
| SMVD | 0.08% | 0.01% | -0.01% | 97% | 100% |
| Geo | 0.04% | 0.08% | 0.09% | 97% | 100% |
| BIO | 0.02% | 0.03% | 0.03% | 98% | 99% |
| SBT | -0.11% | 0.45% | 0.16% | 96% | 100% |
| DMVR | 0.02% | 0.03% | 0.02% | 100% | 99% |
| Affine-PROF | 0.07% | -0.06% | -0.24% | 93% | 98% |
| ALF_CCALF | 1.26% | 2.74% | 3.10% | 97% | 84% |
| DepQuant | 2.55% | 2.60% | 2.50% | 90% | 100% |
| DualITree | 0.07% | 0.07% | 0.11% | 99% | 98% |

|  | RA HDR HLG444 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| IMV | 0.12% | 0.47% | 0.49% | 86% | 101% |
| CIIP | 0.13% | 0.10% | 0.09% | 96% | 100% |
| BCW | 0.04% | 0.13% | 0.15% | 93% | 102% |
| MMVD | 0.02% | 0.03% | 0.04% | 92% | 100% |
| SMVD | 0.06% | 0.07% | 0.07% | 95% | 99% |
| Geo | 0.01% | 0.01% | 0.01% | 95% | 99% |
| BIO | -0.01% | -0.02% | -0.01% | 96% | 98% |
| SBT | 0.01% | -0.01% | 0.00% | 97% | 100% |
| DMVR | 0.00% | -0.01% | 0.00% | 98% | 98% |
| Affine-PROF | 0.01% | 0.03% | 0.04% | 91% | 99% |
| ALF_CCALF | 2.25% | 6.14% | 7.37% | 99% | 82% |
| DepQuant | 3.69% | 5.26% | 5.51% | 81% | 101% |
| DualITree | 0.13% | 0.02% | 0.03% | 98% | 101% |

|  | RA SVT RGB12 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| IMV | 0.01% | 0.03% | 0.03% | 96% | 111% |
| CIIP | 0.04% | 0.04% | 0.04% | 98% | 100% |
| BCW | 0.01% | 0.00% | 0.00% | 95% | 100% |
| MMVD | 0.05% | 0.08% | 0.08% | 97% | 101% |
| SMVD | 0.02% | 0.01% | 0.01% | 99% | 106% |
| Geo | 0.02% | 0.01% | 0.01% | 98% | 100% |
| BIO | -0.05% | 0.01% | 0.01% | 99% | 99% |
| SBT | 0.00% | 0.00% | 0.00% | 99% | 100% |
| DMVR | -0.03% | 0.02% | 0.02% | 100% | 99% |
| Affine-PROF | 0.01% | 0.03% | 0.03% | 99% | 99% |
| ALF_CCALF | 0.43% | 0.00% | 0.00% | 99% | 97% |
| DepQuant | 2.41% | 3.11% | 3.21% | 84% | 97% |
| DualITree | 0.00% | 0.00% | 0.00% | 106% | 99% |

Figure 1. PQ PSNR-Y,U,V vs encoding runtime ratio of VTM (AI) (VTM anchor)

Figure 2. HLG PSNR-Y,U,V vs encoding runtime ratio of VTM (AI) (VTM anchor)

Figure 3. SVT PSNR-G,B,R vs encoding runtime ratio of VTM (AI) (VTM anchor)

Figure 4. PQ PSNR-Y,U,V vs encoding runtime ratio of VTM (LDB) (VTM anchor)

Figure 5. HLG PSNR-Y,U,V vs encoding runtime ratio of VTM (LDB) (VTM anchor)

Figure 6. SVT PSNR-G,B,R vs encoding runtime ratio of VTM (LDB) (VTM anchor)

Figure 7. PQ PSNR-Y,U,V vs encoding runtime ratio of VTM (RA) (VTM anchor)

Figure 8. HLG PSNR-Y,U,V vs encoding runtime ratio of VTM (RA) (VTM anchor)

Figure 9. SVT PSNR-G,B,R vs encoding runtime ratio of VTM (RA) (VTM anchor)

# Recommendations

As run times of simulations have significantly increased for the high bit depth/rate CTC it should be considered whether disabling a number of tools would be justified to reduce simulation times for future tests.  For the most part this contribution only tests disabling one tool at a time so possible interaction between tools should also be considered.

# Reference

[1] W-J. Chien, .J Boyce, W. Chen, Y-W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y-W. Huang, H. Jang, R-L. Liao, S. Liu, “JVET AHG report: Tool reporting procedure (AHG13)”, JVET-T2013, by teleconference, 7–16 Oct 2020

[2] A. Browne, T. Ikai, D. Rusanovskyy, X. Xu, “Common test conditions for high bit depth and high bit rate video coding”, JVET-T2018, by teleconference, 7–16 Oct 2020