| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0077-v1 |
| --- | --- |

| Title: | AHG11: Revisiting SAO in-loop filter with Neural Networks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P.Bordes, F.Galpin, T.Dumas, P.Nikitin | Email: | philippe.bordes@interdigital.com franck.galpin@interdigital.com thierry.dumas@interdigital.com pavel.nikitin@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes a Neural Network-based In-loop filter (NN filter). The proposed NN filter replaces the regular SAO filter while re-using some of its features.

It is reported that averagely 1.76%, 4.96%, and 4.52% BD rate saving for RA, 1.62%, 6.62%, and 5.65% BD rate saving for LDB and 1.91%, 2.91%, and 3.03% BD rate saving for AI, for Y, Cb, and Cr components respectively, are achieved.

# Introduction

In VVC [1], the reconstructed pictures are post-filtered with three cascaded loop filters namely Deblocking Filter, Sample Adaptive Offset (SAO), and Adaptive Loop Filter (ALF). Deblocking filter is mainly used to attenuate blocking artifacts. SAO filter mainly filters ringing artifacts. ALF tries to minimize the mean square error between original samples and reconstructed samples using Wiener-based adaptive filter coefficients.

In [2], it is reported that the relative performance of the SAO and ALF tools is 0.08% and 4.34% in RA for the luma component respectively. This relatively low performance of SAO motivated the study of this NN-based post-filter.

In AHG11, several proposals for improving the performance of VVC with NN-based post-filters have been proposed. They provide significant coding gains but at the expense of significant complexity (number of MAC per pix, number of parameters, encoding/decoding times, heterogeneous frameworks).

This proposal tries to limit the codec complexity while still providing significant BD-rate gains.

# Proposal

The proposed NN-based filter replaces the regular SAO post-filter of VVC. It is composed of K=7 NNs for luma and one NN (K=1) for chroma. The NNs use one CTU 128x128 of one component as input (luma) or 3 components (chroma, Cb,Cr up-sampled). The same NNs are used for all the configurations (RA, LDB, AI and all QPs). The NNs output is one (luma) or two (chroma) mask(s) Wi. (values in range [-512;512]/1024) corresponding to the correction to be added to the reconstructed samples. The correction is weighted with a scaling factor offi .

For luma, a choice is made by the encoder to use one (k=1) or combine two (k=2) NNs:

corr = i=0kWki.offi

Then the proposed NN-based filter leverages on side-information (CTU post-filter parameters) coded in the bitstream for each CTU to tailor the NN correction to the CTU characteristics. These parameters include the indexes of the NNs {k0;k1} and the scaling factors {off0;off1} for luma, the scaling factors only for chroma {offU;offV}.

Figure 1. Proposed NN-based post-filter architecture.

## Inference information

The proposal tried to reduce the overall complexity by minimizing the number of operations per pixels (MACs), the number of parameters (by using NN with the small number of layers (5 or 6) and reduced number of latents/channels (16 to 32 or 64)), use of integer parameters (16-bits) and standalone C++ code implementation. Two versions are provided, v1 and v2 slightly less complex than v1:

Then, all the NNs use same architecture (v1) depicted in Figure 2:

Figure 2. Proposed NN architecture (v1).

MACY = 3*3*(1*138*138*16+16*136*136*32+32*134*134*64+64*132*132*32+32*130*130*16+16*128*128*1)/(128*128) = 50k

MACU,V = 3*3*(3*138*138*16+16*136*136*32+32*134*134*64+64*132*132*32+32*130*130*16+16*128*128*2)/(128*128) = 50.5k

MACper pixel = 2 x 50 + 50.5 = 150 kMACs / pix

Variant with smaller number of layers and parameters (v2):

Figure 3. Proposed reduced NN architecture (v2).

MACY = 3*3*(1*138*138*16+16*134*134*24+24*132*132*32+32*130*130*16+16*128*128*1)/(128*128) = 16,2k

MACU,V = 3*3*(3*138*138*16+16*134*134*24+24*132*132*32+32*130*130*16+16*128*128*2)/(128*128) = 50.5k

MACper pixel = 2 x 16,2 + 16,6 = 49 kMACs / pix

| Inference information |  |  |
| --- | --- | --- |
|  | Architecture V1 | Architecture V2 |
| Param. Number | 46,529 x 7 + 46,962 | 15,353 x 7 + 15,786 |
| Param. Precision | 16 (I) | 16 (I) |
| MAC per pixel | 150k | 49k |
| Total Conv. Layers | 6 | 5 |
| Total FC Layers | - | - |
| Mem.T (MB) | 0.745 MB | 0.246 MB |
| Patch size | 138x138 x 3 | 136x136 x 3 |
| GPU Type | No GPU | No GPU |
| CPU Type | Xeon 2.4GHz | Xeon 2.4GHz |
| Framework | Standalone C++ | Standalone C++ |

## Training information

| Training information |  |
| --- | --- |
| Epoch | 30 – 100 |
| Batch size | 20 |
| Training time | few hours to 30h |
| Training set | BVI-DVC |
| Number of iterations |  |
| Patch size | 128 x 128 x (1 or 3) |
| Learning rate | 5e-3 or 5e-4 |
| Optimizer | Adam |
| Loss function | Mean-squared-error |
| Pre-processing |  |

The BVI-DVC dataset contains several video sequences of 64 images of different sizes (classes A, B, C, D). We encoded the video sequences with VTM-8.0 in Random-Access-main10 and we extracted patches from the reconstructed pictures before SAO. For luma NN-filters training, we generated 5 datasets by grouping patches from different picture sizes and different QPs, with pictures with Tid equal to 0,1,2. We also generated 2 datasets by extracting patches coded with SAO edge-offset 90 and 135.

Table 1: datasets used to train the NNs.

|  | NN | Method | Pictures size | QP |
| --- | --- | --- | --- | --- |
| Luma | 0 | All patches, Tid=0,1,2 | C, D | 22 |
|  | 1 | All patches, Tid=0,1,2 | A | 22 |
|  | 2 | All patches, Tid=0,1,2 | C, D | 37 |
|  | 3 | All patches, Tid=0,1,2 | A, B | 37 |
|  | 4 | All patches, Tid=0,1,2 | B | 22 |
|  | 5 | SAO edge-offset 90 | All sizes | All QPs |
|  | 6 | SAO edge-offset 135 | All sizes | All QPs |
| Chroma | 0 | All patches, Tid=0,1,2 | C, D | 22 |

# Experimental results

## Architecture v1

Table 1: v1 architecture.

|  | Random Access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Psnr 4 QPs |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -0,93% | -1,28% | -1,73% | -1,12% | -1,75% | -2,17% | 106% | 949% |
| Class A2 | -2,35% | -5,26% | -4,20% | -2,78% | -6,57% | -5,20% | 103% | 1368% |
| Class B | -1,72% | -5,19% | -4,30% | -2,04% | -6,30% | -5,50% | 112% | 1789% |
| Class C | -2,00% | -7,21% | -7,14% | -2,44% | -7,88% | -7,97% | 112% | 3416% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -1,76% | -4,96% | -4,52% | -2,11% | -5,87% | -5,43% | 109% | 1775% |
| Class D | -3,07% | -8,88% | -8,94% | -3,61% | -9,87% | -10,2% | 111% | 5368% |
| Class F | -0,18% | -3,89% | -3,56% | -0,62% | -4,10% | -3,98% | 101% | 1653% |

|  | Low Delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Psnr 4 QPs |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -0,57% | -2,86% | -2,15% | -0,63% | -3,51% | -2,67% | 111% | 1045% |
| Class A2 | -1,69% | -7,22% | -5,23% | -1,95% | -8,32% | -5,95% | 109% | 1643% |
| Class B | -1,50% | -7,39% | -5,92% | -1,69% | -8,49% | -6,64% | 101% | 2213% |
| Class C | -2,03% | -11,6% | -11,4% | -2,19% | -11,6% | -11,7% | 103% | 4686% |
| Class E | -2,22% | -1,80% | -1,37% | -2,48% | -2,38% | -2,38% | 113% | 867% |
| Overall | -1,62% | -6,62% | -5,65% | -1,80% | -7,31% | -6,30% | 106% | 1878% |
| Class D | -3,44% | -13,2% | -12,4% | -3,79% | -13,7% | -13,8% | 104% | 6573% |
| Class F | -0,54% | -6,80% | -6,65% | -0,39% | -6,72% | -6,90% | 108% | 1748% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Psnr 4 QPs |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -1,02% | -0,97% | -1,51% | -1,06% | -1,25% | -1,83% | 186% | 4706% |
| Class A2 | -1,95% | -2,73% | -2,49% | -1,81% | -3,35% | -3,03% | 145% | 5862% |
| Class B | -1,68% | -3,07% | -2,87% | -1,51% | -3,70% | -3,65% | 138% | 6082% |
| Class C | -2,14% | -6,07% | -6,51% | -2,05% | -6,44% | -7,03% | 119% | 7207% |
| Class E | -2,82% | -0,58% | -0,71% | -2,70% | -0,84% | -1,02% | 144% | 5633% |
| Overall | -1,91% | -2,91% | -3,03% | -1,81% | -3,37% | -3,56% | 143% | 5938% |
| Class D | -2,64% | -6,28% | -7,03% | -2,49% | -6,64% | -7,60% | 116% | 7643% |
| Class F | -1,13% | -4,39% | -4,56% | -0,97% | -4,65% | -4,91% | 124% | 6847% |

## Architecture v2

Table 2: v2 architecture.

|  | Random Access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Psnr 4 QPs |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -0,66% | -0,79% | -0,47% | -0,80% | -1,02% | -0,53% | 107% | 1094% |
| Class A2 | -1,84% | -2,94% | -2,22% | -2,19% | -3,80% | -2,76% | 110% | 1744% |
| Class B | -1,25% | -3,84% | -2,38% | -1,54% | -4,82% | -3,16% | 114% | 2304% |
| Class C | -1,40% | -4,08% | -3,14% | -1,79% | -4,55% | -3,64% | 117% | 4329% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -1,29% | -3,11% | -2,17% | -1,59% | -3,78% | -2,68% | 113% | 2222% |
| Class D | -2,25% | -4,85% | -4,28% | -2,71% | -5,61% | -5,07% | 118% | 7072% |
| Class F | 0,13% | -1,29% | -0,55% | -0,30% | -1,37% | -0,83% | 113% | 2049% |

|  | Low Delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Psnr 4 QPs |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -0,35% | -1,84% | -0,61% | -0,41% | -2,27% | -0,77% | 114% | 1084% |
| Class A2 | -1,30% | -4,48% | -2,96% | -1,48% | -5,12% | -3,36% | 115% | 2017% |
| Class B | -1,14% | -5,53% | -4,09% | -1,25% | -6,60% | -4,53% | 104% | 2777% |
| Class C | -1,44% | -7,98% | -6,54% | -1,54% | -7,82% | -6,52% | 106% | 5716% |
| Class E | -1,67% | -1,36% | -0,78% | -1,83% | -1,54% | -1,35% | 119% | 1017% |
| Overall | -1,19% | -4,59% | -3,31% | -1,31% | -5,06% | -3,62% | 110% | 2235% |
| Class D | -2,64% | -8,35% | -8,05% | -2,90% | -8,99% | -8,94% | 108% | 8481% |
| Class F | -0,33% | -2,19% | -1,20% | -0,14% | -2,24% | -0,84% | 114% | 2080% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Psnr 5 QPs |  |  | Psnr 4 QPs |  |  | Time |  |
|  | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 | -0,70% | -0,15% | 0,14% | -0,72% | -0,33% | 0,05% | 212% | 5628% |
| Class A2 | -1,58% | -1,42% | -1,29% | -1,47% | -1,79% | -1,58% | 159% | 7693% |
| Class B | -1,32% | -2,27% | -1,66% | -1,18% | -2,78% | -2,16% | 148% | 8271% |
| Class C | -1,69% | -3,14% | -2,96% | -1,60% | -3,49% | -3,29% | 127% | 9479% |
| Class E | -2,22% | -0,32% | -0,44% | -2,15% | -0,52% | -0,68% | 165% | 8061% |
| Overall | -1,49% | -1,64% | -1,38% | -1,41% | -1,99% | -1,70% | 156% | 7866% |
| Class D | -2,17% | -3,08% | -2,88% | -2,01% | -3,52% | -3,37% | 125% | 10612% |
| Class F | -0,81% | -1,59% | -1,29% | -0,65% | -1,70% | -1,36% | 130% | 9054% |

# References

- Versatile Video Coding Editorial Refinements on Draft 10, JVET-T2001, B. Bross, J. Chen, S. Liu, Y.-K. Wang.

- JVET AHG report: Tool reporting procedure and testing (AHG13), JVET-T0013, W.-J. Chien, J. Boyce, Y.-W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y.-W. Huang, H. Jang, R.-L. Liao, S. Liu

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).