| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0080-v1 |
| --- | --- |

| Title: | Balancing Luma-Chroma Channel Coding for DNN-based Intra-Frame Coding in JVET-U0079 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ankitesh K. Singh, Hilmi E. Egilmez, Muhammed Coban, Marta Karczewicz | Email: | ankitesh@qti.qualcomm.com hegilmez@qti.qualcomm.com mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution document presents a reformulation of the loss function used to train end-to-end DNN for YUV 4:2:0 intra-frame compression. It is asserted that by adjusting the weight between luma and chroma distortion (in mean-square error), a more balanced RD trade-off can be achieved.

# Introduction

In JVET-U0079, two DNN architectures are introduced to improve coding efficiency over the solutions presented in JVET-T0123. However, JVET-U0079 shows that luma and chroma coding results are often unbalanced and their performance can deviate considerably on a per-sequence basis.

This contribution document approaches the luma-chroma coding balancing problem by training the network parameters using a different loss function.

# Loss function and Trained Network

In end-to-end DNN-based coding schemes, the following general loss functional is required to optimize network parameters:

RD(β) = β×D+R

where R denotes the estimated bits, D is the average distortion and β is the tuning parameter for the rate-distortion trade-off.

In JVET-U0079, the following distortion term (in mean-square error (MSE)) is used in RD(β):

D = (8×MSEY + 2×MSEU + 2×MSEV)/12

which combines MSEs from Y, U and V components such that the weighted terms are proportional to pixels (i.e., the contribution of Y is 4 times that of U or V, being proportional to the size of each component in YUV 4:2:0 format).

In this proposal, to achieve a different luma-chroma trade-off (in terms of coding gains), the distortion term above is changed as:

D = (6×MSEY + 3×MSEU + 3×MSEV)/12

so that the luma component is relatively more penalized to achieve a better coding gain for chroma.

In this proposal, the parameters of DNN architecture from JVET-U0079 illustrated in Figure 1.

Figure 1: (Method 1 in JVET-U0079) DNN architecture for joint coding with the 1x1 convolution layer and a single PReLU operator integrated to [4].

# Training and Testing

The DNN architectures in Figure 1 are trained and tested on the CompressAI framework [5]. For training the reweighted MSE term is used as the distortion metric as discussed above, and the trained network is tested and evaluated using PSNR. For testing the bitstream generation is done by using the ANS entropy coding (implemented in compressAI) on latent (hyper) variable coefficients, and the bitrates are obtained from the compressed bitstream files.

The training is performed on the Vimeo 90K triplet data set [7] where the RGB images are converted to YUV 4:2:0 using the method specified in JVET-T0041[9].The model was trained for five rate points for each architecture. The training details are summarized in Table 1.

Table 1: Neural network training parameters.

| Dataset | Vimeo90K (converted to YUV420p) |
| --- | --- |
| Input Size | 256x256(Y),128x128(UV) |
| Iterations | 4M |
| Batch Size | 8 |
| Optimizer | Adam |
| Learning rate | 10-4 (main), 10-5(auxiliary) |
| Learning Rate decay schedule (fine tuning) | the initial learning rate times 0.1 after 2M iteration |

The β values of {0.005, 0.01, 0.025, 0.1, 0.2} are used in compressAI. The associated rate points for these β parameters roughly correspond to bitrates for 5 QP parameters, 42, 37, 32, 27 and 22 in HEVC, respectively.

- Experimental Results

The compression experiments are performed are performed on the CTC sequences in [6]. As suggested in JVET-T0041, the results are compared against HM-16.20 at 5 QP points (42, 37, 32, 27 and 22). For the quality metric only PSNR is used.

Tables 2 and 3 show the BD-rate results of Method 1 in JVET-U0079 over HM-16.20, where the results in Table 2 are obtained by training with the new loss function, and Table 3 shows the results copied from JVET-U0079 as the baseline. In comparison, the proposed new formulation provides a better luma-chroma trade-off by trading about a 3% average luma gains for about a 30% chroma gains.

Table 2: Performance of Method 1 in JVET-U0079 retrained by using the new loss function.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Over HM-16.20 |  |  |
|  | Y | U | V |
| Class A1 | -15.32% | 53.80% | -27.24% |
| Class A2 | -17.24% | 41.97% | 41.30% |
| Class B | -6.07% | -12.84% | -27.77% |
| Class C | -4.09% | -5.41% | -9.94% |
| Class E | -9.03% | -7.35% | -36.10% |
| Overall | -9.53% | 9.97% | -13.60% |

Table 3: Performance of Method 1 copied from JVET-U0079 without luma-chroma balancing.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Over HM-16.20 |  |  |
|  | Y | U | V |
| Class A1 | -19.12% | 109.65% | -11.03% |
| Class A2 | -19.68% | 83.79% | 85.44% |
| Class B | -9.47% | 19.83% | -1.85% |
| Class C | -5.91% | 24.96% | 18.98% |
| Class E | -11.98% | 16.68% | -14.73% |
| Overall | -12.41% | 46.07% | 13.65% |

- Conclusion

This contribution proposes a new loss function by reweighting the distortion (i.e., MSE values) measured from luma and chroma components. The experimental results show that a better coding trade-off is achieved between luma and chroma components as compared to the results in JVET-U0079. Further study of handling input YUV chroma formats in DNN-based video codecs with training on larger datasets is encouraged.

- References

- HEVC Test Model (HM) 16.20, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/ HM-16.20/.

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 10),” JVET-S2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 19th Meeting, 22 June–1 July 2020.

- J. Ballé, D. Minnen, S. Singh, S. J. Hwang, and N. Johnston, ‘Variational image compression with a scale hyperprior’, arXiv:1802.01436.

- D. Minnen, J. Ballé, and G. Toderici, ‘Joint Autoregressive and Hierarchical Priors for Learned Image Compression’, arXiv:1809.02736.

- J. Bégaint, F. Racapé, S. Feltman, A. Pushparaja, “CompressAI: A PyTorch library and evaluation platform for end-to-end compression research,” m54467, July 2020.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting, Geneva, CH, 19–27 Mar. 2019.

- Vimeo 90K Triplet dataset, http://toflow.csail.mit.edu/index.html#triplet.

- Draft test conditions for DNNVC, w19583, ISO/IEC JTC 1/SC 29/WG 11, July 2020.

- S. Liu, A. Segall, E. Alshina, J. Boyce, M. Wien, D. Grois, “Methodology and reporting template for neural network coding tool testing”, JVET-T0041, October 2020.

- A. K. Singh, H. E. Egilmez, M. Coban, M. Karczewicz, “A study of handling YUV420 input format for DNN-based video coding”, JVET-T0123, October 2020.

- H. E. Egilmez, A. K. Singh, M. Coban, M. Karczewicz, “A DNN Architecture for Intra-Frame Coding in YUV 4:2:0 format with Cross-Component Prediction”, JVET-U0079, January 2021.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).