| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0083-v1 |
| --- | --- |

| Title: | Signalling of decoder initialization information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Kai Zhang1, Li Zhang1, Yang Wang2, Jizheng Xu1, Zhipin Deng2 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com wangyang.cs@bytedance.com xujizheng@bytedance.com zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to signal decoder initialization information (DII) in order to avoid or minimize the need of decoder reinitialization when decoding a video bitstream. The DII is proposed to be signalled either in the DCI NAL unit or in a new SEI message, named DII SEI message.

# Introduction

In adaptive streaming, the streaming client often requests to switch to a different bitstream with a different spatial resolution, for adapting to the changing network conditions, e.g., bandwidth. Different bitstreams are typically stored in different file format (FF) tracks and encapsulated in different DASH Representations within the same Adaptation Set, thus such bitstream switching typically involves switching of FF tracks and DASH Representations. It is assumed that the switch-to bitstream is of the same codec and the same profile as the switch-from bitstream. Depending on the availability of other information such as the highest spatial resolution and how the decoder is implemented, when the spatial resolution changes, particularly when to be higher, a decoder may be re-initialized. According to the MPEG input documents m54516 and m54522, decoder initialization or reinitialization typically takes 100-200 ms of time and the best case can be just 16 ms, depending on various factors including memory allocation time, secure/nonsecure, mapping time, memory fragmentation of double data rate (DDR), internal fragmentation of system memory management unit (SMMU), etc. Consequently, decoder re-initialization can disrupt a streaming session, affect playback continuity, etc.

In order to address such problems, the DASH-IF initiated the New Codecs Test Cases work item (https://docs.google.com/document/d/1jV1vs2TsietU34EMPIPHsgILAWiKhEr7dk39hmCc18o/edit#heading=h.z5pebcnfqpbj), and later on in October 2020 the MPEG Systems WG started the Systems for Video AHG.

We believe that, to solve the decoder initialization problem, a key aspect is to avoid or reduce decoder re-initializations as much as possible in an application session. One way towards that goal is to signal information needed for decoder initialization for a scope as wide as possible, and at the same time provide guidelines in relevant specifications (e.g., in NOTEs of video coding standards, and in guidelines of systems and application specifications) to encourage or even require decoders implementations to use the signalled DII when initializing the decoder.

While there are some necessary systems-level signalling, such as in file format and DASH media presentation description (MPD), which should be left for the systems groups to work on, signalling of DII in bitstream level would also be helpful, e.g., to be used by applications when encapsulating bitstreams into file tracks and DASH representations.

This contribution presents a proposal of DII signalling, either in the DCI NAL unit or in a new SEI message.

# Proposal

The text changes for both options of the proposals are provided. Added parts are highlighted in turquoise, and deleted parts are highlighted in yellow strikethrough in red fonts.

## DII signalling in DCI

This proposal option is summarized as follows:

- The parameters of the decoder initialization information can only be present in the DCI NAL unit when there is only one profile_tier_level( ) syntax structure in the DCI NAL unit.

- The DII includes the following pieces of information:

  - The maximum number of decoded pictures stored in the DPB when decoding the bitstream;

  - The maximum picture width of all coded pictures in the bitstream, in luma samples;

  - The maximum picture height of all coded pictures in the bitstream, in luma samples;

  - The maximum colour format (i.e., the greatest value of sps_chroma_format_idc) of all coded pictures in the bitstream;

  - The maximum bit depth (i.e., the greatest value of sps_bitdepth_minus8) of all coded pictures in the bitstream.

The text changes for the option of DII signalling in DCI, to the VVC spec, are provided below.

#### 7.3.2.1 Decoding capability information RBSP syntax

| decoding_capability_information_rbsp( ) { | Descriptor |
| --- | --- |
| dci_reserved_zero_4bits | u(4) |
| dci_num_ptls_minus1 | u(4) |
| for( i = 0; i <= dci_num_ptls_minus1; i++ ) |  |
| profile_tier_level( 1, 0 ) |  |
| dci_extension_flag | u(1) |
| if( dci_extension_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| dci_extension_alignment_bit_equal_to_one | u(1) |
| dci_extension( ) |  |
| dci_extension2_flag | u(1) |
| if( dci_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| dci_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

##### 7.3.2.1.1 DCI extension syntax

| dci_extension( ) { | Descriptor |
| --- | --- |
| dci_max_num_dec_pics_in_dpb_minus1 | u(4) |
| dci_max_bitdepth_minus8 | u(4) |
| dci_max_pic_width | u(16) |
| dci_max_pic_height | u(16) |
| dci_max_chroma_format | u(2) |
| dci_ext_reserved_zero_6bits | u(6) |
| } |  |

#### 7.4.3.1 Decoding capability information RBSP semantics

A DCI RBSP could be made available to the decoder, through either being present in the bitstream, included in at least the first AU of the bitstream, or provided through external means.

NOTE 1 – The information contained in the DCI RBSP is not necessary for operation of the decoding process specified in clauses 2 through 9 of this Specification.

When present, all DCI NAL units in a bitstream shall have the same content.

dci_reserved_zero_4bits shall be equal to 0 in bitstreams conforming to this version of this Specification. The values greater than 0 for dci_reserved_zero_4bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall allow values greater than 0 for dci_reserved_zero_4bits to appear in the bitstream and shall ignore the value of dci_reserved_zero_4bits.

dci_num_ptls_minus1 plus 1 specifies the number of profile_tier_level( ) syntax structures in the DCI NAL unit. The value of dci_num_ptls_minus1 shall be in the range of 0 to 14, inclusive. The value 15 for dci_num_ptls_minus1 is reserved for future use by ITU-T | ISO/IEC.

It is a requirement of bitstream conformance that each OLS in a CVS in the bitstream shall conforms to at least one of the profile_tier_level( ) syntax structures in the DCI NAL unit.

NOTE 2 – The DCI NAL unit could include PTL information, possibly carried in multiple profile_tier_level( ) syntax structures, that applies collectively to multiple OLSs, and does not need to include PTL information for each of the OLSs individually.

dci_extension_flag equal to 0 specifies that no dci_extension_data_flag syntax elements are dci_extension( ) syntax structure is present in the DCI RBSP syntax structure. dci_extension_flag equal to 1 specifies that dci_extension_data_flag syntax elements might be the dci_extension( ) syntax structure is present in the DCI RBSP syntax structure. dci_extension_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. However, some use of dci_extension_flag equal to 1 could be specified in some future version of this Specification, and decoders conforming to this version of this Specification shall allow the value of dci_extension_flag equal to 1 to appear in the syntax. When dci_num_ptls_minus1 is greater than 0, the value of dci_extension_flag shall be equal to 0.

dci_extension_alignment_bit_equal_to_one shall be equal to 1.

dci_extension2_flag equal to 0 specifies that no dci_extension_data_flag syntax elements are present in the DCI RBSP syntax structure. vps_extension2_flag equal to 1 specifies that dci_extension_data_flag syntax elements might be present in the DCI RBSP syntax structure. dci_extension_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. However, some use of dci_extension_flag equal to 1 could be specified in some future version of this Specification, and decoders conforming to this version of this Specification shall allow the value of dci_extension_flag equal to 1 to appear in the syntax and shall ignore all data that follow the value 1 for dci_extension2_flag in a DCI RBSP.

dci_extension_data_flag could have any value. Its presence and value do not affect the decoding process specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore all dci_extension_data_flag syntax elements.

##### 7.4.3.1.1 DCI extension semantics

NOTE – When the dci_extension( ) syntax structure is present for a bitstream and the decoder is intialized according to the information provided in the DCI NAL unit, it is expected that no decoder reinitialization is needed for decoding the entire bitstream.

dci_max_num_dec_pics_in_dpb_minus1 plus 1 indicates the maximum number of decoded pictures stored in the DPB at any moment when decoding the bitstream.

dci_max_bitdepth_minus8 plus 8 indicates the maximum bit depth, in units of bits, of each colour component of a decoded picture when decoding the bitstream. The value of dci_max_bitdepth_minus8 shall be in the range of 0 to 8, inclusive.

dci_max_pic_width indicates the maximum width, in units of luma samples, of a decoded picture when decoding the bitstream.

dci_max_pic_height indicates the maximum height, in units of luma samples, of a decoded picture when decoding the bitstream.

dci_max_chroma_format indicates the maximum value of sps_chroma_format_idc of the SPS referenced by a picture when decoding the bitstream.

dci_ext_reserved_zero_6bits shall be equal to 0 in bitstreams conforming to this version of this Specification. The values greater than 0 for dci_ext_reserved_zero_6bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall allow values greater than 0 for dci_ext_reserved_zero_6bits to appear in the bitstream and shall ignore the value of dci_ext_reserved_zero_6bits.

## DII signalling in SEI

This proposal option is summarized as follows:

- The DII SEI message may only be used with a video bitstream when all CVSs of the bitstream are indicated to conform to the same profile.

- The DII includes the following pieces of information:

  - The maximum tier that all CVSs of the bitstream conform to;

  - The maximum level that all CVSs of the bitstream conform to;

  - The same five parameters as in the DCI option above.

The text changes, for the option of DII signalling in SEI, to the VSEI spec, are provided below.

#### 8.1 SEI messages general

Clause 8 specifies the syntax and semantics for SEI messages.

For SEI messages for which the specified syntax structure is empty, such as the dependent random access point SEI message, the mere indication that the SEI message is present (e.g., as indicated by a payload type indicator) is sufficient to convey the associated information (e.g., by indicating that a set of specified constraints are fulfilled).

The semantics and persistence scope for each SEI message are specified in the semantics specification for each particular SEI message.

NOTE – Persistence information for SEI messages is summarized in Table 4.

| Table 4 – Persistence scope of SEI messages (informative) |  |
| --- | --- |
| SEI message | Persistence scope |
| Filler payload | The PU containing the SEI message |
| ... | ... |
| Frame-field information | The PU containing the SEI message |
| Sample aspect ratio information | Specified by the syntax of the SEI message |
| Decoder initialization information | The bitstream containing the access unit that the SEI message is associated with |

...

#### 8.x Decoder initialization information SEI message

##### 8.x.1 Decoder initialization information SEI message syntax

| decoding_init_info( payloadSize ) { | Descriptor |
| --- | --- |
| dii_max_level_idc | u(8) |
| dii_max_tier_flag | u(1) |
| dii_reserved_zero_bit | u(1) |
| dii_max_chroma_format | u(2) |
| dii_max_bitdepth_minus8 | u(4) |
| dii_max_num_dec_pics_in_dpb_minus1 | u(8) |
| dii_max_pic_width | u(16) |
| dii_max_pic_height | u(16) |
| } |  |

##### 8.x.2 Decoding initialization information SEI message semantics

When associated with an AU, the decoder initialization information (DII) SEI message provides information that applies to the entire bitstream. A DII SEI message could be made available to the decoder, through either being present in the bitstream, included in at least the first AU of the bitstream, or provided through external means.

When present, all DII SEI messages in a bitstream shall have the same payload content.

When any two CVSs in the bistream are indicated to conform to different profiles, there shall be no DII SEI message associated with an AU of the bitstream.

NOTE – When the DII SEI message assocaited with an AU of a bitstream and the decoder is intialized according to the profile and the information provided in the DII SEI message, it is expected that no decoder reinitialization is needed for decoding the entire bitstream.

dii_max_level_idc indicates the maximum level to which a CVS in the bitstream conforms.

dii_max_tier_flag, when the DII SEI message is used with an HEVC or VVC bitstream, indicates the maximum tier to which a CVS in the bitstream conforms. When the DII SEI message is used with an AVC bitstream, the semantics of dii_max_tier_flag is unspecified and the decoder shall ignore the value of dii_max_tier_flag.

dii_reserved_zero_bit shall be equal to 0 in bitstreams conforming to this version of this Specification. The value equal to 1 for dii_reserved_zero_bit is reserved for future use by ITU-T | ISO/IEC. Decoders shall allow dii_reserved_zero_bit equal to 1 to appear in the bitstream and shall ignore the value of dii_reserved_zero_bit.

dii_max_chroma_format indicates the maximum value of sps_chroma_format_idc (when the DII SEI message is used with a VVC bitstream) or chroma_format_idc (when the DII SEI message is used with an AVC or HEVC bitstream) of the SPS referenced by a picture when decoding the bitstream.

dii_max_bitdepth_minus8 plus 8 indicates the maximum bit depth, in units of bits, of each colour component of a decoded picture when decoding the bitstream. The value of dii_max_bitdepth_minus8 shall be in the range of 0 to 8, inclusive.

dci_max_num_dec_pics_in_dpb_minus1 plus 1 indicates the maximum number of decoded pictures stored in the DPB at any moment when decoding the bitstream. When the DII SEI message is used with a VVC bitstream, the value of dci_max_num_dec_pics_in_dpb_minus1 shall be in the range of 0 to 15, inclusive. Note that this field is made 8 bits instead of 4 bits as in the DCI syntax above, such that the same syntax can be applicable to other (future) codec or profile that might have more than 16 decoded pictures stored in the DPB. For example, a layered HEVC bitstream might have altogether more than 16 decoded pictures stored in the DPB.

dci_max_pic_width indicates the maximum width, in units of luma samples, of a decoded picture when decoding the bitstream.

dci_max_pic_height indicates the maximum height, in units of luma samples, of a decoded picture when decoding the bitstream.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).