| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0084-v1 |
| --- | --- |

| Title: | AHG9: Cross RAP referencing (CRR) SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Yang Wang2, Li Zhang1, Kai Zhang1, Zhipin Deng2 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com wangyang.cs@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution is a follow-up of JVET-T0071 and proposes a new SEI message, named cross RAP referencing (CRR) indication SEI message. A usage of CRR in adaptive streaming based on DASH is described. Simulation results comparing CRR with DRAP as well as with just using IRAP for providing the random access functionality are also presented.

# Introduction

Video coding approaches based on cross RAP referencing (CRR), also referred to as external decoding refresh (EDR), was proposed in JVET-M0360, JVET-N0119, JVET-O0149, and JVET-P0114.

The basic idea of such a video coding approach is as follows. Instead of coding random access points (RAPs) as intra-coded IRAP pictures (except for the very first picture in the bitstream), they are coded using inter prediction, to circumvent the unavailability of the earlier pictures if the RAPs are coded as IRAP pictures. The trick is to provide a limited number of the earlier pictures, typically representing different scenes of the video content, through a separate video bitstream, which can be referred to as an external means. Such earlier pictures are referred to as the external pictures. Consequently, each external picture can be used for inter prediction referencing by pictures across the random access points. The coding efficiency gain comes from having random access points coded as inter-predicted pictures and having more available reference pictures for pictures that follow EDR pictures in decoding order.

Such EDR picture are CRR pictures per this contribution.

As described in JVET-P0114, a bitstream coded with such a video coding approach can be used in applications based on ISOBMFF and DASH as described below (with the phrase "EDR picture" replaced with "CRR picture").

DASH content preparation operations

- A video content is encoded into one or more representations, each is of a particular spatial resolution, temporal resolution, and quality.

- Each particular representation of a video content is represented by a main stream and an external stream. The main stream contains coded pictures that include CRR pictures and non-CRR pictures. The external stream contains only RAP (IRAP or CRR) pictures.

- Each main stream is carried in a Main Stream Representation (MSR). Each RAP picture in an MSR is the first picture of a Segment.

- Each external stream is carried in an External Stream Representation (ESR).

- For each Segment in an MSR starting with a CRR picture, there is a Segment in the corresponding ESR having the same Segment start time derived from the MPD, carrying the external pictures needed for decoding of that CRR picture and the subsequent pictures in decoding order in the main stream carried in the MSR.

- The MSRs of the same video content are included in one Adaptation Set (AS). The ESRs of the same video content are included in one AS.

DASH streaming operations

- A client gets the MPD of the DASH Media Presentation, parses the MPD, selects an MSR, and determines the starting presentation time from which the content is to be consumed.

- The client requests Segments of the MSR, starting from the Segment containing the picture having presentation time equal to (or close enough to) the starting presentation time.

  - If the first picture in the starting Segment is a CRR picture, the corresponding Segment (having the same Segment start time derived from the MPD) in the associated ESR is also requested, preferably before requesting of the MSR Segments. Otherwise, no Segment of the associated ESR is requested.

- When switching to a different MSR, the client requests Segments of the switch-to MSR, starting from the first Segment having Segment start time greater than that of the last requested Segment of the switch-from MSR.

  - If the first picture in the starting Segment in the switch-to MSR is a CRR picture, the corresponding Segment in the associated ESR is also requested, preferably before requesting of the MSR Segments. Otherwise, no Segment of the associated ESR is requested.

- When continuously operating at the same MSR (after decoding of the starting Segment after a seeking or stream switching operation), no Segment of the associated ESR needs to be requested, including when requesting any Segment starting with a CRR picture.

Not that the functionality of the DRAP indication SEI message is a subset of the CRR approach.

From encoding point of view, although the CRR approach proposed in JVET-P0114 or earlier JVET contributions was not adopted into VVC, the encoder can still encode the video bitstream in such a manner that certain pictures (i.e., the CRR pictures) only rely on some pictures in a set of pictures consisting of the associated IRAP picture and certain CRR pictures earlier in decoding order. These CRR pictures can be used as RAPs as long as the few dependent IRAP or CRR pictures are provided.

However, given a VVC bitstream, it is unknown whether CRR pictures exist in the bitstream. Furthermore, even when it is known that CRR pictures exist in the bitstream, to compose a media file according to the an ISOBMFF and a DASH media presentation based on such a VVC bitstream to enable CRR based random accessing and bitstream switching, the file and DASH media presentation composer would need to parse and derive lots of information, including POC values and active entries in reference picture lists to figure out whether a particular picture is a CRR picture, and, and if yes, which earlier IRAP or CRR pictures are needed for random accessing from this particular picture, such that the proper set of pictures can be included in a separate, time-synchronized file track and DASH representation.

Therefore, to fully support CRR for VVC bitstreams, there is a need for signalling of which pictures are CRR pictures, and for each CRR picture, which IRAP or CRR pictures are needed.

The proposed CRR SEI message provides such information.

Section 2 presents the proposed CRR indication SEI message syntax and semantics. Section 3 includes simulation results comparing CRR with DRAP as well as with just using IRAP for providing the random access functionality.

# Proposal

The syntax and semantics of the proposed CRR indication SEI message are provided below.

### CRR indication SEI message syntax

| crr_indication( payloadSize ) { | Descriptor |
| --- | --- |
| crr_rap_id_in_clvs | u(16) |
| crr_leading_pictures_decodable_flag | u(1) |
| crr_reserved_zero_12bits | u(12) |
| crr_num_ref_rap_pics_minus1 | u(3) |
| for( i = 0; i <= crr_num_ref_rap_pics_minus1; i++ ) |  |
| crr_ref_rap_id[ i ] | u(16) |
| } |  |

### CRR indication SEI message semantics

The picture associated with a CRR indication SEI message is referred to as a CRR picture.

The presence of the CRR indication SEI message indicates that the constraints on picture order and picture referencing specified in this subclause apply. These constraints can enable a decoder to properly decode the CRR picture and the pictures that are in the same layer and follow it in both decoding order and output order without needing to decode any other pictures in the same layer except the list of pictures referenceablePictures, which consists of a list of IRAP or CRR pictures in decoding order that are within the same CLVS and identified by the crr_ref_rap_id[ i ] syntax elements.

The constraints indicated by the presence of the CRR indication SEI message, which shall all apply, are as follows:

–	The CRR picture is a trailing picture.

–	The CRR picture has a temporal sublayer identifier equal to 0.

–	The CRR picture does not include any pictures in the same layer in the active entries of its reference picture lists except the referenceablePictures.

–	Any picture that is in the same layer and follows the CRR picture in both decoding order and output order does not include, in the active entries of its reference picture lists, any picture that is in the same layer and precedes the CRR picture in decoding order or output order, with the exception of the referenceablePictures.

–	When crr_leading_pictures_decodable_flag is equal to 1, the following applies:

–	Any picture that is in the same layer and follows the CRR picture in decoding order shall follow, in output order, any picture that is in the same layer and precedes the CRR picture in decoding order.

–	Any picture that is in the same layer and follows the CRR picture in decoding order and precedes the CRR picture in output order does not include, in the active entries of its reference picture lists, any picture that is in the same layer and precedes the CRR picture in decoding order, with the exception of the referenceablePictures.

–	Any picture in the list referenceablePictures does not include, in the active entries of its reference picture lists, any picture that is in the same layer and is not a picture at an earlier position in the list referenceablePictures.

NOTE – Consequenlty, the first picture in referenceablePictures, even when it is a CRR picture instead of an IRAP picture, does not include any picture from the same layer in the active entries of its reference picture lists.

crr_rap_id_in_clvs specifies the RAP picture identifier, denoted as RapPicId, of the CRR picture.

Each IRAP or CRR picture is associated with a RapPicId value. The RapPicId value for an IRAP picture is inferred to be equal to 0. The RapPicId values for any two IRAP or CRR pictures within a CLVS shall be different.

crr_reserved_zero_12bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for crr_reserved_zero_12bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of crr_reserved_zero_12bits.

crr_num_ref_rap_pics_minus1 plus 1 indicates the number of IRAP or CRR pictures that are within the same CLVS as the CRR picture and may be included in the active entries of the reference picture lists of the CRR picture.

crr_ref_rap_id[ i ] indicates RapPicId of the i-th IRAP or CRR picture that is within the same CLVS as the CRR picture and may be included in the active entries of the reference picture lists of the CRR picture.

# Simulation results

The proposed CRR method was implemented on top of VTM-11.0 [1]. Simulations were performed following the common test conditions (CTC) for the Random Access (RA) configuration [2]. In the test, at most 3 RAP pictures (i.e., IRAP or CRR pictures) earlier in decoding order were allowed to be used as reference pictures for a CRR picture. Only the associated IRAP picture and the first two CRR pictures were allowed to be referenced by the third and subsequent CRR pictures.

For all the three approaches compared, i.e., VTM, DRAP, and CRR, the random access point (RAP) period was about 1 second, which is the same as the current CTC settings, i.e., 64 for frame rates of 50 or 60 Hz and 32 for frame rates of 30 Hz or below. Note that herein a pictrure at a RAP can be an IRAP, DRAP, or CRR picture.

Under the above RAP period setting, we have tried different periods of IRAP pictures for DRAP and CRR (the IRAP period for VTM was certainly the same as the RAP period), including cases of one IRAP every 2, 3, 4, and 5 seconds (note that most of the CTC sequences are about 10 seconds long). For a fair comparison, we then picked up the best-performing DRAP case (with one IRAP every 4 seconds) among this set of tests and the best-performing CRR case (with one IRAP every 4 seconds) among this set of tests, to compare with each other and with VTM.

The inter prediction reference relationship among the 10 RAP pictures in the CRR case is as shown in Figure 1.

Figure 1. Inter prediction reference relationship among the RAP pictures in the CRR case.

The inter prediction reference relationship among the 10 RAP pictures in the DRAP case is as shown in Figure 2.

Figure 2. Inter prediction reference relationship among the RAP pictures in the DRAP case.

The test results of CRR compared to VTM-11.0 and DRAP are shown in Table 1 and Table 2, respectively.

Table 1: Test results of CRR compared to VTM-11.0

|  | Random Access Main 10 |  |  |
| --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |
|  | Y | U | V |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B | -4.19% | -6.74% | -5.66% |
| Class C | -7.56% | -9.82% | -8.43% |
| Class E |  |  |  |
| Overall |  |  |  |
| Class D | -4.99% | -9.06% | -8.48% |
| Class F (optional) | -22.93% | -22.78% | -21.93% |

Table 2: Test results of CRR compared to DRAP

|  | Random Access Main 10 |  |  |
| --- | --- | --- | --- |
|  | Over DRAP |  |  |
|  | Y | U | V |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B | -3.55% | -10.10% | -10.25% |
| Class C | -2.87% | -8.74% | -7.99% |
| Class E |  |  |  |
| Overall |  |  |  |
| Class D | -2.47% | -10.89% | -10.37% |
| Class F (optional) | -7.76% | -8.26% | -8.20% |

There is basically no scene cut in CTC test sequences. To further evaluate CRR in cases with scene cuts, a simple test was carried out by constructing 5 sequences using BasketballDrill, BQMall, and PartyScene from Class C. These 3 sequences were divided into video clips with 2, 4, or 5 seconds, and the clips were concatenated such that clips of different original sequences are interleaved and a scene cut was generated between each pair of consecutive clips. Table 3 shows the test results of the CRR case with one IRAP every 4 seconds compared to the DRAP case with one IRAP every 4 seconds on these constructed sequences, where NX_Ys denotes a sequence constructed from X of the orignal sequences (e.g., 2 for A and B) in a pattern like A0 B0 A1 B1..., and each chunk is Y seconds. For the constructued sequences, we have not yet tried different periods of IRAP pictures, and we may do some trying and provide more results in a revision of this contribution.

Table 3: Test results of CRR compared to DRAP for the constructed sequences

|  | Random Access Main 10 |  |  |
| --- | --- | --- | --- |
|  | Over DRAP |  |  |
|  | Y | U | V |
| N2_5s | -6.29% | -9.33% | -8.82% |
| N3_5s | -6.97% | -10.12% | -10.02% |
| N2_2s | -4.99% | -8.57% | -8.52% |
| N3_2s | -4.28% | -8.93% | -8.88% |
| N3_4s | -4.44% | -8.09% | -8.22% |
| Overall | -5.39% | -9.01% | -8.89% |

As can be seen, the above simulation results already show that CRR can outperform VTM-11.0 (by just using IRAP pictures to provide the random access functtionality) or DRAP. Note that there can be encoding optimizations, including multi-pass encoding, e.g., as discussed in JVET-P0114, to fully exploit the potential provided by CRR. For example, for some drama test sequences of 136s to 152s, JVET-P0114 reported BD gains of 18.95% to 31.44% (24.32% on average) over VTM-5.0.

# Reference

- VTM-11.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-11.0.

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-T2010.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).