| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0090 |
| --- | --- |

| Title: | Crosscheck of JVET-U0064 (CE-1.1 and CE-1.2: On the Rice parameter derivation for high bit-depth coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Adrian Browne | Email: | adrian.browne@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for CE-1.1 and CE-1.2 reported in JVET-U0064[1], a proposed modification Rice parameter derivation in entropy coding for high bit depths. The simulation results give a close but not exact match with those supplied by the proponents with the exception of encoder and decoder relative run-times which show more variation.

# Introduction

CE-1.1 investigated globally derived adjustment of rice parameters. On inception, it was intended to have two sub tests, but this was reduced to a single test when code was made available.

CE-1.2 investigated locally adaptive adjustment of rice parameters, On inception it was intended to have a single test, but another subtest was added when code was made available. This second subtest affects only 16 bit coding and uses a mechanism based on coefficient history.

The aim of the cross-check was to verify that:

- The source code implements the proposal description

- The VVC draft specification changes match the proposal

- Simulation results match those provided in the proposal

# Procedure

The source code for the CE is based on VTM11.0rc1 and the source code for the three branches CE-1.1[2], CE-1.2a[3] and CE-1.2b[4] was obtained from the CE git repository.

CTC testing[5] was performed against anchors of VTM11.0rc1. The code was compiled using GCC 4.8.5. CE-1.1 was conducted on a cluster of dual Intel Platinum 8160 servers running openSUSE 42.3. CE-1.2 was conducted on a cluster of i7-6700 processors running openSUSE 42.1. Due to the memory demands of high bit depth 4K 444 processing, only 50% of cores were utilised for CTC testing, the other cores being idle (CE-1.1) or processing dummy HD encodes (CE-1.2a and CE-1.2b) to ensure reliable timing.

## Observations

For CE-1.1, the PSNR and bitrates results achieved match those given in the proposal except those at QP=-13 for 12 bit processing and those above QP=-13 for 16 bit processes. The differences are not large enough to change BD-rates when quoted to two decimal places. The proponent when contacted suggested the changes were likely due to a minor final code change.

For CE-1.2a and CE-1.2b, small discrepancies were found for some SVT LDB results, all other results matched exactly.

Note: the encoder and decoder run-times achieved deviate by as much as 17% with those supplied by the proponent. The crosscheck results show relative run-times consistently very near to 100% with the exception of SVT-16 which show relative decoder run-time reductions.

# Results

## CE-1.1

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.55% | -0.61% | -0.60% | -0.54% | -0.61% | -0.61% | 103% | 99% |
| PQ422 | -0.66% | -0.73% | -0.74% | -0.65% | -0.73% | -0.75% | 102% | 99% |
| Overall | -0.60% | -0.67% | -0.67% | -0.60% | -0.67% | -0.68% | 102% | 99% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.04% | -0.06% | -0.09% | -0.04% | -0.06% | -0.08% | 101% | 100% |
| PQ422 | -0.06% | -0.07% | -0.05% | -0.06% | -0.06% | -0.05% | 101% | 100% |
| Overall | -0.05% | -0.06% | -0.07% | -0.05% | -0.06% | -0.06% | 101% | 100% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.12% | -0.13% | -0.15% | -0.12% | -0.13% | -0.15% | 101% | 99% |
| PQ422 | -0.13% | -0.15% | -0.16% | -0.13% | -0.15% | -0.16% | 101% | 100% |
| Overall | -0.13% | -0.14% | -0.16% | -0.13% | -0.14% | -0.15% | 101% | 99% |
| Overall PQ | -0.26% | -0.29% | -0.30% | -0.26% | -0.29% | -0.30% | 101% | 99% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.20% | -0.22% | -0.22% | 101% | 99% |
| HLG422 | -0.24% | -0.25% | -0.26% | 103% | 100% |
| Overall | -0.22% | -0.24% | -0.24% | 102% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.03% | -0.04% | -0.04% | 102% | 101% |
| HLG422 | -0.02% | -0.05% | -0.04% | 101% | 100% |
| Overall | -0.03% | -0.04% | -0.04% | 102% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.04% | -0.04% | -0.04% | 102% | 99% |
| HLG422 | -0.04% | -0.04% | -0.04% | 102% | 101% |
| Overall | -0.04% | -0.04% | -0.04% | 102% | 100% |
| Overall HLG | -0.10% | -0.11% | -0.11% | 102% | 100% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -25.10% | -23.98% | -23.99% | 99% | 80% |
| SVT12 | -2.62% | -2.48% | -2.48% | 100% | 97% |
| Overall | -13.86% | -13.23% | -13.23% | 100% | 89% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -16.56% | -15.20% | -15.47% | 102% | 90% |
| SVT12 | -0.29% | -0.38% | -0.38% | 101% | 100% |
| Overall | -8.43% | -7.79% | -7.92% | 102% | 95% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -15.36% | -15.50% | -15.54% | 99% | 90% |
| SVT12 | -0.44% | -0.45% | -0.45% | 102% | 100% |
| Overall | -7.90% | -7.98% | -8.00% | 100% | 95% |
| Overall RGB | -10.06% | -9.67% | -9.72% | 101% | 93% |

## CE-1.2a

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.71% | -0.73% | -0.73% | -0.71% | -0.73% | -0.73% | 104% | 101% |
| PQ422 | -0.80% | -0.84% | -0.86% | -0.80% | -0.85% | -0.86% | 103% | 101% |
| Overall | -0.76% | -0.79% | -0.79% | -0.76% | -0.79% | -0.79% | 103% | 101% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.03% | -0.03% | -0.06% | -0.03% | -0.03% | -0.05% | 102% | 101% |
| PQ422 | -0.02% | -0.05% | -0.03% | -0.03% | -0.05% | -0.02% | 101% | 100% |
| Overall | -0.03% | -0.04% | -0.04% | -0.03% | -0.04% | -0.04% | 101% | 101% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.12% | -0.14% | -0.14% | -0.13% | -0.14% | -0.13% | 102% | 101% |
| PQ422 | -0.14% | -0.15% | -0.16% | -0.14% | -0.15% | -0.16% | 103% | 102% |
| Overall | -0.13% | -0.14% | -0.15% | -0.13% | -0.14% | -0.14% | 102% | 101% |
| Overall PQ | -0.30% | -0.33% | -0.33% | -0.31% | -0.33% | -0.32% | 102% | 101% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.29% | -0.30% | -0.31% | 103% | 102% |
| HLG422 | -0.33% | -0.33% | -0.34% | 102% | 101% |
| Overall | -0.31% | -0.31% | -0.32% | 103% | 102% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.04% | 0.04% | 0.04% | 103% | 102% |
| HLG422 | 0.01% | 0.01% | 0.02% | 101% | 100% |
| Overall | 0.03% | 0.02% | 0.03% | 102% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.01% | 0.02% | 0.01% | 103% | 102% |
| HLG422 | -0.01% | -0.01% | -0.01% | 102% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 103% | 101% |
| Overall HLG | -0.10% | -0.10% | -0.10% | 102% | 101% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -25.96% | -24.73% | -24.73% | 98% | 82% |
| SVT12 | -3.11% | -2.91% | -2.91% | 101% | 101% |
| Overall | -14.54% | -13.82% | -13.82% | 100% | 91% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -17.18% | -16.18% | -16.28% | 105% | 91% |
| SVT12 | -0.55% | -0.61% | -0.61% | 101% | 103% |
| Overall | -8.87% | -8.39% | -8.44% | 103% | 97% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -16.12% | -16.31% | -16.27% | 103% | 91% |
| SVT12 | -0.69% | -0.67% | -0.67% | 102% | 102% |
| Overall | -8.41% | -8.49% | -8.47% | 103% | 97% |
| Overall RGB | -10.60% | -10.23% | -10.24% | 102% | 95% |

## CE-1.2b

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.71% | -0.73% | -0.73% | -0.71% | -0.73% | -0.73% | 105% | 101% |
| PQ422 | -0.80% | -0.84% | -0.86% | -0.80% | -0.85% | -0.86% | 103% | 100% |
| Overall | -0.76% | -0.79% | -0.79% | -0.76% | -0.79% | -0.79% | 104% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.03% | -0.03% | -0.06% | -0.03% | -0.03% | -0.05% | 103% | 102% |
| PQ422 | -0.02% | -0.05% | -0.03% | -0.03% | -0.05% | -0.02% | 100% | 100% |
| Overall | -0.03% | -0.04% | -0.04% | -0.03% | -0.04% | -0.04% | 102% | 101% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.12% | -0.14% | -0.14% | -0.13% | -0.14% | -0.13% | 102% | 100% |
| PQ422 | -0.14% | -0.15% | -0.16% | -0.14% | -0.15% | -0.16% | 103% | 101% |
| Overall | -0.13% | -0.14% | -0.15% | -0.13% | -0.14% | -0.14% | 102% | 101% |
| Overall PQ | -0.30% | -0.33% | -0.33% | -0.31% | -0.33% | -0.32% | 102% | 101% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.29% | -0.30% | -0.31% | 103% | 102% |
| HLG422 | -0.33% | -0.33% | -0.34% | 104% | 102% |
| Overall | -0.31% | -0.31% | -0.32% | 104% | 102% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.04% | 0.04% | 0.04% | 102% | 101% |
| HLG422 | 0.01% | 0.01% | 0.02% | 102% | 101% |
| Overall | 0.03% | 0.02% | 0.03% | 102% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.01% | 0.02% | 0.01% | 103% | 101% |
| HLG422 | -0.01% | -0.01% | -0.01% | 102% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Overall HLG | -0.10% | -0.10% | -0.10% | 103% | 101% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -26.47% | -25.04% | -25.05% | 97% | 84% |
| SVT12 | -3.11% | -2.91% | -2.91% | 101% | 101% |
| Overall | -14.79% | -13.97% | -13.98% | 99% | 92% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -17.37% | -16.57% | -16.60% | 105% | 93% |
| SVT12 | -0.55% | -0.61% | -0.61% | 103% | 104% |
| Overall | -8.96% | -8.59% | -8.60% | 104% | 98% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -16.43% | -16.58% | -16.54% | 104% | 93% |
| SVT12 | -0.69% | -0.67% | -0.67% | 102% | 102% |
| Overall | -8.56% | -8.62% | -8.60% | 103% | 97% |
| Overall RGB | -10.77% | -10.40% | -10.39% | 102% | 96% |

# Conclusions

Crosscheck results for CE-1.1 and CE-1.2 from JVET-U0064[1] have been reported. The results of the crosscheck show a close but not exact match with the simulation results provided by the proponents with the exception of encoder and decoder relative run-times which show more variation.

# References

[1] “CE-1.1 and CE-1.2: On the Rice parameter derivation for high bit-depth coding”, D. Rusanovskyy, L. P. Van, M. Coban, M. Karczewicz, JVET-U0064, JVET, 21st Meeting: by teleconference, 6-15 Jan. 2021

[2] https://vcgit.hhi.fraunhofer.de/jvet-t-ce/VVCSoftware_VTM/-/tree/CE-1.1, CE-1.1 source code

[3] https://vcgit.hhi.fraunhofer.de/jvet-t-ce/VVCSoftware_VTM/-/tree/CE-1.2a, CE-1.2a source code

[4] https://vcgit.hhi.fraunhofer.de/jvet-t-ce/VVCSoftware_VTM/-/tree/CE-1.2b, CE-1.2b source code

[5] “Common test conditions for high bit depth and high bit rate video coding”, A. Browne, T. Ikai, D.  Rusanovskyy, X. Xiu, JVET-T2018, JVET, 20th Meeting: by teleconference, 7-16 Oct. 2020