| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21th Meeting, by teleconference, 6 – 15 January 2021 | Document: JVET-U0096 |
| --- | --- |

| Title: | [EE] Tests on Decomposition, Compression, Synthesis (DCS)-based Technology |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ming Lu Zhan Ma Luhang Xu Dong Wang | Tel: Email: | +86 25 8968 1550 luming@smail.nju.edu.cn mazhan@nju.edu.cn xuluhang@oppo.com wangdong7@oppo.com |
| Source: | Nanjing University, OPPO Inc. |  |  |

_____________________________

Introduction

Recent years have witnessed the explosive growth of deep learning based applications. Particularly, deep neural network (DNN)-based image and video compression schemes have been emerged with encouraging rate-distortion efficiency [1-5]. Though the end-to-end learnt image coding solution has outperformed the VTM10.0 intra when the loss is measured using MSE [3] on a variety of databases (e.g., Kodak, Tecnick), end-to-end learnt video coding approaches still cannot rival the VVC.

Over past decades, tremendous efforts have been devoted from both ITU and ISO experts to develop a serial successful video coding standards, from the MPEG-2, to the H.264/AVC, and till recently emerged VVC. All modularized tools are carefully engineered in these masterpieces for the optimal tradeoff between the coding efficiency and complexity. Given that a majority of video products are already deployed with these standard compliant codecs, we have envisioned that it would take time for the adoption of a complete DNN-based video codec. Alternatively, in this contribution, we would like to introduce another end-to-end coding mechanism, which is fully compliant with existing video standards as well as the prospective DNN-based solutions. We call it “Decomposition, Compression & Synthesis” (DCS)-based Video Coding, or DCS for short, as shown in Figure 1 below.

Figure 1: DCS-based Video Coding: ↓ (in red) is for down-sampling , and ↑ (in red) is for up-sampling; E and D represent video encoder and decoder respectively.

Inspired by the facts that retinal cells actually segregate the visual scene into different attributes (e.g., spatial details, temporal motion) for respective neuronal processing, we propose to first decompose the input video into respective spatial texture frames (STF) at its native spatial resolution that preserve the rich spatial details, and the other temporal motion frames (TMF) at a lower spatial resolution that retain the motion smoothness; then com- press them together using standardized video coder; and finally synthesize decoded STFs and TMFs for high-fidelity video reconstruction at the same resolution as its native input. For this resolution-adaptive synthesis at decoder, a motion compensation network (MCN) is devised on TMFs to efficiently align and aggregate temporal motion features that will be jointly processed with corresponding STFs using a nonlocal texture transfer network (NL-TTN) to better augment spatial details, by which the compression and resolution resampling noises can be effectively alleviated with better rate-distortion efficiency. Such DCS based scheme is codec agnostic, exemplifying ≈1 dB PSNR gain or ≈25% BD-Rate saving on average, against the HEVC LDP anchor using common test conditions. Our recent extension to VVC has also promised the similar gains. Given that this preliminary studies have offered very encouraging results, it is suggested to further study the DCS techniques deeply for future standardization exploration.

Framework

Decomposition. The consumption of video frames over the time is the neuronal processing of natural scene stimuli in our visual pathway [7]. When lights arrive the retina, visual information will be segregated according to its attributes into different retinal cells as suggested by earlier studies. For example, cones are re- sponsible for the photopic vision (e.g., texture details, color) while rods are more sensible for scotopic vision (e.g., illumination changes, and motion). Thus, a raw video can be decomposed as a combined representation of spatial texture and temporal motion. For simplicity, we keep using the conventional “frame” term, having the spatial texture frames (STFs) and temporal motion frames (TMFs) respectively.

Though we can engineer pixel-level motion displacement (e.g., optical flow) in practice, the human visual system (HVS) typically perceives the motion at semantic object basis, suggesting that a coarse block-level motion representation indeed suffices. Thus, we downscale the TMFs to a lower spatial resolution, while keeping the STFs the same native frame size. Ideally, we wish to use STFs to preserve the spatial texture details, and have TMFs to well capture the temporal motion smoothness.

Compression. To support the general compatibility with existing video standards, the STF is enforced as the intra frame of a specific group of pictures (GoP), while the rest frames in this GoP are TMFs. We choose to code STFs and low-resolution TMFs using the mainstream HEVC model in this work. Other standards, e.g., emerging VVC, or emerging DNN-based video coding [5] can be applied as well. The STFs are HEVC intra coded, reconstructed, and downscaled to the same size as the TMF for inter prediction reference in Fig. 1.

Synthesis. In a GoP, decoded STF and TMFs are used to produce high-fidelity video at its native spatiotemporal resolution. This is a resolution-adaptive synthesis problem, aiming for the high-quality restoration to alleviate both compression and re-sampling noises for preserving both spatial details and temporal smoothness. Towards this purpose, a motion compensation network (MCN) is first utilized to generate temporally smooth and spatially fine-grained motion representation of current TMF by aggregating the information across neighboring TMFs. This MCN is comprised of a deformable convolution network-based multi-frame motion feature alignment, and a separable temporal-spatial attention-based motion feature aggregation. Then, temporal motion features generated by MCN, together with decoded TMFs, decoded STF as well as its re-sampled version, are fed into a non-local texture transfer network (NL-TTN) to learn and transfer cross-resolution information for high-fidelity frame restoration with both spatial details and temporal smoothness, where the NL-TTN has utilized high-level semantics (e.g., VGG features [8]) to guide accurate refinement for final reconstruction.

Learnt Resolution-Adaptive Synthesis

We propose a learnt resolution-adaptive syn- thesis method to restore decoded TMFs to their original resolution, by which the image quality is greatly enhanced at the same bit rate budget, and thus consequently improves the R-D efficiency.

Figure 2: Motion Compensation Network (MCN). A deformable convolution is used for temporal feature alignment across neighboring frames with an auto-encoder network to generate the feature offsets; and separable temporal-channel-spatial attentions are consecutively augmented to aggregate most proper motion features. DCN stands for the deformable convolutional network; T, H/2, W/2, and C are the dimensional size of feature maps.

Multi-Frame Motion Compensation To enforce the temporal smoothness, it is desired to accurately learn the motion dynamics across neighbor frames for compensation. It generally includes feature alignment and aggregation.

Alignment. In Fig. 2, we set N = 2, having two preceding and another two succeeding frames besides current frame for a 5-frame temporal window. Recalling that multi-frame motion features are often used for temporal alignment, we choose to use the deformable convolutions [9] to fulfill this purpose.

Aggregation. Then, we apply the attention-based mechanism to efficiently aggregate aforementioned features. We perform the separable temporal and spatial attention-based aggregation sequentially.

Non-Local Texture Transfer Network To restore the spatial texture details of decoded TMFs, one may use deformable convolutions or optical flow-based warping to align STF features to current TMF for a super-resolved reconstruction. However, alignment efficiency degrades sharply when the frame distance between STF and associated TMF increases. Instead, we design a non-local texture transfer network (NL-TTN) that resorts to semantic features for fine-grained texture transfer, as shown in Figure 4. VGG-based feature similarity is utilized to determine the transfer matrix.

Figure 3: Non-Local Texture Transfer Network (NL-TTN). T denotes the transpose, U denotes the unfold and S denotes the index select in (a).

Experiments

Datasets and Implementation Details. We combine the dataset provided in [11] that contains 182 raw video sequences with various resolutions and contents, and another one released by [12] for our model training. First 100 frames of each video are used. Test evaluations are conducted using the HEVC standard test sequences (Class B, C, D and E).

As aforementioned, re-sampling operation directly utilizes the bicubic filter offered in FFmpeg, and videos are compressed using HEVC reference software HM16.0 with low-delay P (LDP) configuration on 2.60 GHz Intel Xeon CPU. The QPs, e.g., {27, 32, 37, 42, 47}, are used for a wide bit rate coverage; The HEVC anchors use QPs from {32, 37, 42, 47, 51} for matched bit rates in comparison. The intra interval or GoP size is set to 1 second.

As shown in Table 1, our method consistently out- performs the standard HEVC with almost 1 dB gain (or equivalent ≈25% BD-Rate improvement). More than 1.5 dB gain can be observed for fast-motion “BasketballDrive’’ at 60 FPS (Frame Per Second) and slow-motion “vidyo1” at 50 FPS. Additionally, up to 1.12 dB improvements is also obtained for “Kimono” at 24 FPS. All of these have reported the generalization of DCS to videos with various content characteristics, e.g., fast/slow-motion, high/low-frame rate, rich textures, etc.

Table I: BD-Rate and BD-PSNR of Proposed DCS against HEVC anchor (1-second GoP used in LDP common test)

| Class | Sequence | HEVC LDP (GOP=1s) |  | VVC RA |  |
| --- | --- | --- | --- | --- | --- |
|  |  | BD-rate (%) | BD-PSNR (dB) | BD-rate (%) | BD-PSNR (dB) |
| B | Kimono | -29.76 | 1.12 | -14.25 | 0.48 |
|  | ParkScene | -23.33 | 0.69 | -16.68 | 0.49 |
|  | Cactus | -29.99 | 1.11 | -12.54 | 0.41 |
|  | BQTerrace | -17.54 | 0.52 | -8.01 | 0.21 |
|  | BasketballDrive | -25.90 | 0.94 | -5.65 | 0.15 |
| C | RaceHorses | -12.61 | 0.36 | - | - |
|  | BQMall | -22.58 | 0.93 | - | - |
|  | PartyScene | -23.48 | 0.73 | - | - |
|  | BasketballDrill | -37.28 | 1.63 | - | - |
| D | RaceHorses | -19.78 | 0.71 | - | - |
|  | BQSquare | -9.02 | 0.33 | - | - |
|  | BlowingBubbles | -23.45 | 0.75 | - | - |
|  | BasketballPass | -25.45 | 1.03 | - | - |
| E | vidyo1 | -31.57 | 1.70 | -14.98 | 0.81 |
|  | vidyo3 | -27.41 | 1.38 | -7.12 | 0.31 |
|  | vidyo4 | -30.32 | 1.53 | -13.77 | 0.71 |

Gains are still observed by directly applying the model trained from the HEVC. A retraining could further improve the performance. (VVC is toooo slow).

We also offer comparative studies to super-resolution- based video coding using the state-of-the-art SISR method - RCAN [13] and video super-resolution method - EDVR [9]. In the meantime, bicubic-based upsampling is also provided as a benchmark. Both RCAN and EDVR are re-trained using the same dataset for fair comparison. As depicted in Fig. 5, we select one sequence from each class for evaluation. Our DCS shows a clear performance lead against others, offering consistent gains across all bitrates, while existing methods only present the superior performance for a limited bit rate range, over the anchor.

Figure 5 Rate-distortion curves of proposed DCS, RCAN, EDVR bicubic-based video coding, and HEVC anchor.

Figure 6 visualized the reconstructions of both HEVC anchor and our DCS method. The selected frames have almost the same bitrates for a fair com- parison with partial regions zoomed in for details. Our DCS scheme offers much better perceptual quality with sharp edges, less motion blur, and more spatial texture details.

Figure 6: Qualitative Visualization: HEVC anchor (top) and proposed DCS-based method (bottom). The respective bitrate (in kbps)/PSNR (in dB) from left to right are: vidyo1 (HEVC: 115/33.14, DCS: 109/35.14), BasketballDrive (HEVC: 767/30.50, DCS: 682/31.48), BlowingBubbles (HEVC: 59/24.90, DCS: 58/26.08).

Ablation Studies

Complexity Taking the “vidyo1” with resolution of 720p as an example, its takes about 5514.17 seconds to encode 600 frames at QP 42 using HEVC reference model. On the contrary, the encoding time of our DCS at corresponding bit rate is about 1157.15 seconds, having almost 80% encoding time reduction. It is mainly because most frames in DCS are encoded as TMFs at a lower resolution. At decoder side, proposed learnt resolution-adaptive synthesis increases the processing time in addition to the HEVC decoding.

Model parameters are 5.63 MBytes and GPU running time is about 1.16 seconds.

GoP Size We further examine application scenarios when GoP size is “2 seconds” and “infinity”. For latter case, only the first frame is intra-coded STF, and all the rest frames in a sequence are inter-coded. Table 3 shows that the performance gain first increases from 1-second GoP to the 2- second one, but then drops when having infinity GoP.

Table 2 Averaged BD-Rate and BD-PSNR of Proposed DCS against HEVC anchor at various GoP size.

VVC Extension

Setting CTC videos (Class A, B, E) are compressed using VVC reference software VTM 10.0 with low-delay P (LDP) configuration. The QPs, e.g., {27, 32, 37, 42}, are used for a wide bit rate coverage of our model; The VVC anchors use QPs from {32, 37, 42, 47} for matched bit rates in comparison. Only the first frame is encoded with the intra coding configuration. The BD-rates are calculated in Y channel with 10 bit depth.

| Class | Sequence | LDP |
| --- | --- | --- |
|  |  | BD-rate (%) |
| A1(4K) | Tango2 | -14.26 |
|  | FoodMarket4 | -14.47 |
|  | Campfire | -25.55 |
| A2(4K) | CatRobot | -21.16 |
|  | DaylightRoad2 | -11.51 |
|  | ParkRunning3 | -18.24 |
| B(1080p) | MarketPlace | -8.33 |
|  | RitualDance | -9.83 |
|  | Cactus | -15.65 |
|  | BasketballDrive | 2.52 |
|  | BQTerrace | 35.53 |
| E(720p) | FourPeople | -27.05 |
|  | Johnny | -13.51 |
|  | KristenAndSara | -25.07 |

The loss mainly comes from the scene cuts in a test sequence (e.g., camera moving in BQTerrace). It suggests the optimal coding efficiency of DCS could be possibly achieved when aligning the GoP with scene cuts.

Conclusion

The DCS algorithm generally belongs to the resolution resampling-based video coding, leveraging STF for spatial texture details and low-resolution TMFs for temporal smoothness. This work utilizes the bicubic filter-based decomposition, HEVC/VVC compatible video compression, and mainly study the learnt resolution-adaptive synthesis. This DCS framework complies with almost all existing standards, reporting remarkable PSNR improvements via multi-frame motion compensation and nonlocal texture transfer, against the HEVC/VVC anchor and other state-of-the-art methods.

Even though, there are a variety of interesting topics for further exploration:

- Fully VVC validation with RA

- Fully DNN-based video coding solution integration

- Attention-based adaptive resolution resampling

- Embedding QP and predictive structures into learning-based synthesis

- Complexity reduction and efficiency tradeoff

- Appropriate syntax and semantics definitions

References

J. Ballé, D. Minnen, S. Singh, S. J. Hwang, and N. Johnston, ‘Variational image compression with a scale hyperprior’, arXiv:1802.01436 [cs, eess, math], Jan. 2018, Accessed: May 25, 2018. [Online]. Available: http://arxiv.org/abs/1802.01436.

D. Minnen, J. Ballé, and G. Toderici, ‘Joint Autoregressive and Hierarchical Priors for Learned Image Compression’, arXiv:1809.02736 [cs], Sep. 2018, Accessed: Feb. 10, 2020. [Online]. Available: http://arxiv.org/abs/1809.02736.

Chen, T., Liu, H., Ma, Z., Shen, Q., Cao, X., & Wang, Y. (2019). Neural image compression via non-local attention optimization and improved context modeling. arXiv preprint arXiv:1910.06244.

Liu, H., Lu, M., Ma, Z., Wang, F., Xie, Z., Cao, X., & Wang, Y. (2020). Neural Video Coding using Multiscale Motion Compensation and Spatiotemporal Context Model. arXiv preprint arXiv:2007.04574.

Liu, H., Huang, L., Lu, M., Chen, T., & Ma, Z. (2019). Learned video compression via joint Spatial-Temporal correlation exploration. AAAI，Jan. 2020.

B. Bross, J. Chen, S. Liu, “Versatile Video Coding draft 7,” JVET-P2001, October 2019.

Schwartz, O.; and Simoncelli, E. 2001. Natural signal statis- tics and sensory gain control. Nature neuroscience 4(8): 819.

Simonyan, K.; and Zisserman, A. 2014. Very Deep Convo- lutional Networks for Large-Scale Image Recognition.

Wang, X.; Chan, K. C.; Yu, K.; Dong, C.; and Change Loy, C. 2019. EDVR: Video Restoration With Enhanced De- formable Convolutional Networks. In Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition (CVPR) Workshops.

Li, T.; Xu, M.; Zhu, C.; Yang, R.; Wang, Z.; and Guan, Z. 2019. A Deep Learning Approach for Multi-Frame In-Loop Filter of HEVC. IEEE Transactions on Image Processing 28(11): 5663–5678.

Song, L.; Tang, X.; Zhang, W.; Yang, X.; and Xia, P. 2013. The SJTU 4K video sequence dataset. 34–35. doi:10.1109/ QoMEX.2013.6603201.

Zhang, Y.; Li, K.; Li, K.; Wang, L.; Zhong, B.; and Fu, Y. 2018. Image Super-Resolution Using Very Deep Residual Channel Attention Networks. In Proceedings of the Euro- pean Conference on Computer Vision (ECCV).

Patent rights declaration(s)

Nanjing University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

OPPO Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).