| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0113 |
| --- | --- |

| Title: | Crosscheck of JVET-U0070 (CE Related: On signalling and encoder optimization for Rice parameter derivation) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu | Email: | jhuhong-jheng@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

Reported in this document is the cross-check results on signalling and encoder optimization for Rice parameter derivation as reported in JVET-U0070. The cross-check partial results match the coding performance reported by the proponent.

# Results

The source code for JVET-U0070 was provided by the proponent. The software is built based on VTM-11.0rc1. The crosscheck results are illustrated as following tables. The detailed results could be seen in the attached excel files.

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.94% | -0.83% | -0.83% | -0.95% | -0.84% | -0.83% | 106% | 99% |
| PQ422 | -1.04% | -0.97% | -1.00% | -1.05% | -0.97% | -1.01% | 102% | 98% |
| Overall | -0.99% | -0.90% | -0.92% | -1.00% | -0.90% | -0.92% | 104% | 98% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.08% | -0.09% | -0.12% | -0.08% | -0.09% | -0.10% | 101% | 105% |
| PQ422 | -0.08% | -0.10% | -0.11% | -0.09% | -0.10% | -0.10% | 105% | 108% |
| Overall | -0.08% | -0.10% | -0.11% | -0.08% | -0.10% | -0.10% | 103% | 106% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.19% | -0.18% | -0.18% | -0.20% | -0.18% | -0.19% | 107% | 101% |
| PQ422 | -0.20% | -0.19% | -0.20% | -0.21% | -0.20% | -0.20% | 104% | 102% |
| Overall | -0.20% | -0.18% | -0.19% | -0.20% | -0.19% | -0.19% | 105% | 101% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.52% | -0.38% | -0.39% | 102% | 101% |
| HLG422 | -0.58% | -0.43% | -0.43% | 102% | 99% |
| Overall | -0.55% | -0.41% | -0.41% | 102% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.01% | 0.00% | -0.01% | 105% | 110% |
| HLG422 | -0.03% | 0.00% | 0.00% | 105% | 103% |
| Overall | -0.02% | 0.00% | 0.00% | 105% | 106% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -27.54% | -25.71% | -25.71% | 98% | 99% |
| SVT12 | -4.18% | -3.77% | -3.78% | 103% | 107% |
| Overall | -15.86% | -14.74% | -14.74% | 100% | 103% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM11.0rc1 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -18.02% | -17.23% | -17.25% | 111% | 108% |
| SVT12 | -1.02% | -1.02% | -1.03% | 109% | 113% |
| Overall | -9.52% | -9.13% | -9.14% | 110% | 110% |

# Conclusion

The partial results obtained in the cross-check match the results presented in JVET-U0070 with the exception of encoder and decoder relative run-times which show variation of up to 10%.