| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0119 |
| --- | --- |

| Title: | Dry run subjective assessment of SDR HD and 360 video verification tests |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Mathias Wien Vittorio Baroncini | Tel: Email: | wien@lfb.rwth-aachen.de baroncini@gmx.com |
| Source: | Verification Test Coordinators |  |  |

_____________________________

# Abstract

This document provides a report on results of subjective quality evaluation for test sequences under consideration for the VVC verification test. The QP selection for the test sequences under consideration in the SDR HD and 360° video categories as proposed in JVET-U0041 [2] is tested for suitability for use in the VVC verification test according to the verification test plan JVET-T2009 [1]. Furthermore, candidate QPs for gaming-type test sequences are proposed and evaluated. In the SDR HD category, subset of the candidate test sequences and candidate rate points for the low delay and random access configurations are evaluated. In the 360° video category, mp4 files of viewports of the test sequences encoded using the PERP projection format are assessed. Here, the full set of rate points was included in the tests. The results indicate that the candidate QPs provide a coverage of the quality range from quite bad to very good quality and some adjustments of the QPs are suggested. In the SDR HD category, overall rate saving estimates cannot be provided from the data. Only potential isolated rate savings are estimated for rate points with overlapping confidence intervals. In the 360° video category, estimated rate savings between approximately 30% and 60% are reported.

# Introduction

The QP selection for the test sequences under consideration in the SDR HD and 360° video categories as proposed in JVET-U0041 [2] is tested for suitability for use in the VVC verification test according to the verification test plan JVET-T2009 [1]. Furthermore, candidate QPs for gaming-type test sequences are proposed and evaluated. According to the test plan, five rate points are selected for each sequence with the intention to cover the range from quite bad to very good quality. Formal subjective assessment dry run experiments are conducted with naïve viewers to assert the selected quality range and further achieve an approximate quality matching of the selected rate points for both, VTM and HM. In order to limit the assessment workload, the low, mid, and highest rate points are evaluated for test sequences listed in JVET-T2009. The test sequence selection and the results of the tests for each of the categories are presented in the following sections. Where possible, an estimate of the expected rate savings is reported. If less than four rate points have been tests, these are computed for rate points with sufficiently matching MOS values.

## Formal subjective assessment

### DSIS Test Method

The DSIS test method is described in recommendation BT 500-14 [3]; an 11 grades impairment scale was used as specified in BT 500-14. The Basic Test Cell of the DSIS method is following the below timing:

- 0,5 sec	Text “A” on mid-grey background

- 10 sec	Playback of uncompressed source file

- 0,5 sec	Text “B” on mid-grey background

- 10 sec	Playback of processed video sequence

- 4 sec	Text “Vote N” on mid-grey background

Thereby, each BTC has a duration of 25 sec.

### Test session design

The test was designed using a single test session, made of 43 BTCs ; the first three BTCs represent the “stabilization phase” (see rec. BT500-14 for details); the scores collected watching the first three BTCs were discarded from the final statistical analysis of the results.

The session was run two times, with different randomization orders to minimise contextual effects. This was done to try to improve the results, even though DSIS reportedly is immune to these effects.

The test was done using raw playout of the YUV files for the SDR HD category and with mp4 files for the 360° video category.

### Laboratory set up and COVID procedures

The test was conducted and evaluated at GBTECH ltd. A total of 18 viewers (6 females and 12 males) participated to the test, aging from 18 to 22 years; all viewers were screened for visual acuity and color blindness. A post screening of the viewers was done performing a statistical analysis of each individual’s responses; no viewers was discarded.

To guarantee a safe test and rest areas, all the rooms used to run the test and to let viewers rest, were accurately cleaned (the day before) using disinfectant cleaner (based on alcohol at 70% concentration); the rooms were sanitized using a one-shot spray disinfectant (by Germo S.P.A.) one hour before the test session.

All test subjects were required to provide a recent Corona test (max 1 week before) with a negative test result.

When viewers entered the test area, they were asked to take off the shoes and to clean their hands using a disinfectant cleaner (same used at hospital).

All viewers were wearing a protection masks for all the time they were in the testing and resting areas.

The resting areas and the test areas were wide enough to guarantee ate least 2 mt distancing arrangement.

For this reason, the test session was run three times with the participation of 6 different viewers each.

Two test rooms and two resting rooms were used, 2 subjects were watching the screen on each test room, seated one meter to each other and 2H from the 65” monitor; the viewing angle was below 60°.

The resting room was wide enough to allow a viewers seating at the due security distance.

All viewers were monetary compensated for their participation to the test.

The scores were collected by means of paper scoring sheets.

# SDR HD category

## SDR HD Low delay: Conversational content

In JVET-U0041 [2], a QP selection was proposed for four test sequences with conversational content as listed in the table below. Out of the four test sequences, the sequences Beatriz and OfficeWalkCeiling were selected for visual assessment in the dry-run tests. In order to further reduced the number of test points, the low, mid, and highest values were selected out of the five available QPs.

| Sequence | HM QPs | VTM QPs |
| --- | --- | --- |
| Beatriz_1920x1080_50fps_8bit_420 (tested) | 41, 37, 33, 28, 23 | 42, 38, 34, 29, 24 |
| OfficeWalkAtWall_1920x1080_30fps_8bit_420pf | 46, 42, 38, 32, 26 | 47, 44, 40, 34, 27 |
| OfficeWalkAndTalk_1920x1080_30fps_8bit_420pf | 45, 40, 34, 30, 24 | 46, 42, 37, 32, 28 |
| OfficeWalkCeiling_1920x1080_30fps_8bit_420pf (tested) | 45, 40, 34, 30, 24 | 46, 42, 36, 32, 26 |

The original YUV files of the sequences are available on the JVET ftp site in the directory ahg/verificationtests/candidates/sdr/hd. The provided bitstreams were encoded using the GOP4 configuration. Bitstreams were provided via upload to the JVET ftp site in JVET-T_VT/sdr-hd-ld/Beatriz and JVET-T_VT/JVET-T0060/10second/bistream/.

### Beatriz

Observations:

- The QPs appear to be well aligned and the quality range well covered.

- Gains for separate rate points: (high, mid, low rate point) -33%, -29%, -28%.

- Comments from the subjective viewing session: The sequence seems to be easy to focus on, widely spread area of interest, subjects concentrate on different areas of the scene.

### OfficeWalkCeiling

Observations:

- QPs well aligned, highest rate point could potentially be a bit higher

- Gains for separate rate points: (high, mid, low rate point): -42%, -38%, -37%.

## SDR HD Low delay: Gaming content

In JVET-T2009, a set of seven test sequences for gaming-type applications as listed as shown in the table below. The original sequences are of 1-minute length each. For the encoding and subjective assessment purposes, the indicated clips of 10sec each have been selected as reported in JVET-U0127 [4]. Selection criteria included avoidance of extreme camera motion and avoidance of scene cuts as much as possible.

| Test sequence | Resolution | fps | Start frame | Frames |
| --- | --- | --- | --- | --- |
| GSGO | 1920×1080 | 60 | 1470 | 600 |
| DOTA2 | 1920×1080 | 60 | 0940 | 600 |
| EuroTruckSimulator2 | 1920×1080 | 60 | 3000 | 600 |
| GTAV | 1920×1080 | 60 | 0091 | 600 |
| Hearthstone | 1920×1080 | 60 | 0000 | 600 |
| Rust | 1920×1080 | 60 | 0360 | 600 |
| Starcraft | 1920×1080 | 60 | 2463 | 600 |

The original YUV files of the sequences are available on the JVET ftp site in the directory ahg/verificationtests/candidates/sdr/hd. The provided bitstreams were encoded using the GOP8 configuration. Bitstreams were provided via upload to the JVET ftp site in JVET-U_VT/sdr-hd-ld/gaming/gop8/.

Out of the set listed above, four sequences have been selected which was deemed to be most suitable for evaluation in a formal subjective assessment. An initial set of QPs has been selected for these sequences based on observations from experts viewing. The sequences and the selected QPs are listed in the table below. The QPs skipped in the visual assessment have been greyed out in the table.

| Sequence | HM QPs | VTM QPs |
| --- | --- | --- |
| DOTA2 | 38,34,29,24,20 | 40,36,31,26,22 |
| EuroTruckSimulator2 | 41,36,31,26,21 | 42,37,32,27,22 |
| RUST | 39,34,30,26,21 | 40,36,32,27,22 |
| STARCRAFT | 38,34,30,26,21 | 40,36,32,28,23 |

Rate-distortion plots for these sequences and using the selected QPs are summarized in the figures below:

DOTA2

EuroTruckSimulator2

RUST

STARCRAFT

### DOTA2

Observations:

- The highest rate point appears to be in the saturation range

- Gains for separate rate points: (high, mid, low rate point): -32%, -38%, -37%

### EuroTruckSimulator2

Observations:

- The highest rate points are in saturation while the lowest rate points are not low enough in terms of MOS. All QPs should be increased.

- The low and mid rate points are not well aligned in terms of MOS values which needs to be regarded.

- An estimation of rate savings on rate point pairs does not appear useful due to the misalignment. The test sequence seems to expose significant rate savings for VTM nonetheless.

### RUST

Observations:

- VTM and HM QPs appear to be well aligned. The lowest QP should not be decreased further.

- Gains for separate rate points: (high, mid, low rate point): -22%, -44%, -43%.

### STARCRAFT

Observations:

- The highest rate points are in saturation while the lowest rate points are not low enough in terms of MOS. All QPs should be increased.

- For the VTM mid-rate point, rate savings of about -40% are observed compared to the corresponding HM rate point. It is noted that the quality of the HM point is a bit lower than for the VTM.

## SDR HD Random Access

In JVET-U0041 [2], a QP selection was proposed for the test sequences under consideration. Out of the seven available test sequences, five were selected for visual assessment in the dry-run tests after visual inspection. In order to further reduced the number of test points, the low, mid, and highest values were selected out of the five available QPs.

| Sequence | HM QPs | VTM QPs |
| --- | --- | --- |
| BarScene | 40, 36, 32, 28, 23 | 43, 39, 35, 31, 26 |
| BarSceneFs360 | 40, 36, 32, 28, 23 | 43, 39, 35, 31, 26 |
| DrivingPOV | 39, 35, 31, 27, 24 | 42, 38, 34, 30, 26 |
| DrivingPOVFs480 | 39, 35, 31, 27, 23 | 42, 38, 34, 30, 26 |
| Meridian1 | 39, 35, 31, 27, 23 | 42, 38, 34, 30, 26 |
| Metro | 39, 35, 31, 27, 24 | 42, 38, 34, 30, 26 |
| RushHour | 42, 38, 33, 29, 24 | 44, 40, 35, 31, 26 |

Bitstreams were provided via upload to the JVET ftp site in JVET-U_VT/sdr-hd-ra and JVET-U_VT/sdr-hd-ra/meridian_dat/.

### BarScene

Observations:

- Quality range is well covered. The QP alignment needs to be improved for lowest rate point

- Gains for separate rate points: (high and mid rate point): -37%, -38%

### DrivingPOV

Observations:

- The QPs appear to be well aligned at the top and low end and the quality range well covered. Middle range needs adjustment. The lowest point could potentially be moved a bit further down.

- Gains for separate rate points: (high, low rate point): -40%, -48%.

### Meridian1

Observations:

- Highest rate point potentially at saturation, QP of HM should be lowered there. Middle range needs adjustment. The lowest point could potentially be moved a bit further down.

- Rate savings at lowest rate point: -39%.

### Metro

Observations:

- Quality range is well covered. QP alignment needs to be improved.

- Rate savings at highest rate point: -31%. Overall gain should be significantly higher.

### RushHour

Observations:

- Quality range is well covered. QP alignment needs to be improved at the low end.

- Gains for separate rate points: (high and mid rate point) -37%, -38%.

# 360° video category (PERP only)

In JVET-U0041 [2], a QP selection was proposed for the four 360° video test sequences as listed in the table below. Tests in the 360° video category have been performed using the available mp4-files of the extracted viewpaths. Based on availability, mp4 files of extracted viewports from the bitstreams for the PERP projection format have been evaluated.

| Test sequence | HM-16.22 QP | VTM-11.0 |
| --- | --- | --- |
| GT_Sheriff | 36, 32, 28, 24, 20 | 38, 34, 30, 26, 22 |
| HarborBiking2 | 35, 31, 27, 23, 20 | 38, 34, 30, 26, 22 |
| KiteFliteWalking2 | 40, 36, 32, 28, 24 | 42, 38, 34, 30, 26 |
| SkateBoardAtBridge | 39, 35, 31, 27, 24 | 42, 38, 34, 30, 26 |

The mp4-files of the uncompressed viewpaths of the sequences are available on the JVET ftp site in the directory ahg/verificationtests/candidates/360/viewports. The provided bitstreams and extracted viewpaths mp4 files were provided via upload to the JVET ftp site in JVET-T_VT/360/<Sequencename>/<Codec>/<Projection format>.

Since five rate points have been used in the visual assessment, indicative BDR numbers are provided with the curves.

### GT_Sheriff

Observations:

- Quality range is well covered. Quality could go lower if deemed useful, with potential improvements of rate point allocation in the mid range and a potential adjustment of the two highest HM rate points.

- Estimated BD rate savings: -42.45%

### HarborBiking2

Observations:

- Quality range is well covered. Quality could go lower if deemed useful.

- Estimated BD rate savings: -60.02%

### KiteFliteWalking2

Observations:

- Quality range is well covered.

- Estimated BD rate savings: -33.20%

### SkateBoardAtBridge

Observations:

- Quality range is well covered. Quality could go lower if deemed useful.

- Estimated BD rate savings: -59.81%.

# Conclusions

This document provides results for test sequences under consideration for the SDR HD and 360° video categories.

It is suggested to further deliberate on the set of test sequences to be used in the SDR HD low delay test category and the size and composition of the applicable set. Based on the observed results, it is suggested that a set for the SDR HD RA category could be determined at this meeting.

In the 360° video category using the PERP projection format, promising results have been achieved based on the available mp4 files. It is suggested to investigate the rate points for the GCMP/PCMP format in a similar way. It is noted that viewport extraction has to be performed and confirmed in order to proceed with this task.

# References

- M. Wien, V. Baroncini, A. Segall, and Y. Ye, “VVC verification test plan (Draft 4),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, output document JVET-T2009, 20th JVET meeting, by teleconference, Oct 2020.

- M. Wien, V. Baroncini, Y. Ye, “Status Report on SDR HD and 360 Video Verification Test Preparation,” 	Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, input document JVET-U0041, 21st JVET meeting, by teleconference, Jan 2021.

- Recommendation ITU-R BT.500-14 (2019), Methodologies for the subjective assessment of the quality of television images.

- F. Le Léannec, G. Martin-Cocher, E. Francois, M. Wien, “Coding results of DERF-TWITCH game sequences for the SDR HD low delay VVC verification test,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, input document JVET-U0127, 21st JVET meeting, by teleconference, Jan 2021.