| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 21st Meeting, by teleconference, 6–15 Jan. 2021 | Document: JVET-U0135-v1 |
| --- | --- |

| Title: | Update on open, optimized VVC implementations VVenC and VVdeC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Adam Wieckowski, Jens Brandenburg, Christian Bartnik, Valeri George, Jens Güther, Gabriel Hege, Christian Helmrich, Anastasia Henkel, Tobias Hinz, Christian Lehmann, Christian Stoffers, Ivan Zupancic, Benjamin Bross, Heiko Schwarz, Detlev Marpe | Email: | firstname.lastname@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI, Einsteinufer 37, 10587 Berlin |  |  |

_____________________________

# Abstract

This document provides updated information on features, coding efficiency and runtime for version 0.2.1 of VVenC released in December 2020. Main changes for VVenC since version 0.1 include:

- License: SW license changed to a copyright 3-clause BSD license

- Presets: Increased coding efficiency and speed for existing presets (faster, fast, medium, slow) and added a new preset “slower” to achieve VTM-11 CTC coding efficiency.

- New coding tools: Intra Sub-Partitions (ISP), Transform Skip Residual Coding (TSRC), Block-level Differential Pulse Code Modulation (BDPCM)

- Rate Control (RC): improved 1-pass RC and added new 2-pass RC.

Without QP adaptation for subjective optimization and 6 threads the following PSNR-based YUV BD-rates and speedup factors compared to HM and VTM-11.0 (GOP32+MCTF) are reported for different presets:

Faster		HD:   −6.0%, 33x (HM), 250x (VTM)	UHD: −15.7%, 54x (HM), 390x (VTM)

Fast		HD: −26.2%, 21x (HM), 160x (VTM)	UHD: −29.4%, 32x (HM), 230x (VTM)

Medium	HD: −37.2%, 9.3x (HM), 71x (VTM)	UHD: −39.3%, 15x (HM), 110x (VTM)

Slow		HD: −40.3%, 3.6x (HM), 27x (VTM)	UHD: −42.1%, 5.9x (HM), 43x (VTM)

Slower		HD: −43.6%, 0.7x (HM), 5.2x (VTM)	UHD: −45.4%, 1.2x (HM), 8.8x (VTM)

With QP adaptation for subjective optimization and 6 threads, the following MS-SSIM-based YUV BD-rates and speedup factors compared to HM and VTM-11.0 (GOP32+MCTF) are reported:

Faster		HD: −19.0%, 32x (HM), 250x (VTM)	UHD: −18.1%	51x (HM), 410x (VTM)

Fast		HD: −30.0%, 21x (HM), 160x (VTM)	UHD: −30.6%	31x (HM), 250x (VTM)

Medium	HD: −40.5%, 9.2x (HM), 73x (VTM)	UHD: −41.4%, 15x (HM), 120x (VTM)

Slow		HD: −43.4%, 3.6x (HM), 28x (VTM)	UHD: −44.4%,	5.8x (HM), 47x (VTM)

Slower		HD: −46.0%, 0.7x (HM), 5.5x (VTM)	UHD: −47.5%,	1.2x (HM), 9.7x (VTM)

Main changes for the VVdeC software decoder since version 0.1 include a number of bug fixes as well as its SW license changed to a copyright 3-clause BSD license.

# Introduction

The VVC test model (VTM) [1] serves as a common reference implementation, i.e. a test bed for evaluation and verification of proposed technologies during standardization. While VTM used to be the only publicly available encoder and decoder implementation of the VVC standard, it is aimed at correctness, completeness and readability and should not serve as a real-world example of a VVC encoder and decoder. The Fraunhofer Versatile Video Encoder (VVenC) [2] and Decoder (VVdeC) [3] development was initiated to provide publicly available, fast and efficient VVC software implementations. A first version 0.1 was released in September 2020 and an overview can be found in JVET-T0099 [4].

Bit-streams encoded with VVenC can be decoded by any VVC standard compliant decoder, e.g. the VTM-11.0 reference software decoder. Alternatively, the VVdeC can be used.

VVenC features (version 0.2.1.0):

- Encoder implementation with four predefined quality/speed presets.

- Optimized VVC encoder providing speedups between 5x and 390x over VTM-11.0 for HD and UHD test sequences depending on the chosen quality/speed preset.

- Perceptual optimization to improve subjective video quality.

- Frame-level rate control supporting VBR encoding.

- Expert mode encoder interface available, allowing fine-grained control of the encoding process.

VVdeC features (version 0.2.0.1):

Supported (requires testing):

- Main10 profile without sub-pictures

In development/experimental:

- Other chroma sub-sampling formats (422, 444)

Not supported:

- Sub-pictures

- Other profiles

# VVenC - Fraunhofer Versatile Video Encoder

## Getting Started

### How to obtain VVenC?

The software is hosted at GitHub under:

https://github.com/fraunhoferhhi/vvenc

To clone the project use:

git clone https://github.com/fraunhoferhhi/vvenc

### How to build VVenC?

The software uses CMake to create platform-specific build files. A working CMake installation is required for building the software. Download CMake from http://www.cmake.org/ and install it. The following targets are supported: Windows (Visual Studio), Linux (gcc) and MacOS (clang).

How to build for Windows?

In order to compile the software for Windows, Visual Studio 15 2017 or higher and CMake Version 3.12 or higher are required. Install gnuwin32 that provides make for Windows. To build the software open a command prompt window, change into the project directory and use:

make install-release

This will create the statically linked release version of the encoder applications in the install/bin/release-static/ subdirectory.

How to build for Linux/MacOS?

In order to compile the software for Linux, gcc-7.0 or higher and CMake Version 3.12 or higher are required. For MacOS Xcode and CMake Version 3.12 or higher are required. To simplify the build process a Makefile with predefined targets is available. To build the VVenC encoder applications open a terminal, change into the project directory and use:

make install-release

This will create the statically linked release version of the encoder applications in the install/bin/release-static/ subdirectory.

### How to use VVenC?

The encoder project includes two encoder executables, a standard encoder (vvencapp) and a full featured expert encoder (vvencFFapp).

How to use the standard encoder?

The standard encoder (vvencapp) can be used in one of five predefined presets. Each preset represents a different tradeoff between encoder runtime and video quality. In the slowest preset, the encoder reaches the highest compression gain, whilst in the fastest preset the runtime is significantly decreased. These preset configurations have been determined based on the Pareto optimal configuration set of the encoder configuration space, which is detailed in [5]. A list of the main encoder command line parameters is shown in Table I (highlighted parts are newly added compared to VVenC v0.1).

Table I: List of important encoder options. For full list please use the --help option.

| Option | Default | Description |
| --- | --- | --- |
| --help,-h | - | Show basic help |
| --input <str> | - | Raw yuv input file |
| --size <wxh> | 1920x1080 | Input file resolution (width x height) |
| --framerate <int> | 60 | Temporal rate of input file. Required for rate control and calculation of output bit-rate |
| --format <str> | yuv420 | Set input format to YUV 4:2:0 8bit (yuv420) or YUV 4:2:0 10bit (yuv420_10) |
| --output <str> | not set | Bit-stream output file |
| --preset <str> | medium | Select preset for specific encoding setting (faster, fast, medium, slow, slower) |
| --qp <int> | 32 | Quantization parameter (0..51) |
| --bitrate <int> | 0 | Bitrate for rate control (0 constant QP encoding rate control off, otherwise bits per second). Rate control requires correct framerate. |
| --passes <int> | 1 | Number of passes used for rate control |
| --qpa <int> | 2 | Perceptual QP adaptation (0: off, on for 1: SDR(WPSNR), 2: SDR(XPSNR), 3: HDR(WPSNR), 4: HDR(XPSNR)) |
| --threads <int> | size <= HD: 4 else : 6 | Number of threads (1-N) |

Example usage: Given a YUV 4:2:0 input file with a bit-depth of 8bit and a resolution of 176x144 pixels, the following call will encode the input file with the medium speedup preset:

vvencapp --preset medium -i BUS_176x144_75@15.yuv -s 176x144 -o str.266

How to use the full featured expert mode encoder?

The expert mode encoder (vvencFFapp) is based on the VTM configuration scheme. Most of the parameters have been kept similar to VTM, but for some parameters, additional modes are available. Furthermore, not supported options have been removed. Example configuration files for the expert mode encoder can be found in the cfg sub-directory (see Table II, highlighted parts are newly added compared to VVenC v0.1).

Example usage: In order to start your first experiments with the expert mode encoder, adapt the sequence.cfg configuration file to your input YUV source file and use the following command:

vvencFFapp -c randomaccess_medium.cfg -c sequence.cfg

Table II: Expert mode encoder configuration files provided in the ./cfg sub-directory.

| Configuration File | Description |
| --- | --- |
| sequence.cfg | Sequence specific configuration parameters. Must be always adapted to the input sequence. |
| randomaccess_faster.cfg randomaccess_fast.cfg randomaccess_medium.cfg randomaccess_slow.cfg randomaccess_slower.cfg | Random access configuration for different presets. Each configuration file corresponds to one of the 4 preset modes. |
| qpa.cfg | Perceptually optimized QPA configuration file. |
| gop32.cfg | Experimental. Additional GOP size 32 configuration, replacing the default GOP size 16 configuration. Must be given at the command line after the random access configuration file. |
| frc.cfg | Frame level single pass rate control configuration, overriding default fix QP setup. Note: Currently incompatible with GOP 32. |
| rc2p.cfg | Frame level two pass rate control configuration, overriding default fix QP setup. |

## Encoder Performance

The encoder performance tests focus on the most relevant HD (1920x1080) and UHD (3840x2160) resolution use cases with random access encoding as defined in the JVET common test conditions (CTC) [6]. All presented results are shown for the CTC test sequences, i.e. classes A1 and A2 for UHD and class B for HD sequences. Constant QP encoding is used with QP values of 22, 27, 32 and 37 according to VTM CTC. Reported rate distortion results are calculated as Bjøntegaard delta rate (BD-rate) [7]. For the multi-threading use case 6 threads have been enabled in the encoder configuration. All tests have been performed on Dell Super Micro servers with Intel Xeon processors E5-2697A v4 @2.6GHz.

### Standard Use Case

The PSNR BD-rate gain of VVenC over the HEVC test model reference software HM-16.22 is shown in Figure 1. PSNR BD-rate represents the approximate average bit-rate savings between two encoders for the same objective quality (PSNR). Here lower values mean larger bit-rate savings with respect to the HM-16.22 anchor. Please note the logarithmic scale of the relative encoder runtime in comparison to HM-16.22.

With the slower preset and multi-threading with 6 threads enabled the BD-rate gain of VVenC over HM is similar to VTM-11 common test conditions (CTC), but a speedup of more than 8.6x for UHD and 5.2x for HD sequences is achieved over VTM.

With the faster preset and multi-threading with 6 threads the BD-rate gain over HM is still approx. 11.3%, with a speedup of more than 390x for UHD and 250x for HD respectively over VTM. Comparing the runtime with HM gives a speedup of 54x for UHD and 33x for HD.

As a good tradeoff between encoder runtime and BD-rate performance, we recommend the medium preset with multi-threading enabled. Here, the BD-rate gain over HM is approx. 38.3%, which is quite close to the slow preset and VTM CTC, but in comparison to VTM-11.0 the encoder runs 110x faster for UHD and 71x faster for HD sequences. Compared to HM-16.22 this is an encoder runtime speedup of 15x for UHD and 9.3x for HD. A summary of all results is shown in Table III.

Table III: PSNR BD-rate and multi-threaded (6 threads) encoder speedup for HD and UHD test sequences.

| Preset | HD |  |  | UHD |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSNR BD-rate vs. HM | Speedup vs. HM | Speedup vs. VTM | PSNR BD-rate vs. HM | Speedup vs. HM | Speedup vs. VTM |
| FASTER | -6.0% | 33x | 250x | -15.7% | 54x | 390x |
| FAST | -26.2% | 21x | 160x | -29.4% | 32x | 230x |
| MEDIUM | -37.2% | 9.3x | 71x | -39.3% | 15x | 110x |
| SLOW | -40.3% | 3.6x | 27x | -42.1% | 5.9x | 43x |
| SLOWER | -43.6% | 0.7x | 5.2x | -45.4% | 1.2x | 8.8x |

Figure 1: PSNR BD-rate gain and relative encoder runtime in comparison to HM-16.22 for VVenC and VTM-11 (HD and UHD test sequences). Results are given for the 5 preset options: faster, fast, medium, slow and slower. Lower PSNR BD-rate values mean a better compression for the same objective quality in terms of PSNR.

Additionally, Figure 2 includes multi-threaded results for the HEVC open-source encoder x265 v3.4 at comparable speed presets [8]. For the comparison with VVenC, also x265 was configured to run with 6 threads. Besides sequence-specific parameters, the following parameter settings have been used:

--preset {1,2,3,…,9}  --frame-threads 6  --wpp no-wpp  --tune psnr  
--crf {17,22,27,32}--keyint 1s  --min-keyint 1s  --profile main10  
--output-depth 10

Figure 2: PSNR BD-rate gain and relative encoder runtime in comparison to HM-16.22 for VVenC and x265 running with 6 threads (HD and UHD test sequences). Lower PSNR BD-rate values mean a better compression for the same objective quality in terms of PSNR.

### Perceptually Optimized Quantization Parameter Adaptation

To improve the perceived (subjective) coding quality, VVenC supports a low-complexity quantization parameter adaptation (QPA) algorithm based on a model of the human visual system. To evaluate the quality of the perceptually optimized quantization parameter adaptation (PQPA), multiscale structural similarity measure (MS-SSIM) [9] is used.

In Figure 3 the MS-SSIM BD-rate gain of VVenC over the HEVC test model reference software HM-16.22 is shown (lower is better). For VTM simulation, common test conditions CTC with additional PQPA is used (--SliceChromaQPOffsetPeriodicity=1, --PerceptQPA=1). With PQPA enabled, the speedups achieved over HM and VTM are similar to the Non-PQPA results presented in the previous section. This demonstrates the low-complexity nature of the PQPA algorithm. Comparing the MS-SSIM based BD-rates, additional bitrate reduction can be achieved. Especially for the slower preset, an MS-SSIM BD-rate gain of more than 4.3% over VTM CTC without PQPA is realized. We recommend using the medium preset with multi-threading and PQPA enabled as a good tradeoff between encoder runtime and resulting perceived video quality. A summary of the MS-SSIM results for PQPA is shown in Table IV

Figure 3: MS-SSIM BD-rate gain and encoder runtime in comparison to HM-16.22 for VVenC with perceptually optimized quantization parameter adaptation (QPA) enabled. VVenC results are given for the 5 preset options: faster, fast, medium, slow and slower. Lower MS-SSIM BD-rate values mean a better compression for the same quality in terms of MS-SSIM.

Additionally, Figure 4 includes multi-threaded results for the HEVC open-source encoder x265 v3.4 tuned for SSIM at comparable speed presets [8]. For the comparison with VVenC, also x265 was configured to run with 6 threads. Besides sequence-specific parameters, the following parameter settings have been used:

--preset {1,2,3,…,9}  --frame-threads 6  --wpp no-wpp  --tune ssim  
--crf {17,22,27,32}  --keyint 1s  --min-keyint 1s  --profile main10  
--output-depth 10

Figure 4: MS-SSIM BD-rate gain and encoder runtime in comparison to HM-16.22 for VVenC with QPA enabled and x265 with --tune=ssim. Lower MS-SSIM BD-rate values mean a better compression for the same quality in terms of MS-SSIM.

Table IV: MS-SSIM BD-rate gain and multi-threaded encoder speedup for HD and UHD test sequences for VVenC with perceptually optimized QPA enabled.

|  |  | HD |  |  | UHD |  |
| --- | --- | --- | --- | --- | --- | --- |
| Preset | MS-SSIM BD-rate vs. HM | Speedup vs. HM | Speedup vs. VTM | MS-SSIM BD-rate vs. HM | Speedup vs. HM | Speedup vs. VTM |
| Faster | -19.0% | 32x | 250x | -18.1% | 51x | 410x |
| Fast | -30.0% | 21x | 160x | -30.6% | 31x | 250x |
| MEDIUM | -40.5% | 9.2x | 73x | -41.4% | 15x | 120x |
| SLOW | -43.4% | 3.6x | 28x | -44.4% | 5.8x | 47x |
| SLOWER | -46.0% | 0.7x | 5.5x | -47.5% | 1.2x | 9.7x |

## License

Please see the file https://github.com/fraunhoferhhi/vvenc/blob/master/LICENSE.txt in the repository for the terms of use of the contents of the VVenC repository.

For more information, please contact: vvc@hhi.fraunhofer.de

# References

- VTM software repository, version VTM-11.0. Available online: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- VVenC software repository. Available online: https://github.com/fraunhoferhhi/vvenc

- VVdeC software repository. Available online: https://github.com/fraunhoferhhi/vvdec

- Adam Wieckowski, et al., “Open optimized VVC encoder (VVenC) and decoder (VVdeC) implementations,” document JVET-T0099, Joint Video Experts Team (JVET), Oct. 2020.

- J. Brandenburg et al., “Towards Fast and Efficient VVC Encoding”, IEEE 22nd Workshop on Multimedia Signal Processing (MMSP 2020), Tampere, Finland, 2020.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” document JVET-T2010, Joint Video Experts Team (JVET), Oct. 2020.

- G. Bjøntegaard, “Calculation of average PSNR differences between RD-curves,” Technical Report VCEG-M33, ITU-T SG16/Q6, Austin, Texas, USA, 2001.

- x265 software repository, version 3.4. Available online: https://github.com/videolan/x265/tree/Release_3.4

- Z. Wang, E. Simoncelli, and A. C. Bovik, “Multi-Scale Structural Similarity for Image QualityAssessment,” inProc. IEEE Asilomar Conf. Signals, Systems, and Comp., Pacific Grove, 2003.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).