| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0041 |
| --- | --- |

| Title: | AHG4: Status Report on SDR HD and 360 Video Verification Test Preparation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mathias Wien Vittorio Baroncini Yan Ye | Tel: Email: | wien@lfb.rwth-aachen.de baroncini@gmx.com yan.ye@alibaba-inc.com |
| Source: | Verification Test Coordinators |  |  |

_____________________________

# Abstract

This document summarizes the status of test sequence selection and rate point determination for the VVC verification test categories SDR HD and 360° video. At the last meeting, dry-run results were provided for multiple sequences out of these test categories. Based on these results, updated QP settings are proposed in this document. This document also seeks answers to a few questions in order to proceed to the formal subjective evaluation step.

# Introduction

The verification test plan for the SDR HD and 360° video categories [1] has been reviewed and updates to the encoder settings are proposed. In the context of AHG4 activities, the verification test coordinators have assessed and discussed the available test sequences in multiple online meeting together with JVET experts. Furthermore, dry-run experiments have been conducted in order to determine the suitability of the test sequences as well as the expected coverage of the MOS range in formal visual tests. The suggestions for test sequences and QP settings are based on dry-run results which were reported in [2].

# SDR HD Low Delay

It is noted that based on pre-assessments conducted last year by JVET experts, a GOP size of 4 was considered to be slightly favorable over the GOP size of 8. The suggestion was based on a slight visual benefit for the shorter GOP size in a direct comparison of the GOP4 and GOP8 results. It is noted that GOP size of 8 is used in the LD configuration of the JVET CTC, and that the dry-run results reported in [2] have been generated using GOP size of 8. No specific remarks related to the GOP structure were noted in the dry-run experiments. It is recommended to decide whether the GOP size of 8 could be used in the formal subjective tests at this AHG meeting.

## Conversational content

A set of four test sequences for conversational applications as listed in the table below has been identified out of the available set of proposed test sequences for this purpose in JVET-T0060 and JVET-T0120 [3][4].

Table 1 – SDR HD LD conversational test sequences under consideration

| Test sequence | Resolution | fps | Frames | md5 |
| --- | --- | --- | --- | --- |
| Beatriz_1920x1080_50fps_8bit_420 | 1920x1080 | 50 | 500 | fe74cd5046fa033b4f743f42b29e69cd |
| OfficeWalkAndTalk_1920x1080_30fps_8bit_420pf | 1920x1080 | 30 | 300 | 9313b009c1f287915c9eb39a989b8f9b |
| OfficeWalkAtWall_1920x1080_30fps_8bit_420pf | 1920x1080 | 30 | 300 | 529c15491ea8e1eb0320244a6ff902bb |
| OfficeWalkCeiling_1920x1080_30fps_8bit_420pf | 1920x1080 | 30 | 300 | 265906bc8f65441fb53d167483b9f726 |

The sequences are available on the JVET ftp site in the /ahg/verificationtests/candidates/sdr directory.

The following QPs are proposed for subjective evaluation in the verification tests:

Table 2 – QP selection for the SDR HD LD conversational test sequences under consideration

| Sequence | HM QPs | VTM QPs |
| --- | --- | --- |
| Beatriz_1920x1080_50fps_8bit_420 | 41, 37, 33, 28, 23 | 42, 38, 34, 29, 24 |
| OfficeWalkAtWall_1920x1080_30fps_8bit_420pf | 46, 42, 38, 32, 26 | 47, 44, 40, 34, 27 |
| OfficeWalkAndTalk_1920x1080_30fps_8bit_420pf | 45, 40, 34, 30, 24 | 46, 42, 37, 32, 28 |
| OfficeWalkCeiling_1920x1080_30fps_8bit_420pf | 45, 40, 34, 30, 24 | 46, 42, 36, 32, 26 |

## Gaming content

The following four test sequences for gaming-type applications are proposed to be considered for evaluation in the VVC verification tests:

Table 3 – SDR HD LD gaming test sequences under consideration

| Test sequence | Resolution | fps | Start frame | Frames |
| --- | --- | --- | --- | --- |
| DOTA2 | 1920×1080 | 60 | 0940 | 600 |
| EuroTruckSimulator2 | 1920×1080 | 60 | 3000 | 600 |
| Rust | 1920×1080 | 60 | 0360 | 600 |
| Starcraft | 1920×1080 | 60 | 2463 | 600 |

The original YUV files of the sequences are available on the JVET ftp site in the directory ahg/verificationtests/candidates/sdr/hd. The provided bitstreams were encoded using the GOP8 configuration. Bitstreams were provided via upload to the JVET ftp site in JVET-U_VT/sdr-hd-ld/gaming/gop8/.

Based on the results reported in [2], the following QPs are suggested:

Table 4 – QP selection for the SDR HD LD gaming test sequences under consideration

| Sequence | HM QPs | VTM QPs |
| --- | --- | --- |
| DOTA2 | 38,34,29,24,20 | 40,36,31,26,22 |
| EuroTruckSimulator2 | 45,40,36,32,26 | 46,41,37,33,27 |
| Rust | 39,34,30,26,21 | 40,36,32,27,22 |
| Starcraft | 38,34,30,26,21 | 40,36,32,28,23 |

# SDR HD Random Access

The proposed set of test sequence candidates in HD resolution for the random-access configuration is listed in the table below.

Table 5 – SDR HD RA test sequences under consideration

| Test sequence | Resolution | fps | Frames |
| --- | --- | --- | --- |
| BarScene | 1920x1080 | 60 | 600 |
| DrivingPOV | 1920x1080 | 60 | 600 |
| Metro | 1920x1080 | 60 | 600 |
| RushHour | 1920x1080 | 30 | 300 |
| Meridian1 | 1920x1080 | 60 | 600 |

Bitstreams were provided via upload to the JVET ftp site in JVET-U_VT/sdr-hd-ra and JVET-U_VT/sdr-hd-ra/meridian_dat/.

Based on the results reported in [2], the following QPs are suggested:

Table 6 – QP selection for the SDR HD RA test sequences under consideration

| Sequence | HM QPs | VTM QPs |
| --- | --- | --- |
| BarScene | 42, 38, 33, 28, 23 | 43, 39, 35, 31, 26 |
| DrivingPOV | 39, 34, 30, 27, 24 | 42, 38, 34, 30, 26 |
| Meridian1 | 39, 34, 30, 27, 25 | 42, 38, 34, 30, 26 |
| Metro | 39, 35, 31, 27, 24 | 42, 38, 34, 30, 26 |
| RushHour | 42, 38, 33, 29, 24 | 44, 40, 35, 31, 26 |

It is noted that in the dry-run experiments of [2],  the employed version of the Meridian sequence included a time code as part of the encoded video.

# 360° video category

The set of test sequences under consideration for the 360° video category are listed below [1].

Table 7 – 360 video test sequences under consideration

| Sequence | Resolution | Frame rate | Bit depth | Camera |
| --- | --- | --- | --- | --- |
| GT_Sheriff | 4320×2160 | 30 | 8 | static |
| HarborBiking2 | 8192×4096 | 30 | 8 | moving |
| KiteFliteWalking2 | 8192×4096 | 30 | 8 | moving |
| SkateBoardAtBridge | 6144×3072 | 30 | 8 | moving |

Based on the results reported in [2], the following QPs are suggested:

Table 8 – QP settings for the HM and VTM software for the 360° video test sequences using the PERP projection format

| Test sequence | HM-16.22 QP | VTM-11.0 |
| --- | --- | --- |
| GT_Sheriff | 36, 32, 28, 24, 20 | 38, 34, 30, 26, 22 |
| HarborBiking2 | 35, 31, 27, 23, 20 | 38, 34, 30, 26, 22 |
| KiteFliteWalking2 | 40, 36, 32, 28, 24 | 42, 38, 34, 30, 26 |
| SkateBoardAtBridge | 39, 35, 31, 27, 24 | 42, 38, 34, 30, 26 |

It is noted that for GCMP/PCMP, the QPs listed in the following table are under consideration for the GCMP/PCMP configuration; they have not been checked in a dry run.

Table 9 – QP settings for the HM and VTM software for the 360° video test sequences using the PCMP / GCMP projection format

| Test sequence | HM-16.22 QP | VTM-11.0 |
| --- | --- | --- |
| GT_Sheriff | 39, 35, 31, 27, 22 | 42, 38, 34, 30, 26 |
| HarborBiking2 | 35, 31, 27, 23, 20 | 38, 34, 30, 26, 22 |
| KiteFliteWalking2 | 40, 36, 32, 28, 24 | 42, 38, 34, 30, 26 |
| SkateBoardAtBridge | 38, 34, 30, 27, 23 | 42, 38, 34, 30, 26 |

The evaluation of the test sequences in the 360° video category has so far been based on viewport mp4 files which were generated by the volunteer who produced the bitstreams. Since the generation of the viewports is an additional processing step in preparation of the visual tests, it is suggested to assert the correctness of the extraction process by generating tables with md5 sums of the extracted viewport files. The results should be crosschecked by JVET experts.

# Conclusions

It is suggested to proceed towards formal subjective evaluation with the presented sets of test sequences or with sub-sets thereof. It is suggested to keep the set of evaluated sequences small in order to limit the effort under the ongoing constraints due to the COVID pandemic.

A set of aspects for further discussion has been identified in this document. Clarification of these is suggested to allow for proceeding to the formal evaluation step.

- For the SDR HD LD category, a decision on the GOP size is to be made.

- For the SDR HD RA test set, the version to be used of the Meridian1 sequence is to be determined

- For the 360° video category, the QP setting for the encoding of the test sequences in the PCMP/GCMP format is to be finalized.

- Volunteers for final encoding and crosschecking tasks should be sought.

# References

- M. Wien, V. Baroncini, A. Segall, and Y. Ye, “VVC verification test plan (Draft 4),” Doc. JVET-T2009, Joint Video Experts Team of VCEG and MPEG, 20th meeting, Oct. 2020.

- M. Wien, V. Baroncini, “Dry run subjective assessment of SDR HD and 360 video verification tests,” Doc. JVET-U0119, Joint Video Experts Team of VCEG and MPEG, 21st meeting, Jan. 2021.

- S. Xu, R.-L. Liao, J. Chen, Y. Ye (Alibaba), “AHG4: new video conference sequences for HD verification testing,” Doc. JVET-T0060, Joint Video Experts Team of VCEG and MPEG, 20th meeting, Oct. 2020.

- K. Andersson, M. Folkesson (Ericsson), “AHG4: Video conference sequence for HD verification testing,” Doc. JVET-T0120, Joint Video Experts Team of VCEG and MPEG, 20th meeting, Oct. 2020.