| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0044 |
| --- | --- |

| Title: | AHG4: Agenda and report of the AHG meeting on the HDR video verification test preparation on 2021-03-25 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrew Segall Mathias Wien Vittorio Baroncini | Tel: Email: | asegall@sharplabs.com wien@lfb.rwth-aachen.de baroncini@gmx.com |
| Source: | AHG |  |  |

_____________________________

# Agenda and Notes

The meeting was held online on Thu, 2021-03-25, 21:00h UTC with the connection information provided via the JVET mailing list and in the JVET calendar.

- Review of contribution JVET-V0043

- Discussion on next steps

| JVET-V0043 | m56356 | AHG4: Status Report on HDR Video Verification Test Preparation | A. Segall, M. Wien, V. Baroncini, K. Andersson | JVET-V0043. |
| --- | --- | --- | --- | --- |

This document summarizes the status of test sequence selection and rate point determination for the HDR VVC verification test category since the last JVET meeting.  Areas of progress include:

- Selection of additional HDR candidate sequences suitable for use in the verification test

- Selection of QP points for these sequences for use in a dry run of the test

- Receipt of additional HDR candidates.

With the identification of the additional HDR candidate sequences, the number of HDR sequence candidates for the verification test has increased from 5 sequences to 10 sequences.

It was commented that the ITE sequence Drama appears to be exposing a good extent of HDR features. P. Topiwala mentioned that it might be possible to generate new HDR HLG material (which would be compressed at 100Mbps HEVC).

It was suggested to proceed with a selection of a subset of rate points to be evaluated in a reduced dry run experiment. The goal would be to assert an appropriate coverage of the MOS range as well as a sufficient alignment of the MOS for HM and VTM for each pair of rate points. The determination of the rate points for this experiment is to be done by the test coordinators.

There was interest expressed to include UHD 24Hz content in the verification test set.

The meeting was closed at approximately 21:50h UTC.

# List of AHG Meeting Participants

(in alphabetical order)

- Ajay Luthra (Picsel Labs)

- Andrew Segall (Sharp - US)

- Dmytro Rusanovskyy (Qualcomm - US)

- Gary Sullivan (Microsoft - US)

- Jens-Rainer Ohm (RWTH - DE)

- Karsten Suehring (HHI - DE)

- Kenneth Andersson (Ericsson - SE)

- Mathias Wien (RWTH - DE)

- Olena Chubach (MediaTek - USA)

- Pankaj Topiwala (FastVDO - US)

- Peng Yin (Dolby - US)

- Vittorio Baroncini (VABtech – GB)

- Yan Ye (Alibaba - US)