| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 April 2021 | Document: JVET-V0045 |
| --- | --- |

| Title: | 4K HDR (HLG) Aerial Test Data |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Pankaj Topiwala FastVDO LLC 3097 Cortona Dr. Melbourne, FL 32940 USA | Tel: Email: | +1 (321) 355-7376 pankaj@fastvdo.com |
| Source: | FastVDO LLC |  |  |

_____________________________

# Abstract

This contribution offers some drone-captured aerial HDR video with 4K resolution, and HLG transfer function, for potential use in HDR coding and other standards-related investigations.

# Introduction, Availability of HDR Test Data

Test sequences for use in compression studies are generally uncompressed for good reason, in order to allow precise analysis of compression loss based on algorithm selection. But there are applications where uncompressed video is simply unavailable, and yet offer adequately high-quality source data as to allow meaningful compression studies at much lower rates (e.g., 10X lower bitrates). One such application is in aerial drone footage. High-end consumer drones today can capture 10-bit 4K (3840x2160), at 30-60 fps, in BT.2020 color space, and with the Hybrid Log Gamma (HLG) transfer function, and HEVC encoding. FastVDO has captured some initial data, using the DJI Mavic II Pro drone, capturing 4Kp30, in HEVC Main 10, at a nominal 100 Mbs data rate. Since this data is already compressed (albeit at a high rate), and thus is likely inflicted with at least some limited compression artifacts, we took the opportunity to also upconvert the p30 to p60, to remain consistent with other HDR test data sequences under study. The original p30 captures are also available upon request. The initial four sequences, all 10s clips, at p60, are available for download at www.fastvdo.com/fvhdr/ , together with a license statement, royalty free for compression studies in standards development. They are called: fv-fair, fv-pond, fv-stadium, fv-suburb. Precise calibration of the dynamic range of the data remains to be calculated, and can be remapped as desired, including generating SDR products.

|  | fv-fair (4Kp60, HLG) |
| --- | --- |
|  | fv-pond (4Kp60, HLG) |
|  | fv-stadium (4Kp60, HLG) |
|  | fv-suburb (4Kp60, HLG) |