| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 April 2021 | Document: JVET-V0060-v1 |
| --- | --- |

| Title: | Constrained RASL encoding for bitstream switching |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Christian Bartnik Adam Wieckowski Karsten Suehring Yago Sanchez Benjamin Bross | Tel: Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

VVC features reference picture resampling (RPR) which was investigated in a CE for conversational use cases with LD configuration. In such a setup, RPR allows avoiding sending bitrate-costly Intra coded pictures for resolution changes.

This contribution investigates the benefit of applying RPR for resolution switching in HTTP streaming use cases, also known as open GOP drift switching, by encoding RASL pictures in a manner allowing client-side RPR with other reference pictures than used on encoder side. The coding efficiency benefit of unconstrained open GOP coding over closed GOP coding is reported to be between -9,22% BD-rate for short IRAP periods and -2,35% for longer IRAP periods.

It is reported that CCLM introduces noticeable artefacts in the chroma components when reference pictures are exchanged due to bitrate or resolution switching which is assumed to be attributable to the underlying linear model relying on extrema of neighbouring samples for its parameter derivation. CCLM can only be indicated to be inactive through sequence-scope signalling and deactivating CCLM for all pictures leads to substantial coding efficiency losses (3.43% BR-rate in RA according to AHG13).

Therefore, it is proposed to add a GCI flag that indicates that RASL pictures adhere to the RPR related constraints as well as that RASL pictures do no use CCLM. The coding efficiency benefit of the proposed constrained open GOP RASL picture coding over closed GOP coding is reported to be between -8.69% BD-rate for short IRAP periods and -2,20% for longer IRAP periods while avoiding severe artefacts in open GOP drift switching.

# Introduction

VVC features reference picture resampling (RPR) which was primarily investigated in a conversational use case with LD configuration [1] for the purpose of avoiding sending bitrate-costly Intra coded pictures for resolution changes. This contribution asserts that RPR with additional constraints can be beneficial for streaming applications and proposes signalling to indicate the required constraints and provide a conformance point for related systems such as MPEG-DASH or ISOBMFF.

HTTP adaptive streaming is a major path of video distribution and is based on download of content segments. The content segments are typically offered in multiple variants, referred to as representations, and may differ in terms of bitrate or coded resolution, e.g., 1Mbps@720p, 2Mbps@1080p, 5Mbps@2160p. End clients may respond to bandwidth fluctuations with up- or down switching between content variants in order to achieve continuous playout under constrained network conditions. It is known that open GOP coding structures using RASL pictures provide substantial coding efficiency benefit. Our experiments have shown to provide up to -9,22% BD-rate gain for open GOP vs closed GOP coding with short intra periods (64 pictures) as in live streaming and -2,35% BD-rate gain for longer intra periods (256 pictures) as typical for VOD streaming using the HHI-Berlin test sequence set [2] in CTC RA configuration with VVenC v0.2.1 (medium preset) and resolutions of 720p, 1080p and 2160p.

When open GOP structures are used and clients perform bitstream switching, different reference pictures are used for RASL pictures on decoder-side compared to the encoder side. For instance, in bitrate switching the decoder-side may encounter a deviation of sample values in some reference pictures, while in resolution switching, it may even be required to perform RPR when decoding the concatenated client-side bitstream segment if a RASL picture has a different size than reference pictures of a previous segment. Lack of RPR functionality would result in discontinuous playout from dropping of RASL pictures which is why coding for streaming with earlier standards H.264/AVC and HEVC had to resort to less efficient closed-GOP structures (using RADL or Trailing only). Open GOP RASL pictures at switching points are decoded with drift compared to the encoder side andu nconstrained open GOP switching leads to substantial propagating artefacts in the RASL pictures from deviated sample values as well motion information, the latter usually causing very noticeable artefacts.

When resolution switching is supposed to be possible in the client-side concatenated bitstream, the RPR related constraints of DMVR, PROF and BDOF as well as (SB)TMVP (no collocated reference picture prior to associated CRA in decoding order) need to be active for the RASL pictures (even though they are unnecessary on encoder-side). For this, the encoder has to indicate that RPR and resolution change are allowed (sps_ref_pic_resampling_enabled_flag and sps_res_change_in_clvs_allowed_flag equal to 1) and refrains from using said tools for the RASL pictures by disabling them in the picture header of the respective RASL pictures and selecting collocated reference pictures accordingly. These measures preclude all drift in the motion information of the RASL pictures.

# Problem Description

Our experiments have shown that Cross Component Linear Model (CCLM) also introduces very noticeable artefacts in the color components of reconstructed samples when performing bitrate or resolution switching in a streaming scenario. Figure 1 shows per component PSNR of test sequence NeptuneFountain2 when performing open GOP bitrate switching at every IRAP between QP22 and QP32 at 2160p. It is evident that the objective quality of the color components of RASL pictures suffers from severe switching drift.

Figure 1 Per-component PSNR of open GOP bitrate switching with sequence NeptuneFountain2 when RASL pictures adhere to RPR constraints (DMVR, PROF, BDOF disabled, no collocated refpic preceding CRA).

Figure 2 shows the picture with POC 420 of the above bitrate switching example with visible color component artefacts towards the high-motion area at the bottom of the picture. It should be noted that similar artefacts occur when switching between resolutions.

Figure 2: Switching drift from CCLM in color component samples in frame 420 of the Figure 1 example.

CCLM allows to intra-predict the chroma components of a block from the respective luma component using a linear model wherein model parameters are derived from the reconstructed luma sample values. The linear model transforms subsampled luma-samples rec’L into chroma prediction through P(i, j) = a ⋅ rec’L(i, j) + b, where the parameters a and b are derived from neighboring luma and chroma samples as a = ( Yl – Ys ) / ( Xl – Xs) and b = Ys – a ⋅ Xs. with Xl and Xs denoting the average of the two largest and the two smallest neighboring samples, respectively, and Yl and Ys denoting the average of the corresponding chroma sample pairs, respectively.

Due to the parameter derivation process only accounting for the extrema of the neighboring samples values, the process is prone to extensive drift even in case of single-sample drift outliers in the neighboring blocks. Also, because of the linear model, if a is large, the luma drift may be amplified in the chroma domain. In context of bitstream switching in streaming, the drift artefacts may only occur in RASL pictures, i.e. motion-compensated inter-predicted pictures. If the encoder decides to use CCLM, i.e. an intra prediction mode, this would usually be due to a lack of an appropriate motion compensated predictor, implying a region of high temporal activity. In such regions the expected reconstruction drift for open GOP switching is expected to be high, contributing even more to the discussed extensive drift.

Unfortunately, CCLM can only be switched off on SPS level (sps_cclm_enabled_flag) for the entire CLVS which comes at substantial coding efficiency losses of roughly 3.43% BR-rate for CTC RA which mostly stems from the Intra coded pictures which use CCLM for roughly half of the CUs (as measured in CTC AI configuration) [3].

# Proposal

## Description

In order to avoid the above-described drift artefacts while limiting the coding efficiency penalty, RASL picture encoding can be constrained as follows:

- DMVR, PROF, BDOF are disabled by setting ph_bdof_disabled_flag, ph_dmvr_disabled_flag and ph_prof_disabled_flag in the picture header equal to 1,

- no RASL picture in OlsInScope uses a collocated reference picture preceding its associated CRA picture in decoding order,

- CCLM is disabled by disallowing CUs with cclm_mode_flag equal to 1.

Disabling CCLM through encoder-side constraints only for the drift-affected RASL pictures results in a negligible coding penalty. Applying the constraints above allows open GOP resolution switching while retaining most of the open GOP coding gains compared to closed GOP coding with -8.69% BD-rate for short intra period (64 pictures) and -2,20% BD-rate for longer intra period (256 pictures) under the same conditions as described above. Table 1 summarizes coding efficiency performance of open GOP coding and the proposed constrained open GOP coding over closed GOP coding for various intra periods and resolutions using the HHI-Berlin test sequence set in CTC RA configuration with VVenC v0.2.1 (medium preset) and resolutions of 720p, 1080p and 2160p.

Table 1: Coding efficiency results for open GOP and constrained open GOP coding over closed GOP.

| Res. | IRAP Period | Anchor: closed GOP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Open GOP |  |  | Constrained open GOP |  |  |
|  |  | Y | U | V | Y | U | V |
| 2160p | 64 | -8.70% | -10.8% | -10.3% | -8.16% | -9.75% | -9.04% |
| 1080p |  | -9.12% | -11.0% | -10.6% | -8.71% | -9.80% | -8.93% |
| 720p |  | -9.38% | -11.4% | -11.0% | -9.00% | -10.1% | -9.24% |
| Overall |  | -9.07% | -11.1% | -10.6% | -8.62% | -9.88% | -9.07% |
|  |  | -9,22% |  |  | -8,69% |  |  |
| 2160p | 128 | -4.26% | -4.89% | -4.62% | -3.97% | -4.38% | -4.01% |
| 1080p |  | -4.53% | -4.90% | -4.70% | -4.31% | -4.28% | -3.85% |
| 720p |  | -4.68% | -4.99% | -4.84% | -4.48% | -4.38% | -4.02% |
| Overall |  | -4.49% | -4.93% | -4.72% | -4.25% | -4.35% | -3.96% |
|  |  | -4,50% |  |  | -4,22% |  |  |
| 2160p | 256 | -2.23% | -2.42% | -2.25% | -2.07% | -2.11% | -1.90% |
| 1080p |  | -2.40% | -2.37% | -2.15% | -2.29% | -1.97% | -1.63% |
| 720p |  | -2.50% | -2.41% | -2.33% | -2.38% | -2.04% | -1.84% |
| Overall |  | -2.38% | -2.40% | -2.24% | -2.25% | -2.04% | -1.79% |
|  |  | -2,35% |  |  | -2,20% |  |  |

The presented constrained encoding method is able to avoid severe drift artefacts from bitrate or resolution switching. Figure 3 shows per-component PSNR for the example of Figure 1 when RASL pictures are encoded with RPR constraints and without CCLM usage.

Figure 3: Per-component PSNR of open GOP bitrate switching with sequence NeptuneFountain2 when RASL pictures adhere to RPR constraints and do not use CCLM.

In general, using the presented method, the drift affected RASL pictures exhibit a gradual quality transition between variants as evident from the following Figure 4 that shows YUV-PSNR for all RASL pictures averaged over all sequences in the test set, switching points and QPs for up- and down-switching between resolution pairs (720p, 1080p) and (1080p, 2160p).

Figure 4:Quality transition of the presented constrained encoding method for open GOP resolution switching averaged over sequences, switching points, QPs and resolution pairs.

## Proposed Specification changes

It is proposed to add a flag to the general constraint information (GCI) that indicates that the constraints described in Section 3.1 apply to all RASL pictures in scope (DCI: full bitstream, VPS: CVS, SPS: CLVS). The following highlighted changes to the GCI syntax and semantics are proposed:

| general_constraints_info( ) { | Descriptor |
| --- | --- |
| gci_present_flag | u(1) |
| if( gci_present_flag ) { |  |
| […] |  |
| gci_num_reserved_bits | u(8) |
| if( gci_num_reserved_bits > 0 ) { |  |
| gci_constrained_rasl_encoding_flag | u(1) |
| for( i = 0; i < gci_num_reserved_bits – 1; i++ ) |  |
| gci_reserved_zero_bit[ i ] | u(1) |
| } |  |
| } |  |
| while( !byte_aligned( ) ) |  |
| gci_alignment_zero_bit | f(1) |
| } |  |

gci_num_reserved_bits specifies the number of the reserved GCI bits. The value of gci_num_reserved_bits shall be equal to 0 or 1 in bitstreams conforming to this version of this Specification. Other values of gci_num_reserved_bits are reserved for future use by ITU-T | ISO/IEC. Although the value of gci_num_reserved_bits is required to be equal to 0 or 1 in this version of this Specification, decoders conforming to this version of this Specification shall allow the value of gci_num_reserved_bits greater than 1 to appear in the syntax and shall ignore the values of all the gci_reserved_zero_bit[ i ] syntax elements when gci_num_reserved_bits is greater than 1.

gci_constrained_rasl_encoding_flag equal to 1 specifies that all RASL pictures in OlsInScope shall have ph_bdof_disabled_flag, ph_dmvr_disabled_flag, ph_prof_disabled_flag equal to 1 in the respective PHs, and no CU of a RASL picture shall have cclm_mode_flag equal to 1, and no RASL picture in OlsInScope shall use a collocated reference picture that precedes its associated CRA picture in decoding order. gci_constrained_rasl_encoding_flag equal to 0 does not impose a constriant.

# References

D. Luo, V. Seregin, W. Wan. “Description of Core Experiment 1 (CE1): Reference picture resampling filters“, doc. JVET-Q2021 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 15th meeting: July 2019

B. Bross et al., "AHG4: Multiformat Berlin Test Sequences", JVET-Q0791, JVET Brussels Meeting, Jan 2020, online: https://jvet-experts.org/doc_end_user/current_document.php?id=9637

W.-J. Chien, J. Boyce, Y.-W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y.-W. Huang, H. Jang, R.-L. Liao, S. Liu. “JVET AHG report: Tool reporting procedure and testing (AHG13)“, doc.  JVET-T0013 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 20th meeting: October 2020

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)