| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0065-v1 |
| --- | --- |

| Title: | AHG9: On the DRAP and EDRAP indication SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Li Zhang1, Yang Wang2 18910 University Center Lane, San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com lizhang.idm@bytedance.com wangyang.cs@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes to the dependent random access point (DRAP) indication and the extended DRAP indication SEI messages:

- The semantics of the DRAP indication SEI message is changed such that the semantics is clear when the SEI message is applied in the context of a multi-layer bitstream.

- Change edrap_rap_id_in_clvs to edrap_rap_id_minus1 and the value plus 1 specified the random access point (RAP) picture ID.

- The semantics of the EDRAP SEI message is changed to allow two IRAP pictures to have the same RapPicId value, and also to allow two EDRAP pictures associated with different IRAP pictures to have the same RapPicId value, while disallowing RAP pictures (IRAP or EDRAP pictures) within an IRAP period to have the same RapPicId value.

- The semantics of edrap_ref_rap_id[ i ] is changed to clarify that a RAP picture that may be referenced by a current EDRAP picture shall be either the IRAP picture associated with the current EDRAP picture or an EDRAP picture associated with the same IRAP picture as the current EDRAP picture.

The text changes for the proposals in JVET-V0065 (this contribution) are included in the attachment to JVET-V0063.

# Introduction

The current syntax and/or semantics of the DRAP indication and EDRA indication SEI messages have the following issues:

- The semantics of the DRAP indication SEI message was written in a manner without considering the use of the SEI message in the context of a multi-layer bitstream.

- The syntax element edrap_rap_id_in_clvs is used to signal the RAP picture ID of an EDRAP picture and the value needs to be greater than 0 as the RAP picture ID is inferred to be equal to 0 for IRAP pictures. However, edrap_rap_id_in_clvs is coded as u(16) and is not disallowed to be equal to 0. It would make more sense to change edrap_rap_id_in_clvs to be edrap_rap_id_minus1.

- Currently, the semantics of edrap_rap_id_in_clvs is as follows:

edrap_rap_id_in_clvs specifies the RAP picture identifier, denoted as RapPicId, of the EDRAP picture.

Each IRAP or EDRAP picture is associated with a RapPicId value. The RapPicId value for an IRAP picture is inferred to be equal to 0. The RapPicId values for any two IRAP or EDRAP pictures within a CLVS shall be different.

The last sentence has a problem, as it does not allow two IRAP pictures to have the same RapPicId value, and it does not allow two EDRAP pictures associated with different IRAP pictures to have the same RapPicId value, either.

- Currently, the semantics of edrap_ref_rap_id[ i ] is as follows:

edrap_ref_rap_id[ i ] indicates RapPicId of the i-th IRAP or EDRAP picture that is within the same CLVS as the EDRAP picture and may be included in the active entries of the reference picture lists of the EDRAP picture.

It should be clarified that a RAP picture that may be referenced by a current EDRAP picture shall be either the IRAP picture associated with the current EDRAP picture or an EDRAP picture associated with the same IRAP picture as the current EDRAP picture.

# Proposal

The proposal for addressing the above-mentioned issues is summarized in the abstract. The proposed text changes are as follows:

# Replace the texts in clause 8.11.2 with the following:

The presence of the DRAP indication SEI message indicates that the constraints on picture order and picture referencing specified in this subclause apply. These constraints can enable a decoder to properly decode the DRAP picture and the pictures that are in the same layer and follow it in both decoding order and output order without needing to decode any other pictures in the same layer except the associated IRAP picture of the DRAP picture.

The constraints indicated by the presence of the DRAP indication SEI message, which shall all apply, are as follows:

–	The DRAP picture is a trailing picture.

–	The DRAP picture has a temporal sublayer identifier equal to 0.

–	The DRAP picture does not include any pictures in the same layer in the active entries of its reference picture lists except the associated IRAP picture of the DRAP picture.

–	Any picture that is in the same layer and follows the DRAP picture in both decoding order and output order does not include, in the active entries of its reference picture lists, any picture that is in the same layer and precedes the DRAP picture in decoding order or output order, with the exception of the associated IRAP picture of the DRAP picture.

# Replace clause 8.23 with the following:

## 8.23 Extended DRAP indication SEI message

8.23.1 Extended DRAP indication SEI message syntax

| extended_drap_indication( payloadSize ) { | Descriptor |
| --- | --- |
| edrap_rap_id_in_clvs_minus1 | u(16) |
| edrap_leading_pictures_decodable_flag | u(1) |
| edrap_reserved_zero_12bits | u(12) |
| edrap_num_ref_rap_pics_minus1 | u(3) |
| for( i = 0; i <= edrap_num_ref_rap_pics_minus1; i++ ) |  |
| edrap_ref_rap_id[ i ] | u(16) |
| } |  |

8.23.1 Extended DRAP indication SEI message semantics

The picture associated with an extended DRAP (EDRAP) indication SEI message is referred to as an EDRAP picture.

The presence of the EDRAP indication SEI message indicates that the constraints on picture order and picture referencing specified in this subclause apply. These constraints can enable a decoder to properly decode the EDRAP picture and the pictures that are in the same layer and follow it in both decoding order and output order without needing to decode any other pictures in the same layer except the list of pictures referenceablePictures, which consists of a list of IRAP or EDRAP pictures in decoding order that are within the same CLVS and identified by the edrap_ref_rap_id[ i ] syntax elements.

The constraints indicated by the presence of the EDRAP indication SEI message, which shall all apply, are as follows:

–	The EDRAP picture is a trailing picture.

–	The EDRAP picture has a temporal sublayer identifier equal to 0.

–	The EDRAP picture does not include any pictures in the same layer in the active entries of its reference picture lists except the referenceablePictures.

–	Any picture that is in the same layer and follows the EDRAP picture in both decoding order and output order does not include, in the active entries of its reference picture lists, any picture that is in the same layer and precedes the EDRAP picture in decoding order or output order, with the exception of the referenceablePictures.

–	When edrap_leading_pictures_decodable_flag is equal to 1, the following applies:

–	Any picture that is in the same layer and follows the EDRAP picture in decoding order shall follow, in output order, any picture that is in the same layer and precedes the EDRAP picture in decoding order.

–	Any picture that is in the same layer and follows the EDRAP picture in decoding order and precedes the EDRAP picture in output order does not include, in the active entries of its reference picture lists, any picture that is in the same layer and precedes the EDRAP picture in decoding order, with the exception of the referenceablePictures.

–	Any picture in the list referenceablePictures does not include, in the active entries of its reference picture lists, any picture that is in the same layer and is not a picture at an earlier position in the list referenceablePictures.

NOTE – Consequenlty, the first picture in referenceablePictures, even when it is an EDRAP picture instead of an IRAP picture, does not include any picture from the same layer in the active entries of its reference picture lists.

edrap_rap_id_in_clvs_minus1 plus 1 specifies the RAP picture identifier, denoted as RapPicId, of the EDRAP picture.

Each IRAP or EDRAP picture is associated with a RapPicId value. The RapPicId value for an IRAP picture is inferred to be equal to 0. The RapPicId values for any two IRAP or EDRAP pictures within a CLVS shall be different. The RapPicId values for any two EDRAP pictures associated with the same IRAP picture shall be different.

edrap_reserved_zero_12bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for edrap_reserved_zero_12bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of edrap_reserved_zero_12bits.

edrap_num_ref_rap_pics_minus1 plus 1 indicates the number of IRAP or EDRAP pictures that are within the same CLVS as the EDRAP picture and may be included in the active entries of the reference picture lists of the EDRAP picture.

edrap_ref_rap_id[ i ] indicates RapPicId of the i-th IRAP or EDRAP picture that is within the same CLVS as the EDRAP picture and RAP picture that may be included in the active entries of the reference picture lists of the current EDRAP picture. The i-th RAP picture shall be either the IRAP picture associated with the current EDRAP picture or an EDRAP picture associated with the same IRAP picture as the current EDRAP picture.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).