| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22st Meeting, by teleconference, 20-28 Apr. 2021 | Document: JVET-V0070 |
| --- | --- |

| Title: | VVC Software Decoder for Mobile Platforms |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | W.-L. Feng, F.-L. Luo, Y.-S. He, Z.-H. Liu, K.-C. Tang, S.-M. Meng, X. Wen | Email: | fengweilun@kuaishou.com wenxing@kuaishou.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution presents K266Dec, a VVC software decoder designed for ARM platforms. The decoder is designed from scratch following the VVC specification and can support all coding tools specified in VVC main profile.

For Android platform, K266Dec can decode 2K 8-bit CTC bitstreams at 31 fps with single thread on Huawei P40, which is 3.92x of VTM-11.0 decoder. In addition, we also test CTC bitstreams and UGC videos on various Android phones based on the 50B+ daily playback data from Kwai App.

For iOS platform, K266Dec can decode 2K 8-bit CTC bitstreams at 83 fps with single thread on iPhone 12 Pro Max, which is 4.22x compared to VTM-11.0 reference decoder.

# Introduction

There have been several VVC software decoder implementations have been presented to JVET. While JVET-Q0386[1], JVET-S0224[2], JVET-T0095[3] and JVET-T0099[4] focus on X86 platforms, JVET-U0071[5], JVET-U0088[6] have presented their optimized decoder on ARM platform. These proposals show their achievements on real-time decoding, without running at full capacity of multi-threading.

In this contribution, we present K266Dec, a software VVC decoder for ARM platform. K266Dec is highly optimized for Android platform decoding because 80-90 % viewing counts are from Android phones on our platform,

It is vital to keep low CPU usage or power consumption to adopt VVC on a wider range of current mobile platforms. K266Dec focus on cache friendly pipeline and high throughput SIMD optimizations targeting the ARM platforms. On the other hands, we also try to enforce single-thread decoding ability, which will significantly affect the real-time CPU usage.

For scenarios using single-thread decoding, K266Dec focus on the following optimizations:

- ARM/NEON assembly: K266Dec fully uses ARM and NEON instruction set, and we optimized the computation intensive modules such as CABAC decoding, inter/intra prediction, inverse transform, deblocking filter, SAO, etc. To further utilize the single-core CPU pipeline, we carefully designed the instruction level pipeline of nearly every assembly functions.

- Cache friendly pipeline and low memory usage: K266Dec utilizes multiple local information shared buffer between blocks to avoid fetching information from frame-level memory to decrease cache miss. K266Dec minimized the decoding memory usage with detailed memory allocating and in-place operation as much as possible.

For scenarios with higher decoding complexity like 4K resolution, K266Dec can make full use of multi-core CPU resource and achieve 3.62x speed up on class-A with 4 decoding threads on Huawei P40 compared to single-thread. The optimization with multi decoding threads are as follows:

- Fine-grained parallelism: In addition to traditional frame and wavefront parallelism, a fine-grained parallelism is utilized in K266Dec. Decoding progress of a CTU was devided into multiple sub-stages: parsing, mode-deriving, reconstruction, filtering and so on. In this way, the K266Dec can significantly reduce the data dependency and increase throughput.

# Results

The decoding performance of K266Dec is presented in this section. The bitstreams can be classified into two categories according to the source: JVET CTC sequences, and Kwai UGC sequences. The Kwai UGC sequences are all in portrait with 720x1280 resolution, and they are the top viewed videos uploaded by users on Kwai App and the viewing counts are ranging from 800k to 2M.

Bitstreams for JVET CTC content are encoded by VTM-11.0 with the following configurations:

- Internal bit depth set to 8 bits

- Disable ALF/CCALF

- The other configurations follow random access (RA) configuration specified in the CTC

Bitstreams for the Kwai UGC sequences are generated with the following configurations:

- Internal bit depth set to 8 bits

- Constant subjective quality rate-control, average bitrate less than 2Mbps

- Disable GEO/BDOF/SbTMVP/DMVR/LMCS/CIIP/MIP/JCCR/SBT/LFNST/MTS

- Disable ALF/CCALF

- The other configurations following RA configuration specified in the CTC

## Speed Test: Android Platform Results

The following three devices are used to test K266Dec decoding speed and they are selected based on the user playback data from Kwai App:

- High-End: Huawei P40

  - Chipset: Kirin 990 5G (7 nm)

  - CPU: Octa-core (2x2.86 GHz Cortex-A76 & 2x2.36 GHz Cortex-A76 & 4x1.95 GHz Cortex-A55)

  - Memory: 8GB RAM

- Mid-End: Oppo R17

  - Chipset: Qualcomm Snapdragon 439 (10 nm)

  - CPU: Octa-core (4x1.95 GHz Cortex-A53 & 4x1.45 GHz Cortex A53)

  - Memory：4GB RAM

- Low-End: Vivo Y93s

  - Chipset: Mediatek MT6762 Helio P22 (12 nm)

  - CPU: Octa-core 2.0 GHz Cortex-A53

  - Memory：4GB RAM

Table 1 to Table 3 show the average decoding speed of K266Dec vs VTM-11.0. Compared to VTM-11.0, K266Dec on Huawei P40 with single thread mode achieves 3.66x decoding speed-up.

Table 1. Average decoding speed (fps) of K266Dec VS VTM-11.0 for the JVET CTC sequences

| Huawei P40 | VTM-11.0 | K266Dec |  |  |  | Speedup |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # threads | 1 | 1 | 2 | 4 | 8 | 1 | 2 | 4 | 8 |
| Class A | 2.04 | 7.52 | 15.53 | 27.03 | 27.93 | 3.68 | 7.62 | 13.29 | 13.79 |
| Class B | 7.99 | 31.35 | 61.59 | 107.16 | 93.52 | 3.92 | 7.72 | 13.46 | 11.85 |
| Class C | 36.94 | 125.87 | 251.14 | 390.01 | 269.44 | 3.35 | 6.75 | 10.58 | 7.38 |
| Class D | 124.04 | 452.73 | 680.90 | 809.84 | 451.11 | 3.61 | 5.45 | 6.58 | 3.66 |
| Average | - | - | - | - | - | 3.66 | 7.00 | 11.35 | 9.80 |

Table 2. Average decoding speed (fps) of K266Dec VS VTM-11.0 for the JVET CTC sequences

| Oppo R17 | VTM-11.0 | K266Dec |  |  |  | Speedup |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # threads | 1 | 1 | 2 | 4 | 8 | 1 | 2 | 4 | 8 |
| Class A | 0.90 | 3.38 | 6.95 | 9.39 | 12.83 | 3.75 | 7.72 | 10.46 | 14.34 |
| Class B | 3.63 | 13.67 | 27.46 | 36.27 | 48.55 | 3.76 | 7.55 | 9.99 | 13.44 |
| Class C | 15.18 | 58.52 | 112.30 | 130.64 | 159.03 | 3.84 | 7.40 | 8.66 | 10.65 |
| Class D | 52.83 | 204.22 | 348.90 | 268.17 | 286.74 | 3.84 | 6.58 | 5.15 | 5.55 |
| Average | - | - | - | - | - | 3.79 | 7.37 | 8.84 | 11.47 |

Table 3. Average decoding speed (fps) of K266Dec VS VTM-11.0 for the JVET CTC sequences

| VIVO Y93s | VTM-11.0 | K266Dec |  |  |  | Speedup |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # threads | 1 | 1 | 2 | 4 | 8 | 1 | 2 | 4 | 8 |
| Class A | 0.48 | 1.51 | 2.89 | 5.17 | 7.10 | 3.14 | 5.99 | 10.74 | 14.80 |
| Class B | 2.00 | 6.16 | 11.68 | 21.01 | 27.34 | 3.11 | 5.90 | 10.64 | 13.91 |
| Class C | 8.28 | 25.29 | 48.62 | 82.65 | 87.98 | 3.09 | 5.92 | 10.10 | 10.86 |
| Class D | 26.89 | 83.12 | 146.72 | 209.80 | 206.54 | 3.11 | 5.48 | 7.86 | 7.76 |
| Average | - | - | - | - | - | 3.11 | 5.85 | 9.97 | 12.25 |

Table 4. shows that the K266Dec can achieve an average decoding speed of 97 fps, 41 fps and 19 fps, for Kwai UCG scenarios using single ARM core on Huawei P40, Oppo R17 and Vivo Y93.

Table 4. Decoding speed (fps) of K266Dec on Android device for the Kwai UGC sequences

|  | VTM-11.0 | K266Dec |  |  |  | Speedup |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # threads | 1 | 1 | 2 | 4 | 8 | 1 | 2 | 4 | 8 |
| Huawei P40 | 26.66 | 97.49 | 173.85 | 270.26 | 178.75 | 4.82 | 8.59 | 13.35 | 8.83 |
| Oppo R17 | 10.59 | 40.95 | 78.40 | 100.52 | 116.97 | 5.17 | 9.89 | 12.68 | 14.76 |
| VIVO Y93s | 5.82 | 19.06 | 35.28 | 60.10 | 56.63 | 4.39 | 8.12 | 13.83 | 13.03 |

## Speed Test: iOS Platform Results

The following iOS device is selected to test K266Dec decoding speed:

- iPhone 12 Pro Max

  - Chipset: Apple A14 Bionic (5 nm)

  - CPU: Hexa-core (2x3.1 GHz Firestorm + 4x1.8 GHz Icestorm)

  - Memory: 6GB RAM

For CTC bitstreams, K266Dec in single thread mode achieves 4.22x decoding speed-up.

Table 5. Average decoding speed (fps) of K266Dec VS VTM-11.0 for the JVET CTC bitstreams

| iPhone 12 Pro Max | VTM-11.0 | K266Dec |  |  |  | Speedup |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # threads | 1 | 1 | 2 | 4 | 6 | 1 | 2 | 4 | 6 |
| Class A | 3.85 | 19.90 | 32.27 | 42.91 | 52.19 | 5.18 | 8.42 | 11.17 | 13.63 |
| Class B | 19.42 | 82.70 | 135.57 | 172.39 | 200.03 | 4.22 | 6.93 | 8.80 | 10.25 |
| Class C | 94.41 | 341.01 | 550.22 | 659.76 | 648.93 | 3.55 | 5.77 | 6.92 | 6.86 |
| Class D | 323.33 | 1117.88 | 1572.40 | 1696.56 | 1656.73 | 3.39 | 4.83 | 5.22 | 5.14 |
| Average | - | - | - | - | - | 4.21 | 6.71 | 8.40 | 9.53 |

Table 6. Average Decoding speed (fps) of K266Dec on iOS device for the Kwai UGC sequences

|  | VTM-11.0 | K266Dec |  |  |  | Speedup |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # threads | 1 | 1 | 2 | 4 | 6 | 1 | 2 | 4 | 6 |
| iPhone 12 pro max | 67.16 | 245.30 | 380.54 | 411.11 | 358.25 | 4.66 | 7.23 | 7.81 | 6.81 |

# Conclusion

An optimized VVC software decoder is proposed in this contribution. Based on this implementation, real-time decoding capability in mobile platform is supported. For Android platform, K266Dec can decode 2K 8-bit CTC bitstreams at 31 fps with single thread on Huawei P40, which is 3.92x of VTM-11.0 decoder. For real applications, the proposed decoder is still on an early stage and we need to further optimize all VVC tools including ALF and CCALF.

# References

- J. Arumugam, S. Kotecha, S. Ramamurthy, “AHG16: Feature-rich implementation of VVC real-time decoding and playback on ARM based mobile clients,” JVET-Q0386, Joint Video Experts Team (JVET), January 2020.

- F. Bossen, “AHG16: Performance of a reasonably fast VVC software decoder,” JVET-S0224, Joint Video Experts Team (JVET), June 2020.

- B. Zhu, S. Liu, X. Xu, X. Zhang, C. Gu, L. Wang, W. Feng., “Performance of a VVC software decoder,” JVET-T0095, Joint Video Experts Team (JVET), October 2020.

- A. Wieckowski, J. Brandenburg, C. Bartnik, V. George, J. Güther, G. Hege, C. Helmrich, A. Henkel, T. Hinz, C. Lehmann, C. Stoffers, I. Zupancic, B. Bross, H. Schwarz, D. Marpe, “Open optimized VVC encoder (VVenC) and decoder (VVdeC) implementations,” JVET-T0099, Joint Video Experts Team (JVET), October 2020.

- Y. Li, S. Liu, Y. Chen, Y. Zheng, S. Chen, B. Zhu, J. Lou, “Performance of a VVC Software Decoder on Mobile Platform”, JVET-U0071, Joint Video Experts Team (JVET), December 2020.

- J. Chen, L. Wang, R.-L. Liao, Y. Ye, “VVC software decoder implementation for mobile devices”, JVET-U0088, Joint Video Experts Team (JVET), December 2020.