| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 |  |  |  | Document: JVET-V0077 |
| --- | --- | --- | --- | --- |
| Title: | EE2-3.8: Multiple Hypothesis Prediction |  |  |  |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Martin Winken Jonathan Pfaff Benjamin Bross Heiko Schwarz | Email: | martin.winken@hhi.fraunhofer.de |  |
| Source: | Fraunhofer HHI |  |  |  |

_____________________________

# Abstract

This document reports results for EE2 test 3.8 on Multiple Hypothesis Prediction (MHP). With MHP, it is possible to use up to four motion-compensated prediction signals per block (instead of two, as in VVC). These individual prediction signals are superimposed in order to form the overall prediction signal. The reported coding gains relative to VTM-11.0 (“tool-on test”) are:

RA:	-0.31% (Y), -0.30% (Cb), -0.21% (Cr)	Enc. time: 106%	Dec. time: 107%
	LB:		-0.49% (Y), -0.15% (Cb), 0.04% (Cr)	Enc. time: 112%	Dec. time: 109%

# Introduction

Multiple Hypothesis Prediction (MHP) has been proposed and studied during the development of the VVC standard (e.g., [1], [2]). At the last JVET meeting, MHP was proposed among a set of other coding tools that show compression efficiency beyond VVC [3]. In order to study the individual coding performance of these tools, an Exploration Experiment (EE) has been established [4]. In this document, the technical details of MHP are briefly reviewed and experimental results are presented.

# Technical description

In the multi-hypothesis inter prediction mode, one or more additional motion-compensated prediction signals are signaled, in addition to the conventional bi prediction signal. The resulting overall prediction signal is obtained by sample-wise weighted superposition. With the bi prediction signal pbi and the first additional inter prediction signal/hypothesis h3, the resulting prediction signal p3 is obtained as follows:

p3=(1-α)pbi+αh3

The weighting factor α is specified by the new syntax element add_hyp_weight_idx, according to the following mapping:

| add_hyp_weight_idx | α |
| --- | --- |
| 0 | 1/4 |
| 1 | -1/8 |

Analogously to above, more than one additional prediction signal can be used. The resulting overall prediction signal is accumulated iteratively with each additional prediction signal.

pn+1=1-αn+1pn+αn+1hn+1

The resulting overall prediction signal is obtained as the last pn (i.e., the pn having the largest index n). Within this EE, up to two additional prediction signals can be used (i.e., n is limited to 2).

The motion parameters of each additional prediction hypothesis can be signaled either explicitly by specifying the reference index, the motion vector predictor index, and the motion vector difference, or implicitly by specifying a merge index. A separate multi-hypothesis merge flag distinguishes between these two signalling modes.

For inter AMVP mode, MHP is only applied if non-equal weight in BCW is selected in bi-prediction mode.

Combination of MHP and BDOF is possible, however the BDOF is only applied to the bi-prediction signal part of the prediction signal (i.e., the ordinary first two hypotheses).

# Experimental results

For the experimental results, the EE2 software that was provided at the following URL has been used:

https://vcgit.hhi.fraunhofer.de/jvet-u-ee2/VVCSoftware_VTM.git

The software version that corresponds to the commit id 59b12bb8 was used.

In the following, the experimental results that are obtained when applying MHP as single additional tool on top of VTM-11.0 (“tool-on test”) are shown.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.19% | -0.30% | 104% | 105% |
| Class A2 | -0.06% | -0.05% | -0.06% | 104% | 106% |
| Class B | -0.59% | -0.60% | -0.43% | 107% | 107% |
| Class C | -0.32% | -0.20% | 0.01% | 108% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.31% | -0.30% | -0.21% | 106% | 107% |
| Class D | -0.24% | 0.16% | 0.27% | 106% | 103% |
| Class F | -0.95% | -0.77% | -0.77% | 105% | 107% |
| Class TGM | -1.63% | -1.14% | -1.10% | 103% | 108% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.71% | -0.56% | -0.22% | 113% | 110% |
| Class C | -0.36% | -0.20% | -0.29% | 110% | 107% |
| Class E | -0.29% | 0.60% | 0.91% | 113% | 111% |
| Overall | -0.49% | -0.15% | 0.04% | 112% | 109% |
| Class D | -0.34% | 0.62% | -0.03% | 108% | 103% |
| Class F | -1.43% | -1.02% | -0.74% | 110% | 111% |
| Class TGM | -1.85% | -1.13% | -1.24% | 111% | 111% |

# References

- C.-W. Hsu, M. Winken, “CE10: Summary report on combined and multi-hypothesis prediction,” Joint Video Experts Team, doc. JVET-M0030, Marrakech, MA, Jan. 2019.

- M. Winken, H. Schwarz, D. Marpe, T. Wiegand, “CE10: Multi-hypothesis inter prediction (Test 10.1.2),” Joint Video Experts Team, doc. JVET-M0425, Marrakech, MA, Jan. 2019.

- Y.-J. Chang et al., “Compression efficiency methods beyond VVC,” Joint Video Experts Team, doc. JVET-U0100, Jan. 2021.

- V. Seregin et al., “Exploration Experiment on Enhanced Compression beyond VVC capability,” Joint Video Experts Team, doc. JVET-U2024, Jan. 2021.

# Patent rights declaration(s)

Fraunhofer may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).