| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0083 |
| --- | --- |

| Title: | EE2 related: asymmetric binary tree splitting on top of VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec K. Naser, T. Dumas, A. Robert, F. Galpin, E. Francois | Tel: Email: | fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

Asymmetric Binary Tree (ABT) splitting was proposed in JVET-D0064, JVET-J0022 and JVET-K0197. It basically consists in a split mode that divides a coding unit into 2 sub-CUs with split ratio (1/4,3/4) or (3/4,1/4), in the horizontal or vertical direction.

This contribution re-introduces ABT on top of VTM-11. In addition to the technical aspects described in JVET-K0197, some adaptations to VVC coding tools are needed, like affine motion compensation, BDOF, DMVR, ISP and MIP.

Several obtained trade-offs between luma coding gain and encoding time over VTM-11 are reported in random access configuration, which are -1.1%, -1.3%, -1.5% and -1.9% luma gain with respectively 117%, 132%, 148% and 168% encoding times.

It is also shown that higher coding gains are obtained with the proposed partitioning scheme than with the QTBTT at equivalent encoder complexity, when varying the MaxMTTHierachyDepth coding parameters of VTM-11.

It is proposed to further explore the ABT partitioning in the EE2 on Enhanced Compression beyond VVC capability.

# Technical description

## Added ABT split modes.

Four new asymmetric binary tree splitting modes are added to the multi-type tree structure of VVC, to allow new splitting configurations. These added split modes are shown on Figure 1.

According to the added split modes, a coding unit with size S is divided into 2 sub-CU with sizes S/4 and 3.S/4, either in the horizontal or in the vertical direction. In practice the added available CU sizes are 6, 12, and 24.

Figure 1: Split modes supported in the QTBT+TT+ABT framework

## ABT signaling

The signaling of added split modes is depicted on Figure 2. Two context-based coded bins are added, respectively indicating the use of ABT split for a given CU, and the type of ABT split in the case ABT is used. 2 CABAC contexts are added overall.

Figure 2:  signaling of QT, BT, TT and ABT split modes used in this contribution.

Furthermore, some redundant series of splits are normatively forbidden, as shown by the following  Figure 3, so that block division resulting from one ABT split and one BT or TT split can be reached through a unique series of split modes.

Figure 3: split redundancy limitation in the proposed ABT partitioning scheme

## Non-power of 2 adaptations

### Quantization

The quantization and de-quantization processes are adapted for new block sizes as described in JVET-K0197.

### Residual coding

A slight adaptation in the residual coding for the last_sig_coeff_x_prefix and last_sig_coeff_y_prefix syntax elements, for their CABAC context derivation. For block sizes not equal to a power of 2, the context of the closest power of 2, lower than the current block size.

### Additional primary transforms to support non-power of 2 block sizes.

The set of primary transforms contained in VVC is extended to includes the DCT2, DST7 and DCT8 transforms with sizes 6, 12 and 24.

### Affine motion compensation

In VVC, affine motion compensation is performed on a 4x4 sub-block basis in luma and chroma. For ABT, an inter coding block with size 12 in luma has a corresponding coding block with size 6 in chroma. For such chroma blocks, the sub-block size is set to 2 for affine motion compensation.

### SbTMVP (Sub-block Temporal Motion Vector Prediction)

In VVC, in SbtTMVP coding mode, motion compensation is performed on an 8x8 sub-block basis. If a CU width is equal to 12, then the sub-block width is set to 6. If a CU height is equal to 12, then the sub-block height is set to 6. See Figure 3.

Figure 4: policy used for sub-block division of a CU in case of block size 12.

### BDOF and DMVR

In VVC, BDOF performs a refinement of inter bi-predicted blocks based on the optical flow. BDOF applies on luma only, and proceeds on 8x8, 8x16, 16x8 or 16x16 subblocks basis depending on the CU size. It takes the maximum possible sub-blocks size up to 16x16.

The sub-block-based BDOF process is adapted to ABT as follows. For a CU with size 24 in a direction (width or height), the sub-block size used is 8 in that direction, see Figure 4. For a CU with size 12 in a direction (width or height), the sub-block size used is 4 in that direction.

Figure 5: BDOF and DMVR sub-block division adaptation for new block sizes introduced by ABT.

For DMVR, the sub-block size is adapted the same way as for BDOF.

### Matrix-based Intra Prediction (MIP)

The MIP coding tool is adapted to new block sizes issued from ABT splitting. First, the 1D down sampling of neighboring samples is extended to supported ratios 3 and 6.

To do so, the MIP size Id parameter is redefined as follows:

idxW,H=0for W=H=41for W=4 or H=4 or W=8 and H=8or W=12 or H=122otherwise

Then, as in VVC specification, the down sampled boundary size is set as follows:

redBdrSize=idxW,H==0?2:4

For the new down-sampling ratios introduced, similar averaging as for other block sizes is performed.

Then the reduced prediction signal is computed through matrix multiplication as in VVC. The size of the reduced prediction signal is thus given by: redPredSize=(idx(W,H)<2)?4:8.

Finally, the up-sampling towards the initial block size is extended to support up-sampling ratios 3 and 6. To do so, linear interpolation is used as in VVC.

Figure 6: MIP adaptation to ABT for down-sampling neighboring samples and up-sampling the reduced predicted block.

### Intra Sub-Partition (ISP)

The last tool that has been adapted to ABT is ISP. For blocks with size 12 in width or height, when ISP is used in the direction of size 12, then 3 TU with size 4 are generated. In case of an ISP split in the direction where the CU size is 24, then 4 Tus of size 6 are generated.

# Experimental results

The proposed ABT partitioning has been tested according the JVET SDR coding test conditions described in JVET-N1010. Table 1 presents the compression performances obtained with 4 trade-offs between encoding time vs. BD-rate variation over VTM-11 in random access. It can be seen that a luma gain of -1.1%, -1.3%, -1.6% and -1.9% is respectively obtained with a coding time ratio of 117%, 132%, 148% and 162%.

Finally, a last random trade-off 5 with increased coding gain is included in the revision of this contribution. See Table 1(e).

Table 1: performances obtained with ABT partitioning on top of VTM-11, for various BD-rate/Complexity trade-offs.

- Trade-off 1

- Trade-off 2

- Trade-off 3

- Trade-off 4

- Trade-off 5

ABT performances in AI and LDB configurations are given on following Table 2. As can be seen, -0.75%/-3.1%/-2.9% coding gain is obtained in Y/Cb/Cr components in AI configuration, with 237% encoding time relative to VTM-11. In low-delay B, -0.8%/-2.2%/-2.8% (Y/Cb/Cr) coding gain is obtained, with 151% encoding time.

Table 2: ABT performances over VTM-11 in AI and LDB configurations.

Finally, Figure 6 illustrates the trade-offs that are obtained respectively with ABT and with the unchanged VTM-11, by increasing the maximum Multi-Type Tree (MTT) depth parameter. With the VTM-11 with increased maximum MTT depth, -0.8% gain is obtained in Luma, with encoding time 171% over default VTM-11 encoder.

Figure 7: BD-rate luma in RA configuration, as a function of the encoding time over VTM-11 respectively obtained with VTM-11 (blue) and VTM-11+ABT (red)

# Conclusion

Asymmetric Binary Tree partitioning (ABT), initially proposed in JVET-D0064, JVET-J0022 and JVET-K0197, is re-introduced over VTM-11.

It is shown that the enriched partitioning scheme increases the coding efficiency of VVC, with very limited impact on the decoding complexity.

Moreover, interesting trade-offs between coding gain and encoding time are obtained in random access configuration. -1.1% to -1.9% gain is obtained for an encoding time ranging from 117% to 162%.

It is proposed to further investigate ABT in the scope of the JVET Exploration Experiment 2 on compression capability beyond VVC.

# Cited references

- “CE1: Asymmetric Binary Tree (tests 1.0.1, 1.0.2, 1.0.3, 1.0.4, 8.0.1 and 8.0.2)”, F. Le Léannec, T. Poirier, F. Galpin (Technicolor), JVET-K0197, July 2018

- “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – medium complexity version”,	P. Bordes, Y. Chen, C. Chevance, E. François, F. Galpin, M. Kerdranvat, F. Hiron, P. de Lagrange, F. Le Léannec, K. Naser, T. Poirier, F. Racapé, G. Rath, A. Robert, F. Urban, T. Viellard (Technicolor), Y. Chen, W.-J. Chien, H.-C. Chuang, M. Coban, J. Dong, H. E. Egilmez, N. Hu, M. Karczewicz, A. Ramasubramonian, D. Rusanovskyy, A. Said, V. Seregin, G. Van Der Auwera, K. Zhang, L. Zhang (Qualcomm), JVET-J0022, March 2018

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).