| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20-28th Apr. 2021 | Document: JVET-V0085 |
| --- | --- |

| Title: | CE-related: On Rice parameter selection for transform skip residual coding (TSRC) at high bit depths |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Steve Keating Karl Sharman | Email: | adrian.browne@sony.com steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This contribution describes modifications to the method for the selection of Rice parameters for transform skip residual coding, for consideration in VVC version 2. The modifications offer BD-rate gains, particularly for screen content, when residual coefficient values become larger but do not affect version 1 operating points. It is suggested that the modification extends the operating point of the existing residual coding method to be applicable at all currently considered bit depths.

Using the high bit depth, high bit rate CTC the following BD-rate results were obtained for screen content coding:

AI:		-0.41%/-0.14%/-0.25%

RA:	-0.86%/-0.85%/-0.86%

LDB:	-0.59%/-1.08%/-0.32%

# Introduction

In VVC version 1[1], all coefficients coded with TSRC use a Rice parameter of 1. This value has been found to be suitable for both 8 and 10 bit data; however, as bit depths increase and QPs become lower, the size of each coded coefficient becomes larger. Such values may be coded more efficiently using a higher Rice parameter.

This proposal represents a re-examination and refinement of CE-1.6 described in the CE of the JVET U meeting[2] which aims to simplify the complexity of the original technique whilst attempting to maintain, or where possible, increase the measured gains.

The modification described here results in gains for 12 and 16-bit content, with the greatest gains observed for 12-bit screen content. The modification provides backwards compatibility with VVC version 1 as the existing Rice coding method can be considered as a subset of the modification and is controlled by a parameter sent in the SPS.

# Algorithm description

The algorithm in this proposal, and the one on which it is based, impact entropy coding and, in particular, the selection of Rice parameters for Golomb-Rice coding. The algorithm can be split into a series of components as follows each of which has been re-examined for this proposal:

- A coefficient size estimation function, which is backed by a series of counters.

- An update function for the selected counter.

- A modification to the current algorithm for selecting Rice parameters.

## Coefficient size estimation

The technique requires an estimation of the log2 size of coefficients being coded. This is derived in a similar manner to Rice parameter estimation in HEVC (RExt) using a series of counters. The counters are mapped to an array with one element per component type, giving a total of 2 elements.

## Counter update function

Each counter is set to a default value computed from the slice QP at the beginning of every slice.  The counters are handled in the same way as CABAC contexts, thus allowing the values to be rewound during the encoder search. This also provides support for wave-front parallel coding as each wave-front copies the counters before starting.

In the previous proposal, a counter was updated each time a coefficient on the leading diagonal of the coefficient block was coded using Golomb-Rice. In the new proposal, this update has been moved outside the coefficient coding process and is now based on four updates per 4x4 sub-block at fixed scan positions. The update is also now done using the complete coefficient value rather the remainder coded with Golomb-Rice. This removes the coupling between updates and Golomb-Rice coding and allows additional flexibility for implementations, as the results of the update process are not required until the start of the next transform unit.

The update process has been simplified in the new proposal as shown below. Previously three comparisons were used, one with zero and up to two with computed thresholds. In addition a clip to zero was used to stop the counter going negative. This technique with upper and lower thresholds was taken from HEVC, but was proven experimentally to be sub-optimal for VVC. This has been reduced in this proposal to two comparisons, one with zero and the other with a single threshold. The clip has been moved to occur once per TU rather than once per coded coefficient.

count += value == 0 ? 0 : abs(value) > threshold ? increment : -decrement // increment= 3 decrement = 1

The threshold used in the update process is updated once per transform unit as below:

count = max(0, count);                                                      // clip now once per TU

int rice = count  >> log2Divisor;				            // log2Divisor = 4

threshold = (max(12, 16 - (rice >> 1)) << rice) >> 3;

The new threshold calculation includes a gradual relative reduction to model the effect of the reducing effect of the “greater than” flags used in coefficient coding.

## Rice parameter selection

The rice parameter selected is a positive offset from the value of 1 used in VVC version 1. The increment depends on whether the coefficient was coded with significance, GT1, parity and GT2 flags; significance, GT1, and parity, or no flags, computed as below:

int riceValueX2 = count >> log2DivisorMinus1;

offsetWithGT2 = max(0, riceValueX2 - 6) >> 1;

offsetWithParity = max(0, riceValueX2 - 5) >> 1;

offsetNoParity = max(0, riceValueX2 - 3) >> 1;

These increments are computed immediately after the computation of the new threshold value, i.e. once per transform unit.

# Results

All results reported here are compared with anchors generated using the CE anchor (VTM12.0 with Rice coding modifications from CE-1.1 and CE-2.1) and the high bit depth, high bit rate and high frame rate CTC[4].

From the results it can be seen that greater gains are obtained for screen content (TGM) rather than standard camera content for which RRC modifications are more beneficial.

## Low QP range

### PQ

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.02% | -0.02% | -0.02% | -0.02% | -0.03% | -0.02% | 98% | 99% |
| PQ422 | -0.02% | -0.05% | -0.06% | -0.02% | -0.06% | -0.06% | 100% | 100% |
| Overall | -0.02% | -0.04% | -0.04% | -0.02% | -0.04% | -0.04% | 99% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | -0.02% | -0.01% | -0.03% | -0.02% | -0.01% | -0.02% | 101% | 102% |
| PQ422 | 0.00% | 0.00% | -0.03% | 0.00% | 0.00% | -0.02% | 100% | 101% |
| Overall | -0.01% | -0.01% | -0.03% | -0.01% | -0.01% | -0.02% | 101% | 102% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM_U_CE |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | -0.02% | -0.01% | 0.00% | -0.02% | -0.01% | 100% | 100% |
| PQ422 | 0.00% | -0.01% | -0.04% | 0.00% | -0.01% | -0.03% | 100% | 100% |
| Overall | 0.00% | -0.01% | -0.02% | 0.00% | -0.02% | -0.02% | 100% | 100% |

### HLG

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.04% | -0.09% | -0.09% | 100% | 100% |
| HLG422 | -0.05% | -0.09% | -0.09% | 99% | 100% |
| Overall | -0.05% | -0.09% | -0.09% | 100% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.05% | 0.00% | 0.02% | 100% | 100% |
| HLG422 | -0.07% | -0.04% | -0.01% | 99% | 100% |
| Overall | -0.06% | -0.02% | 0.00% | 100% | 100% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.05% | -0.01% | 0.00% | 100% | 100% |
| HLG422 | -0.05% | 0.00% | -0.01% | 100% | 99% |
| Overall | -0.05% | 0.00% | -0.01% | 100% | 100% |

### SVT

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.23% | -0.14% | -0.13% | 102% | 103% |
| SVT12 | -0.07% | -0.06% | -0.07% | 100% | 100% |
| Overall | -0.15% | -0.10% | -0.10% | 101% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.03% | 0.01% | 0.00% | 99% | 101% |
| SVT12 | -0.03% | 0.01% | 0.00% | 99% | 101% |
| Overall | -0.03% | 0.01% | 0.00% | 99% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM_U_CE |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | -0.03% | -0.01% | -0.01% | 98% | 100% |
| SVT12 | -0.04% | 0.01% | -0.01% | 100% | 102% |
| Overall | -0.03% | 0.00% | -0.01% | 99% | 101% |

## Standard QP range

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.03% | -0.02% | -0.02% | -0.02% | -0.02% | -0.02% | -0.02% | -0.02% | 100% | 102% |
| Class H2 |  |  |  |  |  | -0.01% | -0.04% | -0.02% | 100% | 102% |
| Overall | -0.03% | -0.02% | -0.02% | -0.02% | -0.02% | -0.02% | -0.03% | -0.02% | 100% | 102% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM_U_CE |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 100% |

## TGM Content

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | 0.36% | 0.30% | 0.35% | 102% | 102% |
| RGB 4:4:4 - TGM 720p | -1.19% | -0.57% | -0.85% | 102% | 101% |
| Overall | -0.41% | -0.14% | -0.25% | 102% | 102% |

|  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | -0.44% | -0.45% | -0.43% | 100% | 102% |
| RGB 4:4:4 - TGM 720p | -1.28% | -1.26% | -2.26% | 100% | 101% |
| Overall | -0.86% | -0.85% | -1.34% | 100% | 101% |

|  |  |  | LDB |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_U_CE |  |  |
|  | Y / G | U / B | V / R | EncT | DecT |
| RGB 4:4:4 - TGM 1080p | -0.02% | -0.02% | 0.01% | 100% | 100% |
| RGB 4:4:4 - TGM 720p | -0.59% | -2.14% | -0.65% | 101% | 101% |
| Overall | -0.30% | -1.08% | -0.32% | 100% | 101% |

# Estimate of Complexity

This section attempts to give an estimate of the additional complexity of the proposal for both hardware and software. The additional complexity may be separated into four sections: memory requirements, coding a value, updating a counter, and updating a parameter set as follows:

## Memory requirements

2*10 bit memory for storage of counters

## Coding a value (every coefficient)

1 * add

## Updating a counter (4 times in a 4x4 sub-block)

2 * conditional

1 * add

## Updating a parameter set (once per block)

9 * adds

7 * shift (fixed size)

1 * shift (variable size)

# Conclusions

This contribution has described modifications to the method for the selection of Rice parameters for transform skip residual coding, for consideration in VVC version 2. The modifications offer BD-rate gains when residual coefficient values become larger but do not affect version 1 operating points.  It is suggested that the modification extends the operating point of the existing residual coding method to be applicable at all currently considered bit depths.

It is proposed that the modifications be included for consideration alongside proposals within the CE for adoption into VVC version 2 and VTM.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] “Versatile Video Coding Editorial Refinements on Draft 10”, JVET-T2001, B. Bross, J. Chen, S. Liu and Y-K. Wang

[2] “CE-1.5, CE-1.6, CE-2.2 and CE-2.3: Rice parameter selection for high bit depths”, JVET-U0050, A. Browne, S. Keating and K. Sharman

[3] “Common test conditions for high bit depth and high bit rate video coding”, JVET-U2018, A. Browne, T. Ikai, D. Rusanovskyy, M. Sarwer, X. Xiu