| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0089 |
| --- | --- |

| Title: | EE2-related: Inter coding modes modifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal, |  |  |
| Author(s) or Contact(s): | A. Robert, F. Galpin, F. Le Leannec T. Poirier | Email: | antoine.robert@interdigital.com franck.galpin@interdigital.com fabrice.leleannec@interdigital.com tangi.poirier@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes some Inter coding modes modifications on top of JVET-U0100. This includes:

- Adding a full pruning considering BCW index, SIF and LIC flags in all Inter coding modes,

- Adding a STMVP candidate in merge list,

- Modifying the pairwise merge candidate,

- Adding pairwise affine merge candidates,

- Using the SIF flag in GEO mode.

It is reported that tool-on test achieves averagely 0.24% BD rate saving in RA while losing 2% encoding and 3% decoding time, and 0.36% BD rate saving in LDB while keeping the encoding and decoding time. Tool-off test is achieving averagely 0.09% BD rate saving in RA while keeping the encoding and decoding time, and 0.xx% BD rate saving in LDB while losing x% encoding and x% decoding time.

# Introduction

JVET-U0100 [1] proposes some pruning in AMVP mode for the regular AMVP list construction that uses TM, and also in merge mode. In merge mode, when the merge list is constructed for the TM case, a distance between available candidates is used. On the other hand, when constructing the list for other merge modes (regular merge, MMVD, CIIP, GEO), a classical pruning is used.

# Proposal

This proposal aims at modifying Inter coding modes in several ways without increasing the complexity.

## Full pruning

In the current AMVP of JVET-U0100, some pruning is performed for the regular case that uses TM. It is proposed to extend this pruning to all AMVP cases, including the multi-hypothesis and IBC cases. Furthermore, the AMVR rounding operations have been adjusted to avoid a full list rounding at the end.

In all other modes, all motion information is considered, meaning that not only motion vectors and reference frames are compared, but it is also proposed to include:

- the BCW index,

- the SIF flag, and

- the LIC flag.

Indeed, two candidates having the same motion vectors and reference frames but with a different BCW index or SIF or LIC flag produce two different predictions.

In merge list construction for TM, the pruning with a distance between candidates has not been modified. But for all the other merge modes (regular merge, MMVD, CIIP, GEO), the BCW index as well as the SIF and LIC flags have been added into the pruning.

In affine modes, AMVP and merge, some pruning has been added. The pruning operations mimic those of the regular AMVP and regular merge modes, respectively. It favors spatial inherited affine candidates by searching two different ones (one from left and one from above) while all following candidates are pruned. In affine merge, only the BCW index and the LIC flag are used in the pruning since affine does not use SIF.

In GEO mode, the input merge list is already pruned, but by selecting uni-directional parts some redundancy can still occur. A pruning process has also been added in the GEO mode to avoid such redundancy.

## STMVP candidate

In the regular merge list, a STMVP (Spatio-Temporal Motion Vector Predictor) is proposed. It is inserted right after the bottom-left spatial candidate.

It is constructed based on up to two of the spatial candidates already present in the merge list selected as the two first candidates using the first reference frame, and on the regular temporal candidate scaled to the first reference frame. For each reference frame list, if the temporal candidate and at least one of the spatial candidates are defined, the STMVP candidate is constructed as an average between these candidates as:

- When only one spatial candidate is defined,

mvSTMVP = (mvspa + mvtemp) / 2

- When both spatial candidates are defined,

mvSTMVP = (3*mvspa1 + 3*mvspa2 + 2*mvtemp) / 8

## Pairwise candidate

In the current JVET-U0100 implementation of pairwise, the BCW index of the pairwise candidate is never set. So, if the previously studied candidate has been pruned, then the pairwise candidate wrongly inherits from its BCW index. To overcome this problem, it is proposed to set a BCW index to the pairwise candidate, inherited from input candidates as:

- If first input candidate is bi-directional, then the pairwise candidate inherits its BCW index,

- Otherwise

  - If the second input candidate is bi-directional and shares the same reference frame as the first input candidate (which is uni-directional), then the pairwise candidate inherits the BCW index of the second input candidate,

  - Otherwise, the BCW index of the pairwise candidate is set to default.

Furthermore, it is also proposed to define the pairwise SIF flag with an “or” operation between the input candidates’ SIF flags instead of an “and” operation, and to set a LIC flag, in case the pairwise candidate is uni-directional, defined with an “or” operation between the input candidates’ LIC flags.

Eventually, up to three pairwise candidates can be constructed using as input pairs of merge candidates with indexes (0, 1), (0, 2) or (1, 2).

## Affine pairwise candidate

In affine merge mode, it is proposed to add some affine pairwise candidates in the same way as in regular merge modes. To this aim, up to three pairwise affine candidates can be constructed with the same BCW index and LIC flag derivation processes as in regular merge case.

The SbTMVP candidate (if present) is avoided of the affine pairwise candidate construction. If the input candidates have models with the same number of parameters (4- or 6-), the pairwise candidate is described as an average model with this number of parameters. But when input candidates have models with a different number of parameters, the pairwise average model is forced to be a 4-parameter model.

## SIF in GEO mode

It is proposed to inherit the SIF flag of the corresponding merge candidate when constructing the GEO uni-directional candidates’ list.

Prior to the motion compensation of each GEO part, the AMVR index of the CU is set to half-pel when the corresponding SIF flag is on or to default otherwise. After motion compensation, the AMVR index is reset to default. In that way, each part can be independently motion compensated using its own SIF flag.

The SIF flag of each part is stored independently in the motion field of the corresponding part. For the blended part, where combined motion information is used, the motion field is filled as:

- If each uni-directional part uses different reference picture lists (one from L0 and the other from L1), then their motion vectors are simply combined to form the bi-prediction motion information and an “or” operation is performed between their SIF flag to get the blended SIF flag.

- Otherwise, both use the same list, only uni-prediction motion information of the second part is stored as well as its SIF flag.

The AMVR index of the GEO CU is then affected to half-pel when the “or” operation between the SIF flags of each part, whatever their reference frame list, is on, otherwise it is set to default.

# Experimental results

## Tool-on tests

Tool-on tests have been performed using VTM-11.0 as anchor compared to the proposed modifications of Inter coding modes of JVET-U0100 without any tools and tool-on configuration files.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | -0.26% | -0.24% | 102% | 103% |
| Class A2 | -0.21% | -0.23% | -0.32% | 101% | 104% |
| Class B | -0.20% | -0.20% | -0.05% | 103% | 104% |
| Class C | -0.32% | -0.36% | -0.24% | 100% | 101% |
| Overall | -0.24% | -0.26% | -0.19% | 102% | 103% |
| Class D | -0.32% | -0.27% | -0.25% | 104% | 107% |
| Class F (optional) | -0.38% | -1.01% | -1.14% | 101% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.28% | -0.30% | -0.18% | 102% | 100% |
| Class C | -0.30% | -0.26% | -0.28% | 101% | 101% |
| Class E | -0.58% | -0.74% | -0.22% | 100% | 97% |
| Overall | -0.36% | -0.39% | -0.22% | 101% | 99% |
| Class D | -0.32% | 0.14% | 0.03% | 101% | 99% |
| Class F (optional) | -0.36% | 0.22% | -0.27% | 100% | 103% |

## Tool-off tests

Tool-off tests of the proposed methods have been performed on top of the JVET-U0100 as described in the EE2 document JVET-U2024 [2].

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-U0100 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.04% | 0.10% | 101% | 100% |
| Class A2 | -0.12% | -0.06% | -0.06% | 100% | 101% |
| Class B | -0.07% | -0.03% | -0.16% | 100% | 98% |
| Class C | -0.13% | -0.01% | -0.09% | 101% | 100% |
| Overall | -0.09% | -0.03% | -0.07% | 100% | 99% |
| Class D | -0.09% | 0.00% | -0.24% | 101% | 97% |
| Class F (optional) | -0.07% | -0.64% | -0.70% | 104% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-U0100 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.14% | -0.05% | 0.03% | 101% | 98% |
| Class E | -0.36% | -0.43% | -0.56% | 103% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.13% | -0.11% | -0.16% | 100% | 99% |
| Class F (optional) | -0.10% | -0.39% | -0.76% | 102% | 99% |

# Conclusion

In this proposal, some parts of the JVET-U0100 Inter coding modes have been modified. It is shown that this can achieve some BD rate saving with a very limited encoding and decoding time impact for both tool-on and tool-off tests. It is proposed to investigate these Inter modifications in the scope of EE2.

# References

- “Compression efficiency methods beyond VVC”, JVET-U0100, Y-J. Chang, C-C. Chen, J. Chen, J. Dong, H. E. Egilmez, N. Hu, H. Huang, M. Karczewicz, J. Li, B. Ray, K. Reuze, V. Seregin, N. Shlyakhov, L. Pham Van, H. Wang, Y. Zhang, Z. Zhang.

- “Exploration Experiment on Enhanced Compression beyond VVC capability”, JVET-U2024, V. Seregin, J. Chen, S. Esenlik, F. Le Leannec, L. Li, M. Winken, J. Strom, X. Xiu, K. Zhang.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).