| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 April 2021 | Document: JVET-V0099-v2 |
| --- | --- |

| Title: | AHG12: Adaptive Reordering of Merge Candidates with Template Matching |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Na Zhang1, Kai Zhang2, Li Zhang2, Hongbin Liu1, Zhipin Deng1,Yang Wang1 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhangna.cynthia@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, an adaptive reordering of merge candidates with template matching (ARMC) method is proposed. With the proposed method, merge candidates are divided into several subgroups, and merge candidates in a subgroup are reordered according to cost values based on template matching. The reordering method is applied to regular merge mode, template matching merge mode, and affine merge mode.

On top of the EE code of JVET-U0100, simulation results of the proposed method are reported as below:

Tool-on test (VTM-11 as the anchor): RA: -0.45%,103%, 111%; LB: -0.52%, 101%,114%

Tool-off test (EE code as the anchor): RA: -0.31%, 100%, 103%; LB: x%, x%, x%

# Introduction

In current VTM design, merge candidate list is generated in a fixed order, which may not be optimal. Adaptive merge candidate list generation process can be used to improve the coding efficiency of the merge candidate index.

# Proposed methods

In this contribution, an adaptive reordering of merge candidates with template matching (ARMC) method is proposed. The reordering method is applied to regular merge mode, template matching (TM) merge mode, and affine merge mode (excluding the SbTMVP candidate). For the TM merge mode, merge candidates are reordered before the refinement process.

After a merge candidate list is constructed, merge candidates are divided into several subgroups. The subgroup size is set to 5. Merge candidates in each subgroup are reordered ascendingly according to cost values based on template matching. For simplification, merge candidates in the last but not the first subgroup are not reordered.

The template matching cost is measured by the sum of absolute differences (SAD) between samples of a template of the current block and their corresponding reference samples. The template comprises a set of reconstructed samples neighboring to the current block. Reference samples of the template are located by the same motion information of the current block.

When a merge candidate utilizes bi-directional prediction, the reference samples of the template of the merge candidate are also generated by bi-prediction as shown in Figure 1.

Figure 1. Template and reference samples of the template in reference list 0 and reference list 1

For subblock-based merge candidates with subblock size equal to Wsub * Hsub, the above template comprises several sub-templates with the size of Wsub × 1, and the left template comprises several sub-templates with the size of 1 × Hsub. As shown in Figure 2. the motion information of the subblocks in the first row and the first column of current block is used to derive the reference samples of each sub-template.

Figure 2. Template and reference samples of the template for block with sub-block motion using the motion information of the subblocks of current block

# Experimental Results

The proposed method has been implemented on top of the EE code of JVET-U0100 [1]. Results under RA/LDB configurations are summarized in Table 1 and Table 2. The simulations were performed following the common test conditions [2]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the ARMC (tool-on test).

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | -0.51% | -0.45% | 102% | 105% |  |  |  |  |  |
| Class A2 | -0.54% | -0.60% | -0.66% | 103% | 107% |  |  |  |  |  |
| Class B | -0.40% | -0.42% | -0.26% | 103% | 109% | -0.39% | -0.15% | 0.12% | 101% | 113% |
| Class C | -0.46% | -0.44% | -0.34% | 103% | 121% | -0.42% | -0.22% | -0.41% | 101% | 117% |
| Class E |  |  |  |  |  | -0.88% | -0.88% | -0.48% | 102% | 110% |
| Overall | -0.45% | -0.48% | -0.40% | 103% | 111% | -0.52% | -0.36% | -0.21% | 101% | 114% |
| Class D | -0.31% | -0.34% | -0.28% | 102% | 125% | -0.18% | 0.06% | -0.15% | 101% | 119% |
| Class F | -0.37% | -0.31% | -0.39% | 103% | 105% | -0.63% | -0.42% | -0.27% | 101% | 107% |

Table 2: Coding performance of the ARMC (tool-off test).

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.41% | -0.22% | 101% | 103% |  |  |  |  |  |
| Class A2 | -0.31% | -0.24% | -0.27% | 101% | 103% |  |  |  |  |  |
| Class B | -0.28% | -0.35% | -0.34% | 99% | 104% | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class C | -0.33% | -0.26% | -0.27% | 98% | 102% | -0.25% | -0.25% | -0.14% | 101% | 107% |
| Class E |  |  |  |  |  | -0.67% | -0.62% | 0.75% | 102% | 103% |
| Overall | -0.31% | -0.31% | -0.28% | 100% | 103% | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | -0.27% | -0.22% | -0.37% | 99% | 103% | -0.16% | -0.07% | -0.48% | 102% | 108% |
| Class F | -0.34% | -0.42% | -0.32% | 99% | 102% | -0.48% | -0.20% | -0.41% | 101% | 106% |

- Conclusions

In this contribution, a ARMC method is proposed. It can bring some coding gain with a good trade-off between coding performance and complexity. It is recommended to further study it on the video coding research platform beyond VVC.

# References

[1]	https://vcgit.hhi.fraunhofer.de/jvet-u-ee2/VVCSoftware_VTM/-/tree/U0100

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video,” JVET-T2010, Oct. 2020.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).