| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0103 |
| --- | --- |

| Title: | AHG12: Geometric prediction mode with motion vector differences |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Kai Zhang2, Li Zhang2, Na Zhang1, Yang Wang1 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | +1-858-380-8522 zhipin.deng@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, Geometric prediction mode with Motion Vector Difference (GMVD) is proposed. With GMVD, the motion information of geometric partitions of a GPM CU are further refined by adding up signalled MVDs to GPM merge candidates. The MVD of a GPM partition is signalled in a similar way as MMVD in VVC, wherein indices of a direction and a distance are coded. Both tool-on (Test 1) and tool-off (Test 2) tests were conducted. Simulation results are reported as below:

Test 1 (VTM-11.0 as the anchor):      RA -0.xx%, xxx%, xxx%, and LB -0.xx%, xxx%, xxx%

Test 2 (EE common base as the anchor): RA -0.xx%, xxx%, xxx%, and LB -0.xx%, xxx%, xxx%

The peak BD-rate impacts for tool-on test in CTC are reported to be -0.xx% and -0.xx% under RA and LB configurations respectively.

# Introduction

The GPM mode in VVC uses the merge-derived motion information to generate the prediction samples for each geometric partition. Directly using the implicitly derived motion in GPM might not lead to a precise motion vector.

# Proposed Solution

To address the problem in GPM, Geometric prediction mode with Motion Vector Difference (GMVD) mode is proposed in this contribution. The GPM in VVC is extended by adding up an MVD to a GPM merge candidate, as proposed in JVET-R0357 [1]. A GMVD flag is firstly signalled for a GPM CU, to specify whether the GMVD mode is used. If the GMVD flag is true, each geometric partition of a GPM CU can further decide whether to use GMVD or not. If GMVD is used to a geometric partition, after a GPM merge candidate is selected, the motion of the partition is further refined by the signalled MVDs information. All other procedures are kept the same as in GPM.

With GMVD, an MVD is signaled as a pair of distance and direction, similar as in MMVD. There are nine candidate distances (¼-pel, ½-pel, 1-pel, 2-pel, 3-pel, 4-pel, 6-pel, 8-pel, 16-pel), and eight candidate directions (four horizontal/vertical directions and four diagonal directions) involved in GMVD. In addition, when pic_fpel_mmvd_enabled_flag is equal to 1, the MVD in GMVD is also left shifted by 2 as in MMVD.

# Experimental Results

The proposed method has been implemented on top of the EE common software base [2], and simulations are conducted under the CTC in JVET-U2017 [3].

Two tests were conducted. Test 1 is a tool-on test, using VTM-11.0 as the reference. Test 2 is a tool-off test, using the EE common software base as the reference. As a response to new coding tools in the EE common software base, Test 2 introduces more encoder search for GMVD as well as a normative change that full pruning the GPM merge list.

Results under RA/LB configurations are summarized in Table 1 and Table 2, for tool-on and tool-off configurations, respectively.

Table 1: Results of GMVD in Test1 (tool-on)

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.16% | -0.25% | -0.15% | 104% | 99% |  |  |  |  |  |
| Class C | -0.39% | -0.64% | -0.50% | 105% | 98% | -0.43% | -0.49% | -0.58% | 105% | 101% |
| Class E |  |  |  |  |  | -0.28% | -0.98% | -0.32% | 102% | 102% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.31% | -0.53% | -0.34% | 105% | 99% | -0.42% | 0.22% | -0.57% | 103% | 100% |
| Class F | -0.08% | -0.22% | -0.24% | 103% | 100% |  |  |  |  |  |

Table 2: Results of GMVD in Test2 (tool-off)

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.07% | -0.17% | -0.20% | 124% | 101% |  |  |  |  |  |
| Class C | -0.21% | -0.24% | -0.37% | 121% | 100% | -0.27% | -0.37% | -0.11% | 126% | 97% |
| Class E |  |  |  |  |  | -0.18% | 0.15% | 0.45% | 132% | 99% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.18% | -0.12% | -0.41% | 119% | 102% | -0.26% | -0.90% | -0.84% | 122% | 104% |
| Class F | -0.05% | -0.06% | -0.07% | 115% | 101% |  |  |  |  |  |

# Conclusion

The proposed method can achieve promising BD-rate gains with almost no burden at decoder side. It is suggested adopting GMVD to the next Exploration Experiment on Enhanced Compression beyond VVC capability.

# References

- K. Zhang, L. Zhang, Z. Deng, H. Liu, Y. Wang, “Geometric prediction mode with motion vector differences”, JVET-R0357, Apr. 2020.

- Y.-J. Chang, C.-C. Chen, J. Chen, J. Dong, H. E. Egilmez, N. Hu, H. Huang, M. Karczewicz, J. Li, B. Ray, K. Reuze, V. Seregin, N. Shlyakhov, L. Pham Van, H. Wang, Y. Zhang, Z. Zhang, “Compression efficiency methods beyond VVC”, JVET-U0100, Jan. 2021.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-U2017, Jan. 2021.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).