| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0107 |
| --- | --- |

| Title: | AhG7: Editorial suggestion for JVET CTC on HDR/WCG |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | D. Rusanovskyy (Qualcomm) T.Hashimoto (Sharp) | Email: | dmytror@qti.qualcomm.com tomonori.hashimoto@sharp.co.jp |
| Source: | Qualcomm, Sharp |  |  |

_____________________________

# Abstract

This contribution informs on inaccuracies identified in the text of JVET CTC on HDR/WCG video coding related to wPSNR calculation. Text improvement is proposed to address identified inaccuracies and enabling HDR/WCG metric interpretation for higher bit depths.

# Introduction

During the work on preparation of the 12 bits HDR test materials and VTM SW update for AhG8, the following inaccuracies were identified in Annex B of the text of the CTC on HDR/WCG video coding, JVET-T2011.

- Weights of wPSNR calculation

The document defines two weighting functions for calculating of the wPSNR. The first function is suitable for a native 10 bits HDR content in BT.2100/PQ representation. The second function, is suitable for original SDR content in BT.709, being converted to the BT.2100/PQ representation.

However, the current text seems to specify usage of the second weighting function (SDR optimized) for ClassH1 test content (native HDR content). Specified function does not cover full range of the 10 bits codewords and is not aligned to what is implemented in VTM12.0 software.

The calculation of the weight is computed as:

yi = 0.015*luma(xorig,i) − 1.5 − 6;

yi = yi < −3 ? −3 : (yi>6 ? 6 : yi);

wi(luma(xorig,i)) = pow(2.0, yi÷3.0);

However, for the testing of sequences in Class H1, an alternative calculation of the weight shall be used and computed as:

yi = 0.03*luma(xorig,i) − 3.0;

yi = yi<0 ? 0 : (yi>12 ? 12 : yi);

wi(luma(xorig,i)) = pow(2.0, yi÷3.0);

- Bit depth support

Current specification for weights function is limited to 10 bits and does not support other bit depths. In the view of ongoing development on high bit depth coding (AhG8) where wPSNR calculation is utilized, and to reflect the metrics calculation implemented in VTM12.0, the metric description provided in HDR/WCG CTC may be improved.

# Suggested text

Suggested changes to the HDR/WCG CTC with are shown below in the yellow highlight.

## Suggested text 1

Suggest text 1 address’ identified inaccuracies and enables bit-depth support in the aligning to SW implementation.

The wPSNR, or weighted PSNR, metric is calculated from the weighted mean squared error of the pixel values. An implementation of the metric for materials in the BT.2100/PQ representation is provided in the VTM software, and that implementation shall be used for calculation of the metric using the weighting functions provided below. It is noted that the HDRMetrics tool is capable of computing the weighted PSNR metric. If the HDRMetric package is used, it shall be configured to provide the same result as the implementation of the metric provided in the VTM software.

As an informative description of the metric follows.

The wPSNR metric is calculated as:

wPSNR=10*logX2wMSE,

where X is the maximum pixel value for the specific bit depth and wMSE is given as

wMSE= all pixels i in blockwilumaxorig,i*(xorig, i-xdec,i)2,

where wilumaxorig,iis a weight that is a function of the luma value corresponding to the pixel xorig,i, xorig,i is the original value at location i, and xdec,i is the reconstructed value at location i.

For reporting of coding results for video data in the BT.2100/PQ representation, the calculation of the weight is computed as following:

x = luma(xorig,i);

x = bitDepth<10 ? x << (10 - bitDepth) : bitDepth>10 ? x >> (bitDepth - 10) :x;

yi = 0.015* x − 1.5 − 6;

yi = yi < −3 ? −3 : (yi>6 ? 6 : yi);

wi(luma(xorig,i)) = pow(2.0, yi÷3.0);

The variable bitDepth is being the bit depth of the luma component of the original signal.

Note: for an SDR test material originally provided in the BT.709 representation and being converted to and coded at the BT.2100/PQ, 10 bits representation, another weighting function to be used:

yi = 0.03*luma(xorig,i) − 3.0;

yi = yi<0 ? 0 : (yi>12 ? 12 : yi);

wi(luma(xorig,i)) = pow(2.0, yi÷3.0);

In all cases, the metric is calculated individually for a single luma or chroma channel and then used to compute a Bjøntegaard Delta-Rate and Delta-PSNR rate.

## Suggested text 2

Alternatively, non-10 bits support in wPSNR calculation can be implemented with a higher precision of weight computation. Suggested text for this computation is shown below, if adopted SW implementation in VTM may need to be implemented.

bdDelta = bitDepth<10 ? x << (10 - bitDepth) : bitDepth>10 ? x >> (bitDepth - 10) :x;

scaler = 0.015 / (2 ^ bdDelta);

yi = scaler * luma(xorig,i) − 1.5 − 6;

yi = yi < −3 ? −3 : (yi>6 ? 6 : yi);

wi(luma(xorig,i)) = pow(2.0, yi÷3.0);

# References

[1] “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-T2011, Oct. 2020.