| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 22nd Meeting, by teleconference, 20–28 Apr. 2021 | Document: JVET-V0108r1 |
| --- | --- |

| Title: | AHG9: Colour Transform Information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François, Miloš Radosavljević, Philippe de Lagrange, Fabrice Le Léannec | Tel: Email: | {edouard.francois,milos.radosavljevic}@interdigital.com |  |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution describes additional study of a new SEI message named Colour Transform Information (CTI) SEI message, proposed in JVET-U0078. The SEI message signals mapping parameters that can apply to the decoded pictures. The mapping can operate intra-component (for luma and chroma) or inter-component (for chroma). In this contribution, the CTI has been tested as a mandatory process for out-of-loop mapping for HDR-PQ content. Tests without and with LMCS have been made. The best results are obtained with CTI and LMCS both applied. BD-rate variations compared to VTM12.0, for the CTCs HDR PQ content, are reported in the table below, for AI, RA, LB and LD configurations.

|  | DE100 | PSNR-L | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AI | -0.46% | -0.91% | -0.30% | -5.27% | -6.21% | 0.40% | -1.79% | -2.15% | 106% | 99% |
| RA | -2.30% | -3.31% | -1.74% | -5.65% | -7.35% | -1.06% | -1.32% | -1.99% | 102% | 96% |
| LB | -5.85% | -3.96% | -2.57% | -9.52% | -14.06% | -1.89% | 0.69% | -3.25% | 100% | 92% |
| LP | -6.68% | -3.83% | -2.49% | -9.90% | -14.50% | -1.86% | 1.18% | -3.40% | 103% | 100% |

# Introduction

This contribution is a follow-up of JVET-U0078, where a new SEI message, named Colour Transform Information (CTI) SEI message, was proposed. The CTI SEI message signals mapping parameters that can apply to the reconstructed pictures after the decoding process. CTI can be used as a tool for out-of-loop content mapping, in order to get improved coding performance. This feature can be of interest for some use cases or content types. For HDR PQ cases, it is observed that the CTI using the same fixed mapping function as LMCS leads to compression gains compared to VTM anchors.

# Proposal

The proposed SEI conveys metadata corresponding to a post-decoding colour transform, hereunder named mapping process. For the luma component (component 0), the mapping process operates as an intra-component mapping.  For the chroma components (components 1 and 2), the mapping process can operate as an intra-component mapping or as a cross-component mapping. When cross-component mapping is used, the chroma cross-component mapping functions can be inferred from the luma mapping function, or explicitly signalled. When cross-component mapping is not used, the chroma intra-component mapping functions are explicitly signalled. The mapping functions are based on piecewise linear models whose parameters are signalled in the CTI. The horizontal axis of the piecewise linear (PWL) functions is uniformly spaced.

The signalling is summarized below.

For colour component 0:

- PWL0 coded

For colour components 1 and 2

- crossCompFlag coded

- If crossCompFlag = 1, chromaInferredFlag coded

- If crossCompFlag = 0 or chromaInferredFlag = 0, PWL1/PWL2 coded

- Else chromaOffset coded, PWL1/PWL2 inferred from PWL0

The colour transform process is summarized below. Mapping functions (noted below “LUT”) are derived from the PWL models.

For colour component 0:

- outC0 = LUT0[inC0]

For colour component x=1, 2

- crossCompFlag = 0	outCx = LUTx[inCx]

- crossCompFlag = 1	outCx = ( (inCx–2^(B–1)) * LUTx[inC0] + 2^(B–1) * 2^11 ) >> 11

## Syntax

The proposed syntax is described in the table below. It signals parameters to derive the mapping functions of the 3 color components. Piecewise linear models are used to define the mapping functions.

| colour_transform_info( payloadSize ) { | Descriptor |
| --- | --- |
| colour_transform_cancel_flag | u(1) |
| if( !colour_transform_cancel_flag ) { |  |
| colour_transform_persistence_flag | u(1) |
| colour_transform_log2_number_of_points_per_lut | u(3) |
| colour_transform_cross_component_flag | u(1) |
| if ( colour_transform _cross_component_flag ) |  |
| colour_transform_cross_comp_inferred_flag | u(1) |
| for( i = 0; i < colourTransformSize; i++ ) |  |
| colour_transform_lut0[ i ] | u(v) |
| if( colour_transform_cross_component_flag = = 0 ¦ ¦ colour_transform_cross_comp_inferred_flag = = 0 ) { |  |
| colour_transform_number_chroma_lut_minus1 | u(1) |
| for( i = 0; i < colourTransformSize; i++ ) |  |
| colour_transform_lut1[ i ] | u(v) |
| if ( colour_transform_number_chroma_lut_minus1 ) |  |
| for( i = 0; i < colourTransformSize; i++ ) |  |
| colour_transform_lut2[ i ] | u(v) |
| } |  |
| else |  |
| colour_transform_chroma_offset | u(v) |
| } |  |
| } |  |

colourTransformSize is set equal to (1 << colour_transform_log2_number_of_points_per_lut).

# Complexity analysis

The mapping functions are based on piecewise linear models, defined by pivot points and one slope value per piece/segment. The horizontal axis of the piecewise linear functions is uniformly spaced, which greatly eases the identification of the index which a sample belongs to (simple right shift).

When the mapping is intra-component, the following process applies for a given decoded sample Yin of bitdepth B, using the pivot points pivotPoint and slope Luts of the intra mapping function:

- idx = Yin >> K1

- tmp = pivotPoint[idx] + ( (slope[idx] * ( Yin – ( idx << K1 ) ) + 1024 ) >> 11 )

- Yout = Clip3(0, 2^B – 1, tmp )

K1 being set to ( B – colour_transform_log2_number_of_points_per_lut ).

The same process applies to components C1 and C2 when their mapping is intra-component.

The number of operations per sample is therefore:

- 1 mult, 2 adds, 2 right shifts, 1 left shift,

- 1 clipping

When the mapping is cross-component, the following process applies for a given decoded sample Cin of bitdepth B, using the pivot points pivotPoint and slope Luts of the cross-component mapping function:

- Ycoloc is the collocated decoded sample of the sample Cin

- idx = Ycoloc >> K1

- scale = pivotPoint[idx] + ( (slope[idx] * ( Ycoloc – ( idx << K1 ) ) ) >> K1 )

- tmp = ( (2^B – 1)<<11 + ( Cin – 2^B – 1 ) * scale + 1024 ) >> 11

- Cout = Clip3(0, 2^B – 1, tmp )

The number of operations per sample is therefore:

- 2 mult, 3 adds, 3 right shifts, 1 left shift, 1 clipping

Note: as a reference, ALF requires 15 multiplications per sample, and CC-ALF 16 multiplications per sample (cf JVET-P1008).

# Simulation results

The proposal is implemented in VTM12.0. The corresponding patch is attached to the contribution. The figure below illustrates the coding chain. Simulations have been performed for the AI, RA configurations as specified in the HDR CTCs (JVET-T2011). LB and LD configuration have also been tested. In the simulations the piecewise linear functions are made of 16 segments. An CTI SEI is inserted for each IRAP.

## Test1 – CTI is enabled, LMCS is disabled

In the results below, CTI is enabled, LMCS is disabled. The CTI luma mapping function is built from the static LMCS luma mapping function used in VTM for HDR-PQ content. Cross-component scaling is applied, with cross-component scaling function ²inferred from the luma mapping function.

The corresponding configuration file is “sei_cti_v01.cfg” attached to this contribution.

Results are summarized in the table below. Detailed results are reported in the attached excel file “VTM12.0_SEICTI_HDR-PQ.xlsm”. Gains are observed for all metrics and all configurations.

|  | DE100 | PSNR-L | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AI | -1.15% | -0.76% | -0.50% | -5.42% | -7.11% | -0.31% | -2.51% | -3.25% | 98% | 95% |
| RA | -2.43% | -1.72% | -1.17% | -5.31% | -7.89% | -1.00% | -1.37% | -2.70% | 100% | 96% |
| LB | -5.47% | -2.50% | -1.57% | -10.63% | -15.01% | -1.46% | -1.05% | -4.83% | 95% | 87% |
| LP | -5.78% | -2.45% | -1.54% | -11.20% | -16.25% | -1.48% | -0.77% | -5.83% | 97% | 97% |

## Test2 – CTI is enabled, LMCS is enabled

In the results below, CTI is enabled with the luma mapping function built from the LMCS luma mapping function used in VTM for HDR-PQ content, as in Test1. Cross-component scaling is applied, with cross-component scaling LUT inferred from the luma mapping function. In addition, LMCS is enabled using the SDR tuning algorithm.

The corresponding configuration file is “sei_cti_v02.cfg” attached to this contribution.

Results are summarized in the table below. Detailed results are reported in the attached excel file “VTM12.0_SEICTI_LMCS_HDR-PQ.xlsm”. Additional gains are observed on top of Test1 thanks to the use of LMCS. This test shows synergy between CTI and LMCS.

|  | DE100 | PSNR-L | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AI | -0.46% | -0.91% | -0.30% | -5.27% | -6.21% | 0.40% | -1.79% | -2.15% | 106% | 99% |
| RA | -2.30% | -3.31% | -1.74% | -5.65% | -7.35% | -1.06% | -1.32% | -1.99% | 102% | 96% |
| LB | -5.85% | -3.96% | -2.57% | -9.52% | -14.06% | -1.89% | 0.69% | -3.25% | 100% | 92% |
| LP | -6.68% | -3.83% | -2.49% | -9.90% | -14.50% | -1.86% | 1.18% | -3.40% | 103% | 100% |

## Visual check

Visual checks have been done on the different clips of HDR CTCs. No clear difference has been observed between the test cases and the VTM12.0 anchor. Also, no negative impact has been identified from the application of CTI.

# Comments and conclusion

The following comments can be made:

- CTI brings noticeable average BD-rate gain for HDR-PQ content on all tested configurations and HDR-PQ metrics, with higher gains in inter-configuration (RA, LB, LP).

- CTI and LMCS show synergies, and lead to additional gains.

- Significant average BD-rate gain is observed in low delay configurations with around 6-7% DE100, 4% psnrL, 2.5% wPsnrY, 10% wPsnrU, 14% wPsnrV.

- Added complexity in terms of operations and runtime variations is very limited.

- Low delay is more and more important for the video industry, with the steady raise of cloud gaming, tele-conference and remote communication applications; HDR-PQ content is also becoming more and more deployed, including in cloud gaming use case; bringing additional compression gains for this type of content and use cases is worthy.

Acknowledgments:

the authors thank Dolby for the discussions and for the cross-checking of the provided results.

This work has been achieved in the context of the 3EMS project funded by the “Region Bretagne”.

# Patent rights declaration

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).